| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0373-v1 |
| --- | --- |

| Title: | CE4-related: Unification for affine motion buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

In VTM-2.0.1, the sub-block motion vector array used for motion compensation of affine coded block is not aligned with the sub-block motion vector array used for skip/merge/AMVP list derivations, because the control point motion vectors (CPMVs) are stored into the corner sub-blocks. In this contribution, the issue is solved by using the sub-block motion vector array with rewritten CPMVs for motion compensation. Compared to VTM-2.0.1 anchor, the average BD-rate differences are 0.06% in RA and 0.01% in LB, respectively.

Introduction

In VTM2.0.1, the decoding process for an affine CU is as following:

- Derive affine CPMVs by merge/skip/AMVP mode

- Used CPMVs to generate sub-block motion vector array

- Perform MC with sub-block MV array as input

- Store CPMVs and sub-block MV array for the following decoding process (merge/skip/AMVP list derivations, TMVP, de-blocking)

In step 4, the sub-block motion vectors around the top-left/top-right/bottom-left corners of a CU coded in affine mode are replaced by CPMVs, which are different for the MC. In this contribution, the sub-block motion buffer in motion compensation and storage of affine mode is aligned, by using the sub-block motion vector array with rewritten CPMVs for motion compensation. After unification, the decoding process for an affine CU is:

- Derive affine CPMVs by merge/skip/AMVP mode

- Used CPMVs to generate sub-block motion vector array; sub-block motion vectors around the top-left/top-right/bottom-left corners of a CU coded in affine mode are replaced by CPMVs

- Perform MC with sub-block MV array as input

- Store sub-block MV array for the following decoding process (merge/skip/AMVP list derivations, TMVP, de-blocking)

Simulation results

The contribution has been implemented on top of VTM-2.0.1. The test condition descripted in JVET-K1024 [1] is used for verifying the contribution. The summary results are provided in Table 1. On average, the BD-rate differences are 0.06% in RA, and 0.01% in LB, respectively.

Table 1 Coding performance over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.13% | 0.01% | 100% | 102% |
| Class A2 | 0.09% | 0.02% | -0.06% | 100% | 105% |
| Class B | 0.08% | 0.09% | 0.03% | 99% | 101% |
| Class C | 0.07% | 0.01% | 0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.06% | 0.01% | 100% | 101% |
| Class D | 0.15% | 0.00% | -0.03% | 100% | 101% |
| Class F (optional) | 0.08% | 0.06% | 0.08% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.03% | -0.18% | 101% | 104% |
| Class C | 0.02% | -0.12% | -0.12% | 100% | 101% |
| Class E | -0.07% | -0.06% | 0.02% | 100% | 100% |
| Overall | 0.01% | -0.04% | -0.11% | 100% | 102% |
| Class D | 0.04% | 0.58% | 0.42% | 100% | 102% |
| Class F (optional) | 0.16% | 0.32% | 0.14% | 100% | 100% |

Conclusions

In this contribution, the sub-block motion buffer in motion compensation and storage of affine mode is aligned. Compared to VTM-2.0.1 anchor, the average BD-rate differences are 0.06% in RA and 0.01% in LB, respectively.

References

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-K1024, the 11th JVET meeting, July 2018, Ljubljana, SI.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).