| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0379 |
| --- | --- |

| Title: | CE7: Block size dependent coefficient scanning (CE7.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 49 278 7417 yo-kidani@kddi.com ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution presents the results of a core experiment CE7.3 included CE7 (JVET-K1027). The proposed coefficient scanning obtains BD-rate gains for luma by averagely -0.08%/-0.03%/-0.06% in AI/RA/LDB conditions respectively without increasing running times and also it is clarified that the BD-rate gains decrease due to the interaction with NSST or under low QP conditions.

Introduction

Mode dependent coefficient scanning (MDCS) is introduced in HEVC and JEM for improving coding efficiency. That is applicable for relatively small block by 4x4, 8x8, 4x8, 8x8 and the scanning orders are selected among horizontal, vertical and diagonal scan depends on intra prediction mode. On the other hand, diagonal scan is used fixedly for relatively large blocks whose block size is larger than 8.

|  | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- |
| 4 | M | M | D | D | D |
| 8 | M | M | D | D | D |
| 16 | D | D | D | D | D |
| 32 | D | D | D | D | D |
| 64 | D | D | D | D | D |

Figure 1. Decision table of JEM-MDCS. The vertical line and horizontal line indicate block height and width respectively and M and D indicate MDCS and diagonal scanning.

Block size dependent coefficient scanning

The proposed coefficient scanning is modified the decision table of selecting MDCS based on the ratio of block size [2]. When the ratio of horizontal and vertical size is equal or larger than four, mode-dependent coefficient scanning is applied. The MDCS itself is the same as in JEM. It is note that MDCS is not applied for both VTM and BMS but the alternative coefficient coding is applied for VTM and BMS respectively instead of MDCS. The proposed decision table is shown in the Figure 2. It is also noted that transform block size 64x64 luma and 32x32 chroma are excluded in VTM and BMS, so these blocks are removed from the tables.

|  | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- |
| 4 | M | M | D | D | D |
| 8 | M | M | D | M | M |
| 16 | D | D | D | D | M |
| 32 | D | M | D | D | D |
| 64 | D | M | M | D | D |

Figure 2. Decision table of proposed method. The vertical line and horizontal line indicate block height and width respectively and M and D indicate MDCS and diagonal scanning.

Experimental Results

Coding conditions

In this contribution, three additional tests are conducted. First one is the proposed MDCS under JVET CTC condition [3]. Second one is the proposed MDCS with Non-Separable Secondary Transform (NSST). The third one is the proposed MDCS under low QPs conditions (QP =12, 17, 22, 27). These tests are numbered CE7.3.1, CE7.3.2 and CE7.3.3 according to the CE7 description [1]. They are implemented on the top of the VTM2.0.1 respectively but for only CE7.3.2 NSST applied to BMS 2.0.1 is additionally introduced. The software of CE7.3.1 and CE7.3.3 are perfectly same, which can be found at https://vcgit.hhi.fraunhofer.de/JVET-K-CE7/VVCSoftware_VTM/tree/CE7.3.1. Regarding CE7.3.3 only, it is noted that to reduce the simulation runtime we subsample the frame for class A/B sequence in AI condition with setting TemporalSubsampleRatio = 16 and also change the total frame numbers for Class A/B sequence in RA and LDB conditions by the 150 frames (0-149 frame) and the 300 frames (0-299 frame) respectively. The software of CE7.3.2 can be found https://vcgit.hhi.fraunhofer.de/JVET-K-CE7/VVCSoftware_VTM/tree/CE7.3.2. It is noted that the anchor of CE7.3.2 is the proposed MDCS off but NSST on.

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Objective results

CE7.3.1 (the test under CTC condition)

From Table 1 to Table 3 indicate the CE7.3.1 objective results of proposed method compared to VTM 2.0.1. The results show the overall BD-rates for luma in AI/RA/LDB conditions are -0.08%/-0.03%/-0.06% without increasing running time.

Table 1 Experimental results for AI condition compared to VTM-2.0.1.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.08% | 0.00% | 101% | 101% |
| Class A2 | -0.05% | -0.04% | -0.06% | 101% | 101% |
| Class B | -0.04% | -0.05% | -0.09% | 101% | 101% |
| Class C | -0.12% | -0.17% | -0.01% | 101% | 101% |
| Class E | -0.19% | -0.11% | -0.23% | 100% | 101% |
| Overall | -0.08% | -0.09% | -0.07% | 101% | 101% |
| Class D | -0.14% | -0.01% | -0.45% | 101% | 101% |

Table 2 Experimental results for RA condition compared to VTM-2.0.1.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.02% | 0.15% | 101% | 100% |
| Class A2 | -0.03% | -0.06% | -0.07% | 101% | 100% |
| Class B | -0.03% | -0.10% | -0.05% | 101% | 100% |
| Class C | -0.02% | -0.10% | -0.01% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.07% | 0.00% | 101% | 100% |
| Class D | -0.04% | -0.10% | 0.05% | 101% | 101% |

Table 3 Experimental results for LDB condition compared to VTM-2.0.1

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.00% | 0.01% | 100% | 101% |
| Class C | 0.02% | 0.07% | -0.29% | 100% | 100% |
| Class E | -0.28% | 0.23% | -0.02% | 100% | 101% |
| Overall | -0.06% | 0.08% | -0.10% | 100% | 101% |
| Class D | -0.01% | 0.40% | -0.12% | 101% | 101% |

CE7.3.2 (the test with NSST)

From Table 4 to Table 6 indicate the CE7.3.2 objective results of proposed method compared to VTM 2.0.1 with NSST. The results show the overall BD-rates for luma in AI/RA/LDB conditions are 0.01%/0.01%/-0.02% without increasing running time. It is said that the BD-rate gains which derive from the proposed MDCS decrease due to the interaction with NSST comparing the CE7.3.1 results.

Table 4 Experimental results for AI condition compared to VTM-2.0.1 with NSST.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.05% | 0.09% | 100% | 100% |
| Class A2 | 0.00% | -0.01% | -0.03% | 101% | 100% |
| Class B | 0.05% | -0.03% | -0.03% | 101% | 100% |
| Class C | 0.02% | -0.03% | -0.06% | 101% | 101% |
| Class E | -0.05% | -0.04% | -0.14% | 99% | 100% |
| Overall | 0.01% | -0.03% | -0.03% | 101% | 100% |
| Class D | 0.03% | -0.18% | -0.22% | 101% | 101% |

Table 5 Experimental results for RA condition compared to VTM-2.0.1 with NSST.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.12% | -0.05% | 100% | 100% |
| Class A2 | -0.03% | 0.07% | -0.18% | 100% | 100% |
| Class B | 0.04% | 0.00% | 0.04% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | -0.02% | 100% | 100% |
| Class D | -0.01% | -0.10% | -0.34% | 100% | 101% |

Table 6 Experimental results for LDB condition compared to VTM-2.0.1 with NSST

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.17% | 0.13% | 100% | 100% |
| Class C | 0.00% | -0.18% | 0.05% | 100% | 100% |
| Class E | -0.10% | 0.69% | 0.04% | 100% | 99% |
| Overall | -0.02% | 0.18% | 0.08% | 100% | 100% |
| Class D | 0.06% | -0.37% | -0.17% | 100% | 100% |

CE7.3.3 (the test under low QP conditions [QP = 12, 17, 22, 27])

From Table 7 to Table 9 indicate the CE7.3.3 objective results of proposed method compared to VTM 2.0.1 under low QP conditions [QP = 12, 17, 22, 27]. The results show the overall BD-rates for luma in AI/RA/LDB conditions are -0.04%/-0.01%/-0.03% without increasing running time. It is said that the BD-rate gains which derive from the proposed MDCS decrease under low QP conditions comparing the CE7.3.1 results.

Table 4 Experimental results for AI condition compared with VTM-2.0.1.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.02% | 101% | 101% |
| Class A2 | -0.05% | -0.05% | -0.06% | 101% | 101% |
| Class B | -0.01% | -0.03% | -0.06% | 101% | 101% |
| Class C | -0.06% | -0.09% | -0.13% | 101% | 101% |
| Class E | -0.10% | -0.17% | -0.14% | 101% | 101% |
| Overall | -0.04% | -0.07% | -0.08% | 101% | 101% |
| Class D | -0.05% | -0.10% | -0.18% | 101% | 100% |

Table 5 Experimental results for RA condition compared with VTM-2.0.1.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.08% | -0.01% | 101% | 100% |
| Class A2 | -0.01% | -0.07% | -0.12% | 101% | 101% |
| Class B | 0.00% | -0.03% | -0.02% | 101% | 101% |
| Class C | -0.02% | -0.08% | 0.01% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.06% | -0.03% | 101% | 101% |
| Class D | 0.00% | -0.10% | 0.05% | 101% | 100% |

Table 6 Experimental results for LDB condition compared with VTM-2.0.1

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.02% | 0.08% | 101% | 99% |
| Class C | -0.02% | -0.04% | 0.01% | 100% | 100% |
| Class E | -0.08% | 0.63% | 0.09% | 100% | 100% |
| Overall | -0.03% | 0.13% | 0.06% | 100% | 100% |
| Class D | -0.01% | -0.04% | 0.08% | 101% | 100% |

Conclusion

In this contribution, the simulation results by the proposed coefficient scanning were presented. The objective results show the proposed method increase BD-rate gains by averagely -0.08%/-0.03%/-0.06% in AI/RA/LDB conditions without increasing running times and also it is clarified that the BD-rate gains decrease due to the interaction with NSST or under low QP conditions, so the adoption of the proposed MDCS for VTM is recommended when NSST would not be introduced into VTM.

- Acknowledgment

We would like to thank Huawei and LGE for doing crosschecking of our proposal.

References

- H. Schwarz, M. Coban and C. Auyeung, “Description of Core Experiment 7 (CE7) on Quantization and coefficient coding”, JVET-K1027, Ljubljana, SI, July 2018.

- K. Kawamura, Y. Kidani and S. Naito, “Description of SDR video coding technology proposal by KDDI”, JVET-J0016, San Diego, US, 10-20, Apr. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Suehring, , “JVET common test conditions and software reference configurations”, JVET-K1010, Ljubljana, SI, July 2018.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).