| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L383 |
| --- | --- |

| Title: | AHG9: Convolution Neural Network Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kei Kawamura Yoshitaka Kidani Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-5066-9075 ki-kawamura@kddi.com yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution presents the results where Convolution Neural Network Filter (CNNF) proposed in JVET-J0016 is implemented on the top of VTM-2.0.1 and used as in-loop filter instead of the current multiple filter such as deblocking filter (DBF), sample adaptive offset (SAO) and adaptive loop filter (ALF). The simulation results show the BD-rate for luma is -0.93% for AI where CNNF is replaced by DBF, SAO and ALF though the BD-rate is -2.21% for AI where CNNF is replaced by DBF and SAO only.

Introduction

In JVET-J0016 [1], the performance of Convolutional Neural Network filter (CNNF) is shown based on JEM-7.0 but this contribution reports that of VTM-2.0.1.

Proposed CNNF

The filter structure is shown in Figure 1. This filter has four layers with 3x3 taps. The input of sum block is residual signal from the left and reconstructed signal before all in-loop filters from the bottom. The output of sum block is filtered pixels. Actual output is weighted sum of after filtered and before filtered pixels based on the distance of edge.

Figure 1 The structure of proposed CNNF

Filter coefficients are common in both the encoder and the decoder so that additional side information is not required.

Experimental Results

Coding conditions

The proposed tool was implemented on top of the VTM-2.0.1 software. In order to verify the performance of our proposed method, we have run simulations according to the test conditions of JVET-K1010 [2]. It is noted that CNN-based in-loop filter is learned in advance to that encoding. The test set is different sequences are used.

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Objective results

VTM aspects

Table 1 and Table 2 show the experimental results of proposed method compared to VTM-2.0.1 anchor without ALF and with ALF respectively. Table 1 show that BD-rate for luma increase -0.93% where CNNF is replaced by DBF, SAO, ALF. In contrast, Table 2 show that the BD-rate for luma increase -2.21 where CNNF is replaced by DBF and SAO but ALF is maintained.

Table 1 Experimental results for AI condition compared to VTM-2.0.1 without ALF

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.40% | 2.81% | 4.93% | 483.5% | 228673.5% |
| Class A2 | -0.06% | 2.07% | 0.10% | 312.0% | 194109.0% |
| Class B | -0.77% | 1.10% | 2.25% | 301.3% | 200181.1% |
| Class C | -2.75% | -1.17% | -2.17% | 208.6% | 138418.5% |
| Class E | -1.97% | 3.37% | 5.46% | 379.8% | 240473.6% |
| Overall | -0.93% | 1.42% | 1.89% | 314.1% | 193416.4% |
| Class D | -3.26% | -1.72% | -3.19% | 186.7% | 119952.1% |

Table 2 Experimental results for AI condition compared to VTM-2.0.1 with ALF

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.70% | -0.55% | 1.61% | 484.2% | 228100.2% |
| Class A2 | -1.80% | 0.02% | -1.61% | 311.9% | 194529.8% |
| Class B | -1.74% | -0.28% | 0.64% | 300.5% | 199694.7% |
| Class C | -3.33% | -2.19% | -3.44% | 207.9% | 137989.9% |
| Class E | -3.41% | -0.43% | 1.38% | 377.6% | 238422.2% |
| Overall | -2.21% | -0.72% | -0.36% | 313.4% | 192865.9% |
| Class D | -3.48% | -2.38% | -3.93% | 186.2% | 119286.7% |

Conclusion

This contribution presents the simulation results of CNNF based on VTM-2.0.1 software and it was confirmed that BD-rates are increased by replacing conventional multiple filter but simulation runtime is still so long that the continuous study is recommended from the point of view how to reduce the complexity and find sweet spot of hardware implementation.

- Acknowledgment

We would like to thank Huawei for doing crosschecking of our proposal.

References

- K. Kawamura, Y. Kidani and S. Naito, “Description of SDR video coding technology proposal by KDDI”, JVET-J0016, San-Diego, US, April 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Suehring, , “JVET common test conditions and software reference configurations”, JVET-K1010, Ljubljana, SI, July 2018.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent