| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0390r3 |
| --- | --- |

| Title: | CE4-related: Simplification of Affine merge common codebase |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Yu-Chiao Yang | Email: | morriss.yj.chang@foxconn.com johnny.yc.yang@fii-foxconn.com |
| Source: | Foxconn |  |  |

_____________________________

# Abstract

The affine merge list in Affine merge common base (CE4-2.1) includes first 5 available candidates out of 5 inherited merge candidates and 10 constructed merge candidates. On top of this Affine merge common base with the same merge list size, i.e., 5, this contribution proposes two aspects to reduce the maximum number of potential merge candidates to 9 by limiting the maximum numbers selected out of inherited merge candidates and constructed merge candidates, and to reduce the number of candidate comparisons from 50 to 14 (72% reduction) by only check with few specified inherited and constructed merge candidates. The proposed simplifications are implemented on top of Affine common codebase (CE4.2.1). Compared to VTM2.0.1, -0.73%/-0.60%/-0.76% YUV-BDrate with 107/104% coding time change are achieved in RA condition, and -0.41%/-0.48%/-0.47% YUV-BDrate with 108/105% coding time change are achieved in LB condition.

# Introduction

Affine merge mode uses the MV information of the neighboring blocks to derive the Affine control points motion vectors (CPMVs) of the current CU. CE4.2.1 is a common codebase [1] for Affine merge mode on top of VTM-2.0.1. This contribution proposes two aspects to simplify the complexity of Affine merge common codebase.

Fig. 1. The inherited and constructed blocks for Affine merge mode

# The proposed simplifications

There are two aspects related to Affine merge simplifications proposed in this proposal.

The first aspect is related to reduction of the maximum number of available merge candidates. The modifications on top of Affine merge common codebase are as follows:

- At most 1 inherited candidate selected out of first available left-bottom blocks F & G

- At most 1 inherited candidate selected out of first available right-top blocks D & E

- At most 1 inherited candidate selected out of first available left-top blocks A, B & C

- At most 2 constructed candidates selected out of first two available 6-parameter Affine model (6-param) constructed candidates

- At most 4 constructed candidates selected out of first four available 4-parameter Affine model (4-param) constructed candidates

The second aspect focuses on the reduction of the candidate comparisons for the Affine merge list construction. The modifications on top of Affine merge common codebase and Modification a ~ e are as follows:

- In Modification a and Modification b, there are no candidate comparisons

- In Modification c, the first available left-top block whose Affine MV information is not the same as those of the left-bottom and right-top blocks is selected (at most 6 candidate comparisons)

- In Modification d, there is no candidate comparison for the first available 6-param constructed candidate. The other available 6-param constructed candidate whose Affine MV information is not the same as that of the first available 6-param constructed candidate is selected (at most 3 candidate comparisons)

- In Modification e, there is no candidate comparisons for the first available 4-param constructed candidate. The other available 4-param constructed candidates whose Affine MV information is not the same as that of the first available 4-param constructed candidate is selected (at most 5 candidate comparisons)

The proposed modifications reduce the number of candidate comparisons (which is reduced from 50 to 14).

# Experiment result

The proposed Affine merge candidate list construction was tested on top of Affine merge common base (CE4.2.1) to compare with the VTM2.0.1 under common test conditions [2]. There are three tests:

Test 1:

Proposed potential candidates: inherited (3) + 6-param constructed (2) + 4-param constructed (4) + reduce 72% candidate comparisons

Test 2:

Proposed potential candidates: inherited (5, exactly the same as CE4.2.1) + 6-param constructed (1) + 4-param constructed (4) + remove candidate comparisons & MV scaling

Test 3:

Proposed potential candidates: inherited (5, exactly the same as CE4.2.1 + 6-param constructed (1) + 4-param constructed (3) + remove candidate comparisons & MV scaling

Table 1. Summary of experimental results (Proposed method Test 1 v.s. VTM anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.93% | -0.51% | -0.86% | 106% | 102% |
| Class A2 | -0.93% | -0.85% | -1.08% | 107% | 107% |
| Class B | -0.65% | -0.58% | -0.66% | 107% | 102% |
| Class C | -0.50% | -0.50% | -0.56% | 107% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.73% | -0.60% | -0.76% | 107% | 104% |
| Class D | -0.62% | -0.68% | -0.63% | 108% | 107% |
| Class F (optional) | -0.14% | -0.16% | -0.17% | 106% | 107% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.37% | -0.39% | -0.35% | 107% | 105% |
| Class C | -0.37% | -0.32% | -0.45% | 107% | 104% |
| Class E | -0.55% | -0.86% | -0.69% | 109% | 106% |
| Overall | -0.41% | -0.48% | -0.47% | 108% | 105% |
| Class D | -0.53% | 0.16% | -0.77% | 106% | 106% |
| Class F (optional) | 0.01% | 0.28% | 0.13% | 106% | 103% |

Table 2. Summary of experimental results (Proposed method Test 2 v.s. VTM anchor)

Table 3. Summary of experimental results (Proposed method Test 3 v.s. VTM anchor)

# Conclusion

This contribution proposes a simplified Affine merge list construction method. It is claimed that the proposed method can keep the same gain with lower complexity as compared with Affine merge common codebase (CE4-2.1) and other similar tests in CE4-2.

# Reference

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-K1024, Ljubljana, Slovenia, Jul. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Ljubljana, Slovenia, Jul. 2018.

# Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).