| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0404 – v2 |
| --- | --- |

| Title: | CE8-related: Restrictions for the search area of the IBC blocks in CPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van, Vadim Seregin, Ted Hsieh, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Tel: Email: | +1-858-658-5664 lphamvan@qti.qualcomm.com |
| Source: | Qualcomm Technologies Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the restrictions to the search area for IBC blocks. The search area is restricted to the current CTU and few CTUs to the left of the current CTU. Additionally, this proposal also proposes to extend the search area to 10 lines above these CTUs. It is reported that with the proposed restricted search area, the Luma BD-rate changes for [CTC, Class F, class SCC 1080p] over BMS2.1-VTM configuration are:

The current and 1 left CTU without and with extended lines, respectively:

- AI: (-0.29%, -15.55%, -46.10%) and (-0.32%, -15.79%, -46.99%).

- RA: (-0.10%, -12.67%, -29.42%) and (-0.11%, -12.88%, -30.17%).

- LB: (0.02%, -7.19%, -19.70%) and (-0.32%, -7.38%, -20.18%).

The current and 2 left CTUs without and with extended lines, respectively:

- AI: (-0.29%, -15.76%, -46.68%) and (-0.32%, -15.98%, -47.56%).

- RA: (-0.10%, -12.84%, -30.23%) and (-0.11%, -13.06%, -30.99%).

- LB: (0.02%, -7.30%, -20.39%) and (0.02%, -7.62%, -20.96%).

The current and 3 left CTUs without and with extended lines, respectively:

- AI: (-0.29, -15.84%, -46.98%) and (-0.32%, -16.06%, -47.85%).

- RA: (-0.10, -12.91%, -30.63%) and (-0.11%, -13.14%, -31.35%).

- LB: (0.02%, -7.52%, -20.94%) and (0.02, -7.67%, -21.52%).

# Introduction

The current picture reference (CPR) mode [1] was adopted into BMS software in the last JVET meeting. In this mode, intra block copy (IBC) block is predicted from already decoded block (before in-loop filtering) of the current picture (Figure 1). The term in-loop filtering here may include both adaptive loop filter (ALF) and Sample Adaptive Offset (SAO). Currently, one major improvement area of the IBC under investigation is the IBC with larger search range, even as far as full-frame.

This scheme makes the process very complicate for searching the best match of the current block and for maintaining the picture in the buffer. In addition, it makes pipe-line/wave-front processing be not friendly to the encoder and decoder. To solve these problems, this contribution proposes the restrictions to the search area for IBC blocks.

Figure 1: An example IBC BV’s distribution for a CU in a picture

# Proposed Method

To reduce the complexity of the encoder and make the codec be friendly with pipeline/wave-front processing, this proposal proposes the restriction applied to the block vector search area for IBC blocks. The search area includes the current CTU, few CTUs to the left and 10 lines above (i.e., the samples needed for intra prediction and filtering). The following cases are analyzed:

- Search area is the current CTU and the left CTU

- Search area is the current CTU, the left CTU, and 10 lines above

- Search area is the current CTUs and two left CTUs

- Search area is the current CTU, two left CTUs, and 10 lines above

- Search area is the current CTUs and three left CTUs.

- Search area is the current CTU, three left CTUs, and 10 lines above.

# Experimental Results

The proposed constraints are implemented in BMS-2.1 software. The tests are under the VTM configurations. The anchor is BMS-2.1.

The performance of the proposed restrictions is summarized in Table 1, Table 2, and Table 3. A comparison of Test b over Test a is presented in Table 4. It should be noted that, the configuration of machine used for tests may be different. Hence, the time comparison in the tables is uninformative. It is reported that with the proposed restricted search area, the Luma BD-rate changes for [CTC, Class F, class SCC 1080p] over BMS2.1 VTM configuration are:

The performances of Test a and Test b are summarized respectively in Table 1 as follows:

- AI: (-0.29%, -15.55%, -46.10%) and (-0.32%, -15.79%, -46.99%).

- RA: (-0.10%, -12.67%, -29.42%) and (-0.11%, -12.88%, -30.17%).

- LB: (0.02%, -7.19%, -19.70%) and (-0.32%, -7.38%, -20.18%).

The performances of Test c and Test d are summarized respectively in Table 1 as follows:

- AI: (-0.29%, -15.76%, -46.68%) and (-0.32%, -15.98%, -47.56%).

- RA: (-0.10%, -12.84%, -30.23%) and (-0.11%, -13.06%, -30.99%).

- LB: (0.02%, -7.30%, -20.39%) and (0.02%, -7.62%, -20.96%).

The performances of Test e and Test f are summarized respectively in Table 1 as follows:

- AI: (-0.29, -15.84%, -46.98%) and (-0.32%, -16.06%, -47.85%).

- RA: (-0.10, -12.91%, -30.63%) and (-0.11%, -13.14%, -31.35%).

- LB: (0.02%, -7.52%, -20.94%) and (0.02, -7.67%, -21.52%).

Table 1. The search area includes the current CTU and the left CTU

|  | Over BMS-2.1 VTM AI configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Without 10 lines above |  |  |  |  | With 10 lines above |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.05% | 0.04% | 131% | 93% | 0.04% | -0.01% | 0.05% | 160% | 100% |
| Class A2 | -0.19% | -0.28% | -0.28% | 130% | 95% | -0.22% | -0.30% | -0.27% | 157% | 100% |
| Class B | -0.43% | -0.53% | -0.48% | 134% | 99% | -0.45% | -0.59% | -0.46% | 160% | 104% |
| Class C | -0.13% | -0.30% | -0.09% | 131% | 92% | -0.16% | -0.25% | -0.07% | 150% | 95% |
| Class E | -0.70% | -0.77% | -0.67% | 136% | 97% | -0.76% | -0.78% | -0.75% | 166% | 99% |
| Overall | -0.29% | -0.38% | -0.30% | 132% | 95% | -0.32% | -0.40% | -0.31% | 158% | 100% |
| Class D | -0.04% | 0.07% | 0.26% | 128% | 96% | -0.06% | 0.04% | 0.26% | 151% | 101% |
| Class F | -15.55% | -15.69% | -15.70% | 162% | 93% | -15.79% | -15.95% | -15.81% | 193% | 97% |
| Class SCC | -46.10% | -45.32% | -45.67% | 138% | 92% | -46.99% | -46.15% | -46.48% | 165% | 96% |
|  | Over BMS-2.1 VTM RA configuration |  |  |  |  |  |  |  |  |  |
|  | Without 10 lines above |  |  |  |  | With 10 lines above |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.05% | 0.04% | 101% | 102% | -0.07% | 0.04% | -0.02% | 124% | 106% |
| Class A2 | -0.20% | -0.20% | -0.14% | 111% | 105% | -0.21% | -0.21% | -0.16% | 119% | 107% |
| Class B | -0.19% | -0.68% | -0.41% | 112% | 104% | -0.19% | -0.58% | -0.46% | 106% | 102% |
| Class C | 0.06% | -0.01% | -0.03% | 107% | 104% | 0.04% | -0.11% | -0.12% | 108% | 104% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.10% | -0.26% | -0.17% | 108% | 104% | -0.11% | -0.26% | -0.22% | 113% | 104% |
| Class D | 0.12% | -0.12% | 0.72% | 111% | 100% | 0.10% | -0.07% | 0.52% | 107% | 100% |
| Class F | -12.67% | -12.73% | -12.79% | 120% | 107% | -12.88% | -13.06% | -13.02% | 111% | 103% |
| Class SCC | -29.42% | -29.36% | -29.71% | 106% | 99% | -30.17% | -30.08% | -30.44% | 114% | 99% |
|  | Over BMS-2.1 VTM LB configuration |  |  |  |  |  |  |  |  |  |
|  | Without 10 lines above |  |  |  |  | With 10 lines above |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% | -0.07% | -0.01% | 109% | 103% | 0.02% | -0.16% | 0.07% | 109% | 103% |
| Class C | 0.13% | 0.08% | -0.08% | 110% | 107% | 0.13% | 0.07% | -0.01% | 108% | 109% |
| Class E | -0.16% | 0.60% | -0.04% | 108% | 117% | -0.14% | 0.05% | -0.31% | 105% | 111% |
| Overall | 0.02% | 0.15% | -0.04% | 109% | 108% | 0.02% | -0.03% | -0.05% | 108% | 107% |
| Class D | 0.18% | 0.57% | -0.33% | 117% | 98% | 0.17% | 0.69% | 0.14% | 109% | 92% |
| Class F | -7.19% | -7.48% | -7.86% | 116% | 113% | -7.38% | -7.28% | -8.25% | 114% | 113% |
| Class SCC | -19.70% | -20.68% | -20.67% | 117% | 111% | -20.18% | -21.16% | -21.24% | 115% | 107% |

Table 2. The search area includes the current CTU and two CTUs to the left

|  | Over BMS-2.1 VTM AI configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Without 10 lines above |  |  |  |  | With 10 lines above |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.05% | 0.04% | 130% | 93% | 0.04% | -0.02% | 0.04% | 129% | 94% |
| Class A2 | -0.19% | -0.28% | -0.28% | 126% | 96% | -0.22% | -0.30% | -0.27% | 131% | 97% |
| Class B | -0.43% | -0.53% | -0.48% | 136% | 100% | -0.45% | -0.59% | -0.46% | 136% | 98% |
| Class C | -0.13% | -0.30% | -0.09% | 131% | 91% | -0.16% | -0.25% | -0.07% | 128% | 94% |
| Class E | -0.70% | -0.77% | -0.67% | 138% | 96% | -0.76% | -0.78% | -0.75% | 135% | 95% |
| Overall | -0.29% | -0.38% | -0.30% | 132% | 96% | -0.32% | -0.40% | -0.31% | 132% | 96% |
| Class D | -0.04% | 0.07% | 0.26% | 126% | 94% | -0.06% | 0.04% | 0.26% | 126% | 95% |
| Class F | -15.76% | -15.93% | -15.94% | 157% | 91% | -15.98% | -16.11% | -16.05% | 164% | 94% |
| Class SCC | -46.68% | -46.02% | -46.36% | 136% | 90% | -47.56% | -46.83% | -47.16% | 144% | 92% |
|  | Over BMS-2.1 VTM RA configuration |  |  |  |  |  |  |  |  |  |
|  | Without 10 lines above |  |  |  |  | With 10 lines above |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.04% | 0.05% | 102% | 102% | -0.07% | 0.04% | 0.00% | 104% | 103% |
| Class A2 | -0.20% | -0.20% | -0.14% | 113% | 106% | -0.21% | -0.21% | -0.16% | 116% | 107% |
| Class B | -0.19% | -0.68% | -0.41% | 108% | 105% | -0.19% | -0.58% | -0.46% | 108% | 105% |
| Class C | 0.06% | -0.01% | -0.03% | 108% | 103% | 0.04% | -0.11% | -0.12% | 106% | 103% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.10% | -0.26% | -0.16% | 108% | 104% | -0.11% | -0.25% | -0.22% | 108% | 104% |
| Class D | 0.12% | -0.12% | 0.72% | 115% | 100% | 0.10% | -0.07% | 0.52% | 112% | 100% |
| Class F | -12.84% | -12.94% | -13.10% | 113% | 107% | -13.06% | -13.23% | -13.25% | 116% | 107% |
| Class SCC | -30.23% | -30.23% | -30.59% | 103% | 100% | -30.99% | -30.95% | -31.30% | 102% | 98% |
|  | Over BMS-2.1 VTM LB configuration |  |  |  |  |  |  |  |  |  |
|  | Without 10 lines above |  |  |  |  | With 10 lines above |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% | -0.07% | -0.01% | 105% | 104% | 0.02% | -0.16% | 0.08% | 106% | 104% |
| Class C | 0.13% | 0.08% | -0.08% | 110% | 109% | 0.13% | 0.07% | -0.01% | 108% | 112% |
| Class E | -0.16% | 0.60% | -0.04% | 105% | 111% | -0.14% | 0.05% | -0.31% | 104% | 113% |
| Overall | 0.02% | 0.15% | -0.04% | 107% | 107% | 0.02% | -0.03% | -0.05% | 106% | 109% |
| Class D | 0.18% | 0.57% | -0.33% | 111% | 95% | 0.17% | 0.69% | 0.14% | 110% | 93% |
| Class F | -7.30% | -7.72% | -7.78% | 112% | 113% | -7.62% | -7.12% | -8.65% | 112% | 112% |
| Class SCC | -20.39% | -21.50% | -21.50% | 110% | 106% | -20.96% | -21.94% | -21.97% | 111% | 111% |

Table 3. The search area includes the current CTU and three CTUs to the left

|  | Over BMS-2.1 VTM AI configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Without 10 lines above |  |  |  |  | With 10 lines above |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.05% | 0.04% | 131% | 94% | 0.04% | -0.02% | 0.04% | 134% | 95% |
| Class A2 | -0.19% | -0.28% | -0.28% | 128% | 96% | -0.22% | -0.30% | -0.27% | 134% | 95% |
| Class B | -0.43% | -0.53% | -0.48% | 134% | 99% | -0.45% | -0.59% | -0.46% | 137% | 98% |
| Class C | -0.13% | -0.30% | -0.09% | 132% | 93% | -0.16% | -0.25% | -0.07% | 128% | 91% |
| Class E | -0.70% | -0.77% | -0.67% | 134% | 97% | -0.76% | -0.78% | -0.75% | 135% | 92% |
| Overall | -0.29% | -0.38% | -0.30% | 132% | 96% | -0.32% | -0.40% | -0.31% | 134% | 95% |
| Class D | -0.04% | 0.07% | 0.26% | 127% | 97% | -0.06% | 0.04% | 0.26% | 127% | 94% |
| Class F | -15.84% | -16.03% | -16.01% | 162% | 91% | -16.06% | -16.18% | -16.10% | 173% | 94% |
| Class SCC | -46.98% | -46.35% | -46.65% | 135% | 90% | -47.85% | -47.15% | -47.46% | 140% | 94% |
|  | Over BMS-2.1 VTM RA configuration |  |  |  |  |  |  |  |  |  |
|  | Without 10 lines above |  |  |  |  | With 10 lines above |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.04% | 0.05% | 104% | 104% | -0.07% | 0.04% | 0.00% | 103% | 104% |
| Class A2 | -0.20% | -0.20% | -0.14% | 112% | 105% | -0.21% | -0.21% | -0.16% | 112% | 105% |
| Class B | -0.19% | -0.67% | -0.40% | 109% | 104% | -0.19% | -0.58% | -0.47% | 108% | 105% |
| Class C | 0.06% | -0.01% | -0.03% | 107% | 104% | 0.04% | -0.11% | -0.12% | 108% | 103% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.10% | -0.26% | -0.16% | 108% | 104% | -0.11% | -0.25% | -0.22% | 108% | 104% |
| Class D | 0.12% | -0.12% | 0.72% | 109% | 99% | 0.10% | -0.07% | 0.52% | 112% | 100% |
| Class F | -12.91% | -13.05% | -13.14% | 118% | 108% | -13.14% | -13.31% | -13.37% | 116% | 107% |
| Class SCC | -30.63% | -30.66% | -30.97% | 104% | 98% | -31.35% | -31.37% | -31.73% | 101% | 97% |
|  | Over BMS-2.1 VTM LB configuration |  |  |  |  |  |  |  |  |  |
|  | Without 10 lines above |  |  |  |  | With 10 lines above |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% | -0.07% | -0.01% | 110% | 103% | 0.02% | -0.16% | 0.08% | 105% | 103% |
| Class C | 0.13% | 0.08% | -0.08% | 113% | 112% | 0.13% | 0.07% | -0.01% | 107% | 105% |
| Class E | -0.16% | 0.60% | -0.04% | 106% | 116% | -0.14% | 0.05% | -0.31% | 103% | 115% |
| Overall | 0.02% | 0.15% | -0.04% | 110% | 109% | 0.02% | -0.03% | -0.05% | 105% | 107% |
| Class D | 0.18% | 0.57% | -0.33% | 116% | 99% | 0.17% | 0.69% | 0.14% | 110% | 98% |
| Class F | -7.52% | -7.70% | -8.03% | 115% | 111% | -7.67% | -8.04% | -8.04% | 112% | 113% |
| Class SCC | -20.94% | -21.97% | -21.98% | 112% | 114% | -21.52% | -22.50% | -22.54% | 111% | 110% |

Table 4. Performance comparison of Test b over Test a

|  | AI configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Overall CTC | -0.03% | -0.02% | 0.00% | 120% | 105% |
| Class F | -0.42% | -0.45% | -0.30% | 127% | 105% |
| Class SCC | -1.71% | -1.60% | -1.58% | 120% | 102% |
|  | RA configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Overall CTC | -0.01% | 0.00% | -0.06% | 104% | 101% |
| Class F | -0.22% | -0.37% | -0.25% | 88% | 94% |
| Class SCC | -1.11% | -1.07% | -1.09% | 107% | 100% |
|  | LB configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Overall CTC | 0.00% | -0.17% | -0.01% | 99% | 99% |
| Class F | -0.26% | 0.22% | -0.41% | 101% | 100% |
| Class SCC | -1.03% | -1.14% | -1.25% | 101% | 97% |

It is reported in Table 4 that, extending 10 lines above the current and left CTU provides an average gain of 1.71%, 1.11%, and 1.03% for Class SCC 1080p under the AI, RA, and LB configuration, respectively.

# Conclusions

This contribution proposes restrictions for search area of IBC mode in CPR.The search area is restricted to few CTUs (e.g., 1, 2, and 3) to the left of current CTU in addition to current CTU. Moreover, the test with 10 lines above these CTU has also been reported.

References

[1] Xiaozhong Xu, Xiang Li and Shan Liu, “CE8-2.2: Current picture referencing using reference index signaling”, JVET-K0076, Ljubljana, SL, July 2018

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).