| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0406r1 |
| --- | --- |

| Title: | CE14: Test on in-loop bilateral filter from JVET-J0021/JVET-K0384 with parametrization (CE14.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Nikolay Shlyakhov Ted Hsieh Marta Karczewicz | Tel: Email: | +1 858 651 4225 dmytror@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution report results of CE14.2 test on in-loop Bilateral Filtering (BIF) method that was proposed in the JVET-J0021 and latter tested in JVET-K0384. In this contribution, the simplified, parametrized version of the original BIF [2], [3] is tested. It is proposed to represent the weighting functions of the BIF with a piece-wise linear model. This drastically reduces memory requirements (size of the stored LUT) from 30KB in [2] to 120 Bytes ROM and can enable a low complexity LUT-free implementation of the BIF. In the latter case, the size of the LUT at the CU level consists of 4 entries (PWL2). Complexity estimates for LUT-based and LUT-free implementation are provided. It is also proposed to skip processing of the 4x4 blocks, thus further reducing computation complexity of the proposed BIF. Maximum size of the filtered block for testes CE14.2 a/b was 8x64 or 64x8. Impact of increased filtered block size is tested in CE14.2.c, and reported separately in the related document JVET-L0584.

Proposed BIF was integrated in the BMS2.01 and tested under CE test conditions in two configurations: CE14.2.a tests BIF with 10-ranges PWL model and CE14.2.b tests the reduced complexity of the PWL with 2 non-zero ranges, with its parameters being adapted to QP value and block partitioning. Filtering of 4x4 blocks was omitted in both tests. The following BD rate results comparing to the VTM2.0.1 CTC are reported in this document:

CE14.2.a: AI : -0.43% 0.20% 0.18%; RA: -0.60% -0.12% -0.07%; LDB: -0.75% 0.20%	0.03%

CE14.2.b: AI : -0.42% 0.13% 0.16%; RA: -0.59% -0.13% -0.11%; LDB: -0.75% 0.37%	0.03%

# Introduction

The bilateral filter is an edge-preserving filter that takes into account the geometric distances between pixels as well as the intensity differences between them. By considering intensity differences between pixels, the bilateral filter prevents blurring of edges in an image during filtering.

This contribution reports results of CE14.2 test on in-loop Bilateral Filtering method that was proposed in the JVET-J0021 and integrated to the VTM2.0.1 code base with simplifications. This method was earlier tested in VTM1.0 and BMS1.0 code bases and those results were provided in document JVET-K00384.

Proposed bilateral filter is a 5-tap bilateral filter which is applied on the reconstructed block when the transform has non-zero coefficients and QP is larger than 17. The strength of the filter is controlled by QP and the center weight of the filter.

# Proposed technology

The proposed bilateral filter is always applied to luma blocks with non-zero transform coefficients and slice quantization parameter larger than 17. Therefore, there is no need to signal the usage of the bilateral filter. Bilateral filter, if applied, is performed on decoded samples right after the inverse transform. In addition, the filter parameters, i.e., weights are explicitly derived from the coded information.

The filtering process is defined as:

P0,  0'=P0, 0+k=1KWkPk, 0-P0, 0×Pk, 0-P0, 0,			(1)

where P0,  0 is the intensity of the current sample and P0, 0' is the modified intensity of the current sample, Pk,  0 and Wk are the intensity and weighting parameter for the k-th neighboring sample, respectively. An example of one current sample and its four neighboring samples (i.e., K=4) is depicted in Figure 1.

Figure 1: Neighboring samples utilized in bilateral filter.

More specifically, the weight Wk(x) associated with the k-th neighboring sample is defined as follows:

Wk(x)=Distancek× Rangek(x) 					(2)

wherein

Distancek=e- 100002σd21+4*e- 100002σd2,				(3)

Rangekx=e– x28*QP – 17*(QP – 17)

and σd is dependent on the coded mode and coding block sizes. The described filtering process is applied to intra-coded blocks, and inter-coded blocks when TU is further split, to enable parallel processing.

To better capture statistical properties of video signal, and improve performance of the filter, weights function resulted from Equation (2) are being adjusted by the σd parameter, tabulated in Table 1 as being dependent on coding mode and parameters of block partitioning (minimal size).

Table 1 Value of σd for different block sizes and coding modes

| Min(block width, block height) | Intra mode | Inter mode |
| --- | --- | --- |
| 4 | 82 | 62 |
| 8 | 72 | 52 |
| Other | 52 | 32 |

To further improve the coding performance, for inter-coded blocks when TU is not split, the intensity difference between current sample and one of its neighboring samples is replaced by a representative intensity difference between two windows covering current sample and the neighboring sample. Therefore, the equation of filtering process is revised to:

P0,  0'=P0, 0+k=1NWk1Mm=-M/2M/2abs(Pk, m-P0, m)×(Pk, 0-P0, 0)		(4)

wherein Pk, m and P0, m represent the m-th sample value within the windows centered at  Pk,  0 and P0,  0, respectively. In this proposal, the window size is set to 3×3. An example of two windows covering P2,  0 and P0,  0 are depicted in Figure 2.

Figure 2: Windows covering two samples utilized in weight calculation.

## Parametrization of the weighting functions:

Enabling weights construction as specified in Eq. 2-4, produces a family of weighted functions consisting of 270 functions or variable length. Figure 3 visualize resulting functions for a set of QP values:

Figure 3. Example of Wk(x) as function of x (shown on horizontal axis) and QP values

This family of weighting functions Wkx, QP, mode produced from Eq. 2-4 can be parameterized by a piece-wise linear model from a basis function W0(x), which is coarsely represented with a small number of pivot points W0 and derivation scaler for QP Sqp and mode Sm.

Below are provided an example of PWL model parameters being tested in CE14.2.a, with full family of W(x) being compactly represented with 120 bytes:

W0[10] = {42950, 37903, 26050, 13944, 5813, 1887, 477, 94, 14, 2}

Sm[5] = { 30195, 19316, 26068, 19316, 1470 };

Sqp[45] = { 65535, 43690, 32768, 26214, 21845, 18724, 16384, 14563, 13107, 11915, 10922, 10082, 9362, 8738, 8192, 7710, 7281, 6898, 6553, 6241, 5957, 5698, 5461, 5242, 5041, 4854, 4681, 4519, 4369, 4228, 4096, 3971, 3855, 3744, 3640, 3542, 3449, 3360, 3276, 3196, 3120, 3048, 2978, 2912, 2849};

The process of W(x) derivation utilizes a piece-wise linear interpolation between j pivot points specified by W0 as following:

idxQP = QP-18;sqp = Sqp[idxQP];
R = idxQP;

sm = Sm[mode];
Wm = W0 *sm >> 15;
R[0] = 0;

For j = 1..M-1{
 	idx1 = j;
	d = (Wm[idx1] - Wm[idx1-1])*sqp >> 17							(5)

R[idx1] = R[idx1-1] + R;

For x within j-range:
	{
   		idx2 = x – R[idx1-1];												    W(x) = max(Wm[idx1], Wm[idx1-1] + idx2 *d); 
	}
}

The W(x) are represented with 16 bits accuracy, thus the size of the LUT W(x,Qp, mode) entry is 2 bytes.

This approach allows to achieve a high accuracy approximation of the weight function W(x,Qp, mode), as shown in Figure 4 below:

Figure 4. Results of approximation of a weight function W(x,Qp,mode) with proposed 10-ranges PWL model

In the test CE14.2.b, a further reduced complexity PWL is utilized to reconstruct the family functions W(x,Qp, mode). Total number of two ranges are defied for PWL, with R[0] value separating two ranges of the PWL are selected to be power of 2, thus can be represented with R[0] = 1<< rShift. Under these constrains a reduced complexity derivation process can be employed.

The variable idxRQP[6] = 18+(1<<i) with i=0..5 establish the dependency between QP value and topShift value employed by the PWL model.

Reduced parameters set provided to the CU level for w(x) derivation consists of rShift, W[2], d[2], totally of 5 16 bits entries is derived as following:

sqp = Sqp[QP];sm = Sm[mode];
rShift = deriveR0 (idxRQP_minus18, QP); 
R[0] = 1 << rShift;
W[0] = W0[0]*sm >> 15;
W[1] = deriveW1(W0[10],sqp) * sm >> 15;							(6)

d[0] = (W[1] - W[0]))*sqp >> 17

d[1] = (-W[1]))*sqp >> 17

The function deriveRS() utilizes 5 comparators for QP against the idxRPQ entries and returns idx of idxRPQ. The function deriveW1() employs 9 parallel comparators to interpolation W1 value from W0[10]

Describe above derivation are invoked once per sequence for each CU type and results in 5 bytes of data for each. Due to this, complexity of this derivation is not included in the complexity analysis provided below.

During the filtering process at CU elvel, for each x value, with x > 0 representing absolute difference of intensity values x=Pk, 0-P0, 0, the w(x) can be derived as following:

R[1] = R[0];

R[0] = 0;

idx1 = (bool) (x >> rShift);

idx2 = x – R[idx1];												(7)

w(x)= max(0, W[idx1] + d[idx1] * idx2 );

Proposed PWL model with adaptive range sizes allows to achieve more coarse, however still sufficiently accurate approximation of the weighting function W(x), as shown in Figure 5 below:

Figure 4. Results of approximation of a weight function W(x,Qp,mode) with proposed 2-ranges PWL model

## Implementation of weights handling:

In this section we discuss potential approaches for parametrized filter implementation and analyze its complexity and memory requirements. One can foresee two possible approaches for implementations: the LUT-based, when LUT is fully constructed prior to CU filtering and LUT-free implementation, when weight w(x) is derived on fly for each x=Pk, 0-P0, 0.

In the first approach, the LUT of the W(x) given in Eq.(2) is generated from reduced parameters set prior to CU processing and provided to the filtering module. Figure 5 shows a block-diagram of such approach.

W0[10], Sm[5], Sqp[45]

Construction
LUT W(Qp,m)
Eq (5)

Filtering
Eq(1) or Eq (4)

Qp/mode

Samples block
WxH

Output
Samples block
WxH

W[N],

7 < N < 370

Figure 5. Block diagram of the LUT-based design of parametrized bilateral filter

The length of the pre-computed LUT W[N] is less than 370 in the case of M=10 and less than 210 in the case of M=2. This approach can be computationally beneficial for large block sizes, e.g. 16x16 and larger or in the case if QP diversity within a coded picture is small, and the same W(x) is shared between many CU blocks (quite common use case). This LUT-based approach was implemented in the BMS based test software and tested in CE14.2.a and CE14.2.b.

The second approach assumes that no LUT of the W(x) is constructed prior to the filtering process. Instead, weight w(x) value for each x would be computed on fly. This design choice is likely to be more useful for memory constrained and hardware implementations A block diagram of such design is shown in Figure 6:

W0[10], Sm[5], Sqp[45]

Derive reduced set W, d, R

Filtering, 
Eq(1) or Eq(4)

Qp/mode

Samples block
NxM

Output
Samples block
NxM

w(x) derivation
Eq(5) or Eq (7)

Figure 6. Block diagram of the LUT-free design of the parametrized bilateral filter

Computational complexity of the w(x) value derived as shown in Eq (5) is estimated as following:

- M-1 parallel comparator (1 cycle) to identify the PWL range idx1

- 2 fetch operations for d[idx1] and W[idx1]

- 1 subtraction operation to derive idx2

- 1 multiplication and 1 summation to derive w(x)

- 1 comparator, to implement max(x)

In summary, value w(x) derivation requires 2 adds, 1 multiplication and 2 comparators operations are required. The special case of PWL model, with M=2 and R[0] being power of 2 can be implemented as shown in Eq(5), thus a single bit-shift operation replaces the parallel comparator. Each weight value w(x) can be derived with 2 adds, 1 mult, 1 bit-shift and 1 max operation (comparator) and it is estimated that HW implementation with 1 cycle is feasible.

Complexity estimates for weights w(x) derivation are summarized in Table 1 and memory requirements are given in Table 2.

Table 1 Number of operations for weight derivation (per weight)

|  | Proposed parametrized |  |
| --- | --- | --- |
|  | M ranges | 2 ranges |
| #multiplications | 1 | 1 |
| #additions | 2 | 2 |
| #bitshifts | 0 | 1 |
| #max | 1 | 1 |
| #comparators | M-1* |  |

Note: For M > 2, 1 group of parallel comparators size of M-1 is counted as 1 comparator

Memory requirements estimates for handling weights are provided in Table 2. The PWL tested in this proposal can be constructed with 60 initial LUT entries, defining the family of weighting function, PWL parameters and provided to the decoder as a side information. For each QP/partition type, a reduced parameters set is derived and passed to the CU level filtering.

In the case of PWL10, tested in CE14.2.a, reduced parameters set need to be provided to CU level include: a single value W[1] to identify the starting value of the LUT, single value R[1] to specify the size of the equal range utilized in the LUT and 10 values to specify the slop of the LUT in the each range. In total 12 parameters of 16 bits representation need to be passed.

In the case of PWL2, tested in CE14.2.b, a reduced parameters set need to be provided to CU level include: a single value W[1], one values specifying the first range size R[1] and 2 values specifying the slop of the LUT in the each range. In total 5 parameters of 16 bits representation need to be passed.

Table 2 Memory requirements estimates for BIF weights handling, in 16 bit words

|  | Proposed parametrized |  |
| --- | --- | --- |
| Storage type | M ranges LUT-Free | 2 ranges LUT-Free |
| Sequence level | 60 entries | 60 entries |
| CU level | M+2 | 4 |

## Implementation of BIF filtering:

No changes to the filtering process or application of the filter is introduced in this contribution, thus analysis conducted in previous contributions [2] and [3] applies.

The filtering process of Eq (1) for every P0, 0 is conducted as following:

Given: eXl, eXt from previous pixel:

dXr= abs(P3, 0-P0, 0)

dXb=abs(P4, 0-P0, 0)

eXr= wabs(dXr)*dXr

eXb=wabs(dXb)*dXb

eXc=eXr+eXb+eXl+eXt

With this, filtered sample output is produced as following:

int16_t fXc = (int16_t)( P0, 0 + (uint16_t)(( eXc + ScaleOffset) >> SCALE_SHIFT_BITS));

This gives complexity estimates of Eq(1) to be equal to: 2 mults, 8 adds, 1 bitshift, 2 abs and it is estimated that HW implementation with 2 cycles is feasible.

In the case of filtering in inter coded block, processing in Eq (4) is invoked, thus for 4 non-center filter taps Pi, a local average of abs(Pk,m-Pi) within 3x3 processing window is computed. With parallel computation in mind, local average in Eq.(4) can reuse absolute difference computed for top/left neighbors, thus compare to complexity described above only 3 additional differences need to be computed for each weight derivation, namely abs(P2,1-Pi), abs(P2,4-Pi)  and abs(P2,2-Pi), where Pi is a center of the blue square in Figure 2. This results in extra 3 adds and 3 abs operations for difference computation and 7 additional adds and 1 bitshift to produce an average for 3x3 window.

Table 3 Number of operations for BIF filtering

|  | Proposed parametrized Filtering |  |
| --- | --- | --- |
|  | Intra filter | Inter filtering |
| #multiplications | 2 | 2 |
| #additions | 8 | 18 |
| #bitshifts | 1 | 2 |
| #abs | 2 | 5 |

## Summary on memory and complexity requirements:

This section would provide summary of memory and complexity requirements for BIF weights handling and filtering process only.

Estimates of memory requirements for LUT BIF handling for proposed method and anchor design are provide in Table 4.

Table 4 Number of entries in LUTs used in proposed and JEM/BMS-2.0 bilateral filter

| Method | Weights LUT size |  | LUTs for division |
| --- | --- | --- | --- |
| JEM/BMS-2.0 | 3468 |  | 325 x 2 |
| BIF in K0384 [3] | 28795 |  | - |
| Proposed Parametrized, M=10 | 60 sequence level storage |  | - |
|  | < 370 CU level LUT-based design | 12 CU level LUT-free design |  |
| Proposed Parametrized, M=2 | 60 sequence level storage |  | - |
|  | < 210 CU level LUT-based design | 4 CU level LUT-free design |  |

Table 5 Number of operations for BIF filtering per sample

|  | Proposed parametrized BIF |  |  |  |
| --- | --- | --- | --- | --- |
|  | Filtering Intra | Filtering Inter | Weights handling in M ranges BIF LUT-Free | Weights handling in 2 ranges BIF LUT-Free |
| #multiplications | 2 | 2 | 2 | 2 |
| #additions | 8 | 18 | 4 | 4 |
| #bitshifts | 1 | 2 | 0 | 2 |
| #abs | 2 | 5 | 0 | 0 |
| #max |  |  | 2 | 2 |
| #comparators |  |  | 2x(M-1) | 0 |

And finally, for the proposed test CE14.2.b ( 2 range PWL approximation) we provide complexity estimate in formalized CE report table:

Table 6 Worst case complexity and memory estimates for elements of proposed CE14.2.b BIF

| Test | filter shape | Compu-tational complex-ity per sample | Precision of mult | Parallel friendly | Latency (in clock cycles) | Memory requirements | How to derive filter coeffs | Minimal CU size for applying filters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14.2.a, LUT based | 5 pixel “plus”-shape Inter: w(x) with NL average | Intra: 2 mult 8 adds 2 checks Inter: 22 ads 5 checks | 32 bits registers | yes | Intra: 2 Inter: 3 | ROM: 120 CU level: <370*2 | Computed prior to CU: idx1= compar(x,R) idx2=x-R[idx1] max0, Widx1+d[idx1]×idx2 | 4×8 and 8×4 |
| 14.2.b LUT based | —”— | —”— | —”— | —”— | —”— | ROM: 120 CU level: <210*2 | Computed prior to CU: idx= x≫BS idx2=x-idx1 max0, Widx1+d[idx1]×idx2 | —”— |
| 14.2.a PWL, M=10 LUT free | —”— | Intra: 4 mult 12 adds 13 checks Inter: 4 mult 22 ads 16 checks | 32 bits registers | yes | Intra: 4 Inter: 5 | 120bytes sequence level 24bytes CU level | idx1= compar(x,R) idx2=x-R[idx1] max0, Widx1+d[idx1]×idx2 | 4×8 and 8×4 |
| 14.2.b PWL, M=2 LUT free | —”— | Intra: 4 mult 12 adds 4 checks Inter: 4 mult 22 ads 7 checks | —”— | —”— | Intra: 3 Inter: 4 | 120bytes sequence level 8bytes CU level | idx= x≫BS idx2=x-idx1 max0, Widx1+d[idx1]×idx2 | —”— |

# Experimental Results

The CE14.2 tests was implemented on top of BMS2.01 with LUT-based approach, Figure 5, and tested under VTM CTC. Two configurations of the proposed method was tested: CE14.2.a – proposed BIF is implemented with PWL employing 10 ranges (pieces), and CE14.a.b – proposed BIF is implemented with reduced complexity of PWL employing 2 ranges. Filtering of 4x4 blocks was omitted in both tests. Simulation results for tested method are provided in Tables 3– 4. Note: more accurate time estimates will be provided in an updated version of the document.

## Sub test CE14.2.a

Table 2. CE14.2.a simulation results comparing to VTM2.01 CTC

## Sub test CE14.2.b

Table 2. CE14.2.b simulation results comparing to VTM2.01 CTC

# Conclusion

This contribution report results of CE14.2 test on in-loop Bilateral Filtering (BIF) method that was proposed in the JVET-J0021 and latter tested in JVET-K0384. In this contribution, the simplified, parametrized version of the original BIF [2], [3] is tested.

Proposed BIF was integrated in the BMS2.01 and tested under CE14 test conditions in two configurations: CE14.2.a tests BIF with 10-ranges PWL model and CE14.2.b tests the reduced complexity of the PWL with 2 non-zero ranges. Filtering of 4x4 blocks was omitted in both tests. Mia

It is proposed to adopt the proposed parametrized BIF method with 2-range PWL model (CE14.2.b) into the next working draft.

# Patent rights declaration(s)

Qualcomm Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Description of Core Experiment 11 (CE14): Deblocking”, JVET-K1034.

- “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions”, 10th JVET Meeting, San Diego, California, USA, April 2018, JVET-J0021.

- “CE2.1.3: In-loop bilateral filter”, 11th JVET Meeting, Ljubljana, Slovenia, July 2018, JVET-K0384.

- “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010.