| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0411r1 |
| --- | --- |

| Title: | CE4-related: Angular merge prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | +82-3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution proposes angular merge prediction which introduces sub-block motion compensation prediction, where the sub-CU MVs are extrapolated from the reconstructed blocks located in the specific direction. Angular merge prediction is controlled by one additional syntax signaled at CU level followed by the syntax which specifies which extrapolating direction is used. The proposed angular merge prediction provides 0.13% and 0.15% bitrate saving for random access and low delay B configuration with 21% and 28% encoding runtime increase, respectively. In revision 1, simulation results with additional fast decision algorithms are added. With fast decision, 0.10% bitrate saving for both random access and low delay B configurations with 9% and 12 % encoder runtime increase, respectively.

Introduction

In VTM-2.0.1, affine motion compensation prediction is adopted which predicts using zoomed and rotated motion model in addition to the conventional translation motion model. However, for the region which includes object boundaries and occlusion, both conventional translation model and affine motion model cannot precisely predict despite that fact that this situation are observed quite often in the real world.

To improve prediction accuracy in such occluded regions near the object boundary, angular merge prediction is introduced in this contribution. The angular merge prediction introduces sub-block motion compensation prediction, where the sub-CU MVs are extrapolated from the reconstructed blocks located in the specific direction.

Firstly, reference MVs used for extrapolation are extracted from the neighboring reconstructed blocks. If neighboring MVs are not available because the neighboring block is an intra-coded block or outside slice boundary, the MVs are substituted from the nearest neighbor available MVs in a similar way to intra reference sample substitution.

For angular merge prediction, five extrapolating directions; Bottom-left(0), Left(1), Above-left(2), Above(3), and Above-right(4), are alternatively applied. For each direction, the following MV extrapolation is curried out:

pMVxu=refMV-1x+y+1				(Bottom-left),

pMV[x][y]=refMV[-1][y]						(Left),

pMV[x][y]=refMV[x-y-1][-1],  &x≥yrefMV[-1][y-x-1],  &x<y		(Above-left),

pMV[x][y]=refMV[x][-1]						(Above),

pMV[x][y]=refMV[x+y+1][-1]				(Above-right),

where refMV denotes reference motion vector extracted from neighboring reconstructed blocks and pMV denotes extrapolated sub-block motion vectors of current CU.

As a syntax, angular_merge_flag which specifies whether angular merge prediction is applied or not is signaled at CU level. If angular_merge_flag is 1, angular_merge_idx which specifies which extrapolating direction is applied is signaled additionally.

Simulation results

The proposed angular merge prediction has been implemented on top of BMS-2.0.1 and compression performance under the common test conditions is evaluated. The experimental results of angular merge prediction in random access and low delay B configurations are shown in Table 1 and 2, respectively.

Table 1. BD bitrate results of angular merge prediction in RA condition

|  | Over BMS-2.0.1 (VTM configuration) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.14% | -0.21% | 117% | 118% |
| Class A2 | -0.14% | -0.29% | -0.31% | 124% | 116% |
| Class B | -0.12% | -0.17% | -0.32% | 120% | 116% |
| Class C | -0.07% | -0.18% | -0.29% | 122% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.19% | -0.29% | 121% | 114% |
| Class D | 0.06% | -0.19% | 0.02% | 125% | 99% |

Table 2. BD bitrate results of angular merge prediction in LB condition

|  | Over BMS-2.0.1 (VTM configuration) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | -0.36% | -0.49% | 127% | 111% |
| Class C | -0.09% | -0.40% | -0.30% | 128% | 117% |
| Class E | -0.29% | -0.23% | -0.42% | 131% | 129% |
| Overall | -0.15% | -0.34% | -0.41% | 128% | 117% |
| Class D | -0.07% | 0.14% | -0.31% | 128% | 100% |

To reduce encoder runtime, fast decision algorithm is introduced in revision 1. The fast decision algorithm is almost same as that of normal merge mode, where the mode decision consists of two stages. In the first stage, approximated RD cost for each candidate is calculated by the linear combination of simply estimated number of flag bit and sum of absolute transformed difference (SATD). In the second stage, only top three candidates are tested with exhaustive RD cost calculation. Table 3 and 4 show the experimental results of angular merge prediction with fast decision.

Table 3. BD bitrate results of angular merge prediction with fast decision in RA condition

|  | Over BMS-2.0.1 (VTM configuration) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.08% | -0.22% | 106% | 115% |
| Class A2 | -0.10% | -0.23% | -0.32% | 109% | 113% |
| Class B | -0.09% | -0.08% | -0.19% | 109% | 113% |
| Class C | -0.06% | -0.17% | -0.17% | 110% | 109% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | -0.13% | -0.21% | 109% | 112% |
| Class D | 0.03% | -0.18% | 0.01% | 111% | 102% |

Table 4. BD bitrate results of angular merge prediction with fast decision in LB condition

|  | Over BMS-2.0.1 (VTM configuration) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | -0.34% | -0.34% | 113% | 113% |
| Class C | -0.05% | -0.34% | -0.27% | 109% | 109% |
| Class E | -0.18% | 0.03% | 0.11% | 113% | 123% |
| Overall | -0.10% | -0.25% | -0.20% | 112% | 114% |
| Class D | -0.04% | 0.24% | -0.72% | 114% | 100% |

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).