| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0429-v1 |
| --- | --- |

| Title: | CE5-related: lookup table-free implementation of the probability update for tests CE5.1.4 and CE5.1.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stefan Matlage, Paul Haase, Heiner Kirchhoffer, Detlev Marpe, Heiko Schwarz, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-184 stefan.matlage@ hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

An extension to the stated-based probability estimator presented in [1] is proposed. The proposed method introduces an optimized transition table and two parameters are added in order to allow for improved control of the state variable update.

# Introduction

The transition table size is kept at 32 elements but is optimized with respect to the video compression application. Shift parameters for both state variables S0 and S1 are introduced to enable controlling the agility of the probability update behaviour of each state.

# Proposed method

Similar to the method proposed in JVET-K430, two signed integer state variables S0 and S1 are stored per context model which require 8 and 12 bit, respectively. However, the state variable update after encoding or decoding a bin is slightly altered, according to the following pseudo code:

if( bin )

{

S0 +=  Tab[ 16 + (S0 >> 3) ] >> shiftS0;

S1 += (Tab[ 16 + (S1 >> 7) ] << 4) >> shiftS1;

}

else

{

S0 -= Tab[ 16 + (-S0 >> 3) ] >> shiftS0;

S1 -= (Tab[ 16 + (-S1 >> 7) ] << 4) >> shiftS1;

}

where lookup table ‘Tab’ is defined as follows:

Tab[32] = { 157, 143, 129, 115, 101,  87,  73,  59,  45,  35,  29,  23,  17,  13,   9,   5,

4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   0 }

and shiftS0 and shiftS1 are defined as variables to be stored per context model in order to improve coding efficiency by supplying different state variable update agilities. However, when shiftS0 and shiftS1 are to be kept fixed over all context models, we define shiftS0 = 1, as well as shiftS1 = 4.

Note that the implementation in the CE software is different in that the state variables S0 and S1 are both defined as 16 bit signed integers. This enables for better flexibility when implementing changes and avoids the need for custom data types. The precision of 8 or 12 bit, respectively, is realized by interpreting S0 and S1 as left aligned 16 bit integer variables and applying respective bit shifting / clearing of the LSBs.

## Lookup table-free state transition

Lookup table ‘Tab’ can be replaced by the following calculation:

Tab16+s=1024+(max0,b1-s≪a1)+  (max0,b2-s≪a2)+ max0,b3-s≪a3

Note that Tab[31] must always be equal to 0.

bi, and ai are parameters that control the probability update behavior.

Optionally, LUT ‘Tab’ can still be used instead of implementing the calculation.

In an effort to optimize lookup table ‘Tab’ for video compression, the following range of configurations was used:

a1=2, 3… 7,  a2=3, 4…7,  a3=5, 6…8

b1=-0, -1, -2, -3, -4, -5, -6, -7

b2=-9, -11, -13, -15, -17, -19, -21, -23

b3=-24, -26, -28, -30, -32, -34, -36, -38

Referring to CE5.1.4, when shiftS0 and shiftS1 are kept fixed over all context models, ‘Tab’ is configured as

Tab[32] = {  161, 147, 133, 119, 105,  91,  77,  63,  49,  35,  29,  23,  17,  13,   9,   5,

4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   0 }

and was derived with

a1=8, a2=7, a3=9

b1=-3, b2=-15, b3=-30

Referring to CE5.1.4, when shiftS0 and shiftS1 can vary between context models, ‘Tab’ is configured as

Tab[32] = {  165, 151, 137, 123, 109,  95,  81,  67,  53,  39,  29,  23,  17,  13,   9,   5,

4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   0 }

and was derived with

a1=8, a2=7, a3=9

b1=-3, b2=-15, b3=-26

Referring to CE5.1.5, when only a single state variable is used and the shift parameter can vary between context models, ‘Tab’ is configured as

Tab[32] = {  225, 203, 181, 159, 137, 115,  93,  71,  49,  35,  29,  23,  17,  13,   9,   5,

4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   0 }

and was derived with

a1=8, a2=7, a3=10

b1=-3, b2=-15, b3=-28

# Experimental Results

BD-Rate results can be obtained from CE5.1.4 and CE5.1.5. Complete results for BMS-2.1 and VTM-2.1 will follow.

# References

- Heiner Kirchhoffer, Jan Stegemann, Detlev Marpe, Heiko Schwarz, Thomas Wiegand, “CE5-related: State-based probability estimator,” JVET-K430, 11th Meeting, Ljubljana, SI, July 2018

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).