| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0490r2 |
| --- | --- |

| Title: | CE12-related: HDR Coding in VVC with Backward Compatibility Options |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | P. Topiwala, M. Krishnan, W. Dai 3097 Cortona Dr. Melbourne, FL 32940 | Tel: Email: | 321-355-7376 pankajtva at gmail dot com |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

This smbmission presents an approach to HDR/WCG video coding developed at FastVDO, which we call FVHDR, and built on top of the Versatile Video Coding (VVC) VTM2.0.1 test model of the Joint Video Exploration Team. A fully automatic adaptive video process that differs from a known HDR video processing chain (analogous to HDR10, and herein called “anchor”) developed previously in the standards committee JCTVC, is used. FVHDR works entirely within the framework of the VTM software model, but adds additional tools. These tools can become an integral part of a future VVC video coding standard, or be developed as additional pre- and post-processing chains (as an example, presented as pre/post processes in this proposal).  Reconstructed videos codec with the proposed system are claimed to show subjective improvement over the anchor system. Moreover, the resultant SDR content generated by the proposed data adaptive grading process is claimed to offer backward compatibility options, which can be developed in follow-on work.

# Introduction

HDR/WCG (herein “HDR”) video data have high levels of luminosity, of at least 1000 nits and potentially up to 10000 nits, and with a much wider color gamut, given by the BT. 2020 standard, compared to standard dynamic range (SDR) video content [1]. A standardized approach to coding such content using HEVC has been recently developed called HDR10, and is now being deployed in broadcast and other video applications. The core coding standard utilized in HDR10 is the Main10 Profile of ITU|ISO/IEC HEVC, High Efficiency Video Coding standard. Currently JVET is exploring the coding of HDR content using VTM [2]. As an active member of the committee, FastVDO has developed several effective approaches for HDR video coding that can be implemented either as pre/post processing steps or as normative changes to the core 10-bit encode/decode processes of VTM [2]. We will use an analog of the HDR10 approach as an anchor for comparisons.

# Anchor Coding Scheme

For our purposes the anchor coding chain is presented in Figure 1. The input video to the encoder is in Y’CbCr 4:2:0 (HDR) format generated from linear light RGB video signal after tone mapping using the ST-2084 transfer function (PQ OETF) [4] followed by color transformation and then downsampling. The encoding is done with VTM1.0 profile. The corresponding VTM1.0 decoder is used to get the reconstructed Y’CbCr 4:2:0 HDR video.

Figure 1: Basic HDR processing chain. The input/output are in a fixed, static transfer function domain (e.g., ST-2084, or HLG). This will serve as our anchor, with VTM 2.0.1.

# FastVDO HDR Coding System

In contrast to figure 1, the basic block diagram of the FVHDR approach is represented in Figure 2, which is here depicted as pre/post processing (alternative approaches that are embedded normatively in the codec are also available). A more extensive discussion of FastVDO approaches to HDR coding, in the context of HEVC, are covered in the Technical Report ISO/IEC TR23008-15 [TR2]. These methods are now adopted for VVC. A data adaptive grading (DAG) process is applied to the Y’CbCr 4:2:0 HDR input video. The graded YCbCr/YFbFr 4:2:0 SDR video along with the associated metadata is then encoded with VTM using the equivalent of the Main 10 profile. After decoding the bitstream an inverse grading process is applied to the reconstructed YCbCr/YFbFr 4:2:0 SDR video to generate the Y’CbCr 4:2:0 HDR reconstructed video. This process utilizes the decoded metadata for adaptive degrading. One variant of this method was presented recently by us at the JVET standards committee, at the Torino, IT, meeting, July, 2017 [8].

Figure 2: FVHDR coding chain. The difference to figure 1 is that we now use pre/post processing with data-adaptive grading, compared to static transfer functions (such as PQ, HLG).

  - DETAILS OF THE DATA ADAPTIVE GRADING (DAG)

The fully automatic DAG process is essentially a point operation on linear light signal. The Y’CbCr 4:2:0 HDR input video is upsampled to Y’CbCr 4:4:4 signal. A color transformation is then done to get a R’G’B’ 4:4:4 signal. PQ EOTF is then applied to get linear light RGBHDR. The RGBHDR input is then graded using a data adaptive OETF to RGBSDR. The RGBSDR content is then mapped to a non-constant luminance (NCL) Y’CbCr or a FastVDO developed YFbFr signal [5]. The resultant YCbCr/YFbFr signal is downsampled to 4:2:0 format using advanced chroma downsampling filters designed by FastVDO [6]. The YCbCr/YFbFr 4:2:0 signal is then encoded using the VTM reference software. Figure 3 describes the sequential steps involved in DAG.

Figure 3: Data Adaptive Grading (DAG) flowchart. A key novel component is the data-adaptive OETF (FVPQ). Note that in this paper we have developed an adaptive version of PQ, though similar ideas can be applied to any transfer function.

Data adaptive OETF: The application of data adaptive OETF involves two steps:

- Determine InputPeakBrightness:  The input Y’CbCr 4:2:0 HDR video is analyzed to determine the peak brightness/luminosity of data.

- Apply a FastVDO modified version of PQ-OETF (herein called FVPQ-OETF) to each channel of the input. The FVPQ-OETF function is described as (where Y now stands for individual signal components of RGBHDR and YOETF for individual signal components of RGBSDR):

YOETF=powf(((c2 *powf(Y, m1) + c1)/(1.0 + c3 *powf(Y, m1))), m2)  where

m1=(2610.0        ) / (4096.0 * 4.0) ,

m2=2523.0 * 128.04096* 1 + 0.25 * log10000InputPeakBrightness,

c1 = (3424.0        ) / 4096.0,

c2= (2413.0 *  32.0) / 4096.0,

c3= (2392.0 *  32.0) / 4096.0

The inverse function (FVPQ-EOTF):

Y=powfdMax0.0,powfYOETF,1.0m2- c1c2 - c3 * powfYOETF,1.0m2,1.0m1;

Here dMax(a,b) returns a if a > b else b.

Here, powfa,b=ab . Thus, the transfer function is adapted based on luminosity. When the parameter equals 10,000, this reduces to the usual PQ function. Figure 4 shows the function for different data luminosity. The entire process described in Figure 3 can be viewed as a single non-linear function. One dimensional (1D) look up tables can be generated to represent the mapping and sent as metadata information. Changes in mean luminosity between frames is used to trigger the transmission of new metadata information.

Figure 4: FVPQ OETF, a FastVDO update on the PQ transfer function, for fixed values of InputPeakBrightness = 10000, 4000, and 1000 nits, respectively (at 10K nits it equals the ST-2084 PQ function), in descending order. But when the input data is below 10000 nits, our TF affords more codewords at the brighter end than the standard PQ transfer function. The difference is significant. In fact, a 1K-nit signal using the standard PQ function would only use a portion of the curve (and codeword space); similarly for a 4K-nit signal.

  - INVERSE DATA ADAPTIVE GRADING (DAG)

At the decoder side, the bitstream is decoded to get the reconstructed YCbCr/YFbFr 4:2:0 SDR signal and metadata (1D lookup tables). The inverse DAG process uses the lookup tables to map YCbCr/YFbFr 4:2:0 SDR to Y’CbCr 4:2:0 HDR video.

# Experimental Setup and Results

The test sequences used in this submission are CTC test sequences. More details on the test sequences are available in [2]. The VTM reference codec VTM 2.0.1 is used for encoding/decoding process. Bitstreams are generated based on the Main 10 profile under Random access coding constraints. A structural delay of 16 pictures (GOP size of 16) was selected.  Intra random access pictures, corresponding to the parameter ‘IntraPeriod’ in the VTM configuration files, are introduced into the bit stream every 32 or 48 pictures for 24 fps, 25 fps and 50 fps sequences, respectively [2]. The codec is run at constant quality with fixed QP values. For each sequence, four rate points are generated corresponding to four different QP values. The simulations were performed in a Rocks cluster distribution with AMD Phenom II X6 1055T’s and Intel i7-6700HQ as the CPU’s and a RAM capacity of 16GB each.

Table 1. Preliminary results of FastVDO HDR coding system vs VTM anchor. Simulations for all 1080p sequences (HDR-B) are reported here. Simulations for the 4K HDR-A seqs are still running at time of submission. Note that wPSNR results are incorrectly calculated here (in the linear light domain, will be fixed by meeting).  These results are being cross-checked by Technicolor.

Our results are presented in Table 1 for the HDR-B sequences (1080p PQ data). We are very grateful to Technicolor for cross-checking our results, as well as pointing out some minor errors. In particular, we ran VTM2.0.1, while CE12 suggested to actually use BMS in VTM emulation mode, leading to minor differences with the cross-check results. In this update (r2), we also corrected some minor calculational errors (on seqs BalloonFestival, and Starting), so that these results for DE100 and PSNRL100 are a good (though not exact) match to the cross-check results of Technicolor. The wPSNR results are still incorrect, (computed in linear, rather than PQ, domain) and will be fixed in another revision.

# Conclusions

FastVDO presents a method of coding HDR video in the context of VVC, analogously to methods previously reported with HEVC in the JCTVC, and in TR2 [TR2]. It is asserted that this method provides visual gains over the anchor.  Moreover, our method provides an avenue to achieve certain backwards compatibility with SDR, which will be explored in follow-on work (and has been explained in [TR2]). Currently tested and reported objective metrics for HDR coding have not proved to be either consistent or reliable. We are prepared to participate in visual testing experiments to determine the efficacy of our proposed method vs the anchor.

# References

- AHG on HDR and WCG, M33622, ISO/IEC JTC1/SC29/WG11 MPEG, July 2014, Sapporo, Japan.

- VTM Test Model. https://jvet.hhi.fraunhofer.de/

- JVET-F1001, Algorithm Description of Joint Exploration Test Model 6 (JEM 6), 6th Meeting: Hobart, AU, 31 March – 7 April 2017.

- Society of Motion Picture and Television Engineers ST 2084 (2014), High Luminance EOTF.

- Pankaj Topiwala and Chengjie Tu, “New Invertible Integer Color Transforms Based on Lifting Steps and Coding of 4:4:4 Video”, JVT-I015r7, San Diego, Sept., 2003.

- Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “Advanced Extended Spatial Re-sampling Filters For SVC”, JVT-V030, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 22nd meeting, Marrakech, Morocco, Jan. 2007.

- G. Bjøntegaard, “Calculation of average psnr differences between rd-curves,” VCEG-M33, 13th VCEG meeting, Austin, Texas, March 2001.

- P. Topiwala, W. Dai, M. Krishnan, “FastVDO Response to the JVET CfE for HDR”, JVET-G0021, Torino, IT, July, 2017.

[TR2] ISO/IEC TR23008-15, “Signalling, Backward Compatibility and Display Adaptation for HDR/WCG Video,” 2018.

[9] X. Xiu et al, “Description of the SDR, HDR, and 360 video coding technology proposal by InterDigital Communications and Dolby Laboratories,” JVET-J0015, San Diego, CA, April, 2018.

[10] P. Bordes et al, “Description of the SDR, HDR, and 360 video coding technology proposal by Qualcomm and Technicolor – medium complexity version,” JVET-J0022, San Diego, CA, April, 2018.

# Patent rights declaration(s)

FastVDO LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).