| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0503 |
| --- | --- |

| Title: | Crosscheck of JVET-L0203: CE-4 related: LIC with reduced memory buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Saurav Bandyopadhyay 9276 Scranton Road, Suite 300 San Diego, CA 92121 USA | Tel: Email: | +1 (858) 210 4813 saurav.bandyopadhyay @interdigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

The contribution reports the cross-check for the results reported in the input document: JVET-L0203: CE-4 related: LIC with reduced memory buffer [1]. The cross-check contribution reports the results of the proposed clean-up on top of the BMS-2.0.1 for the Random Access and Low-Delay B configurations. The source code as well as the configuration files for performed test were provided by the proponent Technicolor. The cross-check confirms the results reported by the proponents.

# Summary of JVET-L0203: LIC with reduced memory buffer

JVET-L0203 propose a method for Local Illumination Compensation (LIC) with reduced memory bandwidth. Prior approaches derive the LIC parameters from the prediction samples and reconstructed samples of same block and store at CU level in order to reduce memory bandwidth. The proposed method further reduces memory bandwidth, by using a reduced memory of size 2 x 32 instead of accessing reference pixels and neighboring reconstructed pixels. BMS-2.0.1 software is used to provide BD-rate over VTM anchor.

# Simulation results

Simulations results reported in this contribution are based on the BMS2.0.1 software used as anchor. The results are generated using configuration for JVET-L0203. The cross-check confirms the results reported by the proponents.

|  | Over BMS-2.0.1 (VTM) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.68% | -0.14% | -0.71% | 226% | 113% |
| Class A2 | -0.23% | -0.24% | -0.35% | 178% | 115% |
| Class B | -0.48% | -0.28% | -0.12% | 195% | 124% |
| Class C | -0.22% | -0.19% | -0.31% | 211% | 141% |
| Class E |  |  |  |  |  |
| Overall | -0.40% | -0.22% | -0.34% | 202% | 124% |
| Class D | 0.02% | -0.17% | -0.16% | 204% | 147% |

Random Access Main 10

|  | Over BMS-2.0.1 (VTM) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.50% | -0.43% | -0.41% | 224% | 119% |
| Class C | -0.18% | -0.20% | -0.51% | 228% | 125% |
| Class E | 0.02% | 0.01% | 0.17% | 140% | 127% |
| Overall | -0.27% | -0.24% | -0.30% | 200% | 123% |
| Class D | 0.06% | 0.31% | -0.25% | 221% | 139% |

Low Delay B Main 10

# References

[1] P. Bordes, F. Léannec, F. Galpin, E. François, “CE4-related: LIC with reduced memory buffer”, JVET-L0203, the 12th JVET meeting, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

InterDigital Communications Inc. does not have any current or pending patent rights relating to the technology described in this contribution.