| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0510 |
| --- | --- |

| Title: | Cross-check of JVET-L0314: CE9-related: Constrained intra prediction with decoder side motion vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jackie Ma | Email: | jackie.ma@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the cross-check of document JVET-L0314 which studies different intra prediction constraints in combination with DMVR. No mismatches were found in the simulation results provided by the proponent.

# Cross-check

A patch for the VTM software was provided by the proponents. The different options can be configured in the config file using the parameter ContrainedIntraPred. The three options correspond to

--ConstrainedIntraPred=0: intra prediction reference may come from intra or inter coded neighboring blocks

--ConstrainedIntraPred=1: intra prediction reference may only come from intra coded neighboring blocks

--ConstrainedIntraPred=2: intra prediction may come from intra or inter coded neighboring blocks where DMVR is not applicable

# Test results

There were three test configurations cross-checked in combination with DMVR

Test 1: --ContrainedIntraPred=0

Test 2: --ContrainedIntraPred=1

Test 3: --ContrainedIntraPred=2

## Test 1

|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1-rc1 (VTM configuration) |  |  |  |  | Over BMS-2.1-rc1 (VTM configuration) + DMVR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1,76% | -1,57% | -1,92% | 106% | 130% | 0,00% | 0,00% | 0,00% | 101% | 101% |
| Class A2 | -2,32% | -2,45% | -2,53% | 105% | 126% | 0,00% | 0,00% | 0,00% | 101% | 99% |
| Class B | -1,32% | -1,48% | -1,53% | 108% | 135% | 0,00% | 0,00% | 0,00% | 102% | 99% |
| Class C | -1,39% | -1,61% | -1,69% | 106% | 132% | 0,00% | 0,00% | 0,00% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1,63% | -1,73% | -1,85% | 106% | 131% | 0,00% | 0,00% | 0,00% | 101% | 100% |
| Class D | -1,74% | -1,75% | -1,84% | 106% | 132% | 0,00% | 0,00% | 0,00% | 100% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

## Test 2

|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1-rc1 (VTM configuration) |  |  |  |  | Over BMS-2.1-rc1 (VTM configuration) + DMVR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 3,63% | 12,71% | 14,23% | 93% | 131% | 5,46% | 14,53% | 16,47% | 89% | 102% |
| Class A2 | 0,52% | 4,55% | 2,92% | 95% | 129% | 2,92% | 7,20% | 5,61% | 91% | 101% |
| Class B | 2,07% | 5,04% | 5,89% | 96% | 134% | 3,44% | 6,63% | 7,56% | 90% | 98% |
| Class C | 2,35% | 5,22% | 5,69% | 95% | 130% | 3,80% | 6,96% | 7,51% | 90% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 2,15% | 6,53% | 6,91% | 95% | 131% | 3,84% | 8,41% | 8,94% | 90% | 100% |
| Class D | 1,22% | 4,01% | 3,51% | 97% | 132% | 3,04% | 5,90% | 5,49% | 91% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

## Test 3

|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1-rc1 (VTM configuration) |  |  |  |  | Over branches_BMS-2.1-rc1 (VTM configuration) + DMVR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1,47% | -0,59% | -0,88% | 104% | 128% | 0,29% | 1,00% | 1,06% | 99% | 100% |
| Class A2 | -2,18% | -2,10% | -2,21% | 104% | 127% | 0,14% | 0,36% | 0,32% | 99% | 99% |
| Class B | -1,03% | -0,95% | -0,89% | 105% | 135% | 0,30% | 0,54% | 0,66% | 99% | 99% |
| Class C | -1,08% | -1,01% | -1,01% | 105% | 133% | 0,32% | 0,62% | 0,69% | 100% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1,36% | -1,12% | -1,18% | 105% | 131% | 0,27% | 0,61% | 0,68% | 99% | 100% |
| Class D | -1,50% | -1,37% | -1,48% | 106% | 133% | 0,25% | 0,39% | 0,37% | 100% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# References

- M. Xu, X. Li, S. Liu, “CE9-related: Constrained intra prediction with decoder side motion vector derivation,” JVET-L00314, October. 2018, Macao.