| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0529-v2 |
| --- | --- |

| Title: | CE11.1.10-related: Smoothness threshold modification for long tap deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | W. Zhu, K. Misra, A. Segall, M. Ikeda, T. Suzuki | Tel: Email: | +1 360 817 8451 zhuw@sharplabs.com misrak@sharplabs.com asegall@sharplabs.com masaru.ikeda@sony.com teruhiko.s@sony.com |
| Source: | Sharp Labs and Sony Corporation |  |  |

_____________________________

# Abstract

This contribution is related to CE 11.1.10.  CE11.1.10 is a joint deblocking design proposed by Sony and Sharp that is asserted to achieve significant visual quality improvement.  While subjective quality is perhaps the most important aspect a deblocking design, the proponents also observe that the configuration of the CE11.1.10 proposal results in a small loss in objective performance compared to VTM 2.0.1.  The purpose of this contribution is to provide supplemental results of the CE11.1.10 design with improved objective performance.  This is achieved by modifying the smoothness threshold for luma stronger filter deblocking in the CE11.1.10 algorithm. Objective evaluations show luma BD rate impact of -0.03%/-0.07%/0.02%/-0.06% for AI/RA/LD/LDP compared to VTM-2.0.1 and -0.04%/-0.12%/0.01%/-0.16% for AI/RA/LD/LDP compared to VTM-2.0.1 when ALF is turned off.

# Introduction

Deblocking in the current test model is based on HEVC deblocking and is applied on an 8x8 grid. In CE11.1.10, we proposed the use of block size decision for filtering different number of samples on block boundary. While the CE11.1.10 design is believed to provide high subjective quality, it has been observed that this comes at the expense of a decrease in objective performance. In the event that the committee would prefer to improve this performance aspect, a modification of the smoothness threshold for long-tap deblocking is described here.

The luma filtering, the chroma filtering and the chroma control operations of CE11.1.10 are unchanged within the context of this contribution and can be found in [5].

# Proposed threshold modification for CE11.1.10 luma stronger filter smoothness decision

In CE11.1.10 the Luma Stronger Filter is used for large blocks where large block corresponds to width >= 32 for a vertical edge, and height >= 32 for a horizontal edge. The adjacent block must be greater than or equal to 16. Additional constraints are also considered, and the selection process is defined below:

Condition L1 is true when p side is greater than or equal to 32 && q side is greater than or equal to 16

Condition L2 is true when q side is greater than or equal to 32 && p side is greater than or equal to 16

The variables dpq0, dpq3, dp, dq, and d are derived as follows:

dp0, dp3, dq0, dq3 are first derived as in HEVC

if (p side is greater than or equal to 32)

dp0 = ( dp0 + Abs( p5,0 − 2 * p4,0 + p3,0 ) + 1 ) >> 1

dp3 = ( dp3 + Abs( p5,3 − 2 * p4,3 + p3,3 ) + 1 ) >> 1

if (q side is greater than or equal to 32)

dq0 = ( dq0 + Abs( q5,0 − 2 * q4,0 + q3,0 ) + 1 ) >> 1

dq3 = ( dq3 + Abs( q5,3 − 2 * q4,3 + q3,3 ) + 1 ) >> 1

dpq0, dpq3, dp, dq, d are then derived as in HEVC

As in HEVC, When d is less than β, the following ordered steps apply:

dpq is derived as in HEVC.

sp3 = Abs( p3 − p0 ), derived as in HEVC

if (Condition L1)

sp3 = ( sp3 + Abs( p7 − p3 ) + 1) >> 1

sq3 = Abs( q0 − q3 ), derived as in HEVC

if (Condition L2)

sq3 = ( sq3 + Abs( q7 − q3 ) + 1) >> 1

As in HEVC derive, StrongFilterCondition = (dpq is less than ( β  >>  2 ), sp3 + sq3 is less than ( β  >>  3 ) ( Condition L1 is true or Condition L2 is true ) ? ( 3*β  >>  5 ) : ( β  >>  3 ) , and Abs( p0 − q0 ) is less than ( 5 * tC + 1 )  >>  1) ? TRUE : FALSE

When StrongFilterCondition is TRUE, use Luma Stronger Filter on the side with length perpendicular to boundary edge greater than or equal to 32.

The proposed threshold modification to CE11.1.10 is highlighted in yellow above.

# Results

The simulations for the proposed threshold modification was performed using the CTC+LDP encoder configuration. Objective performance was also evaluated using ALF=0. The summary results are presented below. Detailed results can be found in attached excel.

Version 2 of the document updates the objective results table below.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -1.52% | -1.07% | 72% | 80% | -0.11% | -2.33% | -1.44% | 99% | 103% |
| Class A2 | -0.02% | -0.35% | -0.32% | 80% | 84% | -0.02% | -0.36% | -0.26% | 104% | 104% |
| Class B | -0.02% | -0.58% | -0.55% | 101% | 99% | -0.02% | -0.57% | -0.54% | 99% | 103% |
| Class C | -0.01% | -0.38% | -0.48% | 102% | 101% | -0.01% | -0.30% | -0.42% | 100% | 101% |
| Class E | -0.08% | -0.62% | -0.68% | 104% | 101% | -0.08% | -0.69% | -0.82% | 100% | 103% |
| Overall | -0.03% | -0.66% | -0.60% | 92% | 94% | -0.04% | -0.79% | -0.66% | 100% | 103% |
| Class D | 0.00% | -0.20% | -0.20% | 104% | 102% | 0.00% | -0.15% | -0.14% | 103% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -2.19% | -2.04% | 83% | 92% | -0.37% | -3.11% | -2.68% | 100% | 103% |
| Class A2 | -0.08% | -0.93% | -0.88% | 80% | 86% | -0.14% | -0.90% | -0.79% | 102% | 104% |
| Class B | 0.01% | -1.09% | -1.17% | 98% | 102% | -0.03% | -1.16% | -1.22% | 101% | 103% |
| Class C | -0.01% | -0.55% | -0.66% | 99% | 102% | -0.01% | -0.56% | -0.61% | 100% | 102% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.07% | -1.13% | -1.15% | 91% | 97% | -0.12% | -1.34% | -1.26% | 101% | 103% |
| Class D | 0.00% | -0.52% | -0.31% | 99% | 102% | -0.01% | -0.44% | -0.39% | 100% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.07% | -1.85% | -2.15% | 91% | 95% | 0.01% | -1.71% | -2.34% | 101% | #NUM! |
| Class C | 0.02% | -0.82% | -1.04% | 101% | 101% | -0.02% | -0.78% | -1.06% | 100% | 101% |
| Class E | -0.07% | -1.83% | -2.05% | 102% | 103% | 0.04% | -2.13% | -2.00% | 101% | 103% |
| Overall | 0.02% | -1.50% | -1.76% | 97% | 99% | 0.01% | -1.50% | -1.83% | 101% | #NUM! |
| Class D | -0.01% | 0.05% | -0.98% | 102% | 102% | 0.03% | -0.66% | -0.68% | 100% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.08% | -1.72% | -1.76% | 90% | 95% | -0.14% | -2.35% | -2.08% | 98% | 103% |
| Class C | -0.04% | -0.95% | -1.26% | 101% | 102% | -0.05% | -0.71% | -1.01% | 102% | 102% |
| Class E | -0.05% | -2.12% | -1.55% | 104% | 101% | -0.35% | -2.36% | -2.57% | 103% | 103% |
| Overall | -0.06% | -1.56% | -1.54% | 97% | 99% | -0.16% | -1.81% | -1.85% | 101% | 103% |
| Class D | 0.07% | -0.81% | -1.09% | 101% | 103% | -0.03% | -0.81% | -0.93% | 98% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusions

We propose to adopt long filters and decisions for luma and long filters for chroma to combat visual artifacts for large blocks.  While we assert that the subjective performance of CE11.1.10 is high, we propose consideration of the describe threshold modification by the committee for improved objective performance

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Description of Core Experiment 11 (CE11): Deblocking”, JVET-K1031.

- “CE2: Deblocking Improvements for Large CUs (Test 2.1.7)”, JVET-K0315.

- “CE2: Long-tap deblocking filter for luma and chroma (CE2-2.1.6)”, JVET-K0152.

- “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010.

- “CE11: Combination of CE11.1.6 and CE11.1.7 (CE11.1.10)”, JVET-L0140