| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0542-v1 |
| --- | --- |

| Title: | Cross check of JVET-L0402 (CE7-related: Complexity reduction of significance map coding and bypass of greater than 4 flags) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Heiko Schwarz | Tel: Email: | +49 30 31002 226 heiko.schwarz@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The cross checker reports that the source code provided by C. Auyeung (Huawei) was carefully analysed and found to be consistent with the description in JVET-L0402. The cross checker could exactly reproduce the results reported in JVET-K0402. The measured encoder and decoder run time ratios are similar to those reported in JVET-K0402.

# Summary of Proposal JVET-L0402

The proposal JVET-L0402 contains the following two aspects:

- Modification of context index selection for sig_flag, par_flag, rem_abs_gt1_flag, rem_abs_gt2_flag:

- The current VVC draft specifies that the context indexes for the binary syntax elements sig_flag, par_flag, rem_abs_gt1_flag and rem_abs_gt2_flag depend on the absolute transform coefficient levels that are partially reconstructed after the first pass over the scan positions inside a subblock. The partially reconstructed absolute transform coefficient are given by

		absLevel1[ k ] = sig_flag[ k ] + par_flag[ k ] + 2 * rem_abs_gt1_flag[ k ]

At the encoder side, these partially reconstructed levels absLevel1[ ] can be obtained from the absolute levels absLevel[ ] by

		absLevel1[ k ] = Max( 4 – ( absLevel[ k ] & 1 ), absLevel[ k ] )

- In JVET-L0402, it is proposed to change the partially reconstructed levels absLevel1[ ], which are summed up (in a local template) for context index derivation to

		absLevel1[ k ] = Max( 3, absLevel[ k ] )

and

		absLevel1[ k ] = Max( 3, sig_flag[ k ] + par_flag[ k ] + 2 * rem_abs_gt1_flag[ k ] )

- The cross checker believes that the modification may reduce implementation complexity at the encoder side, but it might increase implementation complexity at the decoder side.

- Reduction of number of rem_abs_gt2_flag:

- It is proposed to code at maximum 2 rem_abs_gt2_flags in the second pass over the scan positions inside a subblock. The semantics of abs_remainder[ ] transmitted in the third pass over the scan positions of a subblock is modified accordingly.

- This aspect reduces the maximum number of context-coded bins per 4x4 subblock from 64 in VTM-2 to 50.

# Source Code Analysis

The source code provided by C. Auyeung /Huawei) was carefully analysed.  It is confirmed that the implementation matches the description in JVET-L0402.

It should be noted that, in the provided implementation, the modified context derivation is not taken into account in the encoder quantization process for dependent quantization.

# Simulation Results

Simulation results are conducted for the two test conditions specified in the CE7 description [3]:

- high complexity encoder configuration: Same as JVET CTC [2];

- low complexity encoder configuration: JVET CTC [2], but with both dependent quantization and RDOQ disabled and sign data hiding enabled.

Two configurations of the proposal were tested

- Version 1: Only the modified context derivation (1) was enabled;

- Version 2: Both the modified context derivation (1) and the reduction of the number of rem_abs_gt2_flags (2) were enabled.

Simulations were conducted for all configurations and test sequences specified in the CTC. The cross check results are provided in the accompanying Excel documents. The cross checker could exactly reproduce the results provided by the proponent and reported in JVET-K0402. The measured encoder and decoder run time ratios are similar to those reported in JVET-K0402.

# References

- C. Auyeung, J. Chen, “CE7-related: Complexity reduction of significance map coding and bypass of greater than 4 flags,” Joint Video Experts Team, doc. JVET-L0402, Macau, CN, Oct. 2018.

- F Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team, doc. JVET-K1010, Ljubljana, SI, July 2018.

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding,” Joint Video Experts Team, doc. JVET-K1027, Ljubljana, SI, July 2018.