| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0553 |
| --- | --- |

| Title: | Fix of Initial QP Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Xiaozhong Xu, Shan Liu Yiming Li, Zizheng Liu, Zhenzhong Chen | Tel: Email: | xlxiangli@tencent.com zzchen@whu.edu.cn |
| Source: | Tencent, Wuhan University |  |  |

_____________________________

# Abstract

Max QP was extended from 51 to 63 in Ljubljana meeting. However, the signaling of initial QP was not changed accordingly. Two fixes are proposed in this contribution. It is reported that the second fix is with less changes and more practical.

# Introduction

In Ljubljana meeting, JVET-K0251 proposed to extend the range of max QP. Finally, Max QP was agreed to extended from 51 to 63. However, in VTM-2.0.1 the signaling of initial QP in PPS is still kept the same as in HEVC, namely syntax init_qp_minus26 is signaled in PPS with following semantics.

init_qp_minus26 plus 26 specifies the initial value of SliceQpY for each slice referring to the PPS. The initial value of SliceQpY is modified at the slice segment layer when a non-zero value of slice_qp_delta is decoded. The value of init_qp_minus26 shall be in the range of −( 26 + QpBdOffsetY ) to +25, inclusive.

As maxQP has been changed to 63, changes shall be made acoordingly.

# Proposed fixes

Two fixes are proposed. The related changes are highlighted.

Fix#1

Replace syntax element init_qp_minus26 with init_qp_minus32 and revise related semantics as follows.

init_qp_minus32 plus 32 specifies the initial value of SliceQpY for each slice referring to the PPS. The initial value of SliceQpY is modified at the slice segment layer when a non-zero value of slice_qp_delta is decoded. The value of init_qp_minus32 shall be in the range of −( 32 + QpBdOffsetY ) to +31, inclusive.

Note that all semantics and decoding process related to init_qp_minus26 are needed to be revised acoordingly.

Fix#2

Keeping syntax element init_qp_minus26 but revise semantics as follows.

init_qp_minus26 plus 26 specifies the initial value of SliceQpY for each slice referring to the PPS. The initial value of SliceQpY is modified at the slice segment layer when a non-zero value of slice_qp_delta is decoded. The value of init_qp_minus26 shall be in the range of −( 26 + QpBdOffsetY ) to +37, inclusive.

Compared to Fix#1, Fix#2 is claimed better. First, one single change (on the range of init_qp_minus26) is needed by Fix#2 while much more changes are needed by Fix#1. More important, VVC is expected to have much less rate than HEVC given the same QP. With practical rate control, initial QP used for VVC is expected as no larger than that used for HEVC in most use cases. Consequently, using init_qp_minus32 would be less efficient than keeping init_qp_minus26.

# Conclusion

In this contribution, two fixes are proposed to address the issue with initial QP singling after extending max QP from 51 to 63. As Fix#2 is more simple and efficient, it is proposed to include Fix#2 in VTM and working draft.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Wuhan University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).