| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0565-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-L0411 (CE4-related: Angular merge prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh | Tel: Email: | +81-43-299-8526 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The cross checker reports that the source code provided by NHK was carefully analysed and found to be consistent with the description in JVET-L0411. The cross checker could reproduce the results reported in JVET-L0411. The difference from measured encoder and decoder run time ratios to those reported in JVET-L0411 is acceptable.

Summary of Proposal JVET-L0411

JVET-L0411 proposes a new merge mode which is called angular merge prediction. This is a sub-block motion compensation prediction, where the sub-CU MVs are extrapolated from the reconstructed blocks located in the specific direction. It is controlled by an additional syntax signaled at CU level

Simulation Results

Their software is based on BMS2.0.1 and JEM_TOOLS is off. At this cross-checking, a setting which is called “Fast Decision” was used. The anchor is VTM2.0.1.

The results were consistent with the results of fast decision of JVET-L0411. As a result of communicating with the proponent, we have understood that the difference from measured encoder and decoder run time ratios to those reported in JVET-L0411 is acceptable.

Table 1  Summary of results for SDR

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.01 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.08% | -0.22% | 109% | 102% |
| Class A2 | -0.10% | -0.23% | -0.32% | 111% | 101% |
| Class B | -0.09% | -0.08% | -0.19% | 111% | 102% |
| Class C | -0.06% | -0.17% | -0.17% | 112% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | -0.13% | -0.21% | 111% | 102% |
| Class D | 0.03% | -0.17% | 0.01% | 114% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.01 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | -0.34% | -0.34% | 116% | 102% |
| Class C | -0.05% | -0.34% | -0.27% | 115% | 102% |
| Class E | -0.18% | 0.03% | 0.11% | 117% | 100% |
| Overall | -0.10% | -0.25% | -0.20% | 116% | 101% |
| Class D | -0.04% | 0.24% | -0.72% | 116% | 102% |

References

- S. Iwamura, S. Nemoto, A. Ichigaya, CE4-related: Angular merge prediction, Joint Video Experts Team, doc. JVET-L0411, Macau, CN, Oct. 2018.

Patent rights declaration(s)

Sharp Corporation does not have any current or pending patent rights relating to the technology described in this contribution.