| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0614-v1 |
| --- | --- |

| Title: | CE11-related: Additional tests of CE 11.3.3 and 11.3.4 for 4 x N and N x 4 deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra, Semih Esenlik, Biao Wang, Han Gao, Zhijie Zhao, Jianle Chen | Tel: Email: | anand.meher.kotra@huawei.com jianle.chen@huawei.com |
|  | Chia-Ming Tsai, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | Huawei Technologies Co., Ltd., MediaTek Inc. |  |  |

_____________________________

# Abstract

In VTM2.0.1, deblocking is only considered on 8x8 grids. In this contribution, two approaches, Test1 and Test2, are proposed based on the subtest2 of JVET-L0225 and JVET-L0103. In Test1, for vertical edges, for the blocks whose widths are four samples, the proposed filter uses three samples in filtering decision and modifies one sample in filtering process. In Test2, for vertical edges, for the blocks whose widths are four samples, the proposed filter uses two samples in filtering decision and modifies one sample if weak filtering process is used and two samples if strong filtering process is used. Both in Test1 and Test2, for the blocks whose widths are larger than four samples, the numbers of modified samples in filtering process are the same as those in VTM2.0.1. Objective results for Test1 and Test2 are as follows:

Test1:
Over VTM2.0.1 with ALF off, luma BD-rates are 0.01% (AI), -0.26% (RA), -0.19% (LB), and -0.37% (LP) with negligible run-time changes.
Over VTM 2.0.1 with ALF on, luma BD-rates are 0.08% (AI), -0.12% (RA), -0.21% (LB), and -0.19% (LP) with negligible run-time changes.

Test2:
Over VTM2.0.1 with ALF off, luma BD-rates are 0.04% (AI), -0.22% (RA), -0.20% (LB), and -0.40% (LP) with negligible run-time changes.
Over VTM 2.0.1 with ALF on, luma BD-rates are 0.06% (AI), -0.12% (RA), -0.21% (LB), and -0.19% (LP) with negligible run-time changes.

# Introduction

In HEVC and VTM2.0.1 [1], deblocking filtering decisions are made separately for each boundary of four-sample length that lies on the grids dividing the picture into blocks of 8 x 8 samples. Only boundaries on the 8 x 8 grids, which are PU or TU boundaries, are deblocked. However, as binary-tree and ternary-tree partitioning types are used in VTM2.0.1, many 4 x N and N x 4 blocks are generated, and no deblocking is applied if these block boundaries are not on the 8x8 grids. In this contribution, two deblocking approaches Test1 and Test2 for 4 x N and N x 4 block boundaries are proposed.

Some definitions in this contribution are described as follows. The left side and right side of a vertical block edge are called p-side and q-side, respectively. The upper side and lower side of a horizontal block edges are called p-side and q-side, respectively. In addition, each block edge is of 4-sample length by definition, and pji and qji (i=0, 1, 2, or 3; and j=0, 1, 2, …) represent the p-side and q-side samples on the i-th line perpendicular to the block edge, j equal to 0 means the sample is adjacent to the block edge, and a larger j means the sample is at a larger distance from the block edge. The CB length refers to CB width and CB height for horizontal filtering across vertical block edge and vertical filtering across horizontal block edge, respectively.

# Proposed method

Two approaches, Test1 and Test2, are proposed to perform deblocking for 4 x N and N x 4 blocks.

## Test1

Test1 is proposed based on the subtest2 method of JVET-L0225 [2]. In the subtest2 method of JVET-L0225, when weak filtering is used, only one sample is changed at both boundary side if CB length of any side is four. In Test1, the modified samples can be more than one sample at the side with CB length larger than four (i.e., same as in VTM2.0.1). Detailed changes made to HEVC deblocking filters (DF) are listed in the following. Fig. 1 gives four vertical filtering examples of Test1.

- When HEVC filtering decision and filtering operations are performed, if the CB length at p-side is equal to four, then p2i is used to replace p3i.

- When HEVC filtering decision and filtering operations are performed, if the CB length at q-side is equal to four, then q2i is used to replace q3i.

- When HEVC weak or strong filtering is used and CB length at p-side is equal to four, then only samples p0i is modified.

- When HEVC weak or strong filtering is used and CB length at q-side is equal to four, then only samples q0i is modified.

- When HEVC weak or strong filtering is used, for the side with CB length is larger than 4, the modified samples can be more than one sample (i.e., same as in VTM2.0.1).

## Test2

Test2 is proposed based on JVET-L0103 [3]. In JVET-L0103, when CB length of any side is four, then only one sample at both side can be modified in weak filtering, and only two samples at both side can be modified in strong filtering. In Test2, the modified samples can be more than two samples at the side with CB length larger than four (i.e., same as in VTM2.0.1). Detailed changes made to HEVC DF are listed in the following. Fig. 2 gives four vertical filtering examples of Test2.

- When HEVC filtering decision and filtering operations are performed, if the CB length at p-side is equal to four, then p1i is used to replace p2i and p3i.

- When HEVC filtering decision and filtering operations are performed, if the CB length at q-side is equal to four, then q1i is used to replace q2i and q3i.

- When HEVC normal/weak filtering is used and CB length at p-side is equal to four, then sample p0i is modified. Similarly, if CB length at q-side is equal to four, then samples q0i is modified.

- When HEVC strong filtering is used and CB length at p-side is equal to four, then samples p1i and p0i are modified. Similarly, if CB length at q-side is equal to four, then samples q0i and q1i are modified.

- When HEVC weak or strong filtering is used, for the side with CB length is larger than 4, then no filtering sample number restrictions to the side (same as in VTM2.0.1).

Figure 1. Four vertical filtering examples in Test1. Each case shows three connected blocks with CB sizes are {8, 4, 4}, and the two block boundaries are vertical filtered by HEVC weak or strong filtering.

Figure 2. Four vertical filtering examples in Test2. Each case shows three connected blocks with CB sizes are {8, 4, 4}, and the two block boundaries are vertical filtered by HEVC weak or strong filtering.

# Complexity Evaluation

Complexity of the contribution is evaluated based on the following criteria described in JVET-K1031 [4].

- Number of samples from the block boundary that can be modified by deblocking (P+Q):

- For Test1:

  - For blocks size > 4 samples: it is the same as in VTM 2.0.1

  - For block size = 4 samples: one sample is modified

- For Test2:

  - For blocks size > 4 samples: it is the same as in VTM 2.0.1

  - For block size = 4 samples: one sample is modified if weak filtering is used, and two samples are modified if strong filtering is used

- Number of samples from the block boundary that are needed for deblocking decisions:

  - For blocks size > 4 samples: 4 samples are used (same as in VTM 2.0.1)

  - for block size = 4 samples

    - For Test1: three samples are used.

    - For Test2: two samples are only used.

- Number of filtering operations: The same as in VTM 2.0.1

- Number of line buffers: 4 lines (same as VTM 2.0.1)

- Parallelization capabilities: All edges overlapping with 4 x 4 grid can be deblocked in parallel.

- Luma only (Chroma filter is not changed)

# Experimental results

Two approaches, Test1 and Test2, for 4 x N and N x 4 block boundaries deblocking has been implemented on VTM2.0.1 [1], and the objective results are summarized in the Table 1 and Table 2. The simulations were performed following the JVET common test conditions [5].

# Conclusions

In this joint contribution, two extended approaches based on the subtest2 of JVET-L0225 and JVET-L0103 are proposed to support deblocking for all 4 x N and N x 4 block boundaries while keeping the feasibility of parallel deblocking all parallel block boundaries. The proposed DF leads to small BD-rate savings with negligible run-time changes. It is asserted that blocky artifacts are reduced by the proposed DF especially for low resolution sequences with many 4 x N and N x 4 blocks. We suggest to adopt the proposed deblocking technique for 4 x N and N x 4 blocks to VVC.

# References

- VTM2.0.1: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-2.0.1

- A.M. Kotra, S. Esenlik, B. Wang, H. Gao, Z. Zhao, J. Chen, “CE 11.3.3 Deblocking for 4 x N and N x 4 block boundaries,” Document of Joint Video Experts Team, JVET-L0225, Oct. 2018.

- C.-M. Tsai, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE11.3.4: Parallel deblocking for 4 x N and N x 4 block boundaries,” Document of Joint Video Experts Team, JVET-L0103, Oct. 2018.

- A. Norkin, A. M. Kotra, “Description of Core Experiment 11 (CE11): Deblocking”, Document of Joint Video Experts Team, JVET-K1031, July 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-K1010, July 2018.

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Objective results of Test1 on top of VTM2.0.1 for CTC with ALF switched off and for CTC configurations

|  | All Intra Main10 |  |  |  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM2.0.1 (ALF off) |  |  |  |  | Over VTM2.0.1 (ALF on) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.01% | 0.00% | 100% | 100% | 0.07% | 0.00% | -0.01% | 100% | 101% |
| Class A2 | 0.02% | 0.00% | 0.00% | 100% | 99% | 0.09% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.04% | 0.00% | 0.00% | 100% | 101% | 0.11% | -0.01% | -0.01% | 100% | 101% |
| Class C | 0.04% | 0.00% | 0.00% | 100% | 100% | 0.09% | 0.00% | -0.01% | 100% | 101% |
| Class E | -0.13% | -0.01% | 0.00% | 100% | 101% | 0.02% | -0.01% | -0.01% | 100% | 100% |
| Overall | 0.01% | 0.00% | 0.00% | 100% | 100% | 0.08% | 0.00% | -0.01% | 100% | 101% |
| Class D | 0.02% | -0.01% | 0.00% | 100% | 100% | 0.08% | -0.02% | -0.01% | 100% | 101% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
|  | Over VTM2.0.1 (ALF off) |  |  |  |  | Over VTM2.0.1 (ALF on) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.02% | -0.02% | 100% | 98% | -0.06% | 0.07% | -0.02% | 100% | 99% |
| Class A2 | -0.45% | -0.01% | -0.08% | 100% | 101% | -0.17% | -0.03% | -0.13% | 100% | 97% |
| Class B | -0.22% | -0.07% | -0.09% | 100% | 99% | -0.09% | -0.03% | -0.03% | 100% | 99% |
| Class C | -0.25% | -0.01% | -0.12% | 100% | 100% | -0.18% | 0.08% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.26% | -0.03% | -0.08% | 100% | 99% | -0.12% | 0.02% | -0.05% | 100% | 99% |
| Class D | -0.23% | -0.03% | -0.04% | 100% | 101% | -0.13% | -0.13% | -0.03% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Over VTM2.0.1 (ALF off) |  |  |  |  | Over VTM2.0.1 (ALF on) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.19% | -0.10% | -0.24% | 100% | 98% | -0.07% | -0.07% | -0.15% | 100% | 97% |
| Class C | -0.25% | -0.26% | -0.04% | 100% | 99% | -0.24% | 0.13% | -0.11% | 100% | 100% |
| Class E | -0.14% | 0.22% | 0.53% | 100% | 99% | -0.39% | -0.18% | -0.20% | 100% | 100% |
| Overall | -0.19% | -0.07% | 0.02% | 100% | 98% | -0.21% | -0.03% | -0.15% | 100% | 99% |
| Class D | -0.19% | -0.07% | -0.33% | 100% | 100% | -0.11% | 0.25% | -0.42% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  | Low delay P Main10 |  |  |  |  |
|  | Over VTM2.0.1 (ALF off) |  |  |  |  | Over VTM2.0.1 (ALF on) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.35% | -0.14% | -0.13% | 100% | 98% | -0.12% | 0.04% | 0.04% | 100% | 97% |
| Class C | -0.34% | 0.15% | -0.14% | 100% | 100% | -0.19% | 0.20% | -0.19% | 100% | 100% |
| Class E | -0.45% | 0.27% | -0.26% | 100% | 100% | -0.29% | -0.18% | -0.07% | 100% | 99% |
| Overall | -0.37% | 0.06% | -0.17% | 100% | 99% | -0.19% | 0.04% | -0.06% | 100% | 99% |
| Class D | -0.30% | -0.43% | -0.07% | 100% | 101% | -0.11% | 0.02% | -0.11% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 2.  Objective results of Test2 on top of VTM2.0.1 for CTC with ALF switched off and for CTC configurations

|  | All Intra Main10 |  |  |  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM2.0.1 (ALF off) |  |  |  |  | Over VTM2.0.1 (ALF on) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.01% | 0.00% | 100% | 101% | 0.09% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.06% | 0.00% | -0.01% | 100% | 101% | 0.08% | 0.00% | -0.01% | 100% | 100% |
| Class B | 0.08% | 0.00% | -0.01% | 100% | 101% | 0.10% | 0.00% | -0.01% | 100% | 101% |
| Class C | 0.04% | 0.00% | 0.00% | 100% | 102% | 0.06% | 0.00% | 0.00% | 100% | 101% |
| Class E | -0.10% | -0.01% | 0.00% | 101% | 101% | -0.05% | -0.01% | 0.00% | 100% | 101% |
| Overall | 0.04% | 0.00% | 0.00% | 100% | 101% | 0.06% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.04% | -0.01% | 0.00% | 100% | 101% | 0.07% | -0.01% | -0.01% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
|  | Over VTM2.0.1 (ALF off) |  |  |  |  | Over VTM2.0.1 (ALF on) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.01% | -0.13% | 100% | 98% | -0.05% | 0.08% | 0.01% | 100% | 101% |
| Class A2 | -0.40% | -0.07% | -0.06% | 100% | 100% | -0.18% | 0.01% | -0.04% | 100% | 96% |
| Class B | -0.17% | -0.05% | -0.02% | 100% | 99% | -0.07% | -0.03% | -0.11% | 100% | 98% |
| Class C | -0.21% | 0.01% | -0.10% | 100% | 97% | -0.19% | 0.00% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.22% | -0.03% | -0.07% | 100% | 98% | -0.12% | 0.01% | -0.05% | 100% | 99% |
| Class D | -0.17% | -0.07% | -0.05% | 100% | 101% | -0.16% | -0.05% | -0.07% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Over VTM2.0.1 (ALF off) |  |  |  |  | Over VTM2.0.1 (ALF on) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.17% | 0.13% | -0.09% | 100 | 100 | -0.07% | -0.05% | -0.02% | 100% | 98% |
| Class C | -0.25% | -0.25% | 0.01% | 100% | 96% | -0.15% | -0.15% | -0.08% | 100% | 101% |
| Class E | -0.18% | -0.51% | 0.32% | 100% | 96% | -0.50% | -0.32% | -0.22% | 100% | 99% |
| Overall | -0.20% | -0.16% | 0.05% | 100% | 97% | -0.21% | -0.15% | -0.09% | 100% | 99% |
| Class D | -0.15% | -0.55% | -0.34% | 100% | 100% | -0.13% | 0.62% | -0.19% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  | Low delay P Main10 |  |  |  |  |
|  | Over VTM2.0.1 (ALF off) |  |  |  |  | Over VTM2.0.1 (ALF on) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.31% | -0.12% | -0.01% | 99% | 99% | -0.11% | 0.14% | 0.09% | 100% | 98% |
| Class C | -0.35% | 0.12% | -0.25% | 99% | 96% | -0.21% | -0.05% | 0.07% | 100% | 101% |
| Class E | -0.60% | -0.03% | -0.52% | 100% | 97% | -0.27% | -0.49% | 0.27% | 100% | 99% |
| Overall | -0.40% | -0.02% | -0.22% | 99% | 97% | -0.19% | -0.08% | 0.13% | 100% | 99% |
| Class D | -0.37% | 0.06% | -0.34% | 100% | 101% | -0.12% | -0.07% | -0.20% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |