| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0621 |
| --- | --- |

| Title: | Cross-check of JVET-L0226 (CE11-related: Position dependent adaptive Tc clipping range for deblocking filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Patrice Onno Guillaume Laroche Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 patrice.onno@crf.caon.fr guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-L0226 on position dependent adaptive Tc clipping range for deblocking filter. This report confirms the results provided by the authors of JVET-L0226.

# Summary of JVET-L0226

The current deblocking filter in VTM 2.0.1 uses a fixed clipping range, where all the samples after filtering are clipped to the following range of values [P – 2tc, P + 2tc].

In JVET-L0226, the proposed technique defines an adaptive Tc clipping range as a “monotonically decreasing function” where the clipping range is based on the distance of the sample from the edge. It means that closer the sample is to the edge, larger is the clipping range allowed and therefore larger is the deviation allowed after filtering.

Two different “monotonically decreasing functions” are implemented on top of VTM 2.0.1.

The first monotonic function is defined as follows: [Pi – (tc + (tc >> i)), Pi + (tc + (tc >> i))] where “i" is the sample distance from the edge.

The second monotonic function is defined as follows: [Pi – (2tc >> i), Pi + (2tc >> i)] where “i” is the sample distance from the edge.

Additionally, JVET-L0226 proposes a combination of this clipping range modification with the longer tap filter proposed in JVET-K0369 at the last JVET meeting.

# Verification of source code

The proposed modifications are implemented on top of VTM-2.0.1 as recommended by CE11 and they correspond to the description provided in contribution JVET-L0226

# Experimental results

The experimental results have been performed according to the testing conditions recommended by CE11 except that class F has not been performed by the proponent of JVET-L0226. The experimental results have been also performed when deactivating ALF.

“Test 1” corresponds to the first monotonic function.

“Test 2” corresponds to the second monotonic function

“Test 3” corresponds to “Test 1” with combination with JVET-K0369.

“Test 4” corresponds to “Test 2” with combination with JVET-K0369.

The full results are included in the 8 attached Excel files.

The results match exactly with those provided by the proponent for the 4 different macro settings and when activating ALF or not.

# Conclusion

The results obtained in this crosscheck match the results presented in JVET-L0226.