| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0623 |
| --- | --- |

| Title: | Crosscheck of JVET-L0120 "CE4-related: Low pipeline latency LIC" |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vasily Rufitskiy, Alexey Filippov, Jianle Chen | Tel: Email: | +7 925 008 17 96 {vasily.rufitskiy, alexey.filippov, jianle.chen} @huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

The document is a cross-check of the results reported in the proposal JVET-L0120: "CE4-related: Low pipeline latency LIC" [1]. The crosschecked implementation is on top of the BMS-2.0.1 (with bms configuration file) for the Random Access and Low-Delay B configurations. Results reported by the proponent matches closely with the ones obtained by the crosschecker.

# Summary of JVET-L0120

The input document [1] of the crosschecked proposal gives the following abstract:

“This contribution proposes LIC that can solve the pipeline latency issue. Proposed LIC is based on the existing LIC, but it removes all encoder and decoder LIC processes other than the reconstruction stage, and apply LIC process after DMVR refined MC. According to this modification, feedback loop of neighbouring image reference in hardware pipeline can be closed to the reconstruction stage only. Additionally, it is proposed to modify bS calculation of DBF for LIC boundary. Simulation results reportedly show that the proposed method provides 0.75% BD-rate gain for RA, 0.90% BD-rate gain for LDB using the VTM configurations.”

# Simulation results

Simulations results reported in this contribution are based on the BMS2.0.1 software (with bms configuration file) used as anchor. The cross-check reports a match of the results reported by the proponents with the results obtained by the crosschecker (see tables below).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS2.0.1 with bms config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.04% | -0.13% | -0.64% | 149% | 99% |
| Class A2 | -0.52% | -0.03% | -0.33% | 146% | 100% |
| Class B | -0.89% | -0.33% | -0.16% | 141% | 99% |
| Class C | -0.53% | -0.52% | -0.26% | 137% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.75% | -0.28% | -0.31% | 142% | 100% |
| Class D | 0.00% | 0.31% | -0.05% | 138% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS2.0.1 with bms config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.17% | -0.80% | -0.59% | 161% | 100% |
| Class C | -0.82% | -0.68% | -0.53% | 158% | 100% |
| Class E | -0.58% | -0.69% | -0.27% | 189% | 101% |
| Overall | -0.90% | -0.73% | -0.49% | 167% | 100% |
| Class D | -0.13% | -0.55% | -0.37% | 164% | 101% |

# References

[1] K. Abe, T. Toma, J. Li, JVET-L0120 “CE4-related: Low pipeline latency LIC”, the 12th JVET meeting, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd does not have any current or pending patent rights relating to the technology described in this contribution.