| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0632-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-L0142 (CE4: Simplification of the common base for affine merge (Test 4.2.6)) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report of JVET-L0142 (CE4: Simplification of the common base for affine merge (Test 4.2.6)). The simulations were carried out based on the common test condition (CTC). The test results are matched for all points and the software matches the description in the proposal.

A modification on top of CE4.2.6.d is conducted by change the positions of neighboring blocks for inherited candidate. This modification provides -0.02% BD-rate gain on top of CE4.2.6.d.

Introduction

JVET-L0142 reports the results of simplification of the common base for affine merge as follows:

- Test 4.2.6.a: Simplified inherited candidate

- Test 4.2.6.b: Simplified constructed candidate

- Test 4.2.6.c: a) + b)

- Test 4.2.6.d: c) + reduced candidate number of candidate

- Test 4.2.6.e: d) + line buffer restriction

- Test 4.2.6.f: d) + Test 4.1.2.d

- Test 4.2.6.g: e) + Test 4.1.2.e

Simplified inherited candidate

In this method, the maximum two inherited affine candidates are derived from affine motion model of the neighboring blocks and inserted into the candidate list. For the left predictor, the scan order is: {A, D, E} and for the above predictor, the scan order is: {B, C} as shown in Figure 1. Each predictor selects the only one inherited candidate. No pruning check in performed between two inherited candidates.

Figure 1: Spatial positions for inherited candidate

Simplified constructed candidate

In the common base for affine merge, the available combination of motion information of CPs is only considered when the CPs have the same reference index.

Reduced candidate number of candidate

In the available constructed candidates, the following candidates are not considered:

{CP2, CP3}, {CP1, CP4}, {CP2, CP4}, {CP3, CP4}

Line buffer restriction

In this method, to avoid the line buffer storage increase, inherited candidate is restricted to derive from the above block with CTU boundary.

Crosscheck results

Table 1: Test specification of Test 4.2.6

| Test # | Test Description |
| --- | --- |
| 4.2.6.a | Simplified inherited candidate |
| 4.2.6.b | Simplified constructed candidate |
| 4.2.6.c | a) + b) |
| 4.2.6.d | c) + reduced candidate number of candidate |
| 4.2.6.e | d) + line buffer restriction |
| 4.2.6.f | d) + Test 4.1.2.d |
| 4.2.6.g | e) + Test 4.1.2.e |

Table 2: Test results of 4.2.6.a.

Table 3: Test results of 4.2.6.b.

Table 4: Test results of 4.2.6.c.

Table 5: Test results of 4.2.6.d.

Table 6: Test results of 4.2.6.e.

Table 7: Test results of 4.2.6.f.

Table 8: Test results of 4.2.6.g.

Modification on top of CE4.2.6.d

A modification on top of CE4.2.6.d is conducted by change the positions of neighboring blocks for inherited candidate. As shown in Figure 2, for the left predictor, the scan order is changed from {A1, A0, A2} to {A0, A1}; for the above predictor, the scan order is changed from {B1, B0} to {B0, B1, B2}. After this change, the inherited candidate for affine merge list construction is aligned with affine AMVP list construction in CE4.1.6 [2]. This modification provides -0.02% BD-rate gain on top pf CE4.2.6.d.

Figure2: Spatial positions for inherited candidate

Table 9: Test results of 4.2.6.d with modification

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.90% | -0.53% | -0.84% | 104% | 107% |
| Class A2 | -0.89% | -0.89% | -1.07% | 105% | 108% |
| Class B | -0.60% | -0.43% | -0.58% | 104% | 103% |
| Class C | -0.38% | -0.29% | -0.37% | 104% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.66% | -0.50% | -0.68% | 104% | 104% |
| Class D | -0.50% | -0.52% | -0.52% | 105% | 106% |
| Class F (optional) | -0.13% | -0.10% | -0.18% | 102% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.15% | -0.14% | -0.24% | 104% | 103% |
| Class E | -0.34% | -0.59% | -0.39% | 106% | 104% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.15% | 0.41% | -0.08% | 104% | 101% |
| Class F (optional) | -0.07% | -0.29% | -0.32% | 103% | 99% |

References

- J. Lee, J. Nam, N. Park, H. Jang, J. Lim, S. Kim, “CE4: Simplification of the common base for affine merge (Test 4.2.6)”, JVET-L0142, Oct. 2018, Macao, CN.

- H. Huang, W.-J. Chien, Y. Han, Y. Zhang, M. Karczewicz, “CE4.1.6: Simplification of affine AMVP candidate list construction”, JVET-L0142, Oct. 2018, Macao, CN.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).