| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 03–12 Oct. 2018 | Document: JVET-L0688-v1 |
| --- | --- |

| Title: | BoG report on CE8 and CE15 related contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun Xiaozhong Xu | Email: | yuchen.s@alibaba-inc.com xiaozhongxu@tencent.com |
| Source: | BoG |  |  |

_____________________________

- Abstract

Mandates according to the meeting notes:

- To review the contributions from 7.8, 7.15, 7.17, and recommend items to be investigated in the upcoming CE8 and CE15

- To access memory requirements of current-CTU CPR

CE8 related contributions

JVET-L0041 Non-CE8: Rotate Intra Block Copy [Z. Zhang, V. Sze (MIT)]

Proponent didn’t attand the BoG. No presentation.

JVET-L0159 Non-CE8: Block vector predictor for CPR [J. Nam, J. Lim, S. Kim (LGE)]

In merge and AMVP modes, candidate list is first filled from spatial and temporal candidates and zero candidates are then inserted to candidate list until candidate list includes maximum candidates. However, in current picture referencing, block vector for intra block copy cannot become zero. Therefore this contribution proposes, instead of zero candidates, two alternative candidates (-2*width, 0) and (0, -2*height) for block vector prediction candidate for intra block copy (IBC) are introduced. Experimental results reportedly show 0.11%, 0.61%, and 1.18% BD-rate savings for CTC, classF, and SCC over VTM-2.0.1 with CPR under AI configuration.

Q: Do you try other position? A: Yes, the proposed is the best position.

Q: Do you consider CPR restriction? A: full range search is performed in the proposal.

It is commented that it might be interesting to have more results of different positions.

It is commented that there was studied during HEVC, the results showed that the best position might not be the nearest one.

The BoG recommended to study in the next CE.

JVET-L0472 Cross Check report of JVET-L0159: Non-CE8: Block vector predictor for CPR [X. Xu (Tencent)] [late] [miss]

JVET-L0297 CE8-related: CPR mode with local search range optimization [X. Xu, X. Li, S. Liu (Tencent), E. Chai (Ubilinx)]

In this contribution, the effective search range for CPR mode with local reference constraints is increased, while it is asserted that the required memory to store reconstructed samples for CPR referencing is kept unchanged. Compared to the constraint with only 1 CTU size of reference area in CE8 test (CE8.3.1b), the proposed method can achieve higher coding efficiency. The reported BD rate changes from the proposed method (reuse the memory buffer on 64x64 basis) relative to VTM-2.0.1 are as follows:

- In AI, -0.27%/-13.61%/-41.31% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.14%/-11.13%/-25.50% for CTC/Class F/SCC 1080p classes, separately.

- In LB, 0.02%/-6.07%/-15.71% for CTC/Class F/SCC 1080p classes, separately.

The proposal proposed to reuse the reference sample memory on 64x64 basis. Coding performance improvements of the proposed method are reported on top of VTM-2.0.1 and CE8-3-1b (1CTU, no chroma interpolation).

The availability check of reference samples from left CTU is performed on 64x64 basis.

Q: why does decoding time decrease?  A: time information is not accurate.

It is commented that the search range is irregular from encoder perspective; the starting points of encoding search might different.

It is also commented that the search is the same to the current CE design.

The BoG recommended to study in the next CE.

JVET-L0518 Cross check of JVET-L0297 (CE8-related: CPR mode with local search range optimization) [G. Venugopal (HHI)] [late] [miss]

JVET-L0299 CE8-related: CPR mode with merge mode improvements [X. Xu, X. Li, M. Gao, J. Ye, S. Liu (Tencent)]

In this contribution, the CPR mode is combined with some improvements from merge mode, i.e., non-adjacent and history-based merge candidates (CE4.4.2, CE4.4.7). Combination of the two are tested.

With the improved merge mode CE4.4.2, the reported BD rate changes from the base method (BMS-CPR) are as follows:

- In AI, -0.09%/-0.33%/-1.71% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.58%/-0.61%/-2.06% for CTC/Class F/SCC 1080p classes, separately.

- In LB, -0.37%/-0.63%/-1.93% for CTC/Class F/SCC 1080p classes, separately.

With the improved merge mode CE4.4.2, the reported BD rate changes from the base method (CPR with 1 CTU search range, CE8.3.1b) are as follows:

- In AI, -0.06%/-0.21%/-0.95% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.57%/-0.56%/-1.69% for CTC/Class F/SCC 1080p classes, separately.

- In LB, -0.43%/-0.50%/-2.12% for CTC/Class F/SCC 1080p classes, separately.

With the improved merge mode CE4.4.2+CE4.4.7, the reported BD rate changes from the base method (BMS-CPR) are as follows:

- In AI, -0.23%/-0.78%/-2.69% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.87%/-1.14%/-2.99% for CTC/Class F/SCC 1080p classes, separately.

- In LB, -0.52%/-0.94%/-2.87% for CTC/Class F/SCC 1080p classes, separately.

With the improved merge mode CE4.4.2+CE4.4.7, the reported BD rate changes from the base method (CPR with 1 CTU search range, CE8.3.1b) are as follows:

- In AI, -0.16%/-0.57%/-1.83% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.84%/-1.00%/-2.56% for CTC/Class F/SCC 1080p classes, separately.

- In LB, -0.56%/-0.94%/-2.74% for CTC/Class F/SCC 1080p classes, separately.

The proposal provided the information showing that the CPR mode combined the merge mode improvement adopted in this meeting has additional gain.

The proponent suggested that if the CE4.4.7 (adopted merge improvement) is applied to CPR, the addional gain is expected to be similar as the one tested using CE4.4.2 in this contribution.

JVET-L0626 Cross-check of JVET-L0299 [?? (??)] [late] [miss]

JVET-L0404 CE8-related: Restrictions for the search area of the CPR blocks in CPR [L. Pham Van, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm)]

This contribution proposes the restrictions to the search area for IBC blocks. The search area is restricted to the current CTU and few CTUs to the left of the current CTU. Additionally, this proposal also proposes to extend the search area to 10 lines above these CTUs. It is reported that with the proposed restricted search area, the Luma BD-rate changes for [CTC, Class F, class SCC 1080p] over BMS2.1-VTM configuration are:

The current and 1 left CTU without and with extended lines, respectively:

- AI: (-0.29%, -15.55%, -46.10%) and (-0.32%, -15.79%, -46.99%).

- RA: (-0.10%, -12.67%, -29.42%) and (-0.11%, -12.88%, -30.17%).

- LB: (0.02%, -7.19%, -19.70%) and (-0.32%, -7.38%, -20.18%).

The current and 2 left CTUs without and with extended lines, respectively:

- AI: (-0.29%, -15.76%, -46.68%) and (-0.32%, -15.98%, -47.56%).

- RA: (-0.10%, -12.84%, -30.23%) and (-0.11%, -13.06%, -30.99%).

- LB: (0.02%, -7.30%, -20.39%) and (0.02%, -7.62%, -20.96%).

The current and 3 left CTUs without and with extended lines, respectively:

- AI: (-0.29, -15.84%, -46.98%) and (-0.32%, -16.06%, -47.85%).

- RA: (-0.10, -12.91%, -30.63%) and (-0.11%, -13.14%, -31.35%).

- LB: (0.02%, -7.52%, -20.94%) and (0.02, -7.67%, -21.52%).

Based on CPR with local search range (current CTU + x left CTUs), this proposal proposes to extend the search area to N lines above these CTUs. N is set to be 10 in the proposal, but the proposal also proposed that the number of N should be set accoding to the VVC design. Coding performance using these extra N lines is reported.

It is commented that the reference pixels in N lines are before deblocking stage. It was suggestesd to test N=4 in the next CE.

It is commented that there are different approaches: (1) extend reference to the left CTU (2) top N line.

It is commented that multiple tools can share the same additional required resource.

It is commented that it is better that the required line memory could be in the same memory as the one to store the reference samples in the current and left CTUs.

The BoG recommended to study in the next CE.

CE15 related contributions

JVET-L0213 CE15-related: Combination of palette mode and intra prediction [Y.-C. Sun, J. An, J. Lou (Alibaba)]

This document proposes a method combining palette mode and intra prediction. On top of CE15.1, for Class F sequences, in AI/RA/LDB configuration, the results show -0.2%/-0.2%/-0.1% BD-rate luma gain; for 4:2:0 TGM sequences in CE15, the results show -0.7%/-0.4%/-0.1% BD-rate luma gain.

In current palette mode, the block is entirely predicted without any other prediction method. The contribution proposes to combine the predictors from palette mode and intra prediction mode.

- For a decoded index being zero, the pixel is predicted from intra prediction; intra prediction mode needs to be signaled.

- For an index being non-zero, the pixel is predicted using a palette color.

- No residue processing is proposed.

Some gain observed for SCC content. The CE15.1 is the base palette software to apply the proposed method.

It was asked if the results are based on CPR mode enabled. It was answered that CPR mode is not used in the reported test.

It is commented that intra 4x4 is the critical path. For the decoding of the combined mode, palette decoding also needs to finish processing within the same number of the cycles for intra 4x4.

It is noticed that some runtime increase is observed.

It is commented that all CE tests should be based on the same base software.

The BoG recommended to study in the next CE.

JVET-L0574 Crosscheck of JVET-L0213 (CE15-related: Combination of palette mode and intra prediction) [C.-M. Tsai (MediaTek)] [late] [miss]

JVET-L0307 CE15-related: Palette index map scan order constraints [J. Ye, X. Li, S. Liu, X. Xu (Tencent)]

In this contribution, the constraints on palette mode index map scan order are proposed. In VTM 2.0.1, there are non-square coding blocks. Some coding blocks have large height/width or width/height ratios. For these blocks, it is proposed to use one scan order. The proposed method reports negligible loss compared to CE15.2 and reduced complexity.

Q: is it separated palette? A: separated for intra/inter slices.

Q: is other shape restriction tried? A: if the threshold is reduced, loss is observed.

It is commented that decoder depends on CU size.

It is also commented that the information is already ready, the impact on parsing dependency is not critical.

The BoG recommended to study in the next CE.

JVET-L0556 Crosscheck of JVET-L0307 on CE15-related: Palette index map scan order constraints [J. Nam (LGE)]

JVET-L0308 CE15-related: Palette mode when dual-tree is enabled [J. Ye, X. Li, S. Liu, X. Xu (Tencent)]

In this contribution, the modification on palette mode when dual tree is enabled is proposed. In VTM 2.0.1, for I slice, the dual tree is enabled, and luma CTU and chroma CTU have different tree structure. In the HEVC SCC palette mode, the luma sample and chroma sample are interleaved coded. In this case, the HEVC SCC palette mode cannot directly be applied on VTM 2.0.1. In this contribution, two methods are proposed to solve this issue. The proposed method 1 reports -11.64%, -7.75%, -7.55% luma and chroma BD rate changing for AI configuration on class F over VTM 2.0.1. And reports -9.00%, -7.07%, -7.14% luma and chroma BD rate changing for RA configuration on class F over VTM 2.0.1. The proposed method 2 reports -11.60%,	-8.56%, -8.35% luma and chroma BD rate changing for AI configuration on class F over VTM 2.0.1. And reports -8.88%,	-7.75%, -8.12% luma and chroma BD rate changing for RA configuration on class F over VTM 2.0.1.

Two methods are proposed for palette mode when dual tree is enabled:

Method 1: only apply palette mode to luma plane when dual tree is enabled. In this case, only one palette table and one index map for luma CU will be signaled. When coding chroma plane separately, palette mode will not be used. The other available modes, such as intra chroma mode coding, will be used.

Method 2: apply palette mode to luma plane when dual tree is enabled. When coding chroma plane, if the co-located luma blocks are all coded in palette mode, the chroma block are also have the flexibility to use palette mode. A flag is signaled whether the chroma block using palette mode or not. If the chroma block using palette mode, the corresponding palette mode syntax will be signaled.

It is reported that some loss is observed in Method 1, and the loss is less in Method 2.

In Method 2, there is no syntax for the chroma CU if any of collocated luma CU is not coded in palette mode.

It is commented that the chroma CU could be set to palette mode without syntax if the collocated luma CUs are coded by the palette mode.

The BoG recommended to study in the next CE.

JVET-L0526 Crosscheck report of L0308 (CE15-related: Palette mode when dual-tree is enabled) [B. Choi (Sharp)] [late]

JVET-L0427 CE15-related: Separate Palette Coding for Luma and Chroma Components [R. Chernyak, S. Ikonin, J. Chen (Huawei)]

The contribution proposes a unification of Palette Coding tool for Single and Dual Trees. For screen content sequences it demonstrates 9.0% / 3.9% / 1.3% (AI / RA / LDB) on class TGM and 4.0% / 3.3% / 2.4% (AI / RA / LDB) on class F of Luma BD-rate saving on top of CE15.2, if Dual Tree is disabled.

In the current CTC, the palette is separated when the dual tree is enabled in SPS. The proposal proposed to use separate palette when the dual tree is disabled in SPS.

It is reported that coding performance benefit can be observed in non-CTC case by enabling separate palette for luma/chroma as compared to joint palette.

It is commented that joint palette requires sending one set of palette information, while separate palettes for luma/chroma maintain two sets of palette information. There is an increase of the complexity.

It is questioned by an expert that for CTC setting, if joint or separate palette should be used. It was requested that both should be evaluated.

The BoG recommended to further discuss this issue on track.

JVET-L0612 Crosscheck of JVET-L0427: CE-15 related: Separate Palette Coding for Luma and Chroma Components [S. Bandyopadhyay (InterDigital)] [late]

JVET-L0451 CE15-related: Palette predictor list enhancement [J. Ye, X. Li, X. Xu, S. Liu (Tencent)] [late]

In this contribution, it is proposed to derive the spatial palette predictor from the spatial neighboring blocks’ palette table first. Then combine the spatial palette predictor with HEVC SCC palette predictor. The proposed method reports -0.14%,	-0.04%, and -0.04% luma and chroma BD rate changing for AI configuration on class SCC over CE 15.2.

Q: does the proposal consider CTU boundary? A: Yes.

The crosschecker commented that it is interesting to investigate the correlation between the current palette and non-adjectent palette. It is good to study this method in the CE.

The BoG recommended to study in the next CE.

JVET-L0550 Crosscheck of JVET-L0451 (CE15-related: Palette predictor list enhancement) [Y.-C. Sun (Alibaba)] [late]

JVET-L0672 CE15-related: Separated palette test on top of CE15.2 [?? (??)] [late]

This document reports the results of joint and separated palettes for luma and chroma CUs in inter slice when dual tree is enabled in SPS. The test is performed on top of CE15.2 (L0336). The results show that:

- For joint palette, 8.6%, and 4.3% BD-rate gains for luma are shown for class F sequences and 15.6%, and 6.5% BD-rate gains for luma are shown for 4:2:0 TGM sequences under RA, and LD configurations, respectively.

For separated palettes, 8.7%, and 5.2% BD-rate gains for luma are shown for class F sequences and 16.4%, and 8.6% BD-rate gains for luma are shown for 4:2:0 TGM sequences under RA, and LD configurations, respectively.

The proposal showed the information that the separated palettes shows about 1% gain than the joint palette.

Screen content tools (2)

JVET-L0078 AHG11: Block DPCM for Screen Content Coding [M. Abdoli, G. Clare, F. Henry, P. Philippe (Orange)]

This contribution proposes to use DPCM at the block level to encode Screen Content. The proposed Block-DPCM uses the MED predictor of the JPEG-LS (LOCO-I) algorithm. The prediction error is quantized in the spatial domain and encoded using unary binarization of amplitude and bypass coding of signs. This contribution reports the following compression performance changes for the different tested categories (a negative value indicating an overall gain for Block-DPCM):

Class A to E, All Intra: -0.1% (EncT 103%, DecT 99%)

Class F, All Intra: -7.0% (EncT 103%, DecT 89%)

TGM, All Intra: -21.0% (EncT 105%, DecT 73%)

Class A to E, Random Access: 0.0% (EncT 100%, DecT 100%)

Class F, Random Access: -5.1% (EncT 102%, DecT 96%)

TGM, Random Access: -9.8% (EncT 102%, DecT 92%)

The methods were also tested when combining CE8 CPR and CE15 palette mode, separately.

On top of BMS-CPR, additional 1.1% gain is reported; On top of palette (CE15 test), additional 1.2% gain is reported, both in AI config. It is reported that there is small gain in Class C & D.

It is commented that the CPR with restriction should be tested.

Q: is SIMD used in the test? A: No.

It is commented that the method can be processed in parallel alone diagonal lines

It is also commented that there is still impact on thoughtput.

It is commented that, line-based intra-prediction method also showed gain in screen content.

The BoG recommended to study in the next CE. (which CE?).

JVET-L0481 Crosscheck of JVET-L0078 (AHG11: Block DPCM for Screen Content Coding) [C.-Y. Chen (MediaTek)] [late] [miss]

Memory requirement accessment of current-CTU CPR

It is reported and confirmed by experts that CPR memory usage with current CTU restriction is calculated as 22.5KB (=3*(64*64 +2*32*32) *10)/(1024*8) )

Others:

It is commented that it is better to have another name for Screen Content (as it also includes gaming and other content that reflects the market). Experts also commented that Screen Content is actually a good name.

Conclusions

The BoG reviewed the all assigned proposals and futher confirmed the calculation of the memory usage of CPR with 1-CTU (current CTU) restriction to be correct.

The BoG recommend:

- to futher study 8 tests in the next CE.

- to update Class F with more representive materials.

- to use CPR with current CTU restriction (CE8.3.1b) as CPR anchor in the next CE

- to use CE palette of CE15.2 as the palette anchor in the next CE pending on further discussion as to which palette design to be used (the joint palette or separated palette).