| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0693 |
| --- | --- |

| Title: | BoG on CE9-related: decoder-side motion vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | X. Xiu | Email: | xiaoyu.xiu@interdigital.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG on CE9-related: decoder-side motion vector derivation met on October 7, 2018 from 6:00PM to 8:00PM in room 1006. It reviewed input CE9-related contributions on decoder-side motion vector derivation.

# BoG Report

## Decoder-side motion vector refinement

JVET-L0098 CE9-related: Simplified DMVR with reduced internal memory [C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

The method was discussed during the CE9 discussion. The decision was to disable the DMVR for the coding block sizes with either width of height of 128. The method was decided to be included in the CE9 DMVR base software.

JVET-L0174 Non-CE9: Simplifications related to cost function in DMVR [S. Sethuraman (Ittiam)]

In this proposal, two simplifications related to the cost function used in DMVR are proposed and results related to individual methods and the combined methods are presented. The first simplification relates to restricting the bit-depth of the interpolated samples on which the cost functions are evaluated during DMVR. The second simplification relates to replacing the mean removed sum of absolute differences (MR-SAD) with a refinement position independent MR-SAD (P-I-MR-SAD) cost function. Results show that the bit-depth restriction has hardly any impact on BDRATE with EncT ratio of xxx% and DecT ratio of yyy% compared to the reference method. The P-I-MR-SAD results in a 0.08% BDRATE drop when compared to MR-SAD, but offers an EncT ratio of xxx% and DecT ratio of yyy%. The two simplifications together allow EncT ratio of xxx% and DecT ratio of yyy% for DMVR Tool-ON over VTM2.

The first method proposes to intermediate bit-depth of prediction samples of the DMVR from 14-bit to 8-bit. Additional buffer size is needed when reducing the internal bit-depth to 8 with 1% decoding time saving.

It is commented the gain on top of UME of the proposed method is approximately 1.3%.

The proponent suggests to study the proposed methods in the CE9.

Recommendation: Study in the CE.

JVET-L0189 CE9-related: Improved Unidirectional Template based DMVR [F. Chen, L. Wang (Hikvision)] [late]

This contribution presents an improved unidirectional template based DMVR. To avoid latency and reuse refined MV within one CTU, new candidates based on searching pattern are generated. For bidirectional candidates, a better searching method is also developed. It is reported that the proposed method can provide 2.43%, 0.11%, 0.65% gain for RA, LB, LP respectively compared to VTM-2.0 with around 19% encoding time and 10%~24% decoding time increase.

The method was proposed to further improve and simplify CE9.2.10.

One participant mentioned the BD-rate performance in the slides does not match with the results in the contribution document.

The proponent suggested to study in the CE.

Recommendation: Study in the CE.

JVET-L0314 CE9-related: Constrained intra prediction with decoder side motion vector derivation [M. Xu, X. Li, S. Liu (Tencent)]

Decoder side motion vector derivation (DMVD) techniques were proposed to derive motion information at decoder side after parsing stage. As intra prediction may depend on the results of its spatial neighbor’s full reconstruction, it cannot be started until the MV derivation process and the final reconstruction has finished for the previous block, which leads to serious latency in hardware pipeline. It is reported that the issue can be resolved with the method proposed in this contribution at the cost of about 0.27% luma BD-rate increase for BMS-2.1-rc1 with VTM configuration and DMVR tool on.

The proponent proposes to disable DMVR inter predicted samples as references for intra prediction.

One participant commented that as intra prediction is after inter prediction, the proposed method is not helpful for practical hardware design.

The cross-checker commented that the proposed solution could reduce the latency of intra prediction on the DMVR samples.

Recommendation: Study in the CE.

JVET-L0367 CE9-related: An early termination of DMVR [T. Chujoh, T. Ikai (Sharp)]

The DMVR (Decoder Motion Vector Refinement) is a method of improving coding efficiency without explicitly sending overheads by motion vectors refinement of the merge mode on the both encoder and decoder side. One of the tasks is to reduce the complexity of the decoder side. In this contribution, an early termination of DMVR is improved. As experimental results, the decoding time was reduced by about 2% without additional SIMD, and the coding efficiency is almost unchanged. It is also possible to introduce it to integrated CE methods additionally.

Difference compared to the CE9

| Method | Error criterion | Threshold | Applied position |
| --- | --- | --- | --- |
| CE9.2.7 (JVET-L0197) | SAD | BitDepth-9 | Before DMVR process |
| Our proposal (JVET-L0367) | MRSAD | BItDepth-8 | Fix of initial DMVR process |

One participant suggests to replace the SAD with MRSAD with larger threshold (2X) at calculating the initial MRSAD at the center point.

Recommendation: Study in the CE.

JVET-L0382 CE9-related: DMVR with Coarse-to-Fine Search and Block Size Limit [K. Unno, K. Kawamura, S. Naito (KDDI)]

This contribution presents a proposal of search method for bilateral DMVR. In the proposed method, 10 points including half pixels are searched by 3 stages when search range is 2. Furthermore, disallowing DMVR at large CUs is proposed in this contribution. BD-rate gains for luma is -1.07% under the RA conditions in case of comparing with BMS2.1 rc1 with VTM configuration.

The first method reduces the worst-case of the search points from 13 to 10 with the BD-rate performance of 0.43%. no action.

The second aspect of disabling the DMVR for the CU sizes with either width or height of 128 is already included in the CE9 DMVR base software.

JVET-L0670 Simplified DMVR for inclusion in VVC [S. Esenlik, A. M. Kotra, B. Wang, H. Gao, J. Chen (Huawei), S. Sethuraman (Ittiam)] [late]

This contribution document reports the result of the combination of CE9 tests to reduce the complexity of DMVR in BMS2.1. It is asserted that the proposed DMVR is algorithm provides significant reduction in complexity while increasing the coding gain. -1.67% luma BD-rate gain and -1.68% and -1.80% chroma BD-rate gain is achieved with 1% increase in encoding time and 6% increase in decoding time compared to BMS anchor with VTM encoder configuration.

The contribution reports the combination of following CE tests:

- Reference sample padding for eliminating memory BW increase (CE9.2.2)

- Integer based DMVR to eliminate intermediate interpolation filters and buffers (CE9.2.1)

- Use refined MV from top and top-left CU (CE9.1.4)

- Parametric error surface based sub-pel refinement (CE9.2.5)

- Disable DMVR for small blocks and subsampled MRSAD (CE9.2.9f)

- Early-termination based on MV difference between merge candidates(CE9.2.13a)

The proposed modifications are independently tested in CE9.

The proponent proposes to adopt the contribution to VVC.

In the previous CE, the main concerns are internal buffer size. CE9.2.1 solves the problem of the internal buffer problem.

It is commented the performance is interesting; but, due to the adoptions in the meeting, that might be interactions of the method with the new adoptions.

The maximal CU size is 128x128.

The proponent mentioned that if not using MRSAD the loss is 0.24%.

One participant proposes to study the proposal in the next CE.

The proponent suggest to test individual part in the CE.

Recommendation: Study in the CE.