| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0701 |
| --- | --- |

| Title: | Cross-check of contribution JVET-L0628 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jonathan Pfaff | Tel: Email: | jonathan.pfaff@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution reports a cross-checking of contribution JVET-L0628 related reference sample filtering for intra prediction.

The BD-rate gains reported by the proponents and the crosschecker match. The crosschecker received the source code during the meeting and performed the crosscheck during the meeting. The encoding and decoding times measured by the crosschecker are not reliable.

# Introduction

In JVET-L0628, several ways of reference filter sampling for intra prediction are proposed, where the filtering depends on the intra prediction mode and the block-shape. Which filter is to be chosen depends, among other things, on the MDIS-threshold. The proponents conduct two tests. In the second test, one threshold in the MDIS table is slightly modified. All filters used are 4-tap filters. In the second test, the MDIS conditions are unmodified compared to VTM 2.0.1. Only the second test was crosschecked.

# Test Result

The cross-check test is performed on top of BMS2.0.1 with VTM2.0.1 configurations. The results are is shown in the following table, and it matches the result shown in JVET-L0628. Table 1 shows the results in the AI configuration. Table 2 shows the results in the RA configuration.

Table 1. Result for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,12% | -0,16% | -0,20% | 106% | 103% |
| Class A2 | -0,16% | -0,13% | -0,18% | 105% | 101% |
| Class B | -0,42% | -0,49% | -0,51% | 105% | 102% |
| Class C | -0,78% | -1,04% | -0,91% | 103% | 104% |
| Class E | -0,69% | -0,88% | -0,89% | 105% | 102% |
| Overall | -0,45% | -0,56% | -0,55% | 105% | 103% |
| Class D | -0,53% | -0,48% | -0,81% | 103% | 106% |
| Class F (optional) | -0,22% | -0,40% | -0,46% | 104% | 104% |

Table 2. Result for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,08% | -0,06% | -0,10% | 103% | 103% |
| Class A2 | -0,05% | 0,02% | 0,00% | 103% | 103% |
| Class B | -0,20% | -0,13% | -0,03% | 102% | 106% |
| Class C | -0,27% | -0,14% | -0,12% | 104% | 111% |
| Class E |  |  |  |  |  |
| Overall | -0,17% | -0,09% | -0,06% | 103% | 106% |
| Class D | -0,23% | -0,18% | -0,02% | 104% | 118% |
| Class F (optional) | 0,12% | -0,05% | -0,05% | 103% | 108% |

# Reference

[1]	Frank Bossen, Jill Boyce, Karsten Suehring, Xiang Li and Vadim Seregin, “JVET common test conditions and software reference configurations for SDR video”, Document of Joint Video Experts Team (JVET), JVET-K1010, 11th Meeting: Ljubljana, SI, 10–18 Jul. 2018

[2]	A. Fillipov et al., “CE3: A combination of tests 3.1.2 and 3.1.4 for intra reference sample interpolation filter ”, Document of Joint Video Experts Team (JVET), JVET-L0628-v2, 12th Meeting: Macao, CN, 3–12 Oct. 2018