| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L1021 |
| --- | --- |

| Title: | CE1 description: Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Jackie Ma Fraunhofer HHI Fabrice Le Léannec Technicolor Min Woo Park Samsung Electronics | Email: | jackie.ma@hhi.fraunhofer.de fabrice.leleannec@technicolor.com m.w.park@samsung.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document describes the Core Experiment 1 on Partitioning presented at the 12th Meeting in Macao in October 2018.

# Introduction

In [1] several tests on “Split constraints” have been studied that allow for 64x64 pipelining from which SubCE2-2.1.2 (JVET-L0081, [3]) was adopted into the VTM. Two CE1-related documents [4] and [5] were proposed to be further studied in the CE.

# Tests descriptions

## VPDUs

The split constraints were studied with respect to Virtual pipeline data units (VPDUs). VPDUs are originally defined [1] as non-overlapping MxM-luma(L)/NxN-chroma(C) units in a picture to address the feasibility of 64x64 pipelining. Further, the following conditions should hold

- For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU.

- For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU.

- The processing order of CUs shall not leave a VPDU and re-visit it later.

From [2] the solution SubCE2-2.1.2 was adopted into the VTM. This SubCE1 should now study further tests that may improve the performance. The following conditions are especially emphasized for the solutions:

- Solutions should be memory saving

- Solutions must obey the pipelining buffer size 64x64

The following tests do not directly comply with the above definition but have similar advantages and aim at coding gains.

### Test-1.1.1

In [4] non-squared rectangular VPDUs are studied. In this sub-test, the VPDU size is kept 4096 (64x64). However, the VPDU is not necessary to be square. To reduce complexity, VPDU may be further constrained as 1) rectangular, or 2) simple L-shape in which CU widths at most have two different values. For example, consider a CTU size of 128x128 and VPDU size of 64*64 = 4096, cf. Assume further the CTU is split by vertical TT, the center partition is further split by horizontal TT, and, finally, the center partition is split by vertical BT, see Figure 1.

1

2

3

5

4

6

Figure 1 Example of a CTU containing 6 sub CUs (solid lines) and 4 VPDUs (dashed lines). The numbers associated with the CUs indicate the encoding/decoding order.

Then, for this example, the first CU (32x128) falls into the first VPDU; the second CU (64x32) and the third CU (32x64) together fall into the second VPDU; the fourth CU (32x64) and the fifth CU (64x32) together fall into the third VPDU; the sixth CU (32x128) falls into the fourth VPDU. This CTU can be covered by four non-overlapping VPDUs.

| # | Tester | Test configurations | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 1.1.1 | M. Xu (Tencent) | - Use non-squared VPDUs - Disable the constraints from JVET-L0081 - Enable TT splits for 128 split sizes | JVET-L0313 |  |

### Test-1.2.1

In [5] a particular transform tiling for large CUs was proposed. This tiling is used for TT splits, for example at the 128 level. A TT split at the 128 level would generate a center block of size 64x128. This center block is then tiled in four 32x64 TUs instead of two 64x64 TUs. In this way, 64x64 region boundaries are not crossed. The TUs of smaller CUs that span the 64x64 region boundary are similarly tiled to avoid this boundary crossing, see Figure 1 for visualization.

Figure 1. Example TU tilings for different CU sizes within 128x128 CTU ([5]).

The residual coefficient parsing is done at CTU level which is not pipelined. As a CTU is parsed, four sets of “VPDU” coefficients can be emitted for downstream processing. Downstream actions (in particular transform operation, but also prediction and loop filtering) may be done at the 64x64 level, so memory saving is possible at these stages.

| # | Tester | Test configurations | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 1.2.1 | C. Rosewarne (Canon) | - Use transform tiling - Disable the constraints from JVET-L0081 - Enable TT splits for 128 split sizes | JVET-L0128 |  |

# Test configurations

The tests are to be conducted under the common test conditions (CTC) as established at the 12th Meeting in Macao. Further, implementations are to be made into VTM-3 which is the outcome software of the same meeting.

# Timeline

The following timeline is agreed for this CE:

- Nov 2 (T1): Finalization of the CE document

- 2 weeks after VTM-3 SW release (T2): Finished implementation of the technologies into the code basis. Initial study by cross-checkers can begin.

- Dec 7 (T3): Proponents report results to CE coordinators for inclusion in CE report.

# References

- J. Ma, F. Le Léannec, M. W. Park, “Description of Core Experiment 1 (CE1): Partitioning”, JVET-K1021, July 2018, Ljubljana.

- J. Ma, F. Le Léannec, M. W. Park, “Summary report for CE1: Partitioning”, JVET-L0021, Oct. 2018, Macao.

- C.-M. Tsai, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE1.2.1: Constraint for binary and ternary partitions”, Oct. 2018, Macao.

- M. Xu, X. Li, S. Liu, “CE1-related: Non-square virtual pipeline data unit”, JVET-L0313, Oct. 2018, Macao.

- C. Rosewarne, A. Dorrell, “CE1-related: Transform tiling for pipelined processing of large CUs”, JVET-L0128, Oct. Macao.

# A	Participant list

| # | Name | Affiliation | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
|  | Gosala Kulupana | BBC | gosala.kulupana@bbc.co.uk |  |  |
|  | Chris Rosewarne | Canon | chris.rosewarne@cisra.canon.com.au | X |  |
|  | Dan Grois | Comcast | grois@ieee.org |  |  |
|  | Yasser Syed | Comcast | yasser_syed@comcast.com |  |  |
|  | Jung Won Kang | ETRI | jungwon@etri.re.kr |  |  |
|  | Jianqing Zhu | Fujitsu | zhujianqing@cn.fujitsu.com |  |  |
|  | Jie Yao | Fujitsu | yaojie@cn.fujitsu.com |  |  |
|  | Haechul Choi | HNU | choihc@hanbat.ac.kr |  |  |
|  | Heeji Han | HNU | sarhakor@naver.com |  |  |
|  | Daehyeok Gwon | HNU | skyeye0530@naver.com |  |  |
|  | Jaeryun Choe | HNU | tjswo9395@naver.com |  |  |
|  | Han Gao | Huawei | han.gao@huawei.com |  |  |
|  | Roman Chernyak | Huawei | chernyak.roman@huawei.com |  |  |
|  | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com |  |  |
|  | Jianle Chen | Huawei | jianle.chen@huawei.com |  |  |
|  | Timofey Solovyev | Huawei | solovyev.timofey@huawei.com |  |  |
|  | Grace Yu | Intel | grace.yu@intel.com |  |  |
|  | Saurav Bandyopadhyay | InterDigital | saurav.bandyopadhyay@interdigital.com |  |  |
|  | Sriram Sethuraman | Ittiam Systems Pvt. Ltd. | sriram.sethuraman@protonmail.com |  |  |
|  | Jangwon Choi | LGE | jangwon84.choi@lge.com |  |  |
|  | Ling Li | LGE | l.li@lge.com |  |  |
|  | Jaehyun Lim | LGE | jaehyun.lim@lge.com |  |  |
|  | Seunghwan Kim | LGE | seunghwan3.kim@lge.com |  |  |
|  | Hyeongmun Jang | LGE | hm.jang@lge.com |  |  |
|  | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com |  |  |
|  | Chia-ming Tsai | MediaTek | Chia-ming.Tsai@mediatek.com |  |  |
|  | YW Huang | MediaTek | yuwen.huang@mediatek.com |  |  |
|  | ChingYeh Chen | MediaTek | chingyeh.chen@mediatek.com |  |  |
|  | Peter Chuang | MediaTek | peter.chuang@mediatek.com |  |  |
|  | Hilmi Enes Egilmez | Qualcomm | hegilmez@qti.qualcomm.com |  |  |
|  | Ted Hsieh | Qualcomm | thsieh@qti.qualcomm.com |  |  |
|  | Chun-Chi Chen | Qualcomm | chunchic@qti.qualcomm.com |  |  |
|  | Yan Zhang | Qualcomm | yzh@qti.qualcomm |  |  |
|  | Chao-Hsiung Hung | Qualcomm | chaohsiu@qti.qualcomm.com |  |  |
|  | Geert Van der Auwera | Qualcomm | geertv@qti.qualcomm.com |  |  |
|  | Yinji Piao | Samsung | yj1003.piao@samsung.com |  |  |
|  | Min Woo Park | Samsung | m.w.park@samsung.com |  |  |
|  | Minsoo Park | Samsung | mss.park@samsung.com |  |  |
|  | Kiho Choi | Samsung | kiho14.choi@samsung.com |  |  |
|  | Andrew Segall | Sharp | asegall@sharplabs.com |  |  |
|  | Fabrice Le Léannec | Technicolor | fabrice.leleannec@technicolor.com |  |  |
|  | Fabien Racape | Technicolor | fabien.racape@technicolor.com |  |  |
|  | Xiang Li | Tencent | xlxiangli@tencent.com |  |  |
|  | Meng Xu | Tencent | mengxxu@tencent.com | X |  |
|  | John Son | WILUS Inc. | john.son@wilusgroup.com |  |  |
|  | Jason Jung | WILUS Inc. | jason.jung@wilusgroup.com |  |  |
|  | Yiming Li | Wuhan University | userlym@whu.edu.cn |  |  |
|  | Zizheng Liu | Wuhan University | lzz8246@whu.edu.cn |  |  |
|  | Hong Wang | ZTE | philcage@163.com |  |  |