| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L1031 |
| --- | --- |

| Title: | Description of Core Experiment 11 (CE11): Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Andrey Norkin Anand Meher Kotra | Tel: Email: | anorkin@netflix.com anand.meher.kotra@huawei.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to improve and/or enhance deblocking filtering techniques in the test model (VTM). Techniques covered by this CE are related long deblocking filters, filtering on the 4x4 boundaries, and other modifications to deblocking.

The deblocking filter techniques are evaluated according to subjective picture quality as well as BD-rate gains. These are organized in CE11.1 and CE11.2 which address aspects of the long deblocking filters, and filtering at 4x4 block boundaries. Participants in this activity are Ericsson, HHI, Huawei, KDDI, MediaTek, Panasonic, Samsung, Qualcomm, SHARP, and Sony.

The software basis for this CE is VTM-3.0 [2]. For the test sequences, configurations and test conditions, in JVET-L1010 [1] for SDR sequences are used. For subjective viewing, additional results with sequences encoded with higher QPs may be required.

# Participants

| Nr. | Name | Company | Email |
| --- | --- | --- | --- |
| 1 | Alexis Tourapis | Apple | atourapis@apple.com |
| 2 | Patrice Onno | Canon | patrice.onno@crf.canon.fr |
| 3 | Jonathan Taquet | Canon | jonathan.taquet@crf.canon.fr |
| 4 | Christophe Gisquet | Canon | christophe.gisquet@crf.canon.fr |
| 5 | Dan Grois | Comcast | grois@ieee.org |
| 6 | Yongjo Ahn | Digital Insights | yjahn@digitalinsights.co.kr |
| 7 | Peng Yin | Dolby | pyin@dolby.com |
| 8 | Taoran Lu | Dolby | tlu@dolby.com |
| 9 | Kenneth Andersson | Ericsson | kenneth.r.andersson@ericsson.com |
| 10 | Jungwon Kang | ETRI | jungwon@etri.re.kr |
| 11 | Hahyun Lee | ETRI | hanilee@etri.re.kr |
| 12 | Yao-Jen Chang | Foxconn | yaojen.chang.foxconn@gmail.com |
| 13 | Christian Helmrich | HHI | christian.helmrich@hhi.fraunhofer.de |
| 14 | Adam Wieckowski | HHI | adam.wieckowski@hhi.fraunhofer.de |
| 15 | Haechul Choi | HNU | choihc@hanbat.ac.kr |
| 16 | Heeji Han | HNU | sarhakor@naver.com |
| 17 | Daehyeok Gwon | HNU | skyeye0530@naver.com |
| 18 | Jaeryun Choe | HNU | tjswo9395@naver.com |
| 19 | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com |
| 20 | Filippov Alexey | Huawei | alexey.filippov@huawei.com |
| 21 | Vasily Rufitskiy | Huawei | vasily.rufitskiy@huawei.com |
| 22 | Biao Wang | Huawei | biao.wang@huawei.com |
| 23 | Jianle Chen | Huawei | Jianle.chen@huawei.com |
| 24 | Rahul Vanam | InterDigital | Rahul.vanam@interdigital.com |
| 25 | Kyohei Unno | KDDI | ky-unno@kddi-research.jp |
| 26 | Kei Kawamura | KDDI | ki-kawamura@kddi.com |
| 27 | Yi-Wen Chen | Kwai | yiwenchen@kwai.com |
| 28 | Xianglin Wang | Kwai | xianglinwang@kwai.com |
| 29 | Seung Hwan Kim | LG | seunghwan3.kim@lge.com |
| 30 | Junghak Nam | LG | junghak.nam@lge.com |
| 31 | Hyeongmun Jang | LG | hm.jang@lge.com |
| 32 | Jie Zhao | LG | jie.zhao@lge.com |
| 33 | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com |
| 34 | Chia-ming Tsai | MediaTek | Chia-ming.Tsai@mediatek.com |
| 35 | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com |
| 36 | Tzu-Der Chuang | MediaTek | Peter.Chuang@mediatek.com |
| 37 | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |
| 38 | Atsuro Ichigaya | NHK | ichigaya.a-go@nhk.or.jp |
| 39 | Shunsuke Iwamura | NHK | iwamura.s-gc@nhk.or.jp |
| 40 | Shimpei Nemoto | NHK | nemoto.s-fy@nhk.or.jp |
| 41 | Tadamasa Toma | Panasonic | toma.tadamasa@jp.panasonic.com |
| 42 | Dmytro Rusanovskyy | Qualcomm | dmytror@qti.qualcomm.com |
| 43 | Wei-Jung Chien | Qualcomm | wchien@qti.qualcomm.com |
| 44 | Kiho Choi | Samsung | kiho14.choi@samsung.com |
| 45 | Woongil Choi | Samsung | woongil.choi@samsung.com |
| 46 | Weijia Zhu | Sharp | zhuw@sharplabs.com |
| 47 | Andrew Segall | Sharp | asegall@sharplabs.com |
| 48 | Kiran Misra | Sharp | misrak@sharplabs.com |
| 49 | Takeshi Chujoh | Sharp | chujoh.takeshi@sharp.co.jp |
| 50 | Tomohiro Ikai | Sharp | ikai.tomohiro@sharp.co.jp |
| 51 | Dongjae Won | SJU | bigdj2002@sju.ac.kr |
| 52 | Jaemin Ha | SJU | hajama0123@sju.ac.kr |
| 53 | Masaru Ikeda | Sony | masaru.ikeda@sony.com |
| 54 | Teruhiko Suzuki | Sony | teruhiko.s@sony.com |
| 55 | Eric Chai | Ubilinx | eric.chai@ublnx.com |
| 56 | Weimin Zeng | Ubilinx | weimin.zeng@ublnx.com |

# Test conditions and evaluation criteria

The proposals should be encoded according to two test conditions, which correspond to two anchors.

Anchor-1 is VTM-3.0 according to the CTC [1].

Anchor-2 is VTM-3.0 with ALF switched off (other conditions are the same as in Anchor 2).

The proposals in the CE shall be implemented based on VTM-3.0. The results for two test conditions shall be provided: Test Conditions 1 use configuration of Anchor 1, Test Conditions 2 use configuration of Anchor 2. The results for LDP configuration are mandatory for this CE.

Proposals will be compared with respect to subjective, objective quality, and complexity.

# Tool description

## Introduction

The main goal with this Core Experiment is to enhance the subjective quality of the Test Model and to ensure that CUs can be filtered in parallel.

Following aspects will be studied:

- Long deblocking filters and decisions

- Deblocking 4x4 block boundaries

Different technologies will be compared at a subjective viewing at the next meeting.

# Tests specification

| Test# | Description | Document# |
| --- | --- | --- |
| 11.1.1 | Very strong deblocking filtering with conditional activation signalling | JVET-L0523 |
| 11.1.2 | Extended deblocking filter (only luma) | JVET-L0380 |
| 11.1.3 | Long deblocking filters (only luma) | JVET-L0102 |
| 11.1.4 | Test on long deblocking filtering from JVET-J0021/K0334 | JVET-L0403 |
| 11.1.5 | CE11: Test results of CE11.1.5 long-tap deblocking filter | JVET-L0062 |
| 11.1.6 | Long-tap deblocking filter for luma and chroma | JVET-L0327 |
| 11.1.7 | Deblocking modifications for Large CUs both luma and chroma | JVET-L0405 |
| 11.1.8 | Longer tap Luma deblocking filter | JVET-L0224 |
| 11.1.9 | CE11: Long deblocking filters for luma and chroma (CE11.1.9) | JVET-L0072 |
| 11.1.10 | Combination of CE11.1.6 and CE11.1.7 | JVET-L0140 |
| 11.1.11 | Combination of CE11.1.1 and CE11.1.7 | JVET-L0337 |
| 11.1.12 | Combination of CE11.1.8 and CE11.1.7 | JVET-L0572 |
| 11.2.1 | Combination of former CE11.3.1, CE11.3.3, CE11.3.4 | JVET-L0073 JVET-L0225 JVET-L0103 |
| 11.2.2 | Parallel deblocking filter | JVET-L0170 |

# CE 11.1: long-tap deblocking filter

Proposals in this sub-CE investigate longer deblocking filters. Some proposals in this sub-CE only experiment with luma deblocking, while several proposals test changes in both luma and chroma deblocking.

Common aspect of most proposals is applying longer deblocking filters for blocks of larger sizes. By larger block sizes >=32 samples in the direction orthogonal to the block boundary is usually understood. Some proposals differentiate between blocks of size 16 and >=32 with longer filters applied to larger blocks. Another aspect different among the proposals is applying the same or different length of deblocking filters to the sides of the block boundary. The decisions for choosing strong deblocking filtering (in addition to large block sizes) are studied.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE11.1.1 | Christian Helmrich christian.helmrich@hhi.fraunhofer.de JVET-L0523 |  |
| CE11.1.2 | Kei Kawamura ki-kawamura@kddi.com JVET-L0380 |  |
| CE11.1.3 | Chia-Ming Tsai chia-ming.tsai@mediatek.com JVET-L0102 |  |
| CE11.1.4 | Dmytro Rusanovskyy dmytror@qti.qualcomm.com JVET-L0403 |  |
| CE11.1.5 | Woong IL Choi woongil.choi@samsung.com JVET-L0062 |  |
| CE11.1.6 | Masaru Ikeda masaru.ikeda@sony.com JVET-L0327 |  |
| CE11.1.7 | Kiran Misra misrak@sharplabs.com JVET-L0405 |  |
| CE11.1.8 | Anand Meher Kotra Anand.meher.kotra@huawei.com JVET-L0224 |  |
| CE11.1.9 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-L0072 |  |
| CE11.1.10 | Masaru Ikeda masaru.ikeda@sony.com Kiran Misra misrak@sharplabs.com JVET-L0140 |  |
| CE11.1.11 | Kenneth Andersson kenneth.r.andersson@ericsson.com Kiran Misra misrak@sharplabs.com JVET-L0337 |  |
| CE11.1.12 | Anand Meher Kotra Anand.meher.kotra@huawei.com Kiran Misra misrak@sharplabs.com |  |

## Description of technologies in CE11.1

#### CE11.1.1 Very strong deblocking filtering with conditional activation signalling JVET-L0523

The following is proposed to be tested:

- selective signaling of an in-loop filter control parameter (FCP) per CTU to allow the application of very strong deblocking filtering; this FCP shall only be signaled in a CTU if that CTU is partitioned into fewer than A sub-units (TUs) and if residual coefficient coding is used in at least B of the TUs.

- use of a very strong deblocking design, with a filter kernel size of 16 taps, for large transform units. This “superstrong” deblocking is disallowed in CTUs for which no FCP is signaled according to 1.

#### Details including the encoder decision are provided in JVET-L0523.CE11.1.2 JVET-L0380

- Technology to be investigated

Figure 11.1.2.1 Decision process for luma.

Figure 11.1.2.1 shows proposed filter decision process for luma. The processes which are bounded by red dashed line are added to the current process. It is presented that details of added processes in below.

- Size check

When the following condition is true, the proposed decision process is proceeded to “Yes”;

- Edge boundary is located between two blocks of which cross edge sizes are Th or longer than Th,

Th is defined as follows.

- If (Bs == 2), then Th = 16

- else if (Bs == 1), then Th = 32

It is noted that a block size is replaced by an edge size. Direction of edge size depends on that of edge boundary like EDGE_VER or EDGE_HOR.

- Activity check p4-p7, q4-q7

At first, the following condition (1) and (2) are checked.

p70-p00+q70-q00<β8#1

&p73-p03+q73-q03<β8#2

If both (1) and (2) are true, then following conditions (3) and (4) are checked respectively at block P and block Q. If (3) is true, extended filter is applied to block P. In the same manner, if (4) is true, extended filter is applied to block Q.

p70-p40+p73-p43<β8#3

q70-q40+q73-q43<β8#4

- Extended filtering

In extended filtering process, following filter kernels are used.

p6 = 3,5,2,1,1,1,1,1,1,0,0,0,0,0,0,0
p5 = 1,2,5,2,1,1,1,1,1,1,0,0,0,0,0,0
p4 = 0,1,3,3,3,1,1,1,1,1,1,0,0,0,0,0
p3 = 0,1,2,2,2,2,2,1,1,1,1,1,0,0,0,0
p2 = 0,0,1,2,2,2,2,2,2,1,1,1,0,0,0,0
p1 = 0,0,1,1,2,2,2,2,2,2,1,1,0,0,0,0
p0 = 0,0,0,1,2,2,2,2,2,2,2,1,0,0,0,0

Figure 11.1.2.2 Decision process for chroma.

Figure 11.1.2.2 shows proposed filter decision process for chroma. The processes which are bounded by red dashed line are added to the current process. It is presented that details of added processes in below.

- Size check

When the following condition is true, the proposed decision process is proceeded to “Yes”;

- Edge boundary is located between two blocks of which cross edge sizes are Th or longer than Th,

Th is defined as follows.

- If (Bs == 2), then Th = 8

- else if (Bs == 1), then Th = 32

It is noted that a block size is replaced by an edge size. Direction of edge size depends on that of edge boundary like EDGE_VER or EDGE_HOR.

- Strong decision and Extended filtering

Those are exact same procedure as strong filtering for luma.

#### CE11.1.3 JVET-L0102

HEVC deblocking filters are extended with longer strong filters for luma. The flowchart of the proposed filtering decisions if shown below.

When all the following conditions are true, the proposed long strong filtering is applied. Otherwise, the HEVC strong filtering is applied for p-side and q-side

- The slice type is inter.

- The CB length at p-side and the CB length at q-side are either larger than or equal to 16.

- Both of the two inter-side difference values representing difference between p-side and q-side are larger than a threshold, inter_side_diff_th. The two inter-side difference values are defined as

- InterSideDiffi =, where i = 0 or 3.

- Both of the two intra-side difference values representing variation within p-side plus variation within q-side are smaller than a threshold, intra_side_diff_th. The two intra-side difference values are defined as

IntraSideDiffi =, where i = 0 or 3.

- Any one of adjacent block edges also satisfies all the above four conditions.

When the long strong filtering decision output is yes, the long strong filter set selection is activated to select one of the three long strong filter sets for p-side and one of the three long strong filter sets for q-side. The selection based on the p-side and q-side length of the current block edge is described as follows, where lenp and lengq denote the CB length at p-side and the CB length at q-side, respectively.

- If “lenp ≥ 32 and lenq ≥ 32”, R8W7-ultra or R8W7-strong filtering is selected as follows:

  - If any one of adjacent block edges of the current block edge is also satisfied “lenp ≥ 32 and lenq ≥ 32”, R8W7-ultra filtering is applied to both p-side and q-side of the current block edge.

  - Otherwise (i.e., “lenp ≥ 32 and lenq ≥ 32” is only true for the current block edge), R8W7-strong filtering is applied to both p-side and q-side of the current block edge.

- If “lenp > 16 and lenq = 16”, R8W7-strong filtering is applied to p-side of the current block edge, and R4W4-strong filtering is applied to q-side of the current block edge.

- If “lenp = 16 and lenq > 16”, R4W4-strong filtering is applied to p-side of the current block edge, and R8W7-strong filtering is applied to q-side of the current block edge.

- If “lenp = 16 and lenq = 16”, R4W4-strong filtering is applied to both p-side and q-side of the current block edge.

The filtered values are clipped to stay in the bounds of -2tc to 2tc of the original values, as in HEVC strong filtering.

If R8W7-ultra filtering is applied to p-side, filtered sample values from  to  are as follows:

If R8W7-ultra filtering is applied to q-side, filtered sample values from  to  are as follows:

If R8W7-strong filtering is applied to p-side, filtered sample values from  to  are as follows:

If R8W7-strong filtering is applied to q-side, filtered sample values from  to  are as follows:

If R4W4-strong filtering is applied to p-side, filtered sample values from  to  are as follows:

If R4W4-strong filtering is selected at q-side, filtered sample values from  to  are as follows:

Within this CE, the performance of the deblocking method described in JVET-K0232, the filter coefficients, and the samples used in long strong filtering are to be studied.

#### CE11.1.4 JVET-L0403

HEVC deblocking filters are extended with longer deblocking filters for luma. The deblocking process tested in CE11.1.4 is similar to that in JEM with a long-tap filter (instead of strong HEVC filter) being conditionally applied to the boundaries of the blocks with size (orthogonal to the filtered boundary) being larger or equal to 16. A long deblocking filter is applied to large blocks boundary samples if smoothness criteria are met on both sides of the boundary:

- HEVC strong filter is selected

- Boundary separates blocks of size larger than 8, with either side being large than 16

- Long filter smoothness criteria return TRUE

The following pseudo code provides an example of implementation:

minBLkSize = min(lengthP, lengthQ);

maxBLkSize = max(lengthP, lengthQ);

if ((maxBLkSize > LARGE_BLOCK_SIZE_DEBLOCK ) && (minBLkSize >= 8))

if ( isSmoothAsymArea() )

{

if (minBLkSize >= LARGE_BLOCK_SIZE_DEBLOCK)

{

ApplyLongSymmetricFiltering()

}

else if ((MODE_INTRA != cuP.predMode) && (MODE_INTRA != cuQ.predMode))

{

ApplyAsymmetricFiltering();

}

}

else

{

ApplyHEVCFiltering();

}

Long filter smoothness criterion ( isSmoothAsymArea() ) is computed as an absolute value of second derivatives computed for P and Q blocks independently and this estimate is compared against the thresholds thresholdP and thresholdQ. Decision making is conducted for group of 4 lines, with only 2 out of these 4 lines are analyzed.

The thresholds thresholdP and thresholdQ are derived from the block size parameters, coded QP value and the type of the slice. Derivation of these threshold values can be tabulated.

The following pseudo code provides an example of implementation of the function isSmoothAsymArea():

function isSmoothAsymArea()
{

countQ = min(LARGE_BLOCK_SIZE_CLASSIFIER, (lengthQ >> 1)) - 2);

countP = min(LARGE_BLOCK_SIZE_CLASSIFIER, (lengthP >> 1)) - 2);

thresholdP = fun (countP, perCountTh, m_deblockSmoothThQP[iQP]);

thresholdQ = fun (countP, perCountTh, m_deblockSmoothThQP[iQP]);

varP = sum(abs(LaplacianOverP))

varQ = sum(abs(LaplacianOverQ))

if (varP < thresholdP && varQ < thresholdQ)

{

return true;

}

else

return false;

}

When long filter is selected, the following long filters are used to modify pixels q0,q1, q2,q3 and  p0,p1, p2,p3, with filter hi is centered at pixel pi, where i=0,1,2,3:

h0=0, 2, 9, 12, 1, 3, 13, 16, 5, 3, 0,

h1=-1, 6, 7, 8, -2, 13, 15, 10, 8, 0, 0,

h2=-1, 4, 12, -3, 6, 14, 14, 16, 2, 0, 0,

h3=-3, 16, -3, 1, 5, 16, 18, 14, 0, 0, 0

It should be noted, that filters hi defined above are shown in 11-tap representation for simplicity of writing, effective filter support area is spanning to 6 boundary samples.

A side of the boundary for which long filter is not applicable either due to length of the filter or not meeting smoothness criteria, is being filtered with HEVC filter.

Clipping is used to limit the amount of filtering. A boundary strength of 3 is used to determine tc for both luma and chroma. Increase tc offset if QP > 42 and decrease if QP<= 35, used for both luma and chroma. Tc is then scaled by 3 for first two samples near the boundary and scaled by 2 for the other samples.

Long filter can be turned on for respective 4 sample segment. Deblocking applied for 4x4 grid for luma.

#### CE11.1.5 JVET-L0062

HEVC deblocking filters are extended with longer deblocking filters for luma. 7 samples on a side of the boundary is filtered when length orthogonal to block boundary is >= 16, strong/weak filter determines use strong filter, sum of absolute pixel difference for pixel p8 and p0 and q8 and q0 compared to a threshold. Only 3 samples are filtered for the upper block when filtering horizontal boundaries. Decision on 4 sample segments of an 8x8 grid.

Following filters are used for left block in vertical filtering:

p6	{3, 5, 2, 1, 1, 1, 1, 1, 1}	      p7 ~ q0

p5	{1, 2, 5, 2, 1, 1, 1, 1, 1, 1}	   p7 ~ q1

p4	{1, 3, 3, 3, 1, 1, 1, 1, 1, 1}	   p6 ~ q2

p3	{1, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1}	p6 ~ q3

p2	{1, 2, 2, 2, 2, 2, 2, 1, 1, 1}	   p5 ~ q3

p1	{1, 1, 2, 2, 2, 2, 2, 2, 1, 1}	   p5 ~ q3

p0	{1, 2, 2, 2, 2, 2, 2, 2, 1}	      p4 ~ q3

For the upper block in horizontal filtering only p0 to p2.

p2    {3, 5, 2, 2, 1, 1, 1, 1}  p3 ~ q3

p1    {2, 2, 4, 2, 2, 2, 1, 1}  p3 ~ q3

p0    {2, 2, 2, 2, 2, 2, 2, 2}  p3 ~ q3

#### CE11.1.6 JVET-L0327

HEVC deblocking filters are extended with longer deblocking filters for luma and chroma. 7 samples on each side of the boundary is filtered when the minimal length orthogonal to block boundary is >= 32, sum of absolute pixel difference and also Laplacians, and comparison with absolute value of pixel difference across boundary in comparison with tc. Decision is taken on two lines of 4 sample segments on a 8x8 grid.

The decision which has two additional conditions is also modified, accordingly. The first one is for large block decision since the proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 32. The second one is for proposed wide strong filter. The decision is extended based on HEVC strong filter decision (xUseStrongFiltering) as follows.

dp0 = xCalcDP( R[0] ); dp1 = xCalcDP( R[-2] ); dp2 = xCalcDP( R[-4] );

dq0 = xCalcDQ( R[0] ); dq1 = xCalcDQ( R[2] ); dq2 = xCalcDQ( R[4] );

dd0 = 2 * (dp0+dq0); dd2 = 2 * (dp1+dq1); dd4 = 2 * (dp2+dq2);

bdd = ( dd0 < (beta>>2) && dd2 < (beta>>2) && dd4 < (beta>>2) );

dd_strong0 = abs(R[-8] – R[-1]) + abs(R[0] – R[7]);

dd_strong1 = abs(R[-8] – R[-3]) + abs(R[2] – R[7]);

dd_strong2 = abs(R[-8] – R[-5]) + abs(R[4] – R[7]);

dd_strong_flag =

( (dd_strong0<(beta>>3)) && (dd_strong0<(beta>>3)) && (dd_strong0<(beta>>3)) );

If ( dd_strong_flag && bdd && (abs(R[-1]-R[0]) < ((tc*5+1)>>1)) ) is TRUE, and then the following filter is applied to P side in the same with Q side.

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

In this conribution, long-tap filter is also proposed for chroma. The decision which has three additional conditions is also modified, accordingly. The first one is for decision of boundary strength as well as large block. The proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 8 in chroma sample domain. The second and third one are basically the same as for HEVC luma decison, which are on/off decision and strong filter decision, respectively.

In the first decision, boundary strength (bS) is modified for chroma filtering as follows.

| Conditions | Y | U | V |
| --- | --- | --- | --- |
| At least one of the adjacent blocks is intra | 2 | 2 | 2 |
| At least one of the adjacent blocks has non-zero transform coefficients | 1 | 1 | 1 |
| Absolute difference between the motion vectors that belong to the adjacent blocks is greater than or equal to one integer luma sample | 1 | N/A | N/A |
| Motion prediction in the adjacent blocks refers to vectors is different | 1 | N/A | N/A |
| Otherwise | 0 | 0 | 0 |

The following two decisions are the same as HEVC luma strong filter decision:

The second decision:

d < beta

d = Abs(p2,0−2*p1,0+ p0,0) + Abs( q2,0−2*q1,0+q0,0) + Abs(p2,1−2*p1,1+p0,1) + Abs(q2,1−2*q1,1+q0,1);

The third decision:

xUseStrongFiltering( );    // same as luma’s decision

If those criteria are satisfied, and then the following filter is applied to P side in the same with Q side.

p2′= (3*p3+2*p2+p1+p0+q0+4) >> 3

p1′= (2*p3+p2+2*p1+p0+q0+q1+4) >> 3

p0′= (p3+p2+p1+2*p0+q0+q1+q2+4) >> 3

#### CE11.1.7 JVET-L0405 + JVET-L0572

The proposal incorporates stronger filters for both luma and chroma, compared to VTM deblocking. Additionally, the proposed modification includes a control process that considers the block sizes on both sides of the boundary being deblocked. The stronger filters are used for luma samples that correspond to larger block sizes, while the VTM deblocking filters are still used for luma samples corresponding to the smaller block sizes.  For chroma, the filter selection uses a different approach, and a stronger filter is applied when chroma samples on either sides of the deblocking boundary belong to a large block.

Luma filtering.

The flowchart for a stronger filter decision for luma is provided in the picture below:

Luma is deblocked on 8x8 luma sample grid.

Luma stronger filters:

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (7*q7+2*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (6*q7+q6+2*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (5*q7+q6+q5+2*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (4*q7+q6+q5+q4+2*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (3*q7+q6+q5+q4+q3+2*q2+q1+q0+p0+p1+p2+p3+p4+8) >> 4

q1′= (2*q7+q6+q5+q4+q3+q2+2*q1+q0+p0+p1+p2+p3+p4+p5+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+p3+p4+p5+p6+8) >> 4

Luma stronger filters are used for large blocks where large block corresponds to width >= 32 for a vertical edge, and height>=32 for a horizontal edge. The adjacent block must be greater than or equal to 16. The control process is further based on gradients computed for two lines of 4 sample segments; comparison of absolute pixel value difference with tc; and comparison of other absolute pixel value differences with β. More gradients are computed for large block side.

- The variables dpq0, dpq3, dp, dq, and d are derived as follows:

dp0, dp3, dq0, dq3 are first derived as in HEVC

if (p side is greater than or equal to 32)

dp0 = ( dp0 + Abs( p5,0 − 2 * p4,0 + p3,0 ) + 1 ) >> 1

dp3 = ( dp3 + Abs( p5,3 − 2 * p4,3 + p3,3 ) + 1 ) >> 1

if (q side is greater than or equal to 32)

dq0 = ( dq0 + Abs( q5,0 − 2 * q4,0 + q3,0 ) + 1 ) >> 1

dq3 = ( dq3 + Abs( q5,3 − 2 * q4,3 + q3,3 ) + 1 ) >> 1

dpq0, dpq3, dp, dq, d are then derived as in HEVC

- As in HEVC, When d is less than β, the following ordered steps apply:

- dpq is derived as in HEVC.

- sp3 = Abs( p3 − p0 ), derived as in HEVC

if (p side is greater than or equal to 32)

sp3 = ( sp3 + Abs( p7 − p3 ) + 1) >> 1

- sq3 = Abs( q0 − q3 ), derived as in HEVC

if (q side is greater than or equal to 32)

sq3 = ( sq3 + Abs( q7 − q3 ) + 1) >> 1

- As in HEVC derive, StrongFilterCondition = (dpq is less than ( β  >>  2 ), sp3 + sq3 is less than ( β  >>  3 ), and Abs( p0 − q0 ) is less than ( 5 * tC + 1 )  >>  1) ? TRUE : FALSE

- When StrongFilterCondition is TRUE, use luma stronger filter on the side with length perpendicular to boundary edge greater than or equal to 32.

StrongFilterCondition thresholds may be modified to improve objective gains.

Chroma filtering.

Chroma deblocks on a 4x4 chroma sample grid.

Chroma stronger filters (i.e. HEVC luma strong filter):

p0′ = ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >>  3

p1′ = ( p2 + p1 + p0 + q0 + 2 )  >>  2

p2′ = ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >>  3

q0′ = ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 )  >>  3

q1′ = ( p0 + q0 + q1 + q2 + 2 )  >>  2

q2′= ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 )  >>  3

Two sub-tests for chroma are evaluated.

In another test (b), is same as above, except chroma stronger filters are used on both sides of the block boundary. Specifically, the strong filter is used when HEVC luma strong filter condition when computed for chroma is TRUE and any of the following conditions is true:

- The edge type is vertical an p0 belongs to blocks with width >= 16 (chroma samples) and q0 belongs to blocks with width >= 16 (chroma samples)

- The edge type is horizontal and p0 belongs to blocks with height >= 16 (chroma samples) and q0 belongs to blocks with height >= 16 (chroma samples)

Threshold other than 16 may be used. The gradient computation on chroma may be subsampled.

#### CE11.1.8 JVET-L0224

New luma longer tap filters for large blocks are proposed.

The flowchart for the filtering decisions is as follows:

New luma longer tap filters:

Filter condition:                       HEVCstrongweak

&&

p0-p3- p4+p7+q0-q3- q4+q7<3*β32 &&

p0-p2-p3+p5+q0-q2-q3+q5<3*β32

Proposed filter coefficients for longer tap filter:

p6′= (6*p7+3*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (5*p7+p6+3*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (4*p7+p6+p5+3*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (3*p7+p6+p5+p4+3*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (2*p7+p6+p5+p4+p3+3*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (p7+p6+p5+p4+p3+p2+3*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (6*q7+3*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (5*q7+q6+3*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (4*q7+q6+q5+3*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (3*q7+q6+q5+q4+3*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (2*q7+q6+q5+q4+q3+3*q2+q1+q0+p0+p1+p2+p3+p4+8) >> 4

q1′= (q7+q6+q5+q4+q3+q2+3*q1+q0+p0+p1+p2+p3+p4+p5+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+p3+p4+p5+p6+8) >> 4

Proposed longer tap filter condition for horizontal boundaries overlapping with CTU boundaries:

HEVCstrongweak &&

p3-p2-p1+p0+q0-q3- q4+q7<3*β32 &&

p3-p2-p1+p0+q0-q2-q3+q5<3*β32

Proposed longer tap filter coefficients for horizontal boundaries overlapping with CTU boundaries:

p2′= (6*p3+3*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (5*p3+p2+3*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (5*p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (6*q7+3*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (5*q7+q6+3*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (4*q7+q6+q5+3*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (3*q7+q6+q5+q4+3*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (2*q7+q6+q5+q4+q3+3*q2+q1+q0+p0+p1+p2+2*p3+8) >> 4

q1′= (q7+q6+q5+q4+q3+q2+3*q1+q0+p0+p1+p2+3*p3+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+4*p3+8) >> 4

#### CE11.1.9 JVET-L0072

HEVC deblocking filters are extended with longer deblocking filters for luma. One of 7 and 5 sample filtering on respective side of the block boundary can be applied for a 16-sample segment of the block boundary if current and neighboring block size >= 32 and >=16 respectively and corresponding conditions for 8 lines of the boundary are fulfilled as shown below.

| Size | Conditions for each of the 8 lines of a 16 sample segment (line 0, 3, 4, 7, 8, 11, 12, 15) |
| --- | --- |
| 7 | HEVCstrong_weak&& p0-2*p2+p4+q0-2*q2+q4<3*β32 && p0-p2-p3+p5+q0-q2-q3+q5<3*β32&& p0-2*p3+p6+q0-2*q3+q6<3*β32 && p0-p3-p4+p7+q0-q3-q4+q7<3*β32 |
| 5 | HEVCstrong_weak&& p0-p4+q0-q4<β8 && p0-p5+q0-q5<β8 |
| 0-3 | HEVC filter and decisions |

Block boundary samples pi and qi for i=0 to S-1 are then replaced by linear interpolation as follows:

- pi´=(fi*Middles+64-fi*Ps)+32)≫6, clipped to pi±tc

- qi´=(fi*Middles+64-fi*Qs+32)≫6), clipped to qi±tc

where f, Middle, P and Q are given below:

| S | Filter kernels |
| --- | --- |
| 7 | fi=59-i*9, can also be described as f=59,50,41,32,23,14,5 Middle7=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+p5+q5+p6+q6+8)≫4 P7=(p6+p7+1)≫1, Q7=(q6+q7+1)≫1 |
| 5 | fi=58-i*13, can also be described as f=58,45,32,19,6 Middle5=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+8)≫4 P5=(p4+p5+1)≫1, Q5=(q4+q5+1)≫1 |

Use HEVC luma filter and decisions with following modifications for chroma deblocking filtering.

Determine the number of samples to modify in block P and Q as follows:

lengthP = edge==EDGE_HOR ? (CUP_height_chroma >> 2) : (CUP_width_chroma >>2)

lengthQ = edge==EDGE_HOR ? (CUQ_height_chroma >> 2) : (CUQ_width_chroma >>2)

Only filter boundary if both lengthQ and lengthP is larger than 0.

The HEVC strong filter (reading 4 samples and modifying 3 samples on respective side) is only applied if both sides have a maximum deblocking filter length larger than 2.

Strong/weak filter and number of samples to filter is determined for each line. Inter blocks are only filtered if Laplacian gradients near the boundary on respective side is below the structure threshold “beta” but for intra blocks that check is by passed.

“beta” parameter is divided by 8 compared to what is used for luma.

#### CE11.1.10 Combination of CE11.1.6 and CE11.1.7 JVET-L0140 + JVET-L0529

The stronger filters are used for luma samples that correspond to larger block sizes, while the VTM deblocking filters are still used for luma samples corresponding to the smaller block sizes. Luma deblocks on 8x8 luma sample grid.

Luma Stronger Filter :

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

Luma stronger filters are used for large blocks where large block corresponds to width >= 32 for a vertical edge, and height>=32 for a horizontal edge. The adjacent block must be greater than or equal to 16. The control process is further based on gradients computed for two lines of 4 sample segments; comparison of absolute pixel value difference with tc; and comparison of other absolute pixel value differences with β. More gradients are computed for large block side.

- The variables dpq0, dpq3, dp, dq, and d are derived as follows:

dp0, dp3, dq0, dq3 are first derived as in HEVC

if (p side is greater than or equal to 32)

dp0 = ( dp0 + Abs( p5,0 − 2 * p4,0 + p3,0 ) + 1 ) >> 1

dp3 = ( dp3 + Abs( p5,3 − 2 * p4,3 + p3,3 ) + 1 ) >> 1

if (q side is greater than or equal to 32)

dq0 = ( dq0 + Abs( q5,0 − 2 * q4,0 + q3,0 ) + 1 ) >> 1

dq3 = ( dq3 + Abs( q5,3 − 2 * q4,3 + q3,3 ) + 1 ) >> 1

dpq0, dpq3, dp, dq, d are then derived as in HEVC

- As in HEVC, When d is less than β, the following ordered steps apply:

- dpq is derived as in HEVC.

- sp3 = Abs( p3 − p0 ), derived as in HEVC

if (p side is greater than or equal to 32)

sp3 = ( sp3 + Abs( p7 − p3 ) + 1) >> 1

- sq3 = Abs( q0 − q3 ), derived as in HEVC

if (q side is greater than or equal to 32)

sq3 = ( sq3 + Abs( q7 − q3 ) + 1) >> 1

- As in HEVC derive, StrongFilterCondition = (dpq is less than ( β  >>  2 ), sp3 + sq3 is less than ( β  >>  3 ), and Abs( p0 − q0 ) is less than ( 5 * tC + 1 )  >>  1) ? TRUE : FALSE

- When StrongFilterCondition is TRUE, use luma stronger filter on the side with length perpendicular to boundary edge greater than or equal to 32.

StrongFilterCondition thresholds may be modified to improve objective gains.

Chroma deblocks on a 4x4 chroma sample grid

Use of HEVC luma strong filter is also proposed for chroma when both sides of the chroma edge are greater than or equal to 8 and the following decision with three conditions are satisfied. The first one is for decision of boundary strength as well as large block. The proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 8 in chroma sample domain. The second and third one are basically the same as for HEVC luma decison, which are on/off decision and strong filter decision, respectively.

In the first decision, boundary strength (bS) is modified for chroma filtering as follows.

| Conditions | Y | U | V |
| --- | --- | --- | --- |
| At least one of the adjacent blocks is intra | 2 | 2 | 2 |
| At least one of the adjacent blocks has non-zero transform coefficients | 1 | 1 | 1 |
| Absolute difference between the motion vectors that belong to the adjacent blocks is greater than or equal to one integer luma sample | 1 | N/A | N/A |
| Motion prediction in the adjacent blocks refers to vectors is different | 1 | N/A | N/A |
| Otherwise | 0 | 0 | 0 |

The second and third decision is basically the same as HEVC luma strong filter decision as follows.

The second decision:

d < beta

d = Abs(p2,0−2*p1,0+ p0,0) + Abs( q2,0−2*q1,0+q0,0) + Abs(p2,1−2*p1,1+p0,1) + Abs(q2,1−2*q1,1+q0,1);

The third decision:

xUseStrongFiltering( );    // same as luma’s decision, except the gradient computation on chroma may be subsampled.

If those decisions are satisfied, and then the following filter is applied to P side in the same with Q side.

p0′ = ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >>  3

p1′ = ( p2 + p1 + p0 + q0 + 2 )  >>  2

p2′ = ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >>  3

#### CE11.1.11 Combination of CE11.1.9 and CE11.1.7 JVET-L0337 + JVET-L460

The filtering method enables use of long asymmetric filters. For long asymmetric filters the number of samples deblocked on the larger block side is greater than that on the smaller block side. An extended filter set may be used for strong deblocking.

Luma stronger filters are used when either side has a large block and modified strong filter condition is met. Luma large block corresponds to width >= 32 for a vertical edge, or height>=32 for a horizontal edge.

Luma Stronger Filter :

Block boundary samples pi and qi for i=0 to S-1 are then replaced by linear interpolation as follows:

- pi´=(fi*Middles,t+64-fi*Ps+32)≫6), clipped to pi±tc

- qi´=(gi*Middles,t+64-gi*Qs+32)≫6), clipped to qi±tc

where f_i, Middle_s,t, P_s and Q_s are given below:

| 7, 7 (p side: 7, q side: 7) | fi=59-i*9, can also be described as f=59,50,41,32,23,14,5 gi=59-i*9, can also be described as g=59,50,41,32,23,14,5 Middle7,7=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+p5+q5+p6+q6+8)≫4 P7=(p6+p7+1)≫1, Q7=(q6+q7+1)≫1 |
| --- | --- |
| 7, 3 (p side: 7 q side: 3) | fi=59-i*9, can also be described as f=59,50,41,32,23,14,5 gi=53-i*21, can also be described as g=53,32,11 Middle7,3=(2*po+qo+q0+2*(q1+q2)+p1+q1+p2+p3+p4+p5+p6+8)≫4 P7=(p6+p7+1)≫1, Q3=(q2+q3+1)≫1 |
| 3, 7 (p side: 3 q side: 7) | gi=59-i*9, can also be described as g=59,50,41,32,23,14,5 fi=53-i*21, can also be described as f=53,32,11 Middle3.7=(2*qo+po+p0+2*(p1+p2)+q1+p1+q2+q3+q4+q5+q6+8)≫4 Q7=(q6+q7+1)≫1, P3=(p2+p3+1)≫1 |

The control process is further based on gradients computed for two lines of 4 sample segments; comparison of absolute pixel value difference with tc; and comparison of other absolute pixel value differences with β. More gradients are computed for large block side.

- The variables dpq0, dpq3, dp, dq, and d are derived as follows:

dp0, dp3, dq0, dq3 are first derived as in HEVC

if (p side is greater than or equal to 32)

dp0 = ( dp0 + Abs( p5,0 − 2 * p4,0 + p3,0 ) + 1 ) >> 1

dp3 = ( dp3 + Abs( p5,3 − 2 * p4,3 + p3,3 ) + 1 ) >> 1

if (q side is greater than or equal to 32)

dq0 = ( dq0 + Abs( q5,0 − 2 * q4,0 + q3,0 ) + 1 ) >> 1

dq3 = ( dq3 + Abs( q5,3 − 2 * q4,3 + q3,3 ) + 1 ) >> 1

dpq0, dpq3, dp, dq, d are then derived as in HEVC

- As in HEVC, When d is less than β, the following ordered steps apply:

- dpq is derived as in HEVC.

- sp3 = Abs( p3 − p0 ), derived as in HEVC

if (p side is greater than or equal to 32)

sp3 = ( sp3 + Abs( p7 − p3 ) + 1) >> 1

- sq3 = Abs( q0 − q3 ), derived as in HEVC

if (q side is greater than or equal to 32)

sq3 = ( sq3 + Abs( q7 − q3 ) + 1) >> 1

- As in HEVC derive, StrongFilterCondition = (dpq is less than ( β  >>  2 ), sp3 + sq3 is less than ( β  >>  3 ), and Abs( p0 − q0 ) is less than ( 5 * tC + 1 )  >>  1) ? TRUE : FALSE

- When StrongFilterCondition is TRUE, use luma stronger filter.

Combination of StrongFilterCondition thresholds and/or luma control may be modified to improve objective gains.

Use of HEVC luma strong filter is also proposed for chroma (as chroma stronger filter) when both sides of the chroma edge are greater than or equal to 16 (other threshold, such as 8 may be tested) and HEVC luma strong filter condition when computed for chroma is TRUE.

CE11.1.12 Combination of CE11.1.7 and CE11.1.8 CTU line buffer reduction for long filter deblocking JVET-L0572

The proposal uses stronger filters for both luma and chroma. Additionally, the proposed modification includes a control process that considers the block sizes on both sides of the boundary being deblocked. The stronger filters are used for luma samples that correspond to larger block sizes, while the VTM deblocking filters are still used for luma samples corresponding to the smaller block sizes.  For chroma, the filter selection uses a different approach, and a stronger filter is applied when chroma samples on either sides of the deblocking boundary belong to a large block.

Luma is deblocked on 8x8 luma sample grid.

Luma stronger filters:

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (7*q7+2*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (6*q7+q6+2*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (5*q7+q6+q5+2*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (4*q7+q6+q5+q4+2*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (3*q7+q6+q5+q4+q3+2*q2+q1+q0+p0+p1+p2+p3+p4+8) >> 4

q1′= (2*q7+q6+q5+q4+q3+q2+2*q1+q0+p0+p1+p2+p3+p4+p5+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+p3+p4+p5+p6+8) >> 4

When p0 belongs to CTU above current CTU, the following limited support luma filter is used:

p2′= (6*p3+3*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (5*p3+p2+3*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (5*p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (6*q7+3*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (5*q7+q6+3*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (4*q7+q6+q5+3*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (3*q7+q6+q5+q4+3*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (2*q7+q6+q5+q4+q3+3*q2+q1+q0+p0+p1+p2+2*p3+8) >> 4

q1′= (q7+q6+q5+q4+q3+q2+3*q1+q0+p0+p1+p2+3*p3+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+4*p3+8) >> 4

Luma stronger filters are used for large blocks where large block corresponds to width >= 32 for a vertical edge, and height>=32 for a horizontal edge. The adjacent block must be greater than or equal to 16. The control process is further based on gradients computed for two lines of 4 sample segments; comparison of absolute pixel value difference with tc; and comparison of other absolute pixel value differences with β. More gradients are computed for large block side.

- The variables dpq0, dpq3, dp, dq, and d are derived as follows:

dp0, dp3, dq0, dq3 are first derived as in HEVC

LongTapDeblocking  =  (	( p side >= 32 && q side >= 16 ) || ( p side >= 16 && q side >= 32 )  )  ) ? TRUE : FALSE

ControlSamplesAccessible = p0 belongs to CTU above current CTU ? FALSE : TRUE

if ( LongTapDeblocking )

if (p side is greater than or equal to 32 && ControlSamplesAccessible )

dp0 = ( dp0 + Abs( p5,0 − 2 * p4,0 + p3,0 ) + 1 ) >> 1

dp3 = ( dp3 + Abs( p5,3 − 2 * p4,3 + p3,3 ) + 1 ) >> 1

if (q side is greater than or equal to 32)

dq0 = ( dq0 + Abs( q5,0 − 2 * q4,0 + q3,0 ) + 1 ) >> 1

dq3 = ( dq3 + Abs( q5,3 − 2 * q4,3 + q3,3 ) + 1 ) >> 1

dpq0, dpq3, dp, dq, d are then derived as in HEVC

- As in HEVC, When d is less than β, the following ordered steps apply:

- dpq is derived as in HEVC.

- sp3 = Abs( p3 − p0 ), derived as in HEVC

if ( p side is greater than or equal to 32 && LongTapDeblocking && ControlSamplesAccessible )

sp3 = ( sp3 + Abs( p7 − p3 ) + 1) >> 1

- sq3 = Abs( q0 − q3 ), derived as in HEVC

if (q side is greater than or equal to 32 && LongTapDeblocking )

sq3 = ( sq3 + Abs( q7 − q3 ) + 1) >> 1

- As in HEVC derive, StrongFilterCondition = (dpq is less than ( β  >>  2 ), sp3 + sq3 is less than ( β  >>  3 ), and Abs( p0 − q0 ) is less than ( 5 * tC + 1 )  >>  1) ? TRUE : FALSE

- When StrongFilterCondition is TRUE and LongTapDeblocking is TRUE, use luma stronger filter on the side with length perpendicular to boundary edge greater than or equal to 32.

Otherwise, When StrongFilterCondition is TRUE and LongTapDeblocking is FALSE, use HEVC strong filter

StrongFilterCondition thresholds for long filters may be modified to improve objective gains.

Chroma deblocks on a 4x4 chroma sample grid

Chroma stronger filters (i.e. HEVC luma strong filter):

p0′ = ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >>  3

p1′ = ( p2 + p1 + p0 + q0 + 2 )  >>  2

p2′ = ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >>  3

q0′ = ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 )  >>  3

q1′ = ( p0 + q0 + q1 + q2 + 2 )  >>  2

q2′= ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 )  >>  3

Chroma stronger filters are used on both sides of the block boundary. However when p0 belongs to CTU above current CTU, the chroma weak fitler is used for the side above the CTU boundary.

Specifically, the strong filter is used when HEVC luma strong filter condition when computed for chroma is TRUE and any of the following conditions is true:

- The edge type is vertical an p0 belongs to block with width >= 8 (chroma samples) and q0 belongs to block with width >= 8 (chroma samples)

- The edge type is horizontal and p0 belongs to block with height >= 8 (chroma samples) and q0 belongs to block with height >= 8 (chroma samples)

Only one line of 2-sample chroma segment may be used for gradient computation.

# CE 11.3: Deblocking at 4x4 block boundaries

The proposals in this CE study application of deblocking filtering to the 4x4 boundaries. Common feature of proposals studied in this CE is limiting number of modified samples and samples accessed for filtering decisions when the block size is 4 samples in the dimension orthogonal to the block boundary. This is done to avoid introducing a dependency of deblocking for an edge on a parallel edge preceding in a filtering order. Test CE11.3.5 investigates whether filtering any edges adjacent to the blocks of size 4 can be avoided.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE11.2.1 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-L0073 Anand Meher Kotra Anand.meher.kotra@huawei.com JVET-L0225 Chia-Ming Tsai chia-ming.tsai@mediatek.com JVET-L0103 |  |
| CE11.2.2 | Hyeongmun Jang hm.jang@lge.com JVET-L0170 |  |

## Description of technologies in CE11.3

#### CE11.2.1 Combination of JVET-L0073, JVET-L0225, and JVET-L0170

TBD

#### CE11.2.2 Parallel deblocking filter JVET-L0170

Deblocking filter can be processed in parallel because it is not applied to 4x4 TU and PU boundaries. Within this CE, the performance of the parallel deblocking method described in JVET-D0044 is to be studied in terms of subjective quality and computational complexity.

For the luma component, deblocking filtering process is not applied on block boundary of 4xN block for vertical edge and Nx4 block for horizontal edge

For the chroma component, deblocking filtering process is not applied on block boundary of 2xN block for vertical edge and Nx2 block for horizontal edge.

## Expected Gains

- Subjective picture quality improvement while preferably keeping rate-distortion performance.

- Coding efficiency improvement.

## Evaluation Criteria

1. Measure impact on Bitrate/PSNR. The BD-rate is calculated for CTC and for CTC with ALF switched off relative to Anchor-1 and Anchor-2 respectively. The results for LDP configurations are mandatory for both CTC and CTC with ALF switched off.

2. Visual impact on output pictures of representative sequences (including SDR, possibly HDR test sequences). The subjective test is expected to be held to determine the visual impact. The subjective test will use 10 second versions of the sequences. The proponents shall provide the bitstreams and decoders for the sequences that are chosen for evaluation in the subjective test.

3. Complexity of the proposals evaluated in the CE shall be compared. In particular, the following aspects of the proposals shall be summarized:

- decoding/encoding time vs the anchor

- number of samples from the block boundary that can be modified by deblocking (may be per block size)

- number of samples from the block boundary that are needed for deblocking decisions (may be per block size)

- number of filtering operations (such as additions, multiplications, comparisons)

- number of line buffers

- parallelization capabilities, such as size of a unit of a picture that can be processed in parallel to other units.

- whether proposals increase the worst case complexity and if so, by how much.

- data should be provided for both luma and/or chroma if they are modified.

## Subjective test(s)

There are two possibilities for performing a subjective test.

a) A subjective test may be conducted by the test chair before the Marrakesh meeting.

The following test sequences and conditions will be used in the subjective tests. The sequence length is 10 seconds. This means that the versions of some sequences encoded for the subjective test are longer than those used in the CTC [1]. The following encoded sequences shall be provided at QP34 and QP39.

The subjective test sequences shall be provided by YY-YYY (uploaded to the JVET ftp, exact URL will be communicated later), coding configuration with ALF switched off shall be used.

The anchors for the subjective viewing will be provided by [volunteer].

Sequences for long-tap deblocking filters (CE11.1)

The following sequences will be used for subjective evaluation in CE11.1:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | FoodMarket | 3840x2160p | 60fps | 600 | RA | 39, 34 |
| 02 | Campfire | 3840x2160p | 30fps | 300 | RA | 39, 34 |
| 03 | Kimono | 1920x1080p | 24fps | 240 | LDB | 34, 39 |
| 04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 34, 39 |

The BoG should report complexity (considering the current CE summary JVET-L0031 as baseline).

It was agreed to report information regarding what block sizes are, to which long-tap deblocking is applied.

Sequences for 4x4 filtering (CE11.2)

The following sequences will be used for subjective evaluation in CE11.2.

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Campfire | 3840x2160p | 30fps | 300 | RA | 30, 34 |
| 02 | Johnny ? | 1280x720p | 60fps | 600 | LDP | 30, 34 |
| 03 | Kimono | 1920x1080p | 24fps | 240 | LDB | 30, 34 |
| 04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 30, 34 |

b) Expert viewing at the Macao meeting may be performed.

The following sequences shall be provided at the Marrakesh meeting by the proponents: sequences from Table 2 coded with ALF switched on and ALF switched off.

## Subjective test description:

The subjective test will be done using the YYY test method.

The test material at resolution lower than 4K will be expanded to full screen by the player and evaluated together with the 4K test material.

# Test conditions and data to be provided

The test conditions are according to CTC on VTM with ALF turned on and VTM with ALF turned off. In addition, the proponents need to provide results for additional QP and additional test sequences chosen by the CE (see above). The sequences length for the subjective viewing is 10 seconds.

# Time-line and Responsibilities

2018-November-02:		Final CE description uploaded

2018-November-23:		Cross-checking begins, proponents provide software

2018-December-YY: 	Final version of CE software, full test results must be provided at this time.

Sequences for the subjective test are uploaded to the ftp.

# References

- “JVET common test conditions and software reference configurations”, JVET-L1010.

- “Algorithm description for Versatile Video Coding and Test Model 2 (VTM3)”, JVET-L1002.