| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L1033-v1 |
| --- | --- |

| Title: | Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Philippe Hanhart Jian-Liang Lin Chirag Pujara | Tel: Email: | Philippe.Hanhart@InterDigital.com jl.lin@mediatek.com chirag.p@samsung.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The main goal of this Core Experiment (CE) is to study removing subjective artifacts at face boundaries, comparing 360° specific coding tools with existing coding tools, such as tiles, combined with non-normative pre- and post-processing. The tested 360° specific coding tools include modifications to in-loop filtering and inter prediction, as well as projection format adaptation. To alleviate seam artifacts, the tested solutions shall modify in-loop filters by either disabling filtering across face discontinuities and/or by using spherical neighbors in the filtering process. The tested solutions will be implemented for the hybrid equi-angular cubemap (HEC) projection and compared to the HEC with padding of 4 samples around face row with blending (PHEC) and in-loop filters disabled across face discontinuities.

# CE13.1: In-loop filters

## Summary

| Proposal | Description |
| --- | --- |
| L0348/L0350 (MediaTek) | HEC/PHEC with in-loop filters disabled across face discontinuities |
|  | HEC/PHEC using tiles with in-loop filters disabled across tiles |
| L0349 (MediaTek) | HEC with in-loop filters using spherical neighbors |
| L0423 (Zhejiang University) | HEC with deblocking using spherical neighbors |

## Technical description

### HEC/PHEC with in-loop filters disabled across face discontinuities

### For the de-blocking filter, the filtering process is disabled when the filtered pixels cross the discontinuous edge.

For SAO, when the referenced pixels of current pixel in the SAO filtering process cross the discontinuous edge, the proposed modification disables the SAO filtering process of the current pixel to avoid inadequate filtering process.

For ALF, when filtering a CU, reconstructed sample positions used in filtering are clipped to be within the same side of the face discontinuity to which the current CU belongs.

### HEC/PHEC using tiles with in-loop filters disabled across tiles

Assuming tiles will be made available in VTM software.

### In-loop filters using spherical neighbors in L0349 (MediaTek)

#### De-blocking filter

Based on the concept of spherical continuity, the de-blocking filter fetches the spherical pixel positions located at another face for filtering when the filtered pixels cross the frame boundary (layout boundary) or the discontinuous edge as illustrated in Figure 1. The pixel data in the spherical positions are derived directly from the corresponding face and the boundary strength of the corresponding block edge is set to 2 for de-blocking filtering process.

Figure 1. Spherical pixel positions from another face for 360°-based de-blocking filter based on spherical continuity

#### SAO filter

When the referenced neighboring pixels of the current pixel in SAO filtering process cross the discontinuous edge, the proposed implementation uses the spherical pixel positions located at another face for SAO filtering as illustrated in Figure 2. Moreover, the pixel data in the spherical positions are copied directly from the corresponding face without any interpolation.

Figure 2. Spherical pixel positions from another face for 360°-based SAO filter based on spherical continuity

#### ALF

In ALF, during the determination of the filter coefficients or the filtering process, the accessed pixels crossing the frame boundary (layout boundary) or the discontinuous edge are changed to use the pixels data located at the spherical positions as illustrated in Figure 3.

Figure 3. Spherical pixel positions from another face for 360°-based ALF filter based on spherical continuity

### Deblocking using spherical neighbors in L0423 (Zhejiang University)

SAO and ALF are disabled in face row discontinuous boundaries. Blocks located at discontinuous face boundaries are filtered using simplified HEVC deblocking filter. Three cases of different boundary strength (BS) values are tested.

Case a: BS is fixed to1;

Case b: BS is fixed to2;

Case c: BS is determined by the prediction modes of blocks,

Block P and block Q are blocks which are adjacent on the sphere.

## Test specification

| Test # | Description | Proponent | Cross-checker |
| --- | --- | --- | --- |
| 1.1 | In-loop filters disabled across face discontinuities - HEC - PHEC (padding of 4 samples around face row) | MediaTek/ InterDigital |  |
| 1.2 | Tiles with in-loop filters disabled across tiles - HEC, 2 tiles, one around each face row - HEC, 2 tiles, one around each face row, using motion-constrained tile set - HEC, 6 tiles, one around each face - PHEC, 2 tiles, one around each face row - PHEC, 2 tiles, one around each face row, using motion-constrained tile set - PHEC, 6 tiles, one around each face | MediaTek MediaTek MediaTek InterDigital InterDigital InterDigital | Samsung |
| 1.3 | HEC with in-loop filters using spherical neighbors - Deblocking using spherical neighbors, SAO and ALF disabled across face discontinuities - Deblocking and ALF using spherical neighbors, SAO disabled across face discontinuities - All in-loop filters using spherical neighbors | MediaTek |  |
| 1.5 | HEC with deblocking using spherical neighbors, SAO and ALF disabled across face discontinuities - Boundary strength fixed to 1 - Boundary strength fixed to 2 - Boundary strength determined based on the prediction modes | Zhejiang University |  |

# CE13.2: Inter prediction

## Summary

| Proposal | Description |
| --- | --- |
| L0346 (MediaTek) L0230 (InterDigital) | Geometry padding of reference pictures; padding around a group (row) of continuous faces |
| L0211 (RWTH Aachen University) | Geometry padding of reference pictures; padding around each face individually |

## Technical description

### Face row based geometry padding of reference pictures in L0346 (MediaTek) / L0230 (InterDigital)

The inter prediction replaces the replication padding with the geometry padding when the reference pixels are outside the frame boundary (layout boundary) or across the discontinuous edge(s). The geometry padding in Figure 4 demonstrates a pixel position qb outside the Bottom face is represented by its spherical corresponding pixel qa, which is located at the Front face. This spherical corresponding pixel is obtained by the projection mapping functions. When the derived coordinate qa is fractional, the 6-tap and 4-tap Lanczos interpolation filters could be applied for the luminance and chrominance components, respectively.

Figure 4. Geometry padding process.

### Face based geometry padding of reference pictures in L0211 (RWTH Aachen University)

For a region of the size of a CTU around each face, pixels are projected from the neighboring faces. 360Lib is utilized for the projection, adapting it to the used projection format. For CMP this would correspond to finding each pixel by the intersection of a line between the generated pixel pA and the camera center C with the image plane of the neighboring face (Figure 5). For other formats the projection also takes account the changes due to different sampling, etc.. Lanczos 6-tap and 4-tap interpolation filters are used at the found location pB to generate the target pixel pA, for the luminance and chrominance components, respectively.

Figure 5: Extension of face A with face B.

The extended faces are used to create modified reference pictures, which are used instead of the original reference pictures. Thus, no modification of the en/decoder is required at block level. The effect of this extension can be seen in Figure 6 (for CMP).
All faces are padded in this manner, i.e. motion compensation across both continuous and non-continuous face boundaries is changed.

| a) | b) | c) |
| --- | --- | --- |

Figure 6: Example of face extension for CMP a) and b): original faces. c): extended face.

## Test specification

| Test # | Description | Proponent | Cross-checker |
| --- | --- | --- | --- |
| 2.1 | Test 1.1.a with face row based geometry padding of reference pictures - Using projection with Lanczos 3/2 interpolation - Using projection with bilinear interpolation - Using projection with nearest-neighbor interpolation - Using unfolding-based padding without interpolation | MediaTek InterDigital MediaTek MediaTek |  |
| 2.2 | Test 1.1.a with face based geometry padding of reference pictures | RWTH Aachen University |  |

# CE13.3: Post-filtering

## Summary

| Proposal | Description |
| --- | --- |
| L0422 (Zhejiang University) | Post-filtering of seam artifacts using HEVC deblocking |

## Technical description

### Post-filtering of seam artifacts in L0422 (Zhejiang University)

To reduce the seam artifacts of reconstructed viewports including face boundary, a face boundary post-filtering method is proposed. The seam artifacts are caused by coded blocks which are adjacent on the sphere but nonadjacent in the frame-packed image. The proposed filter imitates deblocking filter in HEVC. The difference is that BS is set a fixed strength and the QP in slice level is used to determine the parameters tc and β.

## Test specification

| Test # | Description | Proponent | Cross-checker |
| --- | --- | --- | --- |
| 3.1 | Test 1.1.a with post-filtering of seam artifacts using HEVC deblocking - Boundary strength fixed to 1 - Boundary strength fixed to 2 | Zhejiang University |  |

# CE13.4: Projection format adaptation

## Summary

| Proposal | Description |
| --- | --- |
| L0237 (InterDigital) | Adaptive frame packing |
| L0075 (Samsung) | Pre-rotation |

## Technical description

### Adaptive frame packing in L0237 (InterDigital)

#### Adaptive frame packing

To reduce the visibility of face seam artifacts, adaptive frame packing is proposed to periodically modify the location of discontinuous face boundaries in the frame packed pictures. The advantage of changing frame packing parameters at regular time intervals is that, for a discontinuous face boundary in the current picture, if there is a reference picture which uses a different frame packing where that same face boundary is continuous, then better prediction may be achieved for the areas located near the discontinuous face boundary in the current picture. For example, if the reference and current pictures are packed using different frame packing configurations as depicted in Figure 7(a) and Figure 7(b), respectively, then the face boundary between face #1 and face #4 in 3D geometry, marked by the red dashed lines, can be better predicted from the corresponding areas in the reference picture. This is because the areas near that face boundary in the reference picture, marked by the green line in Figure 7(a), are continuous and therefore generally easier to be coded at a higher quality; thus, better prediction may be provided for the current picture around these areas, reducing the visibility of the seam artifact in the current picture. Additionally, by changing frame packing parameters at regular intervals, locations of the discontinuous face boundaries in the frame packed pictures will change periodically. As a result, though face seams may still appear at some point in time, the duration of a face seam at the same location will be reduced. The result is visually more pleasant compared to having a face seam artifact always at the same location, which could catch the viewer’s attention and degrade perceived video quality when watching the viewport video.

Figure 7. Inter frame prediction in the case of different frame packing configurations between the reference (a) and current (b) pictures. The converted reference picture used to predict the current picture is shown in (c).

As depicted in Figure 7, when the frame packing configuration of the reference picture is different from that of the current picture, the reference picture is first converted into the frame packing configuration of the current picture. This conversion is straightforward and only involves “reshuffling” of the faces within the frame packed pictures. An example of the converted reference picture is shown in Figure 7(c).

To avoid a misalignment of chroma samples after rotation, encoding is performed using chroma sample location type 1. Additionally, the cross-component linear model (CCLM) downsampling filter proposed in [1] is used.

The adaptive frame packing parameters are determined at the encoder side at every IRAP and signaled in the PPS.

#### Adaptive frame packing determination at the encoder

Considering the HEC 3×2 layout, there are six different combinations to select two face groups of three faces each. Using the face definitions from the 360Lib, these combinations are listed in Table 1. Considering that the top and bottom parts of the spherical video are typically less interesting and/or easier to encode, only the combinations where three faces along the equators are grouped together. This reduces the problem to four different combinations (combinations 1-4 in Table 1).

Table 1. Combinations to select two face groups of three faces

| Combination | Group #1 | Group #2 |
| --- | --- | --- |
| 1 | PX, PZ, NX | PY, NZ, NY |
| 2 | PX, NZ, NX | PY, PZ, NY |
| 3 | PZ, PX, NZ | PY, NX, NY |
| 4 | PZ, NX, NZ | PY, PX, NY |
| 5 | PX, PY, NX | PZ, NY, NZ |
| 6 | PZ, PY, NZ | PX, NY, NX |

For any of these four combinations, the three equatorial faces in group #1 are placed horizontally, whereas the remaining three face in group #2 may be rotated in the frame packed picture. However, rotations of ±90° may impact coding efficiency, especially for intra coding. Indeed, a better prediction can be achieved for a pattern composed of diagonal lines, i.e., from upper left to lower right, than for a pattern composed of anti-diagonal lines, i.e., from upper right to lower left, because some intra prediction directions (e.g., those close to the intra mode 2) require reference samples located below left and above right of the current block, respectively, which are more likely to be unavailable.

To determine which rotation is optimal for a face, the horizontal, vertical, diagonal, and anti-diagonal gradients are computed on the luma component using 1-D Laplacian. Then, gradient classification is applied on an 8×8 block basis to determine the dominant gradient in each block. The gradient values are accumulated over all samples within a block. The accumulated horizontal, vertical, diagonal, and anti-diagonal gradient values, denoted gh, gv, gd, and ga, respectively, are compared to each other’s to determine the block’s dominant gradient following the flowchart depicted in Figure 8, where α is a parameter used to classify between active and inactive gradients and is set to 2.

Figure 8. Flowchart for determining the dominant gradient for an 8×8 block.

For each face, a cost function is built as

| ci = wh∙gh,i+wv∙gv,i+wd∙gd,i+wa∙ga,i | (1) |
| --- | --- |

Where i is the face index, wh, wv, wd, and wa are the weights assigned to the horizontal, vertical, diagonal, and anti-diagonal dominant gradient counts gh, gv, gd, and ga, respectively. Considering that intra prediction for patterns composed of anti-diagonal lines may be less efficient than for other directions, the following set of weights are used: wh=wv=wa=-1, wd=1. Then, the overall cost for each combination (see Table 1) is computed as the sum of the individual cost of each face, considering the orientation of each face.

A subset of the first four adaptive frame packing combinations shown in Table 1 is used in a rotating manner at the beginning of each IRAP. Among the four combinations, the cost of each combination is calculated using the first picture of the input video sequence. If the cost of any combination is significantly higher than that of the best combination, then that combination is not used in coding the input video sequence to avoid reducing coding efficiency. Since only the first picture in the sequence is used in the determination, this solution can be used together with parallel encoding. Without loss of generality, after analyzing the first picture, assume that all four combinations are determined to be viable, and they are ranked based on their cost as C1, C2, C3, C4. For the first picture, the best combination, i.e., C1, is used. For the subsequent IRAP pictures, two sets of combinations are formed, i.e., {C1,C2} and {C3,C4}, and the optimal combination is selected as

| Copt=arg min{C1,C2}cif t % 2T=0arg min{C3,C4}cotherwise | (2) |
| --- | --- |

where T is the IRAP period, % is the modulo operator, and c is the cost function computed for the given IRAP picture. Adaptive frame packing parameters are selected by the encoder using the first picture of each IRAP and signaled once per IRAP.

For sequences with large global camera motion, the content may change drastically within a face during one IRAP and frame packing parameters determined at the beginning of the IRAP may need to be updated more frequently. However, if the training period is less than the IRAP period, additional conversions are needed for the B-pictures. Thus, when the global motion is large, the adaptiveness of adaptive frame packing is disabled, i.e., the default frame packing parameters are used. This detection is performed based on the absolute frame difference computed between the pictures at t=0 and t=0.5s.

### Pre-rotation in L0075 (Samsung)

It has been observed that rotation helps in improving the coding gain for 360° video. Rotation of the content before projection can help in reducing the discontinuities in the complex region (hard to encode) and align the edges in the content more suitable for the conventional video compression. Rotation parameters are derived from the first frame and can be signaled in SEI/VUI.

Input image

36 HCP image

Generation

(Low Resolution)

Compute

Gradient angles

Find the image

with max no. of

vertical and

horizontal gradients

Figure 9. Block diagram of optimal rotation parameter estimation algorithm.

To arrive at the optimal rotation angle without encoding the video we devised the following algorithm steps,

- Downsize the image to 2K size and generate 36 HCP images corresponding to rotation angles multiple of 10 degrees starting from 10 to 360 degrees. (only for X axis rotation)

- Compute gradient magnitudes and angles for each of these 36 images

- Find the image with maximum number of vertical and horizontal gradients from these images. Here we consider gradients between -5 to 5 degrees as horizontal and -85 to 95 degrees as vertical.

- After arriving at the best angle at the granularity of 10 degree these steps can be further repeated to reach to 1 degree resolution.

- This algorithm can also be applied at RAP level.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 10. Frame from Gaslamp sequence and its edge image. (a) Image with no rotation, (b) gradient image of a, (c) image with optimal rotation (140 degrees), (d) gradient image of c.

Figure 10 shows the image from the Gaslamp sequence with no rotation and with optimal rotation. It can be seen that the gradients are aligned to vertical and horizontal directions for the later case.

## Test specification

| Test # | Description | Proponent | Cross-checker |
| --- | --- | --- | --- |
| 4.1 | Test 1.1.a with adaptive frame packing | InterDigital | Samsung |
| 4.2 | Pre-rotation - Test 1.1.a with pre-rotation - Test 1.1.b with pre-rotation | Samsung | InterDigital |

# CE13.5: Combinations

## Test specification

| Test # | Description | Proponent | Cross-checker |
| --- | --- | --- | --- |
| 5.1 | Face row based geometry padding of reference pictures and in-loop filters using spherical neighbors - 1.3.a + 2.1.a - 1.3.b + 2.1.a - 1.3.c + 2.1.a - 1.3.a + 2.1.d - 1.3.b + 2.1.d - 1.3.c + 2.1.d | MediaTek |  |
| 5.2 | Face based geometry padding of reference pictures and in-loop filters using spherical neighbors - Test 2.2 + DBF using spherical neighbors, SAO and ALF disabled across face boundaries - Test 2.2 + in-loop filters using spherical neighbors | RWTH Aachen University |  |

## Technical description

### Face based geometry padding of reference pictures and in-loop filters using spherical neighbors in L0211 (RWTH Aachen University)

For loop filters (DBF, SAO, ALF), the pixels accessed by the loop filter are changed if it operates across a face boundary. The correct neighbors according to the 3D geometry are used instead of the neighbours in the coding format (Figure 11), although both can be the same in some cases. No reprojection of the pixels is performed.

DBF is first applied to all edges except face boundaries. In a second step, DBF is applied on face boundaries with boundary strength set to 2.

Figure 11: Pixels accessed by loop filter across face boundaries (block in the middle) are changed such that the actual 3D neighbors (block at the right picture border) are used.

# Test conditions and evaluation criteria

The normative decoding tools and non-normative pre- and post-processing solutions will be tested for the Test Model (VTM). Proposals will be evaluated using BD rates, encoder run time, and decoder run time under the common test conditions and evaluation procedures for 360° video [2]. Additionally, viewports will be extracted near the HEC vertexes for informal subjective viewing at the next meting to assess the visibility of seam artifacts.

# SW integration guideline

All tests (except test 1.2) shall be implemented on top of test 1.1. The codebase for test 1.1 will be released within one week after the VTM-3.0 release.

# Timeline

| T1 | 2018-11-02 | - Final CE description |
| --- | --- | --- |
| T2 | 2018-XX-XX | - Proponents completed initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | 2018-12-19 | - Proponents froze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |

# References

- P. Hanhart, Y. He, Y. Ye, “CE3-related: Enabling different chroma sample location types in CCLM,” JVET-L0239, October 2018, Macau, CN.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-K1012, July 2018, Ljubljana, SI.

# Participants

| Company | Name | Email | Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| Apple | Jae Hoon | i.jaehoon.kim@icloud.com |  |  |
| Arris | Yue Yu Seungwook Hong Limin Wang | Yue.Yu@arris.com Seungwook.Hong@arris.com Limin.Wang@arris.com |  |  |
| BBC | Gosala Kulupana | gosala.kulupana@bbc.co.uk |  |  |
| Broadcom | Minhua Zhou Wade Wan Peisong Chen | minhua.zhou@broadcom.com wade.wan@broadcom.com peisong.chen@broadcom.com |  |  |
| Canon | Jonathan Taquet Patrice Onno | Jonathan.TAQUET@crf.canon.fr Patrice.Onno@crf.canon.fr |  |  |
| Ericsson | Julien Le Tanou Médéric Blestel | julien.le.tanou@ericsson.com mederic.blestel@ericsson.com |  |  |
| ETRI | Gun Bang Woowoen Gwun | gbang@etri.re.kr woogwun@gmail.com |  |  |
| GoPro | David Newman | dnewman@gopro.com |  |  |
| HHI | Robert Skupin | robert.skupin@hhi.fraunhofer.de |  |  |
| Huawei | Sergey Ikonin | sergey.ikonin@huawei.com |  |  |
| Intel | Jill Boyce Zhipin Deng | jill.boyce@intel.com zhipin.deng@intel.com |  |  |
| InterDigital | Philippe Hanhart Yuwen He | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com | X |  |
| ITRI | Ching-Chieh Lin Po-Han Lin | JackLin@itri.com pohan@itri.com |  |  |
| Korea Aerospace University | Yong-Uk Yoon | yuyoon@kau.kr |  |  |
| LG | Ling Li | l.li@lge.com |  |  |
| MediaTek | Jian-Liang Lin Ya-Hsuan Lee Yu-Wen Huang Cheng-Hsuan Shih Sheng-Yen Lin | jl.lin@mediatek.com Louise.Lee@mediatek.com yuwen.huang@mediatek.com ch.shih@mediatek.com Sheng-Yen.Lin@mediatek.com | X |  |
| Nokia | Alireza Aminlou | alireza.aminlou@nokia.com |  |  |
| OPPO | Fannie Yu | yuyuanfang@oppo.com |  |  |
| Qualcomm | Muhammed Coban | mcoban@qti.qualcomm.com |  |  |
| RWTH Aachen Univesity | Johannes Sauer Max Bläser | sauer@ient.rwth-aachen.de blaeser@ient.rwth-aachen.de | X |  |
| Samsung | Chirag Pujara Kiho Choi Woongil Choi Amith Dsouza Anubhav Singh Raj Narayana Akhil Konda | chirag.p@samsung.com kiho14.choi@samsung.com woongil.choi@samsung.com amith.ds@samsung.com anubhav.23@samsung.com raj.gadde@samsung.com akhil.konda@samsung.com | X |  |
| Sharp | Sachin Deshpande Kiran Misra Tomohiro Ikai Kiran Misra | sdeshpande@sharplabs.com misrak@sharplabs.com ikai.tomohiro@sharp.co.jp misrak@sharplabs.com |  |  |
| Technicolor | Fabien Racape | Fabien.Racape@technicolor.com |  |  |
| Tencent | Sehoon Yea | sehoonyea@tencent.com |  |  |
| TNO | Emmanuel Thomas Alexandre da Silva Pratas Gabriel | emmanuel.thomas@tno.nl alexandre.dasilvapratasgabriel@tno.nl |  |  |
| USTC | Dong Liu Yefei Wang | dongeliu@ustc.edu.cn wyfei@mail.ustc.edu.cn |  |  |
| Zhejiang University | Xuchang Huangfu Yule Sun Lu Yu | huangfuxc@zju.edu.cn sunyule@zju.edu.cn yul@zju.edu.cn | X |  |
|  | Koohyar Minoo | koohyar.minoo@gmail.com |  |  |