| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0016-v2 |
| --- | --- |

| Title: | JVET AHG report: Implementation studies (AHG16) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Zhou, J. An, E. Chai, K. Choi, S. Sethuraman, T. Hsieh, X. Xiu | Email: | minhua.zhou@broadcom.com jicheng.ajc@alibaba-inc.com eric.chai@ublnx.com kiho14.choi@samsung.com sriram.sethuraman@ittiam.com thsieh@qti.qualcomm.com xiaoyu.xiu@interdigital.com |
| Source: | AHG16 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG16: implementation studies, between the 12th JVET meeting in Macao, CN (3-12 October 2018) and the 13th JVET meeting in Marrakesh, MA (9–18 January 2019).

The AHG was established with the following mandates:

- Study draft and proposed coding tools to identify implementation issues relating to decoder pipelines, decoder throughput, and other aspects of implementation difficulty.

- Solicit hardware analysis of complex tools.

- Particularly consider intra reconstruction throughput for small blocks.

- Provide feedback on potential solutions to address identified issues.

# Activities

There were not many email exchanges on the main JVET email reflector (jvet@lists.rwth-aachen.de) with an [AHG16] indication on message headers. A summary of the AHG activities is provided as follows:

## Feedback provided during the finalization of CE descriptions:

- Multiple hypothesis inter prediction (MHIP):  suggested to test coding efficiency impact after the line buffer for motion vectors of the additional hypotheses is removed and the total number of distinct reference frames is constrained to 2 for the hypotheses. This is captured in the CE10 description.

- Decoder side motion vector refinement (DMVR): suggested to take the following elements as the baseline DMVR algorithm, i.e.

  - Block sizes for DMVR W*H=64 && H>=8 && W*H<=1024

  - Reference block size (w+7)*(h+7) (for luma)

  - Integer DMVR

  - MV mirroring between list0 and list1 to allow bilateral matching

  - 25 points SAD-based integer-pel search (i.e. (+-) 2 refinement search range)

  - “Parametric error surface equation” based sub-pel refinement

  - Refined MVs used for MC only

  - Luma/chroma MC w/ reference block padding (if needed)

Additional tests were recommended to quantify the benefits of the following DMVR elements in the context of VTM3.0:

  - Mean removed SAD (MRSAD)

  - Bilateral interpolation

  - Use of the refined MVs from the top neighboring CTUs for merging/AMVP/TMVPs/de blocking

  - and splitting a large block to multiple of 16x16 sub-blocks for the DMVR

The suggestion above is integrated into in the CE9 description.

- Bi-directional optical flow (BDOF aka. BIO): commented that it is desirable to avoid the processing irregularity in the extended area during the prediction block generation for the BIO. In the current design the bilinear filters are used in the extended area. If the 8-tap MC filters are consistently used for the prediction block generation including the extended area, the reference block padding should be used to avoid memory bandwidth increase. Such a test is already planned in CE9.

- Overlapped motion compensation (OBMC): suggested to test OBMC when current block is uni-prediction and the neighboring blocks only use uni-prediction to generate OBMC region. When testing sub-block OBMC combined with affine mode, it is suggested to only do blending for top one line and left one column without doing blending for right column and bottom line. Such tests are planned in CE10.

- Current picture referencing (CPR, aka. IBC): commented that sharing the line buffer in the de-blocking filter for IBC will likely lead to the local memory increase (to avoid memory access conflicts between the de-blocking and the IBC). This is generally agreed and reflected in the final CE8 description.

- Planar motion vector prediction (PMVP): commented that using 8x8 sub-block size (or 8x8 for bi-pred and 8x4/4x8 for uni-pred) is most relevant for the CE tests. It is also desirable to avoid further stressing the merging list derivation process and signal the PMVP mode separately (as opposed to signaling it as an additional merging candidate. This comment might be valid for the regression based motion vector field (RMVF) too). Such tests are planned in CE2.

## Other general comments

- Flexible tiling has a profound impact on block-level decoder implementation and should be carefully studied. Potential implications include the increased VDPU processing rate, the increased memory bandwidth if tiling is not VDPU aligned vertically, the increased buffering for frame compression, for in-loop filters and for TMVP storage, and the increased VDPU memory addressing overhead.

- (Affine) merging/AMVP list derivation is a block by block sequential operation within a CTU. Throughput study is needed to make sure that the current VTM3.0 design meets the throughput requirement (e.g. cycle budget of ~50 cycles per 8x8 block for 4K@60 at ~400 MHz clock rate).

- Separate luma/chroma partitioning tree introduces a long latency for chroma intra prediction when the CCLM is on. In the worst case, the chroma intra prediction cannot be kicked off until the whole 64x64 luma block is reconstructed (e.g. when a 64x64 luma block is split horizontally while the chroma blocks are split vertically, and vice versa). Suggest quantifying the gain of separate tree on the top of CCLM to see whether the CCLM and the separate tree can be made mutually exclusive.

- 2x2 Chroma intra-prediction itself is a serious concern from the throughput and processing overhead point of view, given the fact that the CCLM made the luma/chroma intra-prediction sequential, the luma intra prediction has been made more complicated (e.g. 4-tap interpolation filters, PDPC), and the combined intra/inter prediction was already adopted into the intra prediction/reconstruction critical path. It is helpful to test whether the CCLM can be disabled for 4x4 PUs.

- New tools such as diffusion filters, local illumination compensation (LIC), Hadamard transform domain/bilateral filtering of reconstructed blocks, and combined intra/inter prediction are all in the critical path of the intra prediction/reconstruction loop. Given the CCLM, PDPC and combined intra/inter prediction that are already adopted, the additional tools that could be absorbed into this loop is likely very limited from the throughput point of view. Therefore, the benefits of those tools should be carefully studied against each other.

- Line buffer removal from the Adaptive Loop Filter (ALF) is a significant cost reduction which should be pursued.

- The current design of the Bi-directional optical flow (BDOF aka. BIO) prohibits the 64x64 based decoder pipeline due to the early termination which requires a SAD computation up to 128x128 block size.

- 32x32/32x16/16x32 DST7/DCT8 based transform units are the bottleneck for the implementation of primary transforms.

- Transform coefficients “Zero out” of large TUs (e.g. 64x64/32x64/64x32) should be enforced at syntax-level to avoid unnecessary CABAC context storage on the decoder side.

- Combination of Current Picture Referencing (CPR, aka. IBC) with other tools requires careful thinking. CPR combinations with triangle prediction, bi-prediction, weighted prediction, GBI prediction, BDOF and OBMC will require replication of those motion compensation logics into the intra prediction/reconstruction block. Moreover, using other tools (e.g. MMVD) to signal a CPR vector or using the CPR vectors as neighboring spatial and/or temporal candidates may further complicate the logic of the (affine) merging/AMVP list derivations due to additional CPR-related checks.

## Memory bandwidth study on multiple hypothesis inter prediction

Broadcom conducted a memory bandwidth study on multiple hypothesis inter prediction by running both the CE software provided by HHI and the VTM3.0 with a commercial motion compensation cache model integrated. The summary results of the random access configuration are provided in the Table 1 and Table 2 (for informational purposes).

|  | Y | U | V | TCM_diff | ABW_diff | MBW_diff | MBW_I | MBW_D | MBW_E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | -0.24% | -0.46% |  |  | -0.36% |  | 1.81% |  | 1.70% |  | 2.58% |  | 91.67% |  | 8.33% |  | 0.00% |
| Class A2 | -0.16% | -0.16% |  |  | 0.05% |  | 0.77% |  | 0.83% |  | 7.59% |  | 75.00% |  | 16.67% |  | 8.33% |
| Class B | -0.52% | -0.13% |  |  | -0.16% |  | 2.05% |  | 1.92% |  | 4.42% |  | 90.00% |  | 0.00% |  | 10.00% |
| Class C | -0.14% | 0.01% |  |  | -0.03% |  | 0.87% |  | 0.82% |  | 1.86% |  | 75.00% |  | 12.50% |  | 12.50% |
| Class D | -0.08% | 0.00% |  |  | 0.06% |  | 0.45% |  | 0.43% |  | 0.00% |  | 50.00% |  | 37.50% |  | 12.50% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class F | -0.62% | -0.43% |  |  | -0.32% |  | 0.49% |  | 0.78% |  | 6.56% |  | 62.50% |  | 6.25% |  | 31.25% |
| All | -0.29% | -0.16% | -0.12% | 1.07% | 1.08% | 7.59% | 83.33% | 8.33% | 8.33% |

Table 1. Multiple hypothesis inter prediction (up to using 4 reference frames)

|  | Y |  | U | V | TCM_diff | ABW_diff | MBW_diff | MBW_I | MBW_D | MBW_E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | -0.10% | -0.33% |  | -0.15% | 0.95% | 0.87% | 0.97% | 75.00% | 16.67% | 8.33% |
| Class A2 | -0.11% | -0.09% |  | 0.09% | 0.45% | 0.54% | 0.32% | 58.33% | 25.00% | 16.67% |
| Class B | -0.37% | -0.03% |  | -0.16% | 0.87% | 0.80% | -1.29% | 70.00% | 10.00% | 20.00% |
| Class C | -0.12% | -0.04% |  | -0.06% | 0.33% | 0.28% | -0.70% | 50.00% | 37.50% | 12.50% |
| Class D | -0.09% | -0.05% |  | -0.06% | 0.09% | 0.11% | 4.04% | 37.50% | 56.25% | 6.25% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Class F | -0.57% | -0.29% |  | -0.28% | 0.27% | 0.40% | 3.12% | 62.50% | 12.50% | 25.00% |
| All | -0.20% |  | -0.10% | -0.08% | 0.49% | 0.50% | 4.04% | 63.33% | 21.67% | 15.00% |

Table 2. Multiple hypothesis inter prediction (up to using 2 reference frames)

Where

- TCM_diff : Total cache misses (over all the frames coded), percentage increase relative to VTM3.0.

- ABW_diff: Average memory bandwidth (over all the frames coded), percentage increase relative to VTM3.0.

- MBW_diff : Worst case (Max) memory bandwidth (among all the frames coded), percentage increase relative to VTM3.0.

- MBW_I : Percentage of coded  sequences that have the MBW increase relative to VTM3.0.

- MBW_D:Percentage of coded  sequences that have the MBW decrease relative to VTM3.0.

- MBW_E: Percentage of coded  sequences that have equal MBW  relative to VTM3.0.

MBW_diff  is the most important measure, which shows the percentage of the worst case memory bandwidth increase relative to VTM3.0.

# Related contributions

The following contributions are identified for the AHG.

- JVET-M0527, “AHG12: Comments on Tiles and Flexible Tile Partitioning”, W. Wan, M. Zhou, T. Hellman, B. Heng, P. Chen (Broadcom)

  - Elaborated the potential implications of the flexible tiling on decoder designs.

- Contributions aiming for addressing the 64x64 decoder pipeline issue of the BDOF (aka. BIO)

  - JVET-M0284, “CE9-related: BDOF Modifications to Enable 64x64 VPDU”, H. Chen, X. Ma, S. Esenlik, H. Yang, J. Chen (Huawei)

  - JVET-M0073, “Non-CE9: On early termination for BIO”, K. Kondo, M. Ikeda, T. Suzuki (Sony)

  - JVET-M0249”, Non-CE9: Modifications on Bi-Directional Optical Flow”, H. Liu, L. Zhang, K. Zhang, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)

- Contributions relating to ALF line buffer reduction

  - JVET-M0164, “Adaptive loop filter with virtual boundary processing”, C.-Y. Chen, T.-D. Chuang, Z.-Y. Lin, C.-Y. Lai, Y.-W. Huang, S.-M. Lei (MediaTek)

  - JVET-M0301, “Non-CE: Loop filter line buffer reduction”, A.M. Kotra, S. Esenlik, B. Wang, H. Gao, J. Chen (Huawei)

- Contributions aiming to prohibit the transform coefficients from being transmitted in the “zero-out” regions of 64x64/64x32/32x64 transform units at syntax level

  - JVET-M0250, “Non-CE7: Simplified CSBF coding for large block-size transforms”, J. Choi, J. Heo, S. Yoo, J. Choi, L. Li, J. Lim, S. Kim (LGE)

  - JVET-M0257, “CE7-related: coefficient scanning and last position coding for TUs of greater than 32 width or height”, M. Coban, M. Karczewicz (Qualcomm)

- Contributions relating to CPR vector usage and CPR combination with other tools

  - JVET-M0402, “Non-CE8: Comments on Current Picture Referencing”, B. Heng, M.Zhou, W.Wan (Broadcom)

  - JVET-M0175, “CE8-related: Clarification on interaction between CPR and other inter coding tools”, C.-Y. Lai, T.-D. Chuang, Y.-L. Hsiao, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

  - JVET-M0483, “CE8-related: CPR mode signaling and interaction with inter coding tools”, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-M0046, “CE6-related: A study of primary transforms”,  M. Zhou, Y. Hu (Broadcom)

  - Identified the 32-point DST7/DCT8 as the bottleneck for the implementation of primary transforms and recommended a “zero-out” solution.

- JVET-M0245 , “AHG16-related: Chroma block coding and size restriction”, C. Rosewarne, A. Dorrell (Canon)

  - Removal of 2x2/2x4/4x2 block sizes from the chroma intra prediction (for the separate tree case only)

- JVET-M0248, “AHG16: Motion compensation with padded samples for small coding units”, H. Liu, J. Chon, H.-C. Chuang, L. Zhang, K. Zhang, J. Xu (Bytedance)

  - Memory bandwidth reduction for motion compensation of small size PUs.

  - There are many other contributions in this category.

- JVET-M0265, “AHG16: Clean-up on MV Rounding”, K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong(Bytedance)

  - Improves design consistency of the MV rounding.

# Recommendations

The AHG recommends reviewing the input contributions.