| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0027-v1 |
| --- | --- |

| Title: | CE7: Summary report on quantization and coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Heiko Schwarz, Muhammed Coban Cheung Auyeung | Email: | heiko.schwarz@hhi.fraunhofer.de mcoban@qti.qualcomm.com cheung.auyeung@huawei.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

The CE report summarizes the test results and crosschecks reports for CE7 on quantization and coefficient coding.  The CE includes 4 tests on transform coefficient coding.

# Introduction

The purpose of this core experiment is to explore the coding efficiency and complexity impact of proposed algorithms for quantization and transform coefficient coding. This core experiment evaluates the impact of:

- Context modeling/selection/coding order for syntax elements related to transform coefficients

The objectives of this CE include:

- Investigation of the impact of context selection and signaling schemes for transform coefficient levels on the coding efficiency and complexity.

# Participants List

| Affiliation | Person, Email address |
| --- | --- |
| Qualcomm | Muhammed Coban <mcoban@qti.qualcomm.com> Dmytro Rusanovskyy <dmytror@qti.qualcomm.com> Jie Dong <jiedong@qti.qualcomm.com> Ted Hsieh <thsieh@qti.qualcomm.com> Marta Karczewicz <martak@qti.qualcomm.com> |
| Fraunhofer HHI | Heiko Schwarz <heiko.schwarz@hhi.fraunhofer.de> Tung Nguyen <tung.nguyen@hhi.fraunhofer.de> Christian Helmrich <christian.helmrich@hhi.fraunhofer.de> |
| Samsung | Yinji Piao <yj1003.piao@samsung.com> Kiho Choi <kiho14.choi@samsung.com> |
| Huawei | Alexey Filippov <alexey.filippov@huawei.com> Jianle Chen <Jianle.chen@huawei.com> Cheung Auyeung <cheung.auyeung@huawei.com> Yang Haitao <haitao.yang@huawei.com> Yin Zhao <yin.zhao@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> Biao Wang <biao.wang@huawei.com> Vasily Rufitskiy <vasily.rufitskiy@huawei.com> Alexander Karabutov <karabutov.alexander@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> |
| Orange | Pierrick Philippe <pierrick.philippe@orange.com> Felix Henry <felix.henry@orange.com> Gordon Clare <gordon.clare@orange.com> |
| MediaTek | Mohammed Sarwer <Mohammed.Sarwer@mediatek.com> Olena Chubach <Olena.Chubach@mediatek.com> Yu-Wen Huang <yuwen.huang@mediatek.com> Chih-Wei Hsu <cw.hsu@mediatek.com> Ching-Yeh Chen <chingyeh.chen@mediatek.com> T.-D. Chuang <peter.chuang@mediatek.com> Shih-Ta Hsiang <shih-ta.hsiang@mediatek.com> |
| Sharp | Eiichi Sasaki <eiichi.sasaki@sharp.co.jp> Takeshi Chujoh <chujoh.takeshi@sharp.co.jp> Tomohiro Ikai <ikai.tomohiro@sharp.co.jp> Kiran Misra <misrak@sharplabs.com> Andrew Segall <asegall@sharplabs.com> Frank Bossen <frank.bossen@me.com> |
| Apple | Alexis Tourapis <atourapis@apple.com> Jae Hoon Kim <i.jaehoon.kim@icloud.com> |
| KDDI | Kei Kawamura <ki-kawamura@kddi.com> Kyohei Unno <ky-unno@kddi-research.jp> Yoshitaka Kidani <yo-kidani@kddi.com> |
| Kwai | Yi-Wen Chen <yiwenchen@kwai.com> Xianglin Wang <xianglinwang@kwai.com> |
| Bytedance | Li Zhang <lizhang.idm@bytedance.com> Hongbin Liu <liuhongbin.01@bytedance.com> |
| Nokia | Jani Lainema <jani.lainema@nokia.com> |
| Broadcom | Minhua Zhou <minhua.zhou@broadcom.com> Wade Wan <wade.wan@broadcom.com> Peisong Chen <peisong.chen@broadcom.com> |
| Ericsson | Julien Le Tanou <julien.le.tanou@ericsson.com> |
| Sony | Takeshi Tsukuba <Takeshi.Tsukuba@sony.com> Karl Sharman <karl.sharman@eu.sony.com> Masa Ikeda <masaru.ikeda@sony.com> |
| InterDigital | Xiaoyu Xiu <xiaoyu.xiu@interdigital.com> Yuwen He <Yuwen.He@InterDigital.com> Rahul Vanam <Rahul.vanam@interdigital.com> |
| Canon | Chris Rosewarne <chris.rosewarne@cisra.canon.com.au> Andrew Dorrell <andrew.dorrell@cisra.canon.com.au> |
| BBC | Andre Seixas Dias <Andre.SeixasDias@bbc.co.uk> Saverio Blasi <saverio.blasi@bbc.co.uk> |
| Technicolor | Fabien Racape <fabien.racape@technicolor.com> Ya Chen <Ya.chen@technicolor.com> Edouard Francois <Edouard.francois@technicolor.com> Philippe de Lagrange <Philippe.delagrange@technicolor.com> Leleannec Fabrice <Fabrice.Leleannec@technicolor.com> |
| HiSilicon | Jianhua Zheng <zhengjianhua@hisilicon.com> Quanhe Yu <yuquanhe@hisilicon.com> |
| ETRI | Jungwon Kang <jungwon@etri.re.kr> Sung-Chang Lim <sclim@etri.re.kr> |
| Panasonic | Kiyofumi Abe <abe.kiyo@jp.panasonic.com> Han Boon Teo <hanboon.teo@sg.panasonic.com> Chong Soon Lim <chongsoon.lim@sg.panasonic.com> Tadamasa Toma <toma.tadamasa@jp.panasonic.com> |
| LG | Jung-Ah Choi <jungah.choi@lge.com> Jin Heo <jin78.heo@lge.com> Sunmi Yoo <sunmi.yoo@lge.com> Jangwon Choi <jangwon84.choi@lge.com> Ling Li <l.li@lge.com> Seung Hwan Kim <seunghwan3.kim@lge.com> Mehdi Salehifar <mehdi.salehifar@lge.com> Moonmo Koo <moonmo.koo@lge.com> Jaehyun Lim <jaehyun.lim@lge.com> |
| Dolby | Sean McCarthy <smcca@dolby.com> Peng Yin <pyin@dolby.com> |
| Comcast | Dan Grois <grois@ieee.org> |
| Intel | Sabine Roessel <sabine.roessel@intel.com> |
| Tsinghua University | Benben Niu <nbb16@mails.tsinghua.edu.cn> |
| Tencent | Min Gao <maxwellgao@tencent.com> Xiang Li <xlxiangli@tencent.com> |
| WILUS | Dongcheol Kim <dongcheol.kim@wilusgroup.com> |
| Foxconn | Yao-Jen Chang <yaojen.chang.foxconn@gmail.com> |
| Alibaba | Yu-Chen Sun <yuchen.s@alibaba-inc.com> |

# Summary of Tests in CE

Transform coefficient coding

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| 7.1 | Tzu-Der Chuang (MediaTek) | [1] Constraints on the number of context-coded bins depend on whether luma or chroma is coded (and, for chroma, on the subblock size) | Yu-Chen Sun (Alibaba) |
| 7.2 | Tzu-Der Chuang (MediaTek) | [1] Constraints on the number of context-coded bins depend on whether luma or chroma is coded (and, for chroma, on the subblock size) + [2] Constraints on the number of context-coded bins depend on last significant scan position | Jung-Ah Choi (LGE) M. Coban (Qualcomm) |
| 7.3 | Tzu-Der Chuang (MediaTek) | [1] Constraints on the number of context-coded bins depend on whether luma or chroma is coded (and, for chroma, on the subblock size) + [2] Constraints on the number of context-coded bins depend on last significant scan position + [3] Include rem_abs_gt2_flag into first coding pass | H. Schwarz (HHI) M. Coban (Qualcomm) |
| 7.4 | Tzu-Der Chuang (MediaTek) | [3] Include rem_abs_gt2_flag into first coding pass | H. Schwarz (HHI) |

# List of relevant contributions

| document | author(s) | title |
| --- | --- | --- |
| JVET-M0173 | T.-D. Chuang, S.-T. Hsiang, Z.-Y. Lin, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) | CE7 (Tests 7.1, 7.2, 7.3, and 7.4): Constraints on context-coded bins for coefficient coding |
| CE related documents |  |  |
| JVET-M0105 | R. Chernyak, S. Ikonin, J. Chen (Huawei) | Non-CE7: Delta QP for Chroma CU |
| JVET-M0107 | F. Le Léannec, Y. Chen (Technicolor) | CE7-related: reduced local neighborhood usage for transform coefficients coding |
| JVET-M0113 | P. de Lagrange, E. François, P. Bordes (Technicolor) | CE7-related: Quantization Group size uniformity |
| JVET-M0114 | P. de Lagrange, E. François, F. Le Léannec (Technicolor) | CE7-related: implicit QP-offset based on block size |
| JVET-M0119 | K. Sharman, S. Keating (Sony) | CE7-related: Modified dequantization scaling |
| JVET-M0188 | O. Chubach, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) | CE7-related: On quantization parameter signalling considering CU area |
| JVET-M0198 | Y. Piao, K. Choi (Samsung) | CE7-related : Unified rice parameter derivation for coefficient level coding |
| JVET-M0250 | J. Choi, J. Heo, S. Yoo, J. Choi, L. Li, J. Lim, S. Kim (LGE) | Non-CE7: Simplified CSBF coding for large block-size transforms |
| JVET-M0251 | J. Choi, J. Heo, S. Yoo, J. Choi, L. Li, J. Lim, S. Kim (LGE) | Non-CE7: Last position coding for large block-size transforms |
| JVET-M0257 | M. Coban, M. Karczewicz (Qualcomm) | CE7-related: coefficient scanning and last position coding for TUs of greater than 32 width or height |
| JVET-M0278 | S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim (LGE) | Non-CE7 : Residual rearrangement for transform skipped blocks |
| JVET-M0279 | S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim (LGE) | Non-CE7 : Sign coding for transform skip |
| JVET-M0305 | J. Lainema (Nokia) | CE7-related: Joint coding of chrominance residuals |
| JVET-M0318 | P. de Lagrange, P. Bordes (Technicolor) | CE7-related: QP prediction and neighbour availability |
| JVET-M0469 | M. Karczewicz, M. Coban (Qualcomm) | CE7-related: unified Rice parameter derivation for coefficient coding |
| JVET-M0470 | M. Coban, M. Karczewicz (Qualcomm) | CE7-related: Golomb-Rice/exponential Golomb coding for abs_remainder and dec_abs_level syntax elements |
| JVET-M0489 | M Gao, X Li, X Zhao, S Liu (Tencent) | CE7-related: Reduced context models for transform coefficients coding |
| JVET-M0491 | M. Gao, X. Li, X. Zhao, S. Liu (Tencent) | CE7-related: Reduced maximum number of context-coded bins for transform coefficient coding |
| JVET-M0513 | Y. Zhao, S. Hong, H. Yang, J. Chen (Huawei) | CE7-related: Context modeling of pred_mode_flag |
| JVET-M0558 | M. Karczewicz, M. Coban (Qualcomm) | CE7-related: Template based Rice parameter derivation |

# Test Description

## Software, reference configurations, and test conditions

All tools have to be integrated into the VTM-3 software (outcome of the Macau meeting).  Results have to be reported for the following two configurations:

- VTM-3 software and common test conditions (standard configuration files);

- VTM-3 software in low-complexity encoder configuration (common test conditions, but dependent quantization and RDOQ disabled and sign data hiding enabled).

All tools are tested against the corresponding anchor (same set of tools, except the tool to be tested).

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-rate). Proponents are encouraged to provide supplementary results at lower or higher QPs.

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the reference configurations);

- Decoding time (compared to the reference configuration).

In addition, proponents are encouraged to provide the following (but not limited to) information for complexity and entropy throughput analysis:

- Number of context models

- Number of context-coded and bypass bins in average case and the worst case

- Table size of scanning paths

- Analysis of the frequency of context switches

- Analysis of the dependency between two consecutive bins

- Analysis of entropy parsing dependency (whether bits to bins conversion depends on complex reconstruction and/or pixel processing process)

- Analysis of entropy de-binarization dependency (whether bins to symbols conversion depends on complex reconstruction and/or pixel processing process)

# CE Results

## Summary

The average test results (Bjøntegaard delta rates) are summarized in Table 1.

Table 1: Average test results for CE 7.  The table shows results for a high-complexity encoder configurations (same as CTC) as well as a low-complexity encoder configuration (dependent quantization and RDOQ disabled).  All results are relative to VTM-3.0 (using the same configuration as the tested approach).

| CE 7 |  | high complexity (CTC) |  |  |  |  | low complexity (DQ off, RDOQ off) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| AI | 7.1 | 0.08% | 0.64% | 0.44% | 102% | 101% | 0.12% | 0.42% | 0.47% | 100% | 101% |
|  | 7.2 | 0.06% | 0.19% | 0.10% | 104% | 102% | 0.06% | 0.12% | 0.14% | 100% | 102% |
|  | 7.3 | 0.00% | 0.09% | 0.05% | 103% | 101% | 0.00% | 0.03% | 0.11% | 99% | 101% |
|  | 7.4 | -0.08% | -0.09% | -0.10% | 100% | 101% | -0.05% | -0.08% | -0.08% | 99% | 101% |
| RA | 7.1 | 0.07% | 0.87% | 0.69% | 101% | 100% | 0.09% | 0.45% | 0.58% | 100% | 102% |
|  | 7.2 | 0.03% | 0.35% | 0.30% | 101% | 101% | 0.03% | 0.15% | 0.22% | 100% | 102% |
|  | 7.3 | 0.00% | 0.30% | 0.23% | 101% | 102% | 0.02% | 0.14% | 0.17% | 100% | 102% |
|  | 7.4 | -0.06% | 0.03% | -0.06% | 100% | 100% | -0.02% | -0.12% | -0.02% | 100% | 102% |
| LB | 7.1 | 0.09% | 1.58% | 0.89% | 101% | 102% | -0.02% | 1.03% | 0.66% | 100% | 100% |
|  | 7.2 | 0.00% | 0.57% | 0.14% | 102% | 101% | -0.01% | 0.40% | 0.13% | 100% | 99% |
|  | 7.3 | 0.02% | 0.58% | -0.12% | 101% | 100% | -0.01% | 0.50% | 0.09% | 100% | 99% |
|  | 7.4 | -0.04% | 0.00% | 0.12% | 100% | 100% | 0.02% | -0.15% | -0.09% | 100% | 99% |

## Test Description

In JVET-L0145, three methods to constrain the usage of context-coded bins are proposed. The first constraint is dependent on color component and coefficient sub-block size. The second method is to release the constraints values according to the last significant sub-block position to improve the coding efficiency. The third modification is to move the greater than 2 flag into the first pass to further improve the decoding throughput. The details are described as follow.

[1] Different constraints according to color components and sub-block sizes

The number of context-coded bins (including greater than 2 flag) constraints are set to 30 on 4x4 luma subblocks, 16 on 4x4 chroma subblocks, and 4 on 2x2 chroma subblocks, respectively. The number of context-coded bin constraints on greater than 2 are set to 4 on 4x4 luma subblocks, 2 on 4x4 chroma subblocks, and 1 on 2x2 chroma sub-blocks, respectively.

[2] Last significant subblock position dependent constraint

According to the last significant subblock position, the number of to-be-coded sub-blocks in a TB can be derived. The constraint value is determined according to the color component and the coefficient sub-block size. With the number of to-be-coded sub-blocks (NumToBeCodedSb) and the number of total coefficient sub-blocks in a TB (NumTotalSb), the constraint value (ConstraintValue) is modified as follows.

if (NumToBeCodedSb * 2 <= NumTotalSb)

ConstraintValue = ConstraintValue * 2;

else if (NumToBeCodedSb * 5 <= NumTotalSb * 4)

ConstraintValue = (ConstraintValue * 5) >> 2;

[3] Include greater than 2 flag into the first coding pass

The greater than 2 flag is proposed to be moved to the first coding pass after the parity bit. The parsed greater than 2 flag is used to calculate the locSumAbsPass1in the context modeling of sig_coeff_flag, par_level_flag, rem_abs_gt1_flag, and rem_abs_gt2_flag. The reserved number of context-coded bins for greater than 2 flag in the second coding pass is also merged into the number of context-coded bins for the first coding pass. The test that moving the greater than 2 flag before parity bit will also be evaluated.

Specification of tests:

Test 7.1: Aspect [1]

Test 7.2: Aspects [1] + [2]

Test 7.3: Aspects [1] + [2] + [3]

Test 7.4: Aspect [3]

## Test 7.1

Test 7.1 includes the following aspect:

- Different constraints according to color components and sub-block sizes

### Test results

The proposed approach was integrated into the VTM-3.0 software.  The software was made available to all CE participants.  The average test results are summarized in Table 2.

Table 2: Average test results for test 7.1.  The table shows results for a high-complexity encoder configurations (same as CTC) as well as a low-complexity encoder configuration (dependent quantization and RDOQ disabled).  All results are relative to VTM-3.0 (using the same configuration as the tested approach).

|  | all Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | high complexity (CTC) |  |  |  |  | low complexity (DQ off, RDOQ off) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | 0.21% | 0.44% | 0.57% | 103% | 99% | 0.21% | 0.27% | 0.67% | 100% | 103% |
| A2 | 0.20% | 1.22% | 1.15% | 102% | 103% | 0.34% | 0.81% | 0.88% | 100% | 101% |
| B | 0.01% | 0.78% | 0.29% | 103% | 101% | 0.04% | 0.51% | 0.45% | 100% | 100% |
| C | 0.03% | 0.50% | 0.19% | 102% | 101% | 0.04% | 0.34% | 0.27% | 102% | 101% |
| E | 0.03% | 0.20% | 0.22% | 103% | 101% | 0.03% | 0.14% | 0.14% | 100% | 101% |
| avg. | 0.08% | 0.64% | 0.44% | 102% | 101% | 0.12% | 0.42% | 0.47% | 100% | 101% |
| D | 0.05% | 0.26% | 0.15% | 102% | 100% | 0.03% | 0.29% | 0.40% | 99% | 97% |
| F | 0.15% | 0.64% | 0.63% | 102% | 100% | 0.19% | 0.76% | 0.75% | 99% | 99% |
|  | random access |  |  |  |  |  |  |  |  |  |
|  | high complexity (CTC) |  |  |  |  | low complexity (DQ off, RDOQ off) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | 0.30% | 0.71% | 0.71% | 101% | 98% | 0.31% | 0.19% | 0.63% | 100% | 98% |
| A2 | 0.00% | 1.36% | 1.46% | 100% | 98% | 0.05% | 0.80% | 1.00% | 100% | 103% |
| B | -0.01% | 0.97% | 0.36% | 101% | 101% | 0.04% | 0.55% | 0.56% | 100% | 105% |
| C | 0.02% | 0.49% | 0.50% | 101% | 101% | 0.01% | 0.25% | 0.26% | 100% | 99% |
| avg. | 0.07% | 0.87% | 0.69% | 101% | 100% | 0.09% | 0.45% | 0.58% | 100% | 102% |
| D | 0.03% | -0.03% | 0.08% | 101% | 101% | 0.04% | 0.42% | 0.55% | 100% | 100% |
| F | 0.16% | 0.68% | 0.76% | 101% | 102% | 0.21% | 0.67% | 0.66% | 100% | 101% |
|  | low delay B |  |  |  |  |  |  |  |  |  |
|  | high complexity (CTC) |  |  |  |  | low complexity (DQ off, RDOQ off) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| B | 0.04% | 1.78% | 1.03% | 101% | 103% | 0.01% | 1.58% | 0.72% | 100% | 102% |
| C | 0.12% | 1.44% | 1.20% | 101% | 100% | -0.06% | 1.03% | 0.38% | 100% | 98% |
| E | 0.14% | 1.43% | 0.26% | 101% | 102% | -0.03% | 0.13% | 0.92% | 100% | 100% |
| avg. | 0.09% | 1.58% | 0.89% | 101% | 102% | -0.02% | 1.03% | 0.66% | 100% | 100% |
| D | 0.06% | 0.38% | 0.92% | 101% | 101% | 0.00% | 1.07% | 0.94% | 100% | 103% |
| F | 0.18% | 1.23% | 0.98% | 101% | 99% | 0.05% | 0.54% | 1.11% | 100% | 99% |

### Crosschecks

The cross checker Yu-Chen Sun (Alibaba) reported:

- The source code matches the description.

- The reported test results were reproduced for all configurations and test sequences.

## Test 7.2

Test 7.2 includes the following aspects:

- Different constraints according to color components and sub-block sizes

- Last significant subblock position dependent constraint

### Test results

The proposed approach was integrated into the VTM-3.0 software.  The software was made available to all CE participants.  The average test results are summarized in Table 3.

Table 3: Average test results for test 7.2.  The table shows results for a high-complexity encoder configurations (same as CTC) as well as a low-complexity encoder configuration (dependent quantization and RDOQ disabled).  All results are relative to VTM-3.0 (using the same configuration as the tested approach).

|  | all Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | high complexity (CTC) |  |  |  |  | low complexity (DQ off, RDOQ off) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | 0.15% | 0.35% | 0.35% | 104% | 103% | 0.13% | 0.17% | 0.32% | 100% | 102% |
| A2 | 0.18% | 0.37% | 0.33% | 104% | 103% | 0.23% | 0.29% | 0.32% | 100% | 104% |
| B | 0.01% | 0.15% | -0.04% | 104% | 101% | 0.01% | 0.12% | 0.08% | 100% | 108% |
| C | 0.00% | 0.16% | 0.01% | 104% | 101% | 0.01% | 0.11% | 0.12% | 100% | 97% |
| E | 0.00% | -0.06% | -0.05% | 104% | 102% | 0.00% | -0.07% | -0.10% | 100% | 99% |
| avg. | 0.06% | 0.19% | 0.10% | 104% | 102% | 0.06% | 0.12% | 0.14% | 100% | 102% |
| D | 0.00% | 0.14% | 0.15% | 104% | 100% | -0.01% | 0.09% | 0.10% | 100% | 96% |
| F | 0.14% | 0.66% | 0.45% | 104% | 100% | 0.14% | 0.61% | 0.56% | 99% | 107% |
|  | random access |  |  |  |  |  |  |  |  |  |
|  | high complexity (CTC) |  |  |  |  | low complexity (DQ off, RDOQ off) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | 0.13% | 0.46% | 0.24% | 102% | 100% | 0.13% | 0.03% | 0.29% | 100% | 99% |
| A2 | 0.00% | 0.63% | 0.72% | 101% | 100% | 0.02% | 0.39% | 0.43% | 100% | 102% |
| B | 0.00% | 0.34% | 0.09% | 101% | 102% | 0.01% | 0.11% | 0.15% | 100% | 105% |
| C | 0.02% | 0.08% | 0.30% | 102% | 103% | 0.00% | 0.09% | 0.09% | 100% | 99% |
| avg. | 0.03% | 0.35% | 0.30% | 101% | 101% | 0.03% | 0.15% | 0.22% | 100% | 102% |
| D | 0.01% | -0.13% | 0.17% | 102% | 100% | -0.01% | 0.19% | 0.69% | 100% | 100% |
| F | 0.11% | 0.60% | 0.59% | 101% | 100% | 0.13% | 0.40% | 0.52% | 100% | 101% |
|  | low delay B |  |  |  |  |  |  |  |  |  |
|  | high complexity (CTC) |  |  |  |  | low complexity (DQ off, RDOQ off) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| B | -0.06% | 0.17% | -0.06% | 102% | 101% | 0.02% | 0.52% | 0.16% | 100% | 99% |
| C | 0.07% | 0.64% | 0.43% | 102% | 100% | -0.05% | 0.54% | -0.07% | 100% | 98% |
| E | -0.01% | 1.16% | 0.09% | 102% | 102% | -0.02% | 0.03% | 0.37% | 101% | 99% |
| avg. | 0.00% | 0.57% | 0.14% | 102% | 101% | -0.01% | 0.40% | 0.13% | 100% | 99% |
| D | 0.04% | 0.64% | 0.32% | 101% | 100% | -0.01% | 0.36% | 1.07% | 100% | 103% |
| F | 0.09% | 0.59% | 0.34% | 102% | 99% | 0.09% | 0.35% | 0.69% | 100% | 100% |

### Crosschecks

The cross checker Jung-Ah Choi (LGE) reported:

- The cross checker tested the software provided by MediaTek in the git repository. The cross checker ran all required test cases (both common test condition and low-complexity encoder configuration) under VTM configuration and confirmed that there are no problems in the reported test results. Some decoding time results seem not reliable because of inconsistent execution environment leading to inaccurate timings

- The cross checker carefully checked the provided source code and confirmed that the implementation matches the input document

- It is recommended to see the effect of 'last significant subblock position dependent constraint' only (excluding CE7.1)

The cross checker Muhammed Coban (Qualcomm) reported:

- The source code matches the description.

- The reported test results were reproduced for all configurations and test sequences.  The run times are not accurate.

## Test 7.3

Test 7.3 includes the following aspects:

- Different constraints according to color components and sub-block sizes

- Last significant subblock position dependent constraint

- Include greater than 2 flag into the first coding pass

### Test results

The proposed approach was integrated into the VTM-3.0 software.  The software was made available to all CE participants.  The average test results are summarized in Table 4.

Table 4: Average test results for test 7.3.  The table shows results for a high-complexity encoder configurations (same as CTC) as well as a low-complexity encoder configuration (dependent quantization and RDOQ disabled).  All results are relative to VTM-3.0 (using the same configuration as the tested approach).

|  | all Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | high complexity (CTC) |  |  |  |  | low complexity (DQ off, RDOQ off) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | 0.10% | 0.20% | 0.15% | 103% | 103% | 0.07% | 0.09% | 0.26% | 99% | 102% |
| A2 | 0.11% | 0.32% | 0.25% | 103% | 103% | 0.14% | 0.20% | 0.23% | 98% | 101% |
| B | -0.05% | 0.07% | -0.04% | 103% | 101% | -0.04% | 0.02% | 0.06% | 99% | 105% |
| C | -0.08% | 0.03% | -0.09% | 103% | 100% | -0.05% | -0.03% | 0.07% | 98% | 97% |
| E | -0.04% | -0.11% | 0.08% | 102% | 100% | -0.06% | -0.12% | -0.03% | 99% | 99% |
| avg. | 0.00% | 0.09% | 0.05% | 103% | 101% | 0.00% | 0.03% | 0.11% | 99% | 101% |
| D | -0.06% | 0.04% | 0.09% | 103% | 99% | -0.04% | 0.10% | -0.15% | 98% | 98% |
| F | 0.17% | 0.56% | 0.52% | 103% | 99% | 0.17% | 0.60% | 0.58% | 97% | 99% |
|  | random access |  |  |  |  |  |  |  |  |  |
|  | high complexity (CTC) |  |  |  |  | low complexity (DQ off, RDOQ off) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | 0.09% | 0.38% | 0.30% | 101% | 100% | 0.06% | -0.10% | 0.24% | 100% | 99% |
| A2 | 0.02% | 0.63% | 0.59% | 101% | 102% | 0.04% | 0.32% | 0.39% | 100% | 102% |
| B | -0.04% | 0.21% | 0.08% | 101% | 102% | 0.01% | 0.14% | 0.05% | 100% | 106% |
| C | -0.01% | 0.09% | 0.09% | 101% | 103% | -0.02% | 0.18% | 0.10% | 100% | 99% |
| avg. | 0.00% | 0.30% | 0.23% | 101% | 102% | 0.02% | 0.14% | 0.17% | 100% | 102% |
| D | -0.07% | -0.43% | -0.03% | 101% | 99% | 0.01% | 0.23% | 0.29% | 100% | 100% |
| F | 0.18% | 0.67% | 0.60% | 101% | 100% | 0.16% | 0.47% | 0.65% | 100% | 101% |
|  | low delay B |  |  |  |  |  |  |  |  |  |
|  | high complexity (CTC) |  |  |  |  | low complexity (DQ off, RDOQ off) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| B | -0.05% | 0.25% | -0.09% | 101% | 99% | 0.00% | 0.64% | -0.07% | 99% | 100% |
| C | -0.02% | 0.54% | 0.23% | 101% | 99% | -0.05% | 0.42% | -0.18% | 100% | 98% |
| E | 0.20% | 1.17% | -0.66% | 101% | 101% | 0.02% | 0.36% | 0.69% | 100% | 100% |
| avg. | 0.02% | 0.58% | -0.12% | 101% | 100% | -0.01% | 0.50% | 0.09% | 100% | 99% |
| D | 0.03% | 0.97% | 0.52% | 101% | 100% | -0.01% | 0.53% | 0.50% | 100% | 99% |
| F | 0.01% | 0.87% | 0.01% | 101% | 99% | -0.07% | -0.17% | 0.20% | 101% | 100% |

### Crosschecks

The cross checker Heiko Schwarz (Fraunhofer HHI) reported:

- The source code was carefully checked. The software implementation matches the description in JVET-L1027, but includes an additional complexity reduction: For aspect [2] “Last significant subblock position dependent constraint”, the software uses 3 different cases, while the initial description in JVET-L1027 specifies 5 different cases.

- The reported test results could be exactly reproduced for all configurations and test sequences.

The cross checker Muhammed Coban (Qualcomm) reported:

- The source code matches the description.

- The reported test results were reproduced for all configurations and test sequences. The run times are not accurate

## Test 7.4

Test 7.4 includes the following aspect:

- Include greater than 2 flag into the first coding pass

### Test results

The proposed approach was integrated into the VTM-3.0 software.  The software was made available to all CE participants.  The average test results are summarized in Table 5.

Table 5: Average test results for test 7.4.  The table shows results for a high-complexity encoder configurations (same as CTC) as well as a low-complexity encoder configuration (dependent quantization and RDOQ disabled).  All results are relative to VTM-3.0 (using the same configuration as the tested approach).

|  | all Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | high complexity (CTC) |  |  |  |  | low complexity (DQ off, RDOQ off) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | -0.07% | -0.11% | -0.05% | 100% | 101% | -0.04% | -0.04% | -0.02% | 100% | 101% |
| A2 | -0.09% | -0.09% | -0.11% | 100% | 102% | -0.04% | -0.04% | 0.00% | 99% | 102% |
| B | -0.08% | -0.13% | -0.08% | 100% | 101% | -0.05% | -0.07% | -0.06% | 100% | 104% |
| C | -0.11% | -0.04% | -0.23% | 99% | 100% | -0.07% | -0.11% | -0.14% | 99% | 97% |
| E | -0.06% | -0.08% | 0.02% | 99% | 100% | -0.06% | -0.10% | -0.16% | 99% | 100% |
| avg. | -0.08% | -0.09% | -0.10% | 100% | 101% | -0.05% | -0.08% | -0.08% | 99% | 101% |
| D | -0.07% | -0.10% | -0.13% | 103% | 103% | -0.06% | -0.05% | 0.04% | 98% | 96% |
| F | 0.02% | -0.01% | 0.06% | 101% | 100% | 0.05% | 0.22% | 0.16% | 98% | 103% |
|  | random access |  |  |  |  |  |  |  |  |  |
|  | high complexity (CTC) |  |  |  |  | low complexity (DQ off, RDOQ off) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | -0.05% | 0.17% | 0.00% | 100% | 97% | -0.03% | -0.19% | 0.08% | 100% | 98% |
| A2 | -0.06% | -0.08% | -0.06% | 100% | 101% | -0.03% | -0.32% | -0.07% | 100% | 102% |
| B | -0.07% | 0.06% | -0.13% | 100% | 101% | -0.02% | 0.03% | -0.01% | 100% | 106% |
| C | -0.05% | -0.02% | -0.02% | 100% | 100% | -0.02% | -0.10% | -0.07% | 99% | 99% |
| avg. | -0.06% | 0.03% | -0.06% | 100% | 100% | -0.02% | -0.12% | -0.02% | 100% | 102% |
| D | -0.10% | -0.35% | 0.14% | 102% | 100% | -0.04% | -0.09% | 0.08% | 100% | 100% |
| F | 0.01% | 0.09% | -0.17% | 100% | 100% | 0.02% | 0.06% | 0.21% | 100% | 102% |
|  | low delay B |  |  |  |  |  |  |  |  |  |
|  | high complexity (CTC) |  |  |  |  | low complexity (DQ off, RDOQ off) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| B | -0.04% | -0.59% | 0.00% | 100% | 101% | 0.01% | 0.02% | -0.16% | 99% | 99% |
| C | -0.04% | 0.20% | 0.31% | 100% | 100% | -0.02% | -0.30% | -0.32% | 100% | 97% |
| E | -0.03% | 0.72% | 0.07% | 100% | 100% | 0.08% | -0.22% | 0.32% | 100% | 99% |
| avg. | -0.04% | 0.00% | 0.12% | 100% | 100% | 0.02% | -0.15% | -0.09% | 100% | 99% |
| D | -0.05% | -0.26% | 0.12% | 100% | 99% | -0.01% | 0.11% | 0.51% | 100% | 100% |
| F | 0.02% | -0.12% | -0.22% | 100% | 99% | -0.07% | -0.50% | -0.21% | 100% | 100% |

### Crosschecks

The cross checker Heiko Schwarz (Fraunhofer HHI) reported:

- The source code was carefully checked. The software implementation matches the description in JVET-L1027.

- The reported test results could be exactly reproduced for all configurations and test sequences.