| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0032r1 |
| --- | --- |

| Title: | CE12: Summary report on mapping functions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Francois, P. Yin | Tel: Email: | edouard.francois@technicolor.com pyin@dolby.com |
| Source: | CE12 coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report of Core Experiment 12 on mapping functions. CE12 evaluates approaches for mapping of HDR and SDR content, with main focus on SDR. The considered technologies are in-loop reshaping and related variants. Test results against VTM3.0 anchors are provided for each performed test. Crosschecking reports are integrated in this contribution.

# Introduction

CE12 investigates the following proposals/tools:

- [JVET-L0246]: SDR In-loop Reshaping

- [JVET-L0247]: Universal low complexity reshaper for SDR and HDR video

# Tests overview

In JVET-L0247, the universal reshaper has two main components: 1) in-loop reshaping applied to the luma component; 2) complementary luma dependent chroma residue scaling applied to chroma components.

## CE12-1: In-loop Reshaping for SDR on coding efficiency (JVET-L0247)

| Test | Proponent | Description | Xchecker |
| --- | --- | --- | --- |
| CE12-1 | Dolby (JVET-L0247) | Test in-loop reshaping for SDR focused on coding efficiency improvement, such as making reshaping function rate adaptive, disable towards higher rates. | E. François (Technicolor) |

In-loop luma reshaping is implemented as a pair of look-up tables (LUTs), but only one of the two LUTs need to be signaled as the other one can be computed from the signaled LUT. One LUT is a forward LUT, FwdLUT, that maps input luma code values Yi to altered values Yr: Yr=FwdLUT[Yi]. The other LUT is an inverse LUT, InvLUT, that maps altered code values Yr to Yi : Yi=InvLUT[Yr]. (Yi represents the reconstruction values of Yi.)

At the decoder side, for intra slices, only the InvLUT is applied, whereas for inter slices, both FwdLUT and InvLUT are applied.

The reshaper model syntax signals a piece-wise linear (PWL) model with 32 pieces. The PWL model allows precomputing the 1024-entry FwdLUT and InvLUT mapping tables; and it also allows implementations to calculate identical mapping values on-the-fly without pre-computing the LUTs

An overview of intra slice and inter slice decoder workflow is shown in Figure 1 and Figure 2.

Figure 1. Intra slice reconstruction with in-loop luma reshaper in CE12-1 (light-green shaded blocks indicate signal in reshaped domain: luma residue; intra luma predicted; and intra luma reconstructed)

Figure 2. Inter slice reconstruction with in-loop luma reshaper in CE12-1(light-green shaded blocks indicate signal in reshaped domain: luma residue and intra luma reconstructed)

Luma-dependent chroma residue scaling is a simple multiplicative process implemented with fixed-point integer operation. Chroma residue scaling compensates for luma signal interaction with the chroma signal. Chroma residue scaling is applied at the TU level.

Encoder side: cResScale = cRes * cScale = cRes / cScaleInv

Decoder side: cRes = cResScale / cScale = cResScale * cScaleInv

Because a PWL model is used, for any input luma value in each luma PWL range, the same chroma scale value is used. Therefore, the luma dependent chroma scales are simplified to be calculated for each luma PWL range instead of each luma code value and stored in LUT cScaleInv[pieceIdx] (32-piece granularity).

## CE12-2: In-loop Reshaping for SDR on In-loop Reshaping for SDR on implementation investigation

| Test | Proponent | Description | Xchecker |
| --- | --- | --- | --- |
| CE12-2 | Dolby (JVET-L0247) | Test in-loop reshaping for SDR focused on implementation investigation on pipelining of the block-wise prediction loop: such as piece-wise linear interpolation replacing LUT-based mapping, disable intra-block in inter slices. | J. Xu (ByteDance), E. François (Technicolor) |

In-loop luma reshaping: CE12-2 tests a lower complexity pipeline that also eliminates decoding latency for block-wise intra prediction in inter slice reconstruction. As shown in Figure 3, intra prediction is performed in reshaped domain for both inter and intra slices.

CE12-2 also tests 16-piece PWL models for luma and chroma residue scaling instead of the 32-piece PWL models of CE12-1.

Figure 3. Inter slice reconstruction with in-loop luma reshaper in CE12-2 (light-green shaded blocks indicate signal in reshaped domain: luma residue; intra luma predicted; and intra luma reconstructed)

CE12-2 also tests 16-piece PWL models for luma and chroma residue scaling instead of the 32-piece PWL models of CE12-1.

# Test results (JVET-M0427)

## SDR content

The BD-rate performance of CE12-1 and CE12-2 for SDR CTC is shown in Table 1.

Table 1. BD-rate of CE12-1 and CE12-2 for SDR

| All Intra | psnrY | psnrU | psnrV | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| 12-1 | -0.89% | 1.52% | 1.15% | 103% | 100% |
| 12-2 | -0.89% | 1.49% | 1.18% | 102% | 101% |

| Random Access | psnrY | psnrU | psnrV | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| 12-1 | -1.28% | 0.53% | 0.52% | 105% | 101% |
| 12-2 | -1.30% | 1.45% | 1.44% | 105% | 104% |

| Low Delay B | psnrY | psnrU | psnrV | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| 12-1 | -0.72% | 0.46% | 0.11% | 100% | 101% |
| 12-2 | -0.76% | 1.78% | 1.42% | 98% | 105% |

Comments:

- Comparing CE-12-1 and low-complexity CE12-2, the performance is effectively the same for luma for all test conditions: AI, RA, and LDB.  The chroma performance of CE12-1 and CE12-2 are also the same for AI.  For RA and LDB, CE12-2 chroma performance is less than that of CE12-1 by less than 1% for RA, and 1.3% for LDB.  Chroma loss in CE12-2 is mainly due to 32 pieces to 16 pieces simplification and some loss is due to intra prediction in reshaped domain in inter slice.

## Additional HDR Results in CE12-2

Additional test results for HDR-PQ content using CE12-2 in-loop reshaper. For HDR, CE12-2 employs the 16-piece PWL model to signal the static reshaping function designed for HDR. Thus, the parsing and decoding process for HDR and SDR are the same.

The BD-rate performance of CE12-2 for HDR-PQ content is shown in Table 2.

Table 2. BD-rate of CE12-2 for HDR

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wPsnrY | PSNRL100 | DE100 | wPsnrY | PSNRL100 | DE100 |
| Average HDR-B | -1.6% | -2.0% | 0.9% | -1.6% | -1.8% | 1.9% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

## Summary of differences between CE12-1, CE12-2

Table 3 listed the summary of the major differences between CE12-1 and CE12-2 for improvements and simplifications made for reshaping.

Table 3. overview of the differences between CE12-1 and CE12-2

|  | CE12-1 | CE12-2 |
| --- | --- | --- |
| FwdLUT and InvLUT construction | PWL interpolation with 32 pieces, unified for SDR and HDR | PWL interpolation with 16 pieces, unified for SDR and HDR |
| Decoding pipeline implication | need FwdLUT and InvLUT inside intra prediction loop in inter-slice, extra latency | no need for FwdLUT and InvLUT inside intra prediction loop for inter slice, unify intra mode in intra/inter slice, low latency InvLUT not in critical path of inter prediction |
| Chroma residue scaling LUT | 32-entry, index by PWL piece idx to which the TU avg. luma belongs | 16-entry, index by PWL piece idx to which the TU avg. luma belongs |

# Crosscheck reports

## Overview of crosscheck reports of mandatory test results

| Test# | Cross-checker | PSNR&BR Match (Y/N) | RunT matched? (Y/N) | SW implemented by cc? (Y/N) | SW studied? (Y/N) | significant inconsistencies between the description and the provided software? (Y/N) | Visual assessment Y/N Y: comments below | Other tests? (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12-1 | TCH | Y | Y | N | Y | N | N | N |
| 12-2 | TCH | Y | Y | N | Y | N | N | Y* |
| 12-2 | ByteD. | Y | NA** | N | Y | N | N | N |

*further simplifications of CE12-2 have been explored in contributions JVET-M0109 and JVET-M0640.

** The crosschecker’s runtime during the test is not reliable due to cluster fluctuation.

## Detailed cross-check reports

### Cross-check of CE12-1 / CE12-2

Proponent: Dolby

Cross-checker: Technicolor

Objective metrics

The reported objective numbers from the proponent are confirmed by the cross-checker for the two cross-checked configurations: CE12-1, CE12-2. Runtimes are of same order as those reported by the proponent.

Configurations CE12-2 seems to be preferable as the design is simplified without noticeable impact on objective performance.

Code verification

The cross-checker has checked the code and confirms that the implementation is aligned with the tool description in JVET-M0427.

Visual tests

No visual check was performed.

Comments

The cross-checker has investigated further simplifications of CE12-2 aiming either at avoiding the per-sample forward and inverse mapping steps (JVET-M0109), or at simplifying the inverse mapping step by using a PWL with equi-length intervals (JVET-M0640). The cross-checker suggests to review these contributions when reviewing the CE12 results.

### Cross-check of CE12-2

Proponent: Dolby

Cross-checker: ByteDance

Objective metrics

The reported objective numbers from the proponent are confirmed by the cross-checker for the two cross-checked configurations: CE12-1, CE12-2.

Code verification

The cross-checker has checked the code and confirms that the implementation is aligned with the tool description in JVET-M0427.

Visual tests

No visual check was performed.

# CE12 and CE12-related documents

One CE12 and two CE12-related contributions are currently submitted.

| number | Title | Source |
| --- | --- | --- |
| JVET-M0109 | CE12-related: block-based in-loop reshaping | E. Francois, C. Chevance, F. Hiron (Technicolor) |
| JVET-M0427 | CE12: Mapping functions (test CE12-1 and CE12-2) | T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen (Dolby) |
| JVET-M0640 | CE12-related: in-loop reshaping with approximate inverse mapping function | E. Francois (Technicolor) |

# Recommendations

Review all CE document if needed. Review all CE-related documents.