| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0042 |
| --- | --- |

| Title: | CE10: Uniform Directional Diffusion Filters For Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jennifer Rasch, Anastasia Henkel, Jonathan Pfaff, Heiko Schwarz, Detlev Marpe, Thomas Wiegand | Tel: Email: | +49 30 31002-727 Jennifer.Rasch@hhi.fraunhofer.de, Jonathan.pfaff@hhi.fraunhofer.de, anastasia.henkel@hhi-extern.fraunhofer.de. |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute. |  |  |

_____________________________

# Abstract

An encoder speedup for the Uniform Diffusion filters described in JVET-L0157 [4] is investigated. The filter masks are simplified and no iterations are used. There are three types of filters, two directional filters and one weighted interpolation of the directional filters. Further, a restriction of the Diffusion filters to specific types of predictions is investigated.

# Proposed method: Decoder algorithm description

Uniform Directional Diffusion Filters have been integrated to the test model according to the description of sub-tests CE10.4.1- CE10.4.2 [1].

## Uniform Diffusion Filter

Let pred be the prediction signal on a given block obtained by intra or motion compensated prediction. In order to handle boundary points for the filters, the prediction signal needs to be extended to a prediction signal predext. This extended prediction is formed by adding one line of reconstructed samples left and above the block to the prediction signal and then the resulting signal is mirrored in all directions. The Uniform Diffusion Filter is realized by convolving the prediction signal with a fixed mask that is given as hI, defined below. It is proposed to replace the prediction signal pred by

hI*pred,

using the aforementioned boundary extension. Here, the filter mask hI is given as

hI=0.125010141010.

## Directional Diffusion Filter

The directional filters are defined as described in [4].  A horizontal filter hhor and a vertical filter hver,  are used which have a fixed mask. The filtering is restricted to be either applied only along the vertical or along the horizontal direction. The vertical filter is realized by applying the fixed filter mask

hver=0.54 104060401

to the prediction signal and the horizontal filter is realized by using the transposed mask

hhor=hvert.

The extension of the prediction signal is performed in exactly the same way as in the previous section.

# Implementation and Complexity

Different filter configurations, depending on the block-shape, are used. These configurations are described via the syntax element diffFilterIdx as described in the table below.

| diffFilterIdx | filter type | Filters to be used |  |
| --- | --- | --- | --- |
|  |  | Intra | inter |
| 0 | no filter | - | - |
| 1 | Uniform | hI | hI |
| 3 | directional horizontal | hhor | hhor |
| 4 | directional vertical | hver | hver |

Table  Interpretation of the syntax element diffFilterIdx

It should be noted here that the filter is restricted to larger blocks as depicted in the following table.

| BlkSizes | 4 | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8 | 0 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0 |
| 16 | 0 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0 |
| 32 | 0 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0 |
| 64 | 0 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0 |
| 128 | 0 | 0 | 0 | 0 | 0 | 0,1,3,4 |

Table  Restricted number of tested Filters ordered by block sizes, width versus height

The filtering is implemented in a way such that symmetries within the filter-masks are exploited. The multiplications that are factors of two are implemented using bitshifts. The maximal complexity per pixel in terms of number of additions, number of bit-shifts and number of multiplications to produce the overall prediction signal when applying the diffusion filter is depicted for different block sizes below.

| BlkSizes | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- |
| 8 | 5 | 5 | 5 | 5 | - |
| 16 | 5 | 5 | 5 | 5 | - |
| 32 | 5 | 5 | 5 | 5 | - |
| 64 | 5 | 5 | 5 | 5 | - |
| 128 | - | - | - | - | 5 |

Table  Maximal number of additions for different block sizes

| BlkSizes | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- |
| 8 | 2 | 2 | 2 | 2 | - |
| 16 | 2 | 2 | 2 | 2 | - |
| 32 | 2 | 2 | 2 | 2 | - |
| 64 | 2 | 2 | 2 | 2 | - |
| 128 | - | - | - | - | 2 |

Table  Maximal number of bit-shifts for different block sizes

| BlkSizes | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- |
| 8 | 1 | 1 | 1 | 1 | - |
| 16 | 1 | 1 | 1 | 1 | - |
| 32 | 1 | 1 | 1 | 1 | - |
| 64 | 1 | 1 | 1 | 1 | - |
| 128 | - | - | - | - | 1 |

Table  Maximal number of multiplications for different block sizes

# Results

According to the CE10 description in JVET-L1030 [1] and the common test conditions in JVET-L1010 [5], the proposed method is evaluated for intra-only, random-access and low delay configuration with VTM 3.0.

In the following table an overview over the scheduled CE tests with their corresponding config parameters is shown.

|  | Test |  |
| --- | --- | --- |
| CE10.4.1 | All Intra, Random Access and Low Delay | #Diffusion filtering DiffusionFilterMode : 1 FastDiffusionFilter : 3 NMBDRYDiffusion : 0 NumDiffusionFiltersIntra : 4 NumDiffusionFiltersInter : 4 |
| CE10.4.2 | All Intra, Random Access and Low Delay, | #Diffusion filtering DiffusionFilterMode : 1 FastDiffusionFilter : 7 NMBDRYDiffusion : 0 NumDiffusionFiltersIntra : 4 NumDiffusionFiltersInter : 4 |

## CE10.4.1

|  | VTM, All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.44% | -0.14% |  | -0.05% |  |
| Class A2 | -0.22% | 0.04% |  | -0.01% |  |
| Class B | -0.16% | -0.03% |  | -0.01% |  |
| Class C | -0.08% | -0.02% |  | -0.10% |  |
| Class E | -0.14% | -0.03% |  | 0.04% |  |
| Overall | -0.19% | -0.03% |  | -0.03% |  |
| Enc Time [%] | 123% |  |  |  |  |
| Dec Time [%] | 101% |  |  |  |  |
| Class D | -0.04% |  | -0.09% |  | -0.09% |
| Class F | -0.01% |  | -0.04% |  | 0.05% |

|  | VTM, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.58% | -0.89% |  | -0.40% |  |
| Class A2 | -0.45% | -0.53% |  | -0.30% |  |
| Class B | -0.53% | -0.46% |  | -0.64% |  |
| Class C | -0.21% | -0.26% |  | 0.02% |  |
| Overall | -0.44% | -0.51% |  | -0.35% |  |
| Enc Time [%] | 113% |  |  |  |  |
| Dec Time [%] | 98% |  |  |  |  |
| Class D | -0.14% |  | -0.30% |  | -0.07% |
| Class F | -0.15% |  | -0.12% |  | 0.18% |

|  | VTM, Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.24% | -0.19% |  | -0.08% |  |
| Class C | -0.17% | 0.45% |  | 0.66% |  |
| Class E | -0.07% | 0.80% |  | 1.62% |  |
| Overall | -0.17% | 0.27% |  | 0.59% |  |
| Enc Time [%] | 114% |  |  |  |  |
| Dec Time [%] | 98% |  |  |  |  |
| Class D | -0.02% |  | 0.54% |  | 0.55% |
| Class F | 0.07% |  | 0.38% |  | 0.25% |

|  | VTM, Low Delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.94% | -0.50% |  | -0.68% |  |
| Class C | -0.58% | -0.01% |  | 0.15% |  |
| Class E | -1.09% | -0.18% |  | 0.48% |  |
| Overall | -0.86% | -0.26% |  | -0.11% |  |
| Enc Time [%] | 119% |  |  |  |  |
| Dec Time [%] | 97% |  |  |  |  |
| Class D | -0.36% |  | -0.26% |  | -0.10% |
| Class F | -0.37% |  | -0.62% |  | -0.07% |

## CE10.4.2

In case of temporal layer depth larger than 3, no diffusion filter is applied.

|  | VTM, All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.41% | -0.10% |  | -0.12% |  |
| Class A2 | -0.20% | -0.03% |  | -0.04% |  |
| Class B | -0.14% | -0.05% |  | -0.04% |  |
| Class C | -0.06% | 0.00% |  | -0.11% |  |
| Class E | -0.12% | 0.06% |  | 0.08% |  |
| Overall | -0.17% | -0.03% |  | -0.05% |  |
| Enc Time [%] | 119% |  |  |  |  |
| Dec Time [%] | 101% |  |  |  |  |
| Class D | -0.05% |  | -0.04% |  | 0.01% |
| Class F | -0.02% |  | -0.05% |  | 0.09% |

|  | VTM, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.56% | -0.85% |  | -0.41% |  |
| Class A2 | -0.44% | -0.53% |  | -0.35% |  |
| Class B | -0.50% | -0.61% |  | -0.72% |  |
| Class C | -0.21% | -0.22% |  | -0.10% |  |
| Overall | -0.42% | -0.54% |  | -0.42% |  |
| Enc Time [%] | 111% |  |  |  |  |
| Dec Time [%] | 98% |  |  |  |  |
| Class D | -0.15% |  | -0.37% |  | -0.08% |
| Class F | -0.16% |  | -0.21% |  | -0.01% |

|  | VTM, Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.24% | -0.03% |  | -0.36% |  |
| Class C | -0.12% | 0.48% |  | 0.60% |  |
| Class E | -0.12% | 1.06% |  | 1.50% |  |
| Overall | -0.17% | 0.41% |  | 0.43% |  |
| Enc Time [%] | 113% |  |  |  |  |
| Dec Time [%] | 98% |  |  |  |  |
| Class D | -0.03% |  | 0.62% |  | 0.37% |
| Class F | -0.02% |  | 0.16% |  | -0.09% |

|  | VTM, Low Delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.93% | -0.45% |  | -0.33% |  |
| Class C | -0.56% | 0.01% |  | 0.31% |  |
| Class E | -1.17% | 0.14% |  | -0.29% |  |
| Overall | -0.87% | -0.15% |  | -0.10% |  |
| Enc Time [%] | 118% |  |  |  |  |
| Dec Time [%] | 96% |  |  |  |  |
| Class D | -0.36% |  | -0.31% |  | -0.14% |
| Class F | -0.22% |  | -0.61% |  | -0.03% |

# References

- C.W. Hsu, M. Winken, “Description of Core Experiment 10 (CE10): Combined and multi-hypothesis prediction,” JVET-L1030, Macao, CN, 3–12 Oct. 2018.

- J. Pfaff et al., “Signal Adaptive Diffusion Filters For Video Coding,” JVET-J0038, San Diego, USA, April 2018.

- J. Rasch et al., “CE 10: Signal Adaptive Diffusion Filters For Video Coding (Test 10.4.1-10.4.5)” JVET-K323, Ljubljana, SI, 10–18 July 2018.

- J. Rasch et al., “CE10: Uniform Directional Diffusion Filters For Video Coding (Test 10.4.1-10.4.2)” JVET-L0157, Macao, CN, 3–12 Oct. 2018.

- F.Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).