| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0043 |
| --- | --- |

| Title: | CE3: Affine linear weighted intra prediction (test 1.2.1, test 1.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Pfaff, Björn Stallenberger, Michael Schäfer, Philipp Merkle, Philipp Helle, Roman Rischke, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-642 jonathan.pfaff@hhi.fraunhofer.de,philipp.merkle@hhi.fraunhofer.de, philipp.helle@hhi.franunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports results of the core experiment tests CE3-1.2.1 and CE3-1.2.2, both treating affine linear weighted intra prediction modes. These modes generate the luma intra prediction signal out of one line of reference samples left and above a current block by a matrix vector multiplication and the addition of an offset. 	
In the first CE, at most twelve multiplications per sample are needed and the total memory requirement is 0.273 Megabyte of memory. The reported results are -1.36% luma BD-rate-gain for the AI and -0.85% luma BD-rate-gain for the RA configuration.	 The encoding time overheads are 153% resp. 113%. 
In the second CE, at most four multiplications per sample are needed and the total memory requirement is 0.018 Megabyte of memory. The reported results of the second CE are -0.95% luma BD-rate-gain for the AI and -0.57% luma BD-rate-gain for the RA configuration. The encoding time overheads are 153% resp. 110%. It is reported that in terms of operational complexity, the proposed intra prediction modes of the second CE do not exceed the conventional intra prediction modes and that in terms of memory requirement they do not exceed the CPR tool when restricted to the current CTU. 
For the second CE, two further results with reduced encoder complexity are presented: -0.85% and -0.57% luma BD-rate gain at 136% resp. 110% encoder overhead for the AI resp. RA configuration and -0.75% resp. -0.49%  luma BD-rate gain at 127%  resp. 106% encoder overhead for the AI resp. RA configuration.

# Test CE3-1.2.1: Affine linear weighted intra prediction modes with encoder speedup

## Description of the method

The affine linear weighted intra prediction (ALWIP) modes tested in CE3-1.2.1 are the same as proposed in JVET-L0199 [1] under test CE3-2.2.2, except for the following changes:

- Harmonization with multiple reference line (MRL) intra prediction, especially encoder estimation and signaling, i.e. MRL is not combined with ALWIP and transmitting an MRL index is restricted to non-ALWIP blocks.

- Subsampling now mandatory for all blocks W×H ≥ 32×32 (was optional for 32×32 before); therefore, the additional test at the encoder and sending the subsampling flag has been removed.

- ALWIP for 64×N and N×64 blocks (with N ≤ 32) has been added by downsampling to 32×N and N×32, respectively, and applying the corresponding ALWIP modes.

Moreover, test CE3-1.2.1 includes the following encoder optimizations for ALWIP:

- Combined mode estimation: conventional and ALWIP modes use a shared Hadamard candidate list for full RD estimation, i.e. the ALWIP mode candidates are added to the same list as the conventional (and MRL) mode candidates based on the Hadamard cost.

- EMT intra fast and PB intra fast are supported for the combined mode list, with additional optimizations for reducing the number of full RD checks.

- Only MPMs of available left and above blocks are added to the list for full RD estimation for ALWIP, following the same approach as for conventional modes.

## Complexity assessment

In test CE3-1.2.1, excluding computations invoking the Discrete Cosine Transform, at most 12 multiplications per sample are required to generate the prediction signals. Moreover, a total number of 136 492 parameters, each in 16 bits, are required. This corresponds to 0.273 Megabyte of memory.

## Experimental results

Evaluation of the test was performed according to the common test conditions JVET-L1010 [2], for the intra-only (AI) and random-access (RA) configurations with the VTM software version 3.0. . The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler.

Table 1. Result of CE3-1.2.1 for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -2.08% | -1.68% | -1.60% | 155% | 104% |
| Class A2 | -1.18% | -0.90% | -0.84% | 153% | 103% |
| Class B | -1.18% | -0.84% | -0.83% | 155% | 104% |
| Class C | -0.94% | -0.63% | -0.76% | 148% | 106% |
| Class E | -1.71% | -1.28% | -1.21% | 154% | 106% |
| Overall | -1.36% | -1.02% | -1.01% | 153% | 105% |
| Class D | -0.99% | -0.61% | -0.76% | 145% | 107% |
| Class F | -1.38% | -1.23% | -1.04% | 147% | 104% |

Table 2. Result of CE3-1.2.1 for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.25% | -1.80% | -1.95% | 112% | 99% |
| Class A2 | -0.68% | -0.54% | -0.21% | 110% | 100% |
| Class B | -0.82% | -0.72% | -0.97% | 113% | 99% |
| Class C | -0.70% | -0.79% | -0.82% | 113% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.85% | -0.92% | -0.98% | 112% | 99% |
| Class D | -0.65% | -1.06% | -0.51% | 112% | 101% |
| Class F | -1.07% | -1.04% | -0.96% | 117% | 99% |

# Test CE3-1.2.2: Affine linear weighted intra prediction modes with complexity reduction

The technique tested in CE3-1.2.2 is related to “Affine Linear Intra Predictions” described in JVET-L0199 [1], but simplifies it in terms of memory requirements and computational complexity:

- There are only three different sets of prediction matrices and bias vectors covering all block shapes. As a consequence, the number of parameters is reduced to 14400 10-bit values, which is less memory than stored in a 128×128 CTU .

- The input and output size of the predictors is further reduced. Moreover, instead of transforming the boundary via DCT, averaging is performed to the boundary samples and the generation of the prediction signal uses linear interpolation instead of the inverse DCT. Consequently, a maximum of four multiplications per sample is necessary for generating the prediction signal.

## Description of the method

For predicting the samples of a rectangular block of width W and height H, affine linear weighted intra prediction (ALWIP) takes one line of H reconstructed neighbouring boundary samples left of the block and one line of W reconstructed neighbouring boundary samples above the block as input. If the reconstructed samples are unavailable, they are generated as it is done in the conventional intra prediction. 	
The generation of the prediction signal is based on the following three steps:

- Out of the boundary samples, four samples in the case of W=H=4 and eight samples in all other case are extracted by averaging.

- A matrix vector multiplication, followed by addition of an offset, is carried out with the averaged samples as an input. The result is a reduced prediction signal on a subsampled set of samples in the original block.

- The prediction signal at the remaining position is generated from the prediction signal on the subsampled set by linear interpolation.

The total number of multiplications needed in the computation of the matrix-vector product is always smaller or equal than 4*W*H. Moreover, the averaging operations on the boundary and the linear interpolation of the reduced prediction signal are carried out by solely using additions and bit-shifts. In other words, at most four multiplications per sample are needed for the ALWIP modes. 	
The matrices and offset vectors needed to generate the prediction signal are taken from three sets S0, S1, S2 of matrices. The set S0 consists of 18 matrices A0i, i∈{0,…,17} each of which has 16 rows and 4 columns and 18 offset vectors b0i, i∈{0,…,17} each of size 16. Matrices and offset vectors of that set are used for blocks of size 4×4. The set S1 consists of 18 matrices A1i, i∈{0,…,17}, each of which has 16 rows and 8 columns and 18 offset vectors  b1i, i∈{0,…,17} each of size 16. Matrices and offset vectors of that set are used for blocks of sizes 4×8, 8×4 and 8×8. Finally, the set S2 consists of 18 matrices A2i, i∈{0,…,17}, each of which has 64 rows and 8 columns and of 18 offset vectors  b2i, i∈{0,…,17} of size 64. Matrices and offset vectors of that set or parts of these matrices and offset vectors are used for all other block-shapes.

## Averaging of the boundary

In a first step, the input boundaries bdrytop and bdryleft  are reduced to smaller boundaries bdryredtop and  bdryredleft. Here, bdryredtop and  bdryredleft both consists of 2 samples in the case of a 4x4-block and both consist of 4 samples in all other cases.

In the case of a 4x4-block, one defines

bdryredtop0=(bdrytop0+bdrytop1+1)≫1,

bdryredtop1=(bdrytop2+bdrytop3+1)≫1,

and defines  bdryredleft analogously.

In all other cases, if the block-width W is given as W=4*2k, for 0≤i<4 one defines

bdryredtopi=j=02k-1bdrytopi*2k+j+1≪(k-1)≫k.

and defines bdryredleft analogously.	
The two reduced boundaries bdryredtop and bdryredleft are concatenated to a reduced boundary vector bdryred which is thus of size four for blocks of shape 4×4  and of size eight for blocks of all other shapes. Here, if mode<18, one puts

bdryred=bdryredtop , bdryredleft.

If mode≥18, which corresponds to the transposed mode of mode-17, one puts

bdryred= bdryredleft, bdryredtop .

Finally, for the interpolation of the subsampled prediction signal, on large blocks a second version of the averaged boundary is needed. Namely, if minW,H>8  and W≥H, one writes W=8*2l, and for 0≤i<8 defines

bdryredIItopi=j=02l-1bdrytopi*2l+j+1≪(l-1)≫l.

If minW,H>8  and H>W, one defines bdryredIIleft analogously.

## Generation of the reduced prediction signal by matrix vector multiplication

Out of the reduced input vector bdryred one generates a reduced prediction signal predred . The latter signal is a signal on the downsampled block of with Wred and height Hred. Here, Wred and Hred are defined as:

Wred=4,  Hred=4;     if maxW,H≤8,

Wred=minW,8,  Hred=minH,8;       else.

The reduced prediction signal predred is computed by calculating a matrix vector-product and adding an offset:

predred=A∙bdryred+b.

Here, A is a matrix that has Wred* Hred rows and 4 columns if W=H=4 and 8 columns in all other cases and b is a vector of size Wred* Hred.	
If W=H=4, then A has 4 columns and 16 rows and thus 4 multiplications per sample are needed in that case to compute predred. In all other cases, A has 8 columns and one immediately verifies that in these cases one has 8* Wred* Hred≤4*W*H, i.e. also in these cases, at most 4 multiplications per sample are needed to compute predred.  		
The matrix A and the vector b are taken from one of the sets S0, S1, S2 as follows. One defines an index idx=idxW,H by setting idxW,H=0, if W=H=4, idxW,H=1, if maxW,H=8  and idxW,H=2 in all other cases. Moreover, one puts m=mode, if mode<18 and  m=mode-17, else. Then, if idx≤1  or idx=2 and minW,H>4, one puts A=Aidxm and b=bidxm. In the case that  idx=2 and minW,H=4, one lets  A be the matrix that arises by leaving out every row of Aidxm that, in the case W=4, corresponds to an odd x-coordinate in the downsampled block, or, in the case H=4, corresponds	 to an odd y-coordinate in the downsampled block. If mode≥18, one replaces the reduced prediction signal by its transposed signal.

## Linear interpolation to generate the final prediction signal

At the sample positions that were left out in the generation of predred, the final prediction signal arises by linear interpolation from predred. This linear interpolation is not needed if W=H=4.  	
The linear interpolation is given as follows. It is assumed that W≥H. Then, if H>Hred,  a vertical upsampling of predred is performed. In that case, predred is extended by one line to the top as follows. If W=8, predred has width Wred=4 and is extended to the top by the averaged boundary signal bdryredtop, defined above.  If W>8, predred is of width  Wred=8 and it is extended to the top by the averaged boundary signal bdryredIItop, defined above.  One writes  predred x-1 for the first line of predred. Then the signal predredups,ver on a block of  width Wred and height 2*Hred is given as

predredups,ver[x][2*y+1] =predred [x][y],

predredups,ver[x][2*y] = (predred[x][y-1] +  predred[x][y] + 1)>>1,

where 0≤x<  Wred and 	0≤y<Hred. The latter process is carried out k times until 2k*Hred=H. Thus, if H=8 or H=16, it is carried out at most once. If H=32, it is carried out twice. If H=64, it is carried out three times. Next, a horizontal upsampling operation is applied to the result of the vertical upsampling. The latter upsampling operation uses the full boundary left of the prediction signal. Finally, if H>W, one proceeds analogously by first upsampling in the horizontal direction (if required) and then in the vertical direction.

## Illustration of the entire ALWIP process

The entire process of averaging, matrix-vector-multiplication and linear interpolation is illustrated for different shapes in Figs. 1-4. Note, that the remaining shapes are treated as in one of the depicted cases.

- Given a 4×4  block, ALWIP takes two averages along each axis of the boundary. The resulting four input samples enter the matrix-vector-multiplication. The matrices are taken from the set S0. After adding an offset, this yields the 16 final prediction samples. Linear interpolation is not necessary for generating the prediction signal. Thus, a total of (4*16)/(4*4)=4 multiplications per sample are performed.

Figure 1: Illustration of ALWIP for 4×4 blocks

- Given an 8×8  block, ALWIP takes four averages along each axis of the boundary. The resulting eight input samples enter the matrix-vector-multiplication. The matrices are taken from the set S1. This yields 16 samples on the odd positions of the prediction block. Thus, a total of (8*16)/(8*8)=2 multiplications per sample are performed. After adding an offset, these samples are interpolated vertically by using the reduced top boundary. Horizontal interpolation follows by using the original left boundary.

Figure 2: Illustration of ALWIP for 8×8 blocks

- Given an 8×4  block, ALWIP takes four averages along the horizontal axis of the boundary and the four original boundary values on the left boundary. The resulting eight input samples enter the matrix-vector-multiplication. The matrices are taken from the set S1. This yields 16 samples on the odd horizontal and each vertical positions of the prediction block. Thus, a total of (8*16)/(8*4)=4 multiplications per sample are performed. After adding an offset, these samples are interpolated horizontally by using the original left boundary.

Figure 3: Illustration of ALWIP for 8×4 blocks

The transposed case is treated accordingly.

- Given a 16×16  block, ALWIP takes four averages along each axis of the boundary. The resulting eight input samples enter the matrix-vector-multiplication. The matrices are taken from the set S2. This yields 64 samples on the odd positions of the prediction block. Thus, a total of (8*64)/(16*16)=2 multiplications per sample are performed. After adding an offset, these samples are interpolated vertically by using eight averages of the top boundary. Horizontal interpolation follows by using the original left boundary.

Figure 4: Illustration of ALWIP for 16×16 blocks

For larger shapes, the procedure is essentially the same and it is easy to check that the number of multiplications per sample is less than two.

For W×8 blocks, only horizontal interpolation is necessary as the samples are given at the odd horizontal and each vertical positions. Thus, at most (8*64)/(16*8)=4 multiplications per sample are performed in these cases.

Finally for W×4 blocks with W>8, let Akbe the matrix that arises by leaving out every row that correspond to an odd entry along the horizontal axis of the downsampled block. Thus, the output size is 32 and again, only horizontal interpolation remains to be performed. At most (8*32)/(16*4)=4 multiplications per sample are performed.

The transposed cases are treated accordingly.

## Number of parameters needed and complexity assessment

The parameters needed for all possible proposed intra prediction modes are comprised by the matrices and offset vectors belonging to the sets S0, S1, S2. All matrix-coefficients and offset vectors are stored as 10-bit values. Thus, according to the above description, a total number of 14400 parameters, each in 10-bit precision, is needed for the proposed method. This corresponds to 0.018 Megabyte of memory. It is pointed out that currently, a CTU of size 128×128 in the standard 4:2:0 chroma-subsampling consists of 24576 values, each in 10 bit. Thus, the memory requirement of the proposed intra-prediction tool does not exceed the memory requirement of the current picture referencing tool that was adopted at the last meeting. Also, it is pointed out that the conventional intra prediction modes require four multiplications per sample due to the PDPC tool or the 4-tap interpolation filters for the angular prediction modes with fractional angle positions. Thus, in terms of operational complexity the proposed method does not exceed the conventional intra prediction modes.

## Signalization of the proposed intra prediction modes

For luma blocks, 35 ALWIP modes are proposed. For each Coding Unit (CU) in intra mode, a flag indicating if an ALWIP mode is to be applied on the corresponding Prediction Unit (PU) or not is sent in the bitstream. The signalization of the latter index is harmonized with MRL in the same way as for the first CE test. If an ALWIP mode is to be applied, the index predmode of the ALWIP mode is signaled using an MPM-list with 3 MPMS. 	
Here, the derivation of the MPMs is performed using the intra-modes of the above and the left PU as follows. There are three fixed tables map_angular_to_alwipidx, idx∈{0,1,2}  that assign to each conventional intra prediction mode predmodeAngular an ALWIP mode

predmodeALWIP= map_angular_to_alwipidx[predmodeAngular].

For each PU of width W and height H one defines and index

idxPU=idx(W,H)∈{0,1,2}

that indicates from which of the three sets the ALWIP-parameters are to be taken as in section 4 above. If the above Prediction Unit PUabove is available, belongs to the same CTU as the current PU and is in intra mode, if idxPU=idx(PUabove) and if ALWIP is applied on PUabove with ALWIP-mode predmodeALWIPabove, one puts

modeALWIPabove=predmodeALWIPabove .

If the above PU is available, belongs to the same CTU as the current PU and is in intra mode and if a conventional intra prediction mode predmodeAngularabove is applied on the above PU, one puts

modeALWIPabove=map_angular_to_alwipidx(PUabove)[predmodeAngularabove].

In all other cases, one puts

modeALWIPabove=-1

which means that this mode is unavailable. In the same way but without the restriction that the left PU needs to belong to the same CTU as the current PU, one derives a mode

modeALWIPleft .

Finally, three fixed default lists listidx, idx∈{0,1,2}   are provided, each of which contains three distinct ALWIP modes. Out of the default list listidx(PU) and the modes  modeALWIPabove and modeALWIPleft, one constructs three distinct MPMs by substituting -1 by default values as well as eliminating repetitions.

## Adapted MPM-list derivation for conventional luma and chroma intra-prediction modes

The proposed ALWIP-modes are harmonized with the MPM-based coding of the conventional intra-prediction modes as follows. The luma and chroma MPM-list derivation processes for the conventional intra-prediction modes use fixed tables map_lwip_to_angularidx, idx∈{0,1,2},   mapping an ALWIP-mode predmodeLWIP on a given PU to one of the conventional intra-prediction modes

predmodeAngular=map_lwip_to_angularidx(PU)[predmodeLWIP].

For the luma MPM-list derivation, whenever a neighboring luma block is encountered which uses an ALWIP-mode predmodeLWIP, this block is treated as if it was using the conventional intra-prediction mode predmodeAngular. For the chroma MPM-list derivation, whenever the current luma block uses an LWIP-mode, the same mapping is used to translate the ALWIP-mode to a conventional intra prediction mode.

## Experimental results

Evaluation of the test was performed according to the common test conditions JVET-L1010 [2], for the intra-only (AI) and random-access (RA) configurations with the VTM software version 3.0. . The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler.

Table 3. Result of CE3-1.2.2 for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.38% | -0.89% | -0.75% | 152% | 103% |
| Class A2 | -0.75% | -0.25% | -0.24% | 152% | 102% |
| Class B | -0.79% | -0.27% | -0.30% | 155% | 101% |
| Class C | -0.86% | -0.41% | -0.56% | 154% | 102% |
| Class E | -1.11% | -0.40% | -0.49% | 151% | 97% |
| Overall | -0.95% | -0.42% | -0.46% | 153% | 101% |
| Class D | -0.94% | -0.58% | -0.38% | 154% | 102% |
| Class F | -1.01% | -0.64% | -0.63% | 151% | 100% |

Table 4. Result of CE3-1.2.2 for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.90% | -1.56% | -1.61% | 111% | 100% |
| Class A2 | -0.42% | -0.24% | -0.15% | 109% | 102% |
| Class B | -0.52% | -0.52% | -0.99% | 111% | 98% |
| Class C | -0.52% | -0.71% | -0.56% | 110% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.57% | -0.73% | -0.83% | 110% | 98% |
| Class D | -0.54% | -0.77% | -0.70% | 110% | 101% |
| Class F | -0.72% | -0.66% | -0.45% | 112% | 97% |

## Additional results with further encoder speedups

We additionally provide two further results for tests that relied on the same syntax as CE3-1.2.2 but with an optimized encoder search.

Table 5. Result of CE3-1.2.2 with first encoder speedup for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.30% | -0.72% | -0.77% | 137% | 104% |
| Class A2 | -0.65% | -0.21% | -0.20% | 136% | 104% |
| Class B | -0.69% | -0.22% | -0.22% | 137% | 102% |
| Class C | -0.74% | -0.37% | -0.47% | 134% | 103% |
| Class E | -1.00% | -0.42% | -0.39% | 137% | 101% |
| Overall | -0.85% | -0.37% | -0.39% | 136% | 103% |
| Class D | -0.86% | -0.32% | -0.39% | 131% | 103% |
| Class F | -0.92% | -0.68% | -0.63% | 132% | 102% |

Table 6. Result of CE3-1.2.2 with first encoder speedup for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.91% | -1.59% | -1.71% | 110% | 100% |
| Class A2 | -0.39% | -0.26% | -0.06% | 109% | 102% |
| Class B | -0.51% | -0.42% | -1.03% | 110% | 100% |
| Class C | -0.53% | -0.68% | -0.55% | 111% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.57% | -0.69% | -0.84% | 110% | 101% |
| Class D | -0.51% | -0.75% | -0.59% | 110% | 103% |
| Class F | -0.73% | -0.74% | -0.58% | 112% | 101% |

Table 7. Result of CE3-1.2.2 with second encoder speedup for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.13% | -0.71% | -0.67% | 127% | 103% |
| Class A2 | -0.58% | -0.18% | -0.13% | 127% | 102% |
| Class B | -0.59% | -0.25% | -0.29% | 128% | 99% |
| Class C | -0.67% | -0.37% | -0.45% | 125% | 101% |
| Class E | -0.90% | -0.46% | -0.37% | 127% | 100% |
| Overall | -0.75% | -0.38% | -0.38% | 127% | 101% |
| Class D | -0.75% | -0.40% | -0.37% | 124% | 102% |
| Class F | -0.71% | -0.46% | -0.39% | 124% | 100% |

Table 8. Result of CE3-1.2.2 with second encoder speedup for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.79% | -1.54% | -1.44% | 107% | 98% |
| Class A2 | -0.36% | -0.28% | -0.02% | 106% | 100% |
| Class B | -0.43% | -0.41% | -0.64% | 107% | 97% |
| Class C | -0.44% | -0.68% | -0.42% | 106% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.49% | -0.68% | -0.62% | 107% | 97% |
| Class D | -0.43% | -1.09% | -0.66% | 106% | 101% |
| Class F | -0.59% | -0.47% | -0.41% | 107% | 94% |

# Cross-check

Tests CE3-1.2.1 and CE3-1.2.2 were cross-checked by Alexey Filippov (Huawei) and Fabien Racape (Technicolor) and the results are included in the CE3 report [3].

# Conclusion

The proposed affine linear intra prediction modes of test CE3-1.2.1 lead to coding gains of -1.36% resp. -0.85% in the AI resp. RA configuration while the proposed affine linear intra prediction modes of test CE3-1.2.2 lead to coding gains of -0.95% resp. -0.57% in the AI resp. RA configuration. Moreover, the second CE has shown that keeping most of the coding gain one can reduce the operational complexity and the memory requirements of the proposed affine linear intra prediction modes so that they do not exceed already adopted intra prediction tools.

# References

- P. Helle et al., “Non-linear weighted intra prediction”, JVET-L0199, Macao, China, October 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Macao, China, October 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “CE3: Summary report on intra prediction and mode coding”, JVET-M0023, Marrakech, Morocco, January 2019.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).