| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0046-v1 |
| --- | --- |

| Title: | CE6-related: A study of primary transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou and Yi Hu 16340 West Bernardo DR San Diego, CA-92127, USA | Tel: Email: | +1 858 521 5838 minhua.zhou@broadcom.com yi.hu@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

A 32x32 DST7/DCT8 based transform unit (TU) is about 60% more computationally intensive than a 32x32 DCT2 based TU. To bring the implementation cost of 32-point DST7/DCT8 to a reasonable level, it is proposed to keep non-zero transform coefficients only in the top-left 16x16 region of a 32x32/32x16 or 16x32 DST7/DCT8 based TU. Experimental results reveal that in CTC (i.e. the MTS is enabled in intra-coded CUs only) the zero-out leads to an average coding loss of 0.12% in AI, 0.04% in RA, 0.02% in LD_B and -0.02% in LD_P, respectively. When the MTS is enabled in both the intra- and inter-coded CUs, the average coding efficiency loss by using the zero-out is 0.12% in AI, 0.05% in RA, 0.14% in LD_B and 0.12% in LD_P, respectively.

# Introduction

In this document, a computational complexity analysis has been conducted for the current VVC primary transform design; the 32-point DST7/DCT8 based transforms have been identified as the bottleneck for implementation. It is proposed to keep non-zero coefficients only in the top-left 16x16 region  and zero-out the rest of coefficients in a 32x32/32x16 or 16x32 DST7/DCT8 based transform unit (TU), so as to reduce the implementation cost of 32-point DST7/DCT8 to an acceptable level.

# Problem statement

In the current draft VVC spec [2], DST7/DCT8 transforms up to 32-point, and DCT2 transforms up to 64-point are defined for primary transforms. All the transforms use 8-bit transform matrix coefficients and can be implemented with direct matrix multiply. Furthermore, the DCT2 transforms are exactly same as the ones defined in the HEVC, which can be implemented in a structure of half butterfly and half matrix multiply. To reduce the implementation cost of large size transforms, only the top-left 32x32 non-zero transform coefficients are kept and the rest of coefficients are set to zero for 64x64/64x32/32x64 DCT2 based TUs.

Figure 1. Illustration of keeping non-zero transform coefficients in the top-left mxn region of a MxN size TU

For a MxN TU and if non-zero transform coefficients are kept for the top-left mxn region of the TU, the number of multiplications per coefficient for the inverse transform implemented in the direct matrix multiply structure can be computed by

Muls_per_coeffdirectMatrixMultiply=mMn+m

Likewise, the number of multiplications per coefficient for the inverse transform implemented in the structure of half butterfly and half direct matrix multiply can be computed by

Muls_per_coeffhalfButterFly=mM(n2+log2n2)+(m2+log2m2)

Table 1. Number of multiplications per transform coefficient of different transforms sizes

For 64x64/64x32/32x64/32x32 DCT2 based TUs and for 32x32/32x16/16x32 DST7/DCT8 based TUs, the numbers of multiplications per transform coefficient are summarized in Table 1. Table 1 reveals that

- Although the transform size of DCT2 is increased from 32-point of the HEVC to 64-point in the VVC, the computational complexity of 64x64/64x32/32x64 DCT2 based TUs is still less than or equal to that of a HEVC 32x32 DCT2 TU, which is around 40 multiplications per coefficient if the inverse transform is implemented in a structure of half butterfly and half matrix multiply.  Keeping non-zero transform coefficients in the top-left 32x32 region in those large TUs reduces not only the intermediate transform buffer size but also the computational complexity.

- DST7/DCT8 can only be implemented in direct matrix multiply,  the computational complexity 32x32/32x16/16x32 DST7/DCT8 based TUs exceeds that of a HEVC 32x32 DCT2 TU (marked yellow in Table 1). In particular, a 32x32 DST7/DCT8 based TU requires 64 multiplications per coefficient, which is significantly higher than 40 multiplications per coefficient of a HEVC 32x32 DCT2 based TU.

The HEVC 32x32 DCT2 based transform is very costly because it already requires running about 64 multipliers per cycle for 4K@60 video.  Supporting 32x32 DST7/DCT8 transforms will likely to push the design to implement 128 parallel multipliers (i.e. 128 multiplications per cycle) to meet the required sample processing rate of 4K@60 video. It is hence desirable to reduce the implementation cost of 32-point DST7/DCT8 in the current design.

A straightforward method for cost reduction is to completely remove the 32-point DST7/DCT8 transforms from the current design. Alternatively, keeping non-zero transform coefficients in the top-left 16x16 region only in the 32x32/32x16/16x32 DST7/DCT8 based TUs will also bring the implementation cost of those TUs down to a level that is less than a HEVC 32x32 DCT2 TU as shown in Table 1 (marked green in Table 1).

# Experimental results

Tests were carried out by using the VTM3.0 software, and the common test conditions (CTC) defined in [1] were followed. Additional tests were done by enabling the MTS in both the intra and inter-coded CUs. Note that in the current CTC, the MTS is enabled in intra-coded CUs only.

The following five tests were carried out:

- Test 1: VTM3.0 plus enabling the MTS in both the intra- and inter-coded CUs (the 2nd anchor).

- Test 2: disabling 32-point DST7/DCT8 and enabling the MTS in intra-coded CUs only.

- Test 3: disabling 32-point DST7/DCT8 and enabling the MTS in both intra- and inter-coded CUs.

- Test 4: keeping the non-zero transform coefficients only in top-left 16x16 region of a 32x32/32x16/16x32 DST7/DCT8 based TU, and enabling the MTS in intra-coded CUs only.

- Test 5: keeping the non-zero transform coefficients only in top-left 16x16 region of a 32x32/32x16/16x32 DST7/DCT8 based TU, and enabling the MTS in both intra- and inter-coded CUs.

The summary results of the tests are provided in Table 2. As shown in  Table 2, in CTC the coding efficiency loss by keeping non-zero coefficients only in the top-left 16x16 region of a 32x32/32x16 or 16x32 DSt7/DCT8 based transform unit is 0.12% in AI, 0.04% in RA, 0.02% in LD_B and -0.02% in LD_P, respectively. When the MTS is enabled in both the intra- and inter-coded CUs (i.e. relative to the 2nd anchor), the coding efficiency loss is 0.12% in AI, 0.05% in RA, 0.14% in LD_B and 0.12% in LD_P, respectively.

Table 2: Summary results

The detailed results are provided in Table 3 to Table 7, and can be found in the attached spreadsheets. Note that the run time may not be accurate due to the heterogeneous nature of the LSF farm.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 91% | 93% |
| Class A2 | 0.00% | 0.00% | 0.00% | 106% | 106% |
| Class B | 0.00% | 0.00% | 0.00% | 107% | 109% |
| Class C | 0.00% | 0.00% | 0.00% | 93% | 94% |
| Class E | 0.00% | 0.00% | 0.00% | 106% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 91% | 92% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 116% | 132% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | 0.45% | 0.50% | 120% | 108% |
| Class A2 | -0.60% | 0.55% | 0.53% | 121% | 107% |
| Class B | -0.42% | 0.66% | 0.52% | 117% | 117% |
| Class C | -0.25% | 0.50% | 0.66% | 119% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.37% | 0.55% | 0.56% | 119% | 109% |
| Class D | -0.40% | 0.57% | 0.51% | 116% | 95% |
| Class F (optional) | -0.08% | 0.36% | 0.27% | 106% | 93% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.76% | 0.46% | 0.99% | 142% | 114% |
| Class C | -0.51% | 1.22% | 1.08% | 137% | 106% |
| Class E | -0.22% | 1.23% | 1.30% | 135% | 115% |
| Overall | -0.54% | 0.91% | 1.10% | 138% | 112% |
| Class D | -0.62% | 1.06% | 1.38% | 136% | 102% |
| Class F (optional) | -0.18% | 0.75% | 0.75% | 120% | 95% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.80% | 1.16% | 1.27% | 132% | 102% |
| Class C | -0.55% | 1.03% | 1.04% | 141% | 98% |
| Class E | -0.31% | 0.80% | 1.13% | 155% | 115% |
| Overall | -0.59% | 1.03% | 1.16% | 141% | 104% |
| Class D | -0.67% | 1.22% | 0.58% | 167% | 130% |
| Class F (optional) | -0.37% | 0.03% | 0.35% | 146% | 102% |

Table 3: Summary results of Test 1 vs. VTM3.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.05% | 1.42% | 1.50% | 76% | 80% |
| Class A2 | 0.63% | 0.69% | 0.59% | 90% | 96% |
| Class B | 0.55% | 0.56% | 0.61% | 81% | 84% |
| Class C | 0.17% | 0.13% | 0.12% | 89% | 100% |
| Class E | 0.48% | 0.49% | 0.54% | 102% | 107% |
| Overall | 0.55% | 0.62% | 0.63% | 87% | 92% |
| Class D | 0.14% | 0.14% | 0.05% | 90% | 96% |
| Class F (optional) | 0.10% | 0.11% | 0.17% | 102% | 117% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.61% | 0.89% | 1.25% | 97% | 99% |
| Class A2 | 0.26% | 0.41% | 0.42% | 93% | 99% |
| Class B | 0.36% | 0.90% | 0.77% | 96% | 100% |
| Class C | 0.09% | 0.22% | 0.32% | 87% | 88% |
| Class E |  |  |  |  |  |
| Overall | 0.32% | 0.62% | 0.68% | 93% | 96% |
| Class D | 0.05% | -0.28% | -0.03% | 96% | 98% |
| Class F (optional) | 0.08% | 0.36% | 0.18% | 88% | 90% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.15% | 0.08% | 0.44% | 96% | 100% |
| Class C | 0.11% | 0.34% | 0.44% | 99% | 98% |
| Class E | 0.08% | 1.39% | -0.69% | 111% | 105% |
| Overall | 0.12% | 0.49% | 0.16% | 101% | 100% |
| Class D | 0.03% | 0.37% | 0.40% | 108% | 108% |
| Class F (optional) | 0.05% | 0.14% | 0.15% | 98% | 93% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.15% | 0.52% | 0.83% | 82% | 83% |
| Class C | 0.11% | 0.40% | 0.44% | 87% | 89% |
| Class E | -0.05% | 0.76% | -1.03% | 117% | 110% |
| Overall | 0.09% | 0.54% | 0.23% | 92% | 91% |
| Class D | 0.05% | -0.13% | 0.00% | 98% | 102% |
| Class F (optional) | 0.17% | -0.46% | 0.09% | 128% | 127% |

Table 4: Summary results of Test 2 vs. VTM3.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0+ MTS enabled in all CU types |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.05% | 1.42% | 1.50% | 100% | 98% |
| Class A2 | 0.63% | 0.69% | 0.59% | 79% | 84% |
| Class B | 0.55% | 0.56% | 0.61% | 79% | 88% |
| Class C | 0.17% | 0.13% | 0.12% | 103% | 114% |
| Class E | 0.48% | 0.49% | 0.54% | 92% | 95% |
| Overall | 0.55% | 0.62% | 0.63% | 90% | 95% |
| Class D | 0.14% | 0.14% | 0.05% | 93% | 91% |
| Class F (optional) | 0.10% | 0.11% | 0.17% | 83% | 78% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0+ MTS enabled in all CU types |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.72% | 0.73% | 0.98% | 78% | 83% |
| Class A2 | 0.47% | 0.25% | 0.16% | 78% | 89% |
| Class B | 0.53% | 0.58% | 0.61% | 84% | 80% |
| Class C | 0.15% | 0.14% | 0.02% | 82% | 91% |
| Class E |  |  |  |  |  |
| Overall | 0.45% | 0.43% | 0.44% | 81% | 85% |
| Class D | 0.16% | -0.17% | 0.08% | 98% | 109% |
| Class F (optional) | 0.11% | 0.24% | 0.14% | 90% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0+ MTS enabled in all CU types |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.55% | 0.17% | 0.34% | 83% | 97% |
| Class C | 0.20% | -0.24% | 0.12% | 88% | 100% |
| Class E | 0.22% | 0.64% | -1.12% | 92% | 105% |
| Overall | 0.35% | 0.15% | -0.10% | 87% | 100% |
| Class D | 0.20% | -0.03% | -0.36% | 90% | 99% |
| Class F (optional) | 0.10% | -0.31% | 0.63% | 89% | 98% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0+ MTS enabled in all CU types |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.50% | -0.04% | 0.01% | 76% | 86% |
| Class C | 0.17% | 0.12% | 0.01% | 82% | 97% |
| Class E | 0.12% | 0.82% | -0.85% | 69% | 89% |
| Overall | 0.30% | 0.23% | -0.21% | 76% | 91% |
| Class D | 0.17% | -0.80% | 0.27% | 73% | 78% |
| Class F (optional) | 0.34% | -0.01% | -0.11% | 84% | 105% |

Table 5: Summary results of Test 3 vs. Test 1 (the 2nd anchor)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.03% | 0.03% | 91% | 84% |
| Class A2 | 0.20% | 0.03% | -0.05% | 94% | 97% |
| Class B | 0.11% | 0.05% | 0.00% | 92% | 91% |
| Class C | 0.07% | 0.10% | -0.01% | 108% | 113% |
| Class E | 0.16% | 0.10% | 0.13% | 109% | 134% |
| Overall | 0.12% | 0.05% | 0.02% | 98% | 102% |
| Class D | 0.06% | 0.06% | -0.04% | 102% | 104% |
| Class F (optional) | 0.01% | -0.03% | -0.01% | 104% | 105% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.08% | 0.06% | 102% | 98% |
| Class A2 | 0.02% | -0.03% | 0.06% | 96% | 98% |
| Class B | 0.04% | 0.23% | 0.03% | 96% | 96% |
| Class C | 0.07% | -0.01% | 0.18% | 101% | 105% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.05% | 0.08% | 99% | 99% |
| Class D | 0.02% | -0.40% | -0.17% | 99% | 97% |
| Class F (optional) | 0.00% | 0.05% | 0.09% | 94% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.50% | -0.17% | 103% | 108% |
| Class C | 0.00% | 0.20% | 0.23% | 98% | 97% |
| Class E | 0.07% | 0.88% | -0.02% | 97% | 94% |
| Overall | 0.02% | 0.08% | 0.00% | 100% | 101% |
| Class D | 0.03% | 0.42% | 0.32% | 96% | 93% |
| Class F (optional) | 0.06% | 0.35% | -0.17% | 95% | 91% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.05% | -0.07% | 89% | 93% |
| Class C | 0.02% | 0.24% | 0.06% | 95% | 92% |
| Class E | -0.06% | 0.89% | -0.80% | 96% | 94% |
| Overall | -0.02% | 0.33% | -0.21% | 93% | 93% |
| Class D | 0.02% | -0.21% | -0.13% | 94% | 100% |
| Class F (optional) | 0.08% | -0.22% | -0.10% | 97% | 94% |

Table 6: Summary results of Test 4 vs. VTM3.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0+ MTS enabled in all CU types |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.03% | 0.03% | 100% | 91% |
| Class A2 | 0.20% | 0.03% | -0.05% | 89% | 92% |
| Class B | 0.11% | 0.05% | 0.00% | 86% | 84% |
| Class C | 0.07% | 0.10% | -0.01% | 116% | 121% |
| Class E | 0.16% | 0.10% | 0.13% | 103% | 129% |
| Overall | 0.12% | 0.05% | 0.02% | 98% | 100% |
| Class D | 0.06% | 0.06% | -0.04% | 112% | 112% |
| Class F (optional) | 0.01% | -0.03% | -0.01% | 89% | 79% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0+ MTS enabled in all CU types |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.09% | -0.07% | 92% | 91% |
| Class A2 | 0.04% | -0.13% | 0.03% | 88% | 90% |
| Class B | 0.08% | 0.10% | 0.18% | 97% | 83% |
| Class C | 0.07% | 0.09% | 0.18% | 97% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.01% | 0.10% | 94% | 89% |
| Class D | 0.05% | -0.41% | -0.43% | 97% | 95% |
| Class F (optional) | 0.03% | -0.02% | 0.08% | 94% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0+ MTS enabled in all CU types |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | -0.03% | -0.03% | 90% | 92% |
| Class C | 0.13% | -0.01% | 0.26% | 85% | 83% |
| Class E | 0.19% | 0.46% | -0.51% | 96% | 95% |
| Overall | 0.14% | 0.10% | -0.05% | 90% | 90% |
| Class D | 0.13% | -0.25% | 0.06% | 90% | 94% |
| Class F (optional) | -0.06% | 0.27% | -0.02% | 97% | 96% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0+ MTS enabled in all CU types |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.33% | 0.02% | 86% | 85% |
| Class C | 0.10% | -0.10% | -0.08% | 96% | 103% |
| Class E | 0.19% | 0.87% | -0.80% | 75% | 83% |
| Overall | 0.12% | 0.05% | -0.22% | 86% | 90% |
| Class D | 0.09% | -0.71% | 0.17% | 74% | 72% |
| Class F (optional) | 0.05% | 0.31% | 0.80% | 88% | 98% |

Table 7: Summary results of Test 5 vs. Test 1 (the 2nd anchor)

# Recommendation

Based on the experimental results, it is recommended to keep non-zero transform coefficients in the top-left 16x16 region only in the 32x32/32x16/16x32 DS7/DCT8 based transform units.

The authors would like to thank InterDigital for cross-check.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001-v7, the 12th JVET meeting, the 12th JVET meeting, Oct. 2018, Macao, CN.

[3] A. Said and X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, JVET-L1026, the 12th JVET meeting, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.

# Spec Text Changes relative to JVET-L1001-v7

In section 8.4.4.1 General

Replace

The variables nonZeroW and nonZeroH are derived as follows:

nonZeroW = Min( nTbW, 32 )		(8-564)

nonZeroH = Min( nTbH, 32 )		(8-565)

with

The variable nonZeroW is derived as follows:

- If trTypeHor is equal to 0

nonZeroW = Min( nTbW, 32 )		(8-564)

- Otherwise, if trTypeHor is equal to 1 or 2

nonZeroW = Min( nTbW, 16 )		(8-565)

The variable nonZeroH is derived as follows:

- If trTypeVer is equal to 0,

nonZeroH = Min( nTbH, 32 )		(8-566)

- Otherwise, if trTypeVer is equal to 1 or 2

nonZeroH = Min( nTbH, 16 )		(8-567)