| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0049-v1 |
| --- | --- |

| Title: | CE2-related: A restriction on memory bandwidth consumption of affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo DR San Diego, CA-92127, USA | Tel: Email: | +1 858 521 5838 minhua.zhou@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

To constrain the memory bandwidth consumption of the affine mode at PU level, it is proposed to impose a bitstream constraint to disallow the affine model parameters that can lead to the reference block bounding box size exceeding a pre-defined threshold. For the setting in which the threshold of luma reference block bounding box is set to 16*16 for 2x2 sub-block vectors of bi-directional affine mode, and  to16*12 for 2x1 sub-block vectors of unidirectional affine mode, the BD-rate difference is 0.05% in RA, 0.04% in LD_B and -0.02% in LD_P, respectively. For the setting of threshold 17*17 for 2x2 sub-block vectors of bi-directional affine mode and  17*13 for 2x1 sub-block vectors of unidirectional affine mode, the BD-rate difference is 0.01% in RA, 0.06% in LD_B and 0.01% in LD_P, respectively.

# Introduction

The affine motion compensation prediction introduces a more complex motion model for better compression efficiency. In the affine motion model, the motion vector v=(vx,vy) at a sample position (x,y) inside a PU is defined as:

vx=ax+cy+evy=bx+dy+f               Equation 1

Where a,b,c,d,e   and f are the affine motion model parameters. In the VTM3.0, 4x4 sub-block motion vectors are generated and used in the motion compensation of a PU coded in affine mode

The model parameters a,b,c,d determine how far the sub-block motion vectors can go apart from each other within a PU. In the worst case, the sub-block vectors can go far apart, resulting in zero overlapping among reference blocks used for the motion compensation, and hence non-affordable peak memory bandwidth consumption of the affine mode.

Efforts are being made to constrain the worst case memory bandwidth consumption of the affine mode by the sub-block vector clipping or by the 8x8/4x8/8x4 sub-block size adaptation [3]. Those methods have one thing in common, namely, they just constrain the sub-block motion vector spread (i.e. how far the sub-block vectors can go apart from each other) within the 8x8 non-overlapped regions of a PU, but not the affine motion model parameters. Within the PU the sub-block motion vectors from one 8x8 region to the next 8x8 region can still go widely apart, which can lead to a huge peak memory bandwidth consumption in the worst case. Figure 1 depicts such an example, in which sub-block motion vector spread is constrained within 8x8 regions of a PU and hence a tight reference block bounding box can be drawn for each 2x2 sub-block vector group, but the reference block bounding boxes can still be completely non-overlapped in the worst case.

Figure 1: Reference block bounding boxes can still go far apart even if the sub-block motion vector spared is constrained within each 8x8 region of a PU

Unlike the regular motion vectors which can be fully independent from block to block, the sub-block motion vectors of the affine mode are interconnected by a linear equation, which provides an opportunity to constrain the sub-block motion vector spread at the PU level to avoid the above mentioned problem.

# Algorithm description

To constrain the memory bandwidth consumption of the affine mode, we propose a method to constrain the model parameters a,b,c,d directly on the encoder side (i.e. a bitstream restriction). The basic idea is to constrain the reference block bounding box size by not selecting the affine model parameters that can lead to the reference block bounding box sizes exceeding the pre-defined thresholds.

Figure 2: Illustration of reference block bounding box of 4 sub-block motion vectors in a PU coded in bi-directional affine mode

For motion compensation, reference blocks are loaded around the co-located sub-block locations with the offsets determined by the sub-block motion vectors. Figure 2 illustrates the reference blocks for the four sub-block vectors and how a bounding box which contains all four reference blocks is generated.   Assume x0 , y0 be the coordinates of the sub-block 0 in Figure 2 and all the sub-blocks in the PU have equal size m*n, and fx*fy be filter taps used in the motion compensation, the coordinates of upper-left and bottom-right corners of the reference block bounding box in Figure 2, i.e. (xbul,ybul) and (xbbr,ybbr), are defined as:

xbul=min0, 1+am,cn,1+am+cn+(x0+vxs0-fx2+1)ybul=min0, bm, 1+dn,bm+1+dn+(y0+vys0-fy2+1)       Equation 2

xbbr=max0, 1+am,cn,1+am+cn+(x0+vxs0+fx2+m)ybbr=max0, bm, 1+dn,bm+1+dn+(y0+vys0+fy2+n)      Equation 3

Where max() and min() are functions used to return the largest and the smallest value from a set of data, respectively.

By using Equation 2 and 3, the width and height of the reference block bounding box, i.e. (bxW4,bxH4)  can be computed by:

bxW4=max0, 1+am,cn,1+am+cn-min0, 1+am,cn,1+am+cn+fx+m-1bxH4=max0, bm, 1+dn,bm+1+dn-min0, bm, 1+dn,bm+1+dn+fy+n-1 Equation 4

Figure 3: Illustration of reference block bounding box of 2 sub-block motion vectors in a PU coded in unidirectional affine mode (horizontal direction)

Likewise, the width and height of the reference block bounding box (bxWh,bxHh)  for 2 sub-block motion vectors in a PU, which is coded in affine mode and unidirectional inter prediction, can be computed by (see Figure 3, and sub-block 0):

bxWh=max0, 1+am-min0, 1+am+fx+m-1bxHh=max0, bm -min0, bm +fy+n-1                            Equation 5

It can be seen from Equation 4 and Equation 5 that the reference block bounding block size is independent of sub-block locations inside the PU. Instead, it purely depends on the parameters of affine motion model (i.e. a, b, c, d), sub-block size (i.e. m*n) and filter tap lengths (i.e. fx*fy) used for the motion compensation.

The pre-defined thresholds are defined as

Thred4=(fx+2m-1+δx)*(fy+2n-1+δy)Thredh=(fx+2m-1+δx)*(fy+n-1+δy)               Equation 6

Where δx*δy defines the margin for controlling the memory bandwidth consumption.

In the current VTM3.0 design, the sub-block size used for the affine mode is 4x4 (i.e. m=n=4), the filter tap used for luma motion compensation is 8x8 (i.e. fx=fy=8). The thresholds become

Thred4=(15+δx)*(15+δy)Thredh=(15+δx)*(11+δy)               Equation 7

The restriction is imposed as a bitstream constraint; a conforming bitstream shall satisfy the following conditions:

- If a PU uses bi-directional affine mode (i.e. bi-pred), the reference block bounding box size bxW4*bxH4 shall be  less than or equal to Thred4.

- Otherwise, if the PU is coded in unidirectional affine mode, the reference block bounding box size bxWh*bxHh shall be less than or equal to Thredh.

# Experimental results

Tests were carried out by using the VTM3.0 software, and the common test conditions (CTC) defined in [1] was followed. The restriction is implemented on the encoder side by disallowing the affine model parameters that cause the reference block size exceeding the pre-defined threshold. No changes were introduced to the decoder.

The following two tests were carried out:

- Test 1: set threshold of luma reference block bounding box to 16*16 for 2x2 sub-block vectors of bi-directional affine mode, and to 16*12 for 2x1 sub-block vectors of unidirectional affine mode (i.e. setting δx=δy=1).

- Test 2: set threshold of luma reference block bounding box to 17*17 for 2x2 sub-block vectors of bi-directional affine mode, and to 17*13 for 2x1 sub-block vectors of unidirectional affine mode (i.e. setting δx=δy=2).

The summary results of the tests are provided in Table 1 and Table 2. For the threshold setting of (δx=δy=1), the BD-rate difference is 0.05% in RA, 0.04% in LD_B and -0.02% in LD_P, respectively. For the threshold setting of (δx=δy=2), the BD-rate difference is 0.01% in RA, 0.06% in LD_B and 0.01% in LD_P, respectively. Note that the run time may not be accurate due to the heterogeneous nature of the LSF farm.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.07% | 0.08% | 109% | 105% |
| Class A2 | 0.10% | 0.16% | 0.18% | 95% | 100% |
| Class B | 0.03% | 0.06% | 0.06% | 98% | 96% |
| Class C | 0.04% | 0.02% | 0.09% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.07% | 0.09% | 100% | 100% |
| Class D | 0.00% | -0.04% | -0.03% | 99% | 100% |
| Class F (optional) | 0.05% | 0.04% | 0.12% | 92% | 92% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.17% | 0.03% | 98% | 102% |
| Class C | 0.05% | 0.13% | 0.02% | 96% | 93% |
| Class E | 0.02% | 0.51% | 0.18% | 105% | 104% |
| Overall | 0.04% | 0.10% | 0.06% | 99% | 99% |
| Class D | -0.01% | 0.11% | -0.17% | 98% | 97% |
| Class F (optional) | 0.02% | -0.22% | -0.17% | 102% | 99% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.13% | 0.14% | 92% | 89% |
| Class C | 0.00% | 0.24% | 0.34% | 92% | 88% |
| Class E | -0.08% | -0.13% | -0.74% | 99% | 98% |
| Overall | -0.02% | 0.10% | -0.01% | 94% | 91% |
| Class D | 0.05% | -0.04% | 0.30% | 98% | 102% |
| Class F (optional) | 0.13% | 0.22% | -0.23% | 100% | 99% |

Table 1: Summary results of Test 1( δx=δy=1)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.03% | 99% | 100% |
| Class A2 | 0.01% | 0.03% | 0.07% | 97% | 109% |
| Class B | 0.01% | 0.06% | -0.06% | 101% | 101% |
| Class C | 0.00% | 0.02% | 0.07% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.02% | 100% | 102% |
| Class D | 0.01% | -0.10% | 0.03% | 91% | 87% |
| Class F (optional) | 0.01% | -0.02% | -0.03% | 92% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.24% | 0.07% | 103% | 108% |
| Class C | 0.03% | 0.21% | 0.03% | 100% | 100% |
| Class E | 0.16% | 0.11% | -0.45% | 107% | 108% |
| Overall | 0.06% | -0.01% | -0.07% | 103% | 105% |
| Class D | -0.01% | 0.16% | 0.00% | 109% | 108% |
| Class F (optional) | -0.02% | 0.03% | 0.56% | 99% | 93% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.12% | 0.26% | 95% | 93% |
| Class C | 0.04% | 0.16% | -0.09% | 98% | 96% |
| Class E | 0.00% | -0.25% | -0.60% | 91% | 92% |
| Overall | 0.01% | 0.04% | -0.07% | 95% | 94% |
| Class D | 0.03% | -0.19% | 0.04% | 101% | 108% |
| Class F (optional) | 0.07% | -0.61% | -0.26% | 99% | 98% |

Table 2: Summary results of Test 2( δx=δy=2)

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001-v7, the 12th JVET meeting, the 12th JVET meeting, Oct. 2018, Macao, CN.

[3] Y. he, C.-Y. Chen and C-C Chen, “Description of Core Experiment 6 (CE2): Sub-block based motion prediction”, JVET-L1022, the 12th JVET meeting, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Spec Text Changes relative to JVET-L1001-v7

#### In section 8.3.4.9 Derivation process for motion vector arrays from affine control point motion vectors, add

Text for Test 1

It is a requirement of bitstream conformance that

- If predFlagL0[ xCb ][ yCb] is equal to 1 and predFlagL1[ xCb ][ yCb] is equal to 1, bxWX4*bxHX4 shall be  less than or equal to (256≪22).

- Otherwise, if predFlagL0[ xCb ][ yCb] is equal to 1 or predFlagL1[ xCb ][ yCb] is equal to 1, bxWXh*bxHXh shall be less than or equal to (192≪22).

Where the values of (bxWX4,bxHX4) and (bxWXh,bxHXh) are computed by

maxW4=max0, 42048+dHorX,4dHorY,42048+dHorX+4dHorYminW4=min0, 42048+dHorX,4dHorY,42048+dHorX+4dHorYmaxH4=max0, 4dVerX, 42048+dVerY,4dVerX+42048+dVerYminH4=min0, 4dVerX, 42048+dVerY,4dVerX+42048+dVerY

bxWX4=maxW4-minW4+(11≪11)bxHX4=maxH4-minH4+(11≪11)

bxWXh=max0, 42048+dHorX-min0, 42048+dHorX+(11≪11)bxWXh=max0, 4dVerX-min0, 4dVerX+11≪11

Text for Test 2

It is a requirement of bitstream conformance that

- If predFlagL0[ xCb ][ yCb] is equal to 1 and predFlagL1[ xCb ][ yCb] is equal to 1, bxWX4*bxHX4 shall be  less than or equal to (289≪22).

- Otherwise, if predFlagL0[ xCb ][ yCb] is equal to 1 or predFlagL1[ xCb ][ yCb] is equal to 1, bxWXh*bxHXh shall be less than or equal to (221≪22).

Where the values of (bxWX4,bxHX4) and (bxWXh,bxHXh) are computed by

maxW4=max0, 42048+dHorX,4dHorY,42048+dHorX+4dHorYminW4=min0, 42048+dHorX,4dHorY,42048+dHorX+4dHorYmaxH4=max0, 4dVerX, 42048+dVerY,4dVerX+42048+dVerYminH4=min0, 4dVerX, 42048+dVerY,4dVerX+42048+dVerY

bxWX4=maxW4-minW4+(11≪11)bxHX4=maxH4-minH4+(11≪11)

bxWXh=max0, 42048+dHorX-min0, 42048+dHorX+(11≪11)bxWXh=max0, 4dVerX-min0, 4dVerX+11≪11