| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0057 |
| --- | --- |

| Title: | CE8: BDPCM with horizontal/vertical predictor and independently decodable areas (test 8.3.1b) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohsen Abdoli Gordon Clare Félix Henry Pierrick Philippe Orange Labs 4 rue Clos Courtel 35510 Cesson Sevigne France b<>com 1219 Avenue des Champs Blancs, 35510 Cesson-Sévigné France | Tel: Email: | +33 2 99 12 45 58 mohsen.abdoli@orange.com gordon.clare@orange.com gordon.clare@b-com.com felix.henry@orange.com felix.henry@b-com.com pierrick.philippe@orange.com |
| Source: | Orange, b<>com |  |  |

_____________________________

# Abstract

This contribution describes CE test 8.3.1b where the BDPCM coding mode proposed in JVET-L0078 is modified. This modification consists of two parts: first the LOCO-I predictor is replaced with a simple vertical or horizontal predictor (chosen and signaled at block level), and, second, blocks having one dimension equal to 4 are divided into two independently decodable regions. It is asserted that this modification produces a worst case throughput of 8 pixels per cycle. Compared to JVET-L0078, this contribution also introduces two encoder-side-only changes. The performance of this test in Y-BDR gain in All Intra is as follows.

CPR Off, Class F:        -9,2% (Y), 88% (EncT),	96% (DecT)

CPR Off, Class SCC: -24,7% (Y), 73% (EncT),  100% (DecT)

CPR On, Class F:         -4,6% (Y), 93% (EncT), 107% (DecT)

CPR On, Class SCC:    -8,5% (Y), 79% (EncT), 110% (DecT)

# Introduction

JVET-L0078 contribution proposes to use a classical DPCM approach at the block level [1]. During the presentation of this proposal at the Macao meeting, the group expressed the need to increase the throughput of the technology. This CE test proposes to study BDPCM with increased throughput , investigating:

- Block size limitation of BDPCM and impact on throughput

- Independently decodable regions inside a BDPCM block (in a manner similar to the RDPCM throughput increase proposed in JCT-VC M0439 [16]) and impact on throughput

- Replacement of the LOCO-I predictor with vertical and horizontal predictors (in a way similar to RDPCM), allowing the reconstruction to proceed line by line / column by column and thus increase throughput

# Summary of BDPCM

In this section, we describe BDPCM as proposed in JVET-L0078 [1]

A bdpcm_flag is transmitted at the CU level whenever it is a luma intra CU having each dimension smaller or equal to 32. This flag indicates whether regular intra coding or DPCM is used. This flag is encoded using a single CABAC context.

Block DPCM uses the Median Edge Detector of LOCO-I. For a current pixel X having pixel A as left neighbor, pixel B as top neighbor and C as top-left neighbor, the prediction P(X) is determined by

P(X)=	min(A,B) if C≥max(A,B)

max(A,B) if C≤min(A,B)

A+B-C otherwise

The predictor uses unfiltered reference pixels when predicting the top row and left column of the CU. The predictor then uses reconstructed pixels for the rest of the CU. Pixels are processed in raster-scan order inside the CU. The prediction error is quantized in the spatial domain, after rescaling, in a way identical to the Transform Skip quantizer. Each pixel is reconstructed by adding the dequantized prediction error to the prediction. Thus, the reconstructed pixels are used to predict the next pixels in raster-scan order. Amplitude and signs of the quantized prediction error are encoded separately.  A cbf_bdpcm_flag is coded. If it is equal to 0, it indicates that all amplitudes of the block are to be decoded as zero. If the flag is equal to 1, all amplitudes of the block are encoded individually in raster-scan order. In order to keep complexity low, the amplitude is limited to at most 31 (inclusive). The amplitude is encoded using unary binarization, with three contexts for the first bin, then one context for each additional bin until the 12th bin, and one context for all remaining bins. A sign is encoded in bypass mode for each non-zero residue.

In order to maintain the coherence of the regular intra mode prediction, the first mode in the MPM list is associated with a Block-DPCM CU (without being transmitted) and is available for MPM generation for subsequent blocks.

The deblocking filter is de-activated on a border between two Block-DPCM blocks, since neither of the blocks uses the transform stage usually responsible for blocking artifacts.

# Modifications tested in CE 8.3.1b

The following modifications were implemented on top of VTM-3.0

## Encoder modifications

The Full-RD check of the intra modes has been modified by systematically checking the BDPCM mode before the first intra mode. Then, the first intra mode is checked. If it is worse than the BDPCM in terms of RD cost, the Full-RD check is then stopped. Otherwise it is continued normally until all intra modes selected for full-RD check are processed. Note that in our tests this is done systematically, and not only for Screen Content sequences.

Also, a simple form of RDOQ has been implemented in the encoder. Alternate quantization levels are checked in addition to the minimum-distortion one.

## Normative modifications

The LOCO-I predictor of [1] is replaced with a simple vertical or horizontal predictor (chosen and signaled by a flag at block level).

Due to the shape of the horizontal (resp. vertical) predictors, which use the left (A)  (resp. top (B)) pixel for prediction of the current pixel, the most throughput-efficient way of processing the block is to process all the pixels of one column (resp. line)  in parallel, and to process these columns (resp. lines) sequentially. In order to increase throughput, we introduce the following process: a block of width 4 is divided into two halves with a horizontal frontier when the predictor chosen on this block is vertical, and a block of height 4 is divided into two halves with a vertical frontier when the predictor chosen on this block is horizontal.

When a block is divided, samples from one area are not allowed to use pixels from another area to compute the prediction: if this situation occurs, the prediction pixel is replaced by the reference pixel in the prediction direction. This is shown on Figure 1 for different positions of current pixel X in a 4x8 block predicted vertically.

Figure 1: dividing a block of 4x8 samples into two independently decodable areas.

Thanks to this property, it becomes now possible to process a 4x4 block in 2 cycles, and a 4x8 or 8x4 block in 4 cycles, and so on, as shown on Figure 2.

Figure 2: order of processing of the rows of pixels to maximize throughput for 4xN blocks with vertical predictor.

Table 1 summarizes the number of cycles required to process the block, depending on the block size. It is trivial to show that any block which has both dimensions larger or equal to 8 can be processed in 8 pixels per cycle or more.

Table 1: Worst case throughput for blocks of size 4xN, Nx4

| Block size | 4x4 | 4x8, 8x4 | 4x16, 16x4 | 4x32, 32x4 |
| --- | --- | --- | --- | --- |
| Cycles | 2 | 4 | 8 | 16 |
| Pixels | 16 | 32 | 64 | 128 |
| Throughput (pixels/cycle) | 8 | 8 | 8 | 8 |

# Performance

The modified software has been run under the CE8 test conditions [2] with to different configurations: CPR off and CPR on. The main goal of testing BDCM with CPR off is to show that BDPCM can reach a substantial proportion of the gains of other Screen Content tools at a fraction of the complexity.

Table 1 shows the performance of BDPCM in test 8.3.1b with CPR off in All Intra

Table 2 shows the performance of BDPCM in test 8.3.1b with CPR off in Random Access

Table 3 shows the performance of BDPCM in test 8.3.1b with CPR on in All Intra

Table 4 shows the performance of BDPCM in test 8.3.1b with CPR on in Random Access

Table 1: performance of BDPCM in test 8.3.1b with CPR off in All Intra

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,08% | 0,16% | 107% | 98% |
| Class A2 | 0,07% | 0,06% | 0,10% | 107% | 100% |
| Class B | 0,01% | 0,06% | 0,12% | 106% | 98% |
| Class C | -0,27% | -0,11% | -0,25% | 103% | 96% |
| Class E | -0,19% | -0,05% | 0,11% | 105% | 96% |
| Overall | -0,08% | 0,01% | 0,04% | 106% | 98% |
| Class D | -0,31% | -0,17% | -0,24% | 99% | 98% |
| Class F | -9,16% | -6,79% | -6,77% | 88% | 96% |
| Class TGM | -24,70% | -19,08% | -19,36% | 73% | 100% |

Table 2: performance of BDPCM in test 8.3.1b with CPR off in Random Access

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | 0,14% | 0,21% | 102% | 99% |
| Class A2 | 0,02% | 0,10% | 0,21% | 102% | 99% |
| Class B | -0,01% | 0,28% | -0,07% | 102% | 97% |
| Class C | -0,12% | -0,03% | 0,15% | 102% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | 0,14% | 0,10% | 102% | 97% |
| Class D | -0,11% | -0,21% | -0,04% | 102% | 97% |
| Class F | -6,93% | -5,21% | -5,16% | 103% | 95% |
| Class TGM | -12,59% | -8,46% | -8,63% | 102% | 96% |

Table 3: performance of BDPCM in test 8.3.1b with CPR on in All Intra

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | 0,23% | 0,10% | 105% | 103% |
| Class A2 | 0,05% | 0,02% | 0,11% | 104% | 104% |
| Class B | 0,00% | 0,07% | 0,06% | 104% | 103% |
| Class C | -0,29% | -0,22% | -0,14% | 103% | 106% |
| Class E | -0,07% | 0,06% | -0,04% | 103% | 100% |
| Overall | -0,07% | 0,02% | 0,01% | 104% | 103% |
| Class D | -0,37% | -0,17% | -0,24% | 101% | 103% |
| Class F | -4,64% | -2,69% | -2,64% | 93% | 107% |
| Class TGM | -8,48% | -5,28% | -5,42% | 79% | 110% |

Table 4: performance of BDPCM in test 8.3.1b with CPR on in Random Access

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,05% | 0,05% | 102% | 99% |
| Class A2 | 0,02% | -0,02% | -0,04% | 101% | 99% |
| Class B | 0,03% | 0,00% | 0,07% | 103% | 98% |
| Class C | -0,12% | -0,01% | 0,04% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0,02% | 0,00% | 0,04% | 102% | 99% |
| Class D | -0,08% | 0,09% | 0,17% | 103% | 96% |
| Class F | -3,28% | -1,80% | -1,94% | 103% | 102% |
| Class TGM | -4,46% | -2,22% | -2,24% | 102% | 103% |

# Conclusion

CE test 8.3.1b shows that, using BDPCM, it is possible to reach a throughput of at least 8 pixels per cycle, while producing the following performance in All Intra:

CPR Off, Class F:        -9,2% (Y), 88% (EncT),	96% (DecT)

CPR Off, Class SCC: -24,7% (Y), 73% (EncT),  100% (DecT)

CPR On, Class F:         -4,6% (Y), 93% (EncT), 107% (DecT)

CPR On, Class SCC:    -8,5% (Y), 79% (EncT), 110% (DecT)

It is proposed to adopt BDPCM in the VVC draft text and VTM-4.0.

# Bibliography

[1] JVET-L0078, AHG11: Block DPCM for Screen Content Coding, M. Abdoli, G. Clare, F. Henry, P. Philippe (Orange)

[2] JVET-L1028 Description of Core Experiment 8 (CE8): Screen Content Coding Tools, X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu

# Patent rights declaration(s)

Orange and b<>com may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).