| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0061_r1 |
| --- | --- |

| Title: | CE4: Combination of CE4.4.4a and CE4.4.5c (Test 4.4.4.e) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Eiichi Sasaki Tomohiro Ikai Li Jingya Ru-Ling Liao Chong Soon Lim | Email: | +81-43-299-8526 tomonori.hashimoto@sharp.co.jp eiichi.sasaki@sharp.co.jp ikai.tomohiro@sharp.co.jp +65-65505346 jingya.li@sg.panasonic.com ruling.liao@sg.panasonic.com chongsoon.lim@sg.panasonic.com |
| Source: | Sharp Corporation Panasonic |  |  |

_____________________________

- Abstract

This contribution reports a result of CE4.4.4.e (a combination of CE4.4.4.a and CE4.4.5.c). It is reported that the bd-rate is -0.38% with 101% encoding time under RA condition and -0.15% with 102% encoding time under LDB condition.

Proposed Methods

Directional candidates in CE4.4.4

Directional candidates are used in addition to horizontal / vertical candidates of VTM-3.0. The directional candidates are shown as below.

Table 1: Motion direction

| Direction IDX | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-axis | +2 | –2 | 0 | 0 | +1 | -1 | -1 | +1 |
| y-axis | 0 | 0 | +2 | –2 | +1 | -1 | +1 | -1 |

Note: the value of x-axis and y-axis of diagonal direction are half of that of horizontal and vertical direction respectively.

Adaptive distance table in CE4.4.5

(1) Use adaptive distance table based on picture resolution, i.e., if picture resolution is not larger than 2K, i.e., 1920×1080, Table 2 is used as base distance table; Otherwise, Table 3 is used as base distance table.

(2) Use adaptive distance table based on occurrence-based distance table reordering. Distance index is reordered according to the occurrence of usage of each distance in previous coded picture in list 0 and reference index 0.

Details of (2) are as below:

- An optimal reordered distance table is determined after coding each inter picture. If current picture is intra picture, base distance table (shown in Table 2 and Table 3) is used as optimal reordered distance table.

- The optimal reordered distance table of reference picture in list 0 and reference index 0 is used as distance table for current inter picture.

- Weighted table is used for determining optimal reordered distance table. And weighted table is determined using gain table as shown in Table 4 and Table 5. Table 4 is used for picture with resolution non larger than 2K, otherwise Table 5 is used.

In Table 4 and Table 5, each row represents a distance ([1/4-32] in Table 4, [1-128] in Table 5) and each column represents a postion [0-7]

For example, if Table 4 is used,

If order is [1/4, 1/2, 1, 2, 4, 8, 16, 32], weighted table will be [0, 1, 2, 3, 4, 5, 6, 8]

If order is [1/2, 1/4, 1, 2, 4, 8, 16, 32], weighted table will be [2, -1, 3, 4, 5, 6, 8]

Table 2: MMVD distance table candidate

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |

Table 3: MMVD distance table

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel | 64-pel | 128-pel |

Table 4: weighted table for distance table reordering

gain_table[8][8] = {

{ 0, -1, -2, -3, -4, -5, -6, -6},

{ 2,  1,  0, -1, -2, -3, -4, -4},

{ 4,  3,  2,  1,  0, -1, -2, -2},

{ 6,  5,  4,  3,  2,  1,  0,  0},

{ 8,  7,  6,  5,  4,  3,  2,  2},

{10,  9,  8,  7,  6,  5,  4,  4},

{12, 11, 10,  9,  8,  7,  6,  6},

{14, 13, 12, 11, 10,  9,  8,  8} };

Table 5: weighted table for distance table reordering

gain_table_4K[8][8] = {

{ 4,  3,  2,  1,  0, -1, -2, -2},

{ 6,  5,  4,  3,  2,  1,  0,  0},

{ 8,  7,  6,  5,  4,  3,  2,  2},

{10,  9,  8,  7,  6,  5,  4,  4},

{12, 11, 10,  9,  8,  7,  6,  6},

{14, 13, 12, 11, 10,  9,  8,  8},

{16, 15, 14, 13, 12, 11, 10, 10},

{18, 17, 16, 15, 14, 13, 12, 12} };

Experimental Results

The simulations were conducted according to JVET-L1024 [3]. Following Table 1 shows the results.

Table 1. All results CE4.4.4

4.4.4.e

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM30-SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.57% | -0.62% | -0.75% | 98% | 100% |
| Class A2 | -0.52% | -0.69% | -0.75% | 100% | 100% |
| Class B | -0.29% | -0.18% | -0.32% | 101% | 99% |
| Class C | -0.23% | -0.27% | -0.28% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.38% | -0.39% | -0.48% | 101% | 100% |
| Class D | -0.36% | -0.28% | -0.22% | 105% | 100% |
| Class F | -0.16% | -0.21% | -0.20% | 107% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM30-SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | -0.42% | 0.15% | 102% | 100% |
| Class C | -0.19% | 0.01% | -0.15% | 103% | 100% |
| Class E | -0.05% | 0.25% | -0.09% | 102% | 100% |
| Overall | -0.15% | -0.11% | -0.01% | 102% | 100% |
| Class D | -0.26% | -0.63% | -0.08% | 103% | 100% |
| Class F | -0.10% | -0.23% | -0.63% | 103% | 100% |

Conclusion

This contribution reports the results of CE4.4.4.e (a combination of CE4.4.4.a and CE4.4.5.c).

References

[1] Tomonori Hashimoto, Eiichi Sasaki, Tomohiro Ikai, “Non-CE4: Enhanced ultimate motion vector expression,” JVET-L0355, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macau, CN, 3–12 October 2018.

[2] Li Jingya, Ru-Ling Liao, Chong Soon Lim “CE4-related: Improvement on ultimate motion vector expression,” JVET-L0408, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macau, CN, 3–12 October 2018.

[3] Haitao Yang, Shan Liu, Kai Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-L1024, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).