    - Allowed binary split process

Inputs to this process are:

- a binary split mode btSplit,

- a coding block width cbWidth,

- a coding block height cbHeight,

- a location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture,

- a multi-type tree depth mttDepth,

- a maximum multi-type tree depth with offset maxMttDepth,

- a maximum binary tree size maxBtSize,

- a partition index partIdx.

Output of this process is the variable allowBtSplit.

Table 6-2 – Specification of parallelTtSplit and cbSize based on btSplit.

|  | btSplit = = SPLIT_BT_VER | btSplit = = SPLIT_BT_HOR |
| --- | --- | --- |
| parallelTtSplit | SPLIT_TT_VER | SPLIT_TT_HOR |
| cbSize | cbWidth | cbHeight |

The variables parallelTtSplit and cbSize are derived as specified in  Table 6-2.

The variable allowBtSplit is derived as follows:

- If one or more of the following conditions are true, allowBtSplit is set equal to FALSE:

- cbSize is less than or equal to MinBtSizeY

- cbWidth is greater than maxBtSize

- cbHeight is greater than maxBtSize

- mttDepth is greater than or equal to maxMttDepth

- Otherwise, if all of the following conditions are true, allowBtSplit is set equal to FALSE

- btSplit is equal to SPLIT_BT_VER

–	treeType == DUAL_TREE_CHROMA && ( cbWidth / SubWidthC * cbHeight / SubHeightC equal to or less than 8 )

- Otherwise, if all of the following conditions are true, allowBtSplit is set equal to FALSE

- btSplit is equal to SPLIT_BT_HOR

- treeType == DUAL_TREE_CHROMA && ( cbWidth / SubWidthC * cbHeight / SubHeightC equal to or less than 8 )

- x0 + cbWidth is greater than pic_width_in_luma_samples

- y0 + cbHeight is less than or equal to pic_height_in_luma_samples

- Otherwise, if all of the following conditions are true, allowBtSplit is set equal to FALSE:

- mttDepth is greater than 0

- partIdx is equal to 1

- MttSplitMode[ x0 ][ y0 ][ mttDepth − 1 ] is equal to parallelTtSplit

- Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE

- btSplit is equal to SPLIT_BT_VER

- cbWidth is less than or equal to MaxTbSizeY

- cbHeight is greater than MaxTbSizeY

- Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE

- btSplit is equal to SPLIT_BT_HOR

- cbWidth is greater than MaxTbSizeY

- cbHeight is less than or equal to MaxTbSizeY

–	Otherwise, allowBtSplit is set equal to TRUE.

    - Allowed ternary split process

- inputs to this process are:

- a ternary split mode ttSplit,

- a coding block width cbWidth,

- a coding block height cbHeight,

- a location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture,

- a multi-type tree depth mttDepth

- a maximum multi-type tree depth with offset maxMttDepth,

- a maximum binary tree size maxTtSize.

Output of this process is the variable allowTtSplit.

Table 6-3 – Specification of cbSize based on ttSplit.

|  | ttSplit = = SPLIT_TT_VER | ttSplit = = SPLIT_TT_HOR |
| --- | --- | --- |
| cbSize | cbWidth | cbHeight |

The variable cbSize is derived as specified in Table 6-3.

The variable allowTtSplit is derived as follows:

- If one or more of the following conditions are true, allowTtSplit is set equal to FALSE:

- treeType == DUAL_TREE_CHROMA && (cbWidth / SubWidthC * cbHeight / SubHeightC) is equal to or less than 16

- cbSize is less than or equal to 2 * MinTtSizeY

- cbWidth is greater than Min( MaxTbSizeY, maxTtSize )

- cbHeight is greater than Min( MaxTbSizeY, maxTtSize )

- mttDepth is greater than or equal to maxMttDepth

- x0 + cbWidth is greater than pic_width_in_luma_samples

- y0 + cbHeight is greater than pic_height_in_luma_samples

–	Otherwise, allowTtSplit is set equal to TRUE.

- 7.3.4.4 Coding quadtree syntax

| coding_quadtree( x0, y0, log2CbSize, cqtDepth, treeType ) { | Descriptor |
| --- | --- |
| minQtSize = ( treeType = = DUAL_TREE_CHROMA ) ? MinQtSizeC : MinQtSizeY |  |
| maxBtSize = ( treeType = = DUAL_TREE_CHROMA ) ? MaxBtSizeC : MaxBtSizeY |  |
| if( ( ( ( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples ) ? 1 : 0 ) + ( ( y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples ) ? 1 : 0 ) + ( ( ( 1 << log2CbSize ) <= maxBtSize ) ? 1 : 0 ) ) >= 2 && ( 1 << log2CbSize ) > minQtSize ) |  |
| qt_split_cu_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_qp_delta_enabled_flag && cqtDepth <= diff_cu_qp_delta_depth ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| if( qt_split_cu_flag[ x0 ][ y0 ] ) { |  |
| qt_split_cu_flag[ x0 ][ y0 ] == false |  |
| x1 = x0 + ( 1 << ( log2CbSize − 1 ) ) |  |
| y1 = y0 + ( 1 << ( log2CbSize − 1 ) ) |  |
| coding_quadtree( x0, y0, log2CbSize − 1, cqtDepth + 1, treeType ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_quadtree( x1, y0, log2CbSize − 1, cqtDepth + 1, treeType ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x0, y1, log2CbSize − 1, cqtDepth + 1, treeType ) |  |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x1, y1, log2CbSize − 1, cqtDepth + 1, treeType ) |  |
| } else |  |
| multi_type_tree( x0, y0, 1 << log2CbSize, 1 << log2CbSize, cqtDepth, 0, 0, 0, treeType ) |  |
| } |  |

- 7.3.4.5 Multi-type tree syntax

| multi_type_tree( x0, y0, cbWidth, cbHeight, cqtDepth, mttDepth, depthOffset, partIdx, treeType ) { | Descriptor |
| --- | --- |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ) && ( x0 + cbWidth <= pic_width_in_luma_samples ) && (y0 + cbHeight <= pic_height_in_luma_samples ) ) |  |
| mtt_split_cu_flag | ae(v) |
| if( cu_qp_delta_enabled_flag && ( cqtDepth + mttDepth ) <= diff_cu_qp_delta_depth ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| if( mtt_split_cu_flag ) { |  |
| if( ( allowSplitBtHor ¦ ¦ allowSplitTtHor ) && ( allowSplitBtVer ¦ ¦ allowSplitTtVer ) ) |  |
| mtt_split_cu_vertical_flag | ae(v) |
| if( ( allowSplitBtVer && allowSplitTtVer && mtt_split_cu_vertical_flag ) ¦ ¦ ( allowSplitBtHor && allowSplitTtHor && !mtt_split_cu_vertical_flag ) ) |  |
| mtt_split_cu_binary_flag | ae(v) |
| if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_BT_VER ) { |  |
| depthOffset += ( x0 + cbWidth > pic_width_in_luma_samples ) ? 1 : 0 |  |
| x1 = x0 + ( cbWidth / 2 ) |  |
| multi_type_tree( x0, y0, cbWidth / 2, cbHeight, cqtDepth, mttDepth + 1, depthOffset, 0, treeType ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| multi_type_tree( x1, y0, cbWidth / 2, cbHeightY, cqtDepth, mttDepth + 1, depthOffset, 1, treeType ) |  |
| } else if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_BT_HOR ) { |  |
| depthOffset += ( y0 + cbHeight > pic_height_in_luma_samples ) ? 1 : 0 |  |
| y1 = y0 + ( cbHeight / 2 ) |  |
| multi_type_tree( x0, y0, cbWidth, cbHeight / 2, cqtDepth, mttDepth + 1, depthOffset, 0, treeType ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| multi_type_tree( x0, y1, cbWidth, cbHeight / 2, cqtDepth, mttDepth + 1, depthOffset, 1, treeType ) |  |
| } else if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_TT_VER ) { |  |
| x1 = x0 + ( cbWidth / 4 ) |  |
| x2 = x0 + ( 3 * cbWidth / 4 ) |  |
| multi_type_tree( x0, y0, cbWidth / 4, cbHeight, cqtDepth, mttDepth + 1, depthOffset, 0, treeType ) |  |
| multi_type_tree( x1, y0, cbWidth / 2, cbHeight, cqtDepth, mttDepth + 1, depthOffset, 1, treeType ) |  |
| multi_type_tree( x2, y0, cbWidth / 4, cbHeight, cqtDepth, mttDepth + 1, depthOffset, 2, treeType ) |  |
| } else { /* SPLIT_TT_HOR */ |  |
| y1 = y0 + ( cbHeight / 4 ) |  |
| y2 = y0 + ( 3 * cbHeight / 4 ) |  |
| multi_type_tree( x0, y0, cbWidth, cbHeight / 4, cqtDepth, mttDepth + 1, depthOffset, 0, treeType ) |  |
| multi_type_tree( x0, y1, cbWidth, cbHeight / 2, cqtDepth, mttDepth + 1, depthOffset, 1, treeType ) |  |
| multi_type_tree( x0, y2, cbWidth, cbHeight / 4, cqtDepth, mttDepth + 1, depthOffset, 2 , treeType) |  |
| } |  |
| } else |  |
| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |

- 7.3.4. 6 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| if( treeType != DUAL_TREE_CHROMA ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && (cbWidth / SubWidthC) + (cbHeight / SubHeightC) ! = 4 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) { |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_data( x0, y0, cbWidth, cbHeight ) |  |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |

7.4.4.1 General tile group header semantics

The variables MinQtLog2SizeY, MinQtLog2SizeC, MinQtSizeY, MinQtSizeC, MaxBtSizeY, MaxBtSizeC, MinBtSizeY, MaxTtSizeY, MaxTtSizeC, MinTtSizeY, MaxMttDepthY and MaxMttDepthC are derived as follows:

MinQtLog2SizeY  =  MinCbLog2SizeY + tile_group_log2_diff_min_qt_min_cb_luma	(7-33)

MinQtLog2SizeC  =  MinCbLog2SizeY + tile_group_log2_diff_min_qt_min_cb_chroma	(7-34)

MinQtSizeY  =  1  <<  MinQtLog2SizeY		(7-35)

MinQtSizeC  =  Max (1  <<  MinQtLog2SizeC	, 8)	(7-36)

MaxBtSizeY  =  1  <<  ( MinQtLog2SizeY + tile_group_log2_diff_max_bt_min_qt_luma )	(7-37)

MaxBtSizeC  =  1  <<  ( MinQtLog2SizeC + tile_group_log2_diff_max_bt_min_qt_chroma )	(7-38)

MinBtSizeY  =  1  <<  MinCbLog2SizeY		(7-39)

MaxTtSizeY  =  1  <<  ( MinQtLog2SizeY + tile_group_log2_diff_max_tt_min_qt_luma )	(7-40)

MaxTtSizeC  =  1  <<  ( MinQtLog2SizeC + tile_group_log2_diff_max_tt_min_qt_chroma )	(7-41)

MinTtSizeY  =  1  <<  MinCbLog2SizeY		(7-42)

MaxMttDepthY  =  tile_group_max_mtt_hierarchy_depth_luma	(7-43)

MaxMttDepthC  =  tile_group_max_mtt_hierarchy_depth_chroma	(7-44)

8.3.3 Derivation process for chroma intra prediction mode

Input to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived.

If (cbWidth / SubWidthC) + (cbHeight / SubHeightC) is equal to 4,

intra_chroma_pred_mode[ xCb ][ yCb ] is set equal to INTRA_DC

Otherwise ( (cbWidth / SubWidthC) + (cbHeight / SubHeightC) is not equal to 4 ), the following applies.

The chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] as specified in Table 8-2 and Table 8-3.

Table 8-2 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] when sps_cclm_enabled_flag is equal to 0

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 0 | 50 | 18 | 1 | X |

Table 8-3 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] when sps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 | 83 | 83 |
| 7 | 0 | 50 | 18 | 1 | X |