| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0075 |
| --- | --- |

| Title: | CE11: Extended Deblocking Filter (test 11.1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Unno Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution presents a proposal of extended deblocking filtering method. In the proposed method, long tap deblocking filters are applied to both luma and chroma samples. BD-rates for luma and chroma (Y, U, V) are (-0.01%, -0.56, -0.52), (-0.06%, -0.41%, -0.32), (0.03%, 0.53%, 0.23%), and (-0.01%, 0.15%, -0.32) for AI, RA, LDB, and LDP conditions, respectively, in case of comparing with VTM-3.0 ALF ON. In a similar way, BD-rates for luma and chroma (Y, U, V) are (-0.01%, -0.72, -0.63), (-0.09%, -0.45%, -0.36), (-0.01%, 0.17%, -0.41%), and (-0.09%, -0.43%, -0.61) under the AI, RA, LDB, and LDP conditions, respectively, in case of comparing with VTM-3.0 ALF OFF.

Introduction

Core Experiment 11 (CE11) related to deblocking filtering is established in 12th meeting at Macao on October, 2018 [1]. This contribution presents a proposal of extended deblocking filtering method. The proposed method is related to CE11.1. The remainder of this contribution is divided into two parts;

- Proposed method, and

- Experimental results under the CTC.

Proposed Method

In the proposed method, long tap deblocking filters are applied to both luma and chroma samples [2]. Each procedure is shown as follows.

Luma Deblocking Filter

Figure 1 Deblocking filter decision process for luma in proposed method.

Figure 1 shows proposed filter decision process for luma. The processes which are bounded by red dashed line are added to the process of anchor (VTM-3.0). It is presented that details of added processes in below.

- Size check

When the following condition is true, the proposed decision process is proceeded to “Yes”;

- Edge boundary is located between two blocks of which cross edge sizes are Th or longer than Th,

Th is defined as follows.

- If (Bs == 2), then Th = 16

- else if (Bs == 1), then Th = 32

It is noted that a block size is replaced by an edge size. Direction of edge size depends on that of edge boundary like EDGE_VER or EDGE_HOR.

- Activity check p4-p7, q4-q7

At first, the following condition (1) and (2) are checked.

p70-p00+q70-q00<β8#1

&p73-p03+q73-q03<β8#2

If both (1) and (2) are true, then following conditions (3) and (4) are checked respectively at block P and block Q. If (3) is true, extended filter is applied to block P. In the same manner, if (4) is true, extended filter is applied to block Q.

p70-p40+p73-p43<β8#3

q70-q40+q73-q43<β8#4

- Extended filtering

In extended filtering process, following filter kernels are used.

p6 = 3,5,2,1,1,1,1,1,1,0,0,0,0,0,0,0
p5 = 1,2,5,2,1,1,1,1,1,1,0,0,0,0,0,0
p4 = 0,1,3,3,3,1,1,1,1,1,1,0,0,0,0,0
p3 = 0,1,2,2,2,2,2,1,1,1,1,1,0,0,0,0
p2 = 0,0,1,2,2,2,2,2,2,1,1,1,0,0,0,0
p1 = 0,0,1,1,2,2,2,2,2,2,1,1,0,0,0,0
p0 = 0,0,0,1,2,2,2,2,2,2,2,1,0,0,0,0

Chroma Deblocking Filter

Figure 2 Deblocking filter decision process for chroma in proposed method.

Figure 2 shows proposed filter decision process for chroma. The processes which are bounded by red dashed line are added to the process of anchor (VTM-3.0). It is presented that details of added processes in below.

- Size check

When the following condition is true, the proposed decision process is proceeded to “Yes”;

- Edge boundary is located between two blocks of which cross edge sizes are Th or longer than Th,

Th is defined as follows.

- If (Bs == 2), then Th = 8

- else if (Bs == 1), then Th = 32

It is noted that a block size is replaced by an edge size. Direction of edge size depends on that of edge boundary like EDGE_VER or EDGE_HOR.

- Strong decision and Extended filtering

Those are exact same procedure as strong filtering for luma.

Experimental Results

The extended deblocking filter was implemented on top of the VTM-3.0 software. In order to verify the performance of the extended deblocking filter, we have run simulations according to the common test conditions of JVET-L1010 [3] with ALF switched on / off.

Results under the common test conditions

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Objective evaluation for luma and chroma

From Table 2 to Table 9 show the experimental results of proposed method for luma and chroma. It can be seen that BD-rates for luma and chroma (Y, U, V) are (-0.01%, -0.56, -0.52), (-0.06%, -0.41%, -0.32), (0.03%, 0.53%, 0.23%), and (-0.01%, 0.15%, -0.32), for AI, RA, LDB, and LDP can be observed, respectively, in case of comparing with VTM-3.0 ALF ON. In the same manner, BD-rates for luma and chroma (Y, U, V) are (-0.01%, -0.72, -0.63), (-0.09%, -0.45%, -0.36), (-0.01%, 0.17%, -0.41%), and (-0.09%, -0.43%, -0.61), for these four cases can be observed, respectively, in case of comparing with VTM-3.0 ALF OFF.

Table 2 Experimental results for AI condition compared with VTM-3.0 ALF ON

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -1.32% | -1.02% | 100% | 102% |
| Class A2 | 0.00% | -0.25% | -0.21% | 100% | 102% |
| Class B | 0.00% | -0.49% | -0.47% | 100% | 102% |
| Class C | 0.00% | -0.23% | -0.29% | 100% | 101% |
| Class E | -0.04% | -0.64% | -0.71% | 100% | 102% |
| Overall | -0.01% | -0.56% | -0.52% | 100% | 102% |
| Class D | 0.00% | -0.15% | -0.18% | 100% | 101% |
| Class F | -0.01% | -0.17% | -0.28% | 101% | 102% |

Table 3 Experimental results for RA condition compared with VTM-3.0 ALF ON

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -1.73% | -1.49% | 100% | 103% |
| Class A2 | -0.08% | -0.23% | 0.03% | 100% | 102% |
| Class B | -0.01% | -0.12% | -0.10% | 100% | 102% |
| Class C | -0.01% | 0.07% | 0.03% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.41% | -0.32% | 100% | 102% |
| Class D | -0.03% | -0.05% | -0.12% | 100% | 101% |
| Class F | 0.00% | 0.31% | 0.40% | 100% | 101% |

Table 4 Experimental results for LDB condition compared with VTM-3.0 ALF ON

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.17% | 0.06% | 100% | 102% |
| Class C | 0.03% | 0.53% | 0.64% | 100% | 101% |
| Class E | 0.04% | 1.69% | -0.05% | 100% | 102% |
| Overall | 0.03% | 0.53% | 0.23% | 100% | 102% |
| Class D | 0.01% | 0.11% | 0.31% | 100% | 102% |
| Class F | 0.14% | 1.05% | 0.63% | 100% | 102% |

Table 5 Experimental results for LDP condition compared with VTM-3.0 ALF ON

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.35% | -0.50% | 100% | 102% |
| Class C | 0.00% | 0.31% | 0.17% | 100% | 102% |
| Class E | 0.03% | 0.78% | -0.65% | 100% | 102% |
| Overall | -0.01% | 0.15% | -0.32% | 100% | 102% |
| Class D | 0.01% | -0.16% | -0.11% | 100% | 102% |
| Class F | 0.11% | -0.06% | 0.48% | 100% | 102% |

Table 6 Experimental results for AI condition compared with VTM-3.0 ALF OFF

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 ALFOff |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -2.09% | -1.47% | 100% | 102% |
| Class A2 | -0.01% | -0.28% | -0.21% | 100% | 102% |
| Class B | 0.00% | -0.52% | -0.51% | 100% | 102% |
| Class C | 0.00% | -0.22% | -0.28% | 100% | 101% |
| Class E | -0.04% | -0.76% | -0.89% | 100% | 102% |
| Overall | -0.01% | -0.72% | -0.63% | 100% | 102% |
| Class D | 0.00% | -0.15% | -0.18% | 100% | 101% |
| Class F | -0.01% | -0.17% | -0.27% | 99% | 103% |

Table 7 Experimental results for RA condition compared with VTM-3.0 ALF OFF

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 ALFOff |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -2.07% | -1.68% | 100% | 103% |
| Class A2 | -0.09% | -0.10% | -0.04% | 100% | 102% |
| Class B | -0.02% | -0.06% | -0.09% | 100% | 102% |
| Class C | -0.02% | 0.01% | 0.06% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.45% | -0.36% | 100% | 102% |
| Class D | 0.00% | -0.07% | 0.07% | 100% | 101% |
| Class F | 0.00% | 0.31% | 0.35% | 100% | 102% |

Table 8 Experimental results for LDB condition compared with VTM-3.0 ALF OFF

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 ALFOff |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.26% | -0.67% | 100% | 103% |
| Class C | -0.04% | 0.31% | 0.06% | 100% | 102% |
| Class E | 0.04% | 0.71% | -0.58% | 100% | 102% |
| Overall | -0.01% | 0.17% | -0.41% | 100% | 102% |
| Class D | -0.02% | -0.25% | -0.64% | 100% | 101% |
| Class F | 0.12% | 0.99% | 0.81% | 100% | 102% |

Table 9 Experimental results for LDP condition compared with VTM-3.0 ALF OFF

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 ALFOff |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | -0.80% | -0.99% | 100% | 102% |
| Class C | -0.08% | 0.13% | -0.07% | 100% | 102% |
| Class E | -0.12% | -0.58% | -0.70% | 100% | 102% |
| Overall | -0.09% | -0.43% | -0.61% | 100% | 102% |
| Class D | -0.05% | -0.30% | -0.13% | 100% | 101% |
| Class F | 0.03% | 1.27% | 0.85% | 100% | 101% |

Conclusion

This contribution presented results of proposed method related to CE11.1 (JVET-L1031). It was presented that test results under CTCs. In the proposed method, extended deblocking filters were applied to both luma and chroma. BD-rates for luma and chroma (Y, U, V) were (-0.01%, -0.56, -0.52), (-0.06%, -0.41%, -0.32), (0.03%, 0.53%, 0.23%), and (-0.01%, 0.15%, -0.32) for AI, RA, LDB, and LDP conditions, respectively, in case of comparing with VTM ALF ON. In a similar way, BD-rates for luma and chroma (Y, U, V) were (-0.01%, -0.72, -0.63), (-0.09%, -0.45%, -0.36), (-0.01%, 0.17%, -0.41%), and (-0.09%, -0.43%, -0.61) under the AI, RA, LDB, and LDP conditions, respectively, in case of comparing with VTM ALF OFF.

- Acknowledgment

We would like to thank Samsung for doing crosschecking of our proposal.

References

- A. Norkin and A. M. Kotra, “Description of Core Experiment 11 (CE11): Deblocking”, JVET-L1031, Macao, CN, Oct. 2018.

- K. Unno, K. Kawamura, and S. Naito, “CE11-related: Improvement of Extended Deblocking Filter”, JVET-L0393, Macao, CN, Oct. 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-L1010, Macao, CN, Oct. 2018.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).