| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-L0080-v1 |
| --- | --- |

| Title: | CE6: MTS simplification with TAF (tests 1.5a-d) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pierrick Philippe | Email: | pierrick.philippe@orange.com |
| Source: | b<>com, Orange |  |  |

_____________________________

# Abstract

This contribution reports a method for complexity reduction of the DST7/DCT8 kernels for MTS. The kernels are approximated using a low complexity adjustment stage called TAF (Transform Adjustment Filters), implemented as a sparse matrix, used as a preprocessor to the partial butterfly DCT2 algorithm. The technique is applied for MTS with size 16 and above. Four operating points are reported against VTM3.

It is argued that the TAF applied for both MTS of size 32 and 64 (test CE6-1.5b) provides a good quality/complexity trade-off with y-bdrate of 0.02%/-0.03%/-0.01% for AI/RA/LD with respective relative encoding time of 98%/101%/101% and decoding time of 88%/99%/100%. When MTS is activated in Inter coding modes, -0.08%/-0.23% Y-bdrate are measured against VTM3 operating in the same configuration.

# Outline of the proposal

This proposal is a continuation of the proposals JVET-J0066, JVET-K0272, JVET-K0299 and JVET-L0135 [2,3,4,5]. It is proposed to approximate DCT8 and DST7 with an adjustment stage followed by a DCT2 (or inverse DCT3).

The DCT8/DST7 transforms are approximated by the combination of a block band matrix and a DCT2 transform, which can be implemented with the partial butterfly algorithm (as for HEVC).

The size M forward DST7 (S7) is approximated as follows:

S7=S⋅C2t⋅F⋅A

- S is a sign matrix, with elements Sk,n=0 if n!=k-1notherwise

- F is a flip matrix, with elements Fk,n=1 if n=M-1-k0 otherwise

- C2t is the transpose of the DCT2 matrix.

- A is a sparse adjustment matrix as specified below.

The size M forward DCT8 (C8) writes:

C8=C2t⋅F⋅A⋅F

Consequently, the inverse DST7 and DCT8 write the following:

iS7=At⋅F⋅C2⋅S and iC8=F⋅At⋅F⋅C2

The flip (F) and sign (S) matrices do not involve any arithmetic operation. Consequently, the complexity of TAF is the sum of the complexity of the adjustment matrix and the DCT2.

The key-features of the proposals are:

- The HEVC DCT2 partial butterfly algorithm is reused

- The adjustment matrix is orthogonal and can be symmetric

- The size-16, size-32 and size-64 adjustment matrices require overall the storage of 356 8-bits coefficients. This amount represents approximately 15th less storage of the DST7 matrices for the 3 sizes.

- The DST7, DCT8 and their inverse transforms use the same coefficients.

- The adjustment stage can be efficiently implemented using SIMD, any multiply operation requires 16-bit input or less.

A proposal for the draft description of the normative changes is attached to this contribution.

# Implementation details

The code base has the following characteristics:

- the adjustment matrices used for the forward and inverse transform contain a maximum of 5 taps per row.

- symmetries are imposed for the size-32 and size-64 adjustment matrix: instead of storing 5 diagonals (see an example below), 3 diagonals are actually stored, the storage requirements for the coefficients is consequently 93 and 189 8-bit coefficients for size 32 and 64 respectively. For size-16, 74 coefficients are required.

- All the multipliers are 16 bit or less. The adjustment matrix coefficients require 8-bit storage.

- the DST7 and DCT8 use the same adjustment matrices with reverse row order (DCT8 and DST7 being the dual of each other, i.e. reversed rows and sign changes)

- zero-out is applied for size-64 MTS, 32 frequency coefficients are retained in the transform process.

Figure 1: The size-32 adjustment matrix, a maximum of 5 significative values per row/column is required, the upper diagonals deduce from the lower ones (with sign inversion for the first upper diagonal). As such, 32+31+30=93 coefficients are stored.

Applying the symmetric adjustment matrix can be efficiently done with SIMD instructions:

for(k=0;k<M    ;k++)  yk =d0,k*xk;

for(k=0;k<M-1  ;k++) {yk+=d1,k*xk+1; yk+1-=d1,k*xk;}

for(k=0;k<M-2  ;k++) {yk+=d2,k*xk+2; yk+2+=d2,k*xk;}

yk denotes the adjusted signal, xk is the input. d0,k, d1,k and d2,k are the diagonals of the adjustment matrix, respectively, the main, the first upper diagonal and second upper diagonal.

DCT2 and DCT3 with partial butterfly are also used. As shown in document JCT-VC G132 [7], the hardware impact for embedding both forward and inverse DCT2 requires around 20% more area compared to the inverse DCT2 alone.

# Results

The experimental results are presented under the CTC and under the CTC while MTS is enabled for Inter Coding as well. The reference for the later experiment is VTM3 operating with MTS on for Inter Coding as well.

TAF is experiments for size 16, 32 and 64, the following configurations are tested, in compliance with the CE6 test plan.

## Under the common test conditions

Test1.5a applies TAF on MTS with size 32, the performances are 0.07%/0.06%/0.03% for AI/RA/LD with respective relative encoding time of 96%/99%/100% and decoding time of 88%/99%/100%.

Test1.5b applies TAF on MTS with size 32 and 64, the performances are 0.02%/-0.03%/-0.01% for AI/RA/LD with respective relative encoding time of 98%/101%/101% and decoding time of 88%/99%/100%.

Test1.5c applies TAF on MTS with size 16 and 32, the performances are 0.15%/0.09%/0.06% for AI/RA/LD with respective relative encoding time of 94%/99%/100% and decoding time of 85%/98%/100%.

Test1.5d applies TAF on MTS with size 16, 32 and 64, the performances are 0.09%/0.01%/0.07% for AI/RA/LD with respective relative encoding time of 96%/101%/101% and decoding time of 85%/98%/100%.

### CE6-1.5a: TAF applied to size 32

### CE6-1.5b: TAF applied to sizes 32 and 64

### CE6-1.5c: TAF applied to sizes 16 and 32

### CE6-1.5d: TAF applied to sizes 16, 32 and 64

## With MTS enabled for Inter coding

### CE6-1.5a: TAF applied to size 32

### CE6-1.5b: TAF applied to sizes 32 and 64

### CE6-1.5c: TAF applied to sizes 16 and 32

### CE6-1.5d: TAF applied to sizes 16, 32 and 64

# Complexity metrics

For the understanding of complexity, the following information is provided according to CE6.

- Operation counts

- Memory requirements for storing transform cores (in bytes).

| Transform dimension | M = 16 | M = 32 | M=64 |
| --- | --- | --- | --- |
| Memory | 74 * 8bits | 93*8 bits | 189*8 bits |

- Minimum bit-precision required by the transforms

  - To represent transform coefficients or additional parameters: 8 bits

  - Precision of arithmetic operations needed during transform computation: 16 bit for multipliers (int_16s)

  - For storing intermediate data representation (e.g., between two stages), 16 bit (clipping between DCT2 and Adjustment stage.

- List of all combinations of transform types (e.g., DCT-2 and DST-7) and transforms block sizes (e.g., 4x4, 4x8, 8x4 and 8x8) used for primary transformation. DCT2, DST7 and DCT8 are computed through the combination of DCT2 and adjustment.

# Recommendation

The Transform Adjustment Filtering technique provides significant decoding runtimes decreases relative to the direct multiplication approach, arithmetic complexity savings are reported.

The operating point CE6-1.5b seems a good compromise in terms of quality/complexity trade-off:

Hence, it is recommended to adopt TAF for size-32 and 64 MTS.

# Proposed adjustment matrices

Hereunder are listed the additional ROM required by this proposal.

dxx represents the main diagonal for size xx.

dxx_ly dxx_uy represent the yth lower and upper diagonal respectively.

d16[16]={128,128,127,127,126,126,125,123,124,125,124,123,121,119,117,122};

d16_u1[15]={4,8,10,14,16,16,24,26,19,21,24,27,31,35,39};

d16_l1[15]={-4,-8,-10,-14,-16,-16,-23,-25,-19,-21,-24,-27,-31,-34,-38};

d16_u2[14]={-1,0,-1,0,1,1,-1,1,2,2,3,3,5,2};

d16_l2[14]={1,1,2,2,1,2,6,3,1,2,2,4,4,9};

d32[32]={128,128,128,128,128,128,127,127,127,127,126,126,126,126,125,125,124,124,123,123,122,122,121,121,120,119,118,118,117,117,116,121};

d32_u1[31]={2,3,4,5,7,8,9,11,12,14,15,16,16,18,20,21,22,23,25,26,27,28,29,30,32,34,34,36,37,36,41}=-d32_l1;

d32_u2[30]={0,0,0,0,0,0,0,1,1,1,1,1,1,1,2,2,2,2,3,3,3,3,4,4,5,5,5,6,6,6}=d32_l2;

d64[64]={128,128,128,128,128,128,128,128,128,128,128,128,127,127,127,127,127,127,127,127,127,126,126,126,126,126,126,126,125,125,125,125,124,124,124,124,124,123,123,123,122,122,122,122,122,121,121,120,120,120,120,119,119,119,118,118,118,117,117,116,116,116,115,121};

d64_u1[63]={1,2,2,3,3,4,5,5,6,6,7,8,9,9,10,10,11,12,12,13,14,15,15,16,16,16,17,18,19,20,20,21,22,22,22,23,23,25,25,26,27,27,27,27,29,29,30,31,31,31,32,33,33,34,34,34,36,36,37,37,37,38,42}=-d64_l1;

d64_u2[62]={0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,1,1,1,1,1,1,1,1,1,1,1,1,2,2,2,2,2,2,2,2,2,3,3,3,3,3,3,3,4,4,4,4,4,4,5,5,5,5,5,5,6,6,6,6,6,7}=d64_l2;

# Annex: Performance under the common test conditions, at low QP values

In CE6, an additional test to verify the impact of the approximation of core transforms was decided. The QP range for this experiment was set to 1,5,9 and 13 instead of 22,27,32 and 37.

### CE6-1.5a: TAF applied to size 32

### CE6-1.5b: TAF applied to sizes 32 and 64

### CE6-1.5c: TAF applied to sizes 16 and 32

### CE6-1.5d: TAF applied to sizes 16, 32 and 64

# References

- JVET-L0026, Description of Core Experiment 6 (CE6): Transforms and transform signalling

- JVET-J0066, Complexity Reduction for Adaptive Multiple Transforms (AMT) using Adjustment Stages, A. Said et al.

- JVET-K0272, CE6.1.2: Efficient Implementations of AMT with Transform Adjustment Stages, A. Said et al.

- JVET-K0299, CE6-related: Further simplification for AMT complexity reduction (CE6.1.2), P. Philippe et al.

- JVET-K0375, CE6.1.1: Extended AMT, A. Said et al.

- JVET-L0135, CE6: Further simplification of AMT with adjustment stages (Test CE6.1.6b), V. Lorcy, et al.

- JCT-VC G132, Hardware analysis of transform and quantization, M. Budagavi et al.

# Patent rights declaration(s)

b<>com, Orange may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).