| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0096-v3 |
| --- | --- |

| Title: | CE10-related: Inter-intra prediction combination |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van Geert Van der Auwera Adarsh K. Ramasubramonian Vadim Seregin Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Email: | lphamvan@qti.qualcomm.com geertv@qti.qualcomm.com aramasub@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a modified blending technique for inter-intra prediction. Firstly, only the intra planar mode is allowed in the inter-intra mode. Secondly, the weights for planar and merge predicted samples and neighbouring reconstructed samples are adaptively selected in the blending process. It is asserted that the proposed modifications simplify the inter-intra prediction and improve the performance. The average luma BD-rate gain is 0.10%, 0.15%, and 0.22% for the RA, LDB, and LDP configurations, respectively. The average encoding time is reportedly reduced without an average increase of decoding time. (EncT, DecT) is (96%, 99%), (97%, 100%), and (98%, 98%) under the RA, LDB, and LDP respectively.

# Introduction

The inter-intra mode that improves the merge mode by combining one intra prediction and one merge indexed prediction has been adopted in the JVET Macao meeting. Candidates from a set of 4 intra modes, including planar, DC, horizontal and vertical predictions, are evaluated to select the optimal one to combine with a merge indexed prediction. In the combining process, if planar or DC mode is selected, or the size of the block is smaller than 4, equal weights are deployed. Otherwise, the weights for intra and inter predicted samples (wIntra, wInter) are different based on the region of the samples within the coding block.

# Proposed Method

This contribution simplifies and improves the performance of the inter-intra mode. In the first aspect, the mode is simplified by reducing the number of intra modes allowed in inter-intra prediction from 4 to only l, i.e., planar mode. In the second aspect, the performance of the inter-intra mode is enhanced by a position dependent inter-intra combination. In this combination scheme, the top and left reconstructed samples (Figure 1) are blended with the merge and planar predicted samples using a position-dependent weight. Moreover, the weight for intra and inter predicted samples are adaptively selected based on the number of neighbouring intra-coded blocks (Figure 2). The blending is performed using the following equation:

pred(x, y) = (wT*top << 2 + wL*left << 2 + (32 − wL − wT)*(wInter*Merge(x, y) + wIntra*PL(x, y)) + 64) >> 7;

where

wT = 16 >> ((x << 1) >> scl)

wL = 16 >> ((y << 1) >> scl)

scl  = (log2(width) + log2(height) − 2) >> 2

The (wIntra, wInter) weights are adaptively set as follows. If both top and left neighbors are intra-coded, (wIntra, wInter) is set equal to (3, 1). Otherwise, if one of these blocks is intra-coded, these weights are identical, i.e., (2,  2), else the weights are set equal to (1, 3).

Figure 1 An example of top and left reconstructed samples used in inter-intra blending.                               Blue shaped cell is the current sample.

Figure 2 The position of the top and left neighboring blocks that are checked to decide the weights for intra and inter predicted samples in blending.

# Experimental Results

The proposed technique has been implemented on top of VTM3.0. In the simulation, PDPC is disabled for the planar prediction in the inter-intra mode. The experimental results under the RA and LDB configuration are summarized in Table 1.

It is reported that the proposed method has an average luma BD-rate gain of 0.10%, 0.15%, and 0.22% for the RA, LDB, and LDP configuration, respectively. The average encoding time is reportedly reduced without an average increase of decoding time. (EncT, DecT) is (96%, 99%), (97%, 100%), and (98%, 98%) under the RA, LDB, and LDP respectively.

Table 1 Experimental results

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT | DecT |  |
| Class A1 | -0.10% |  | 0.10% |  | 0.26% |  | 96% | 102% |  |
| Class A2 | -0.12% |  | 0.04% |  | 0.14% |  | 97% | 101% |  |
| Class B | -0.13% |  | 0.11% |  | 0.02% |  | 96% | 98% |  |
| Class C | -0.03% |  | 0.14% |  | 0.17% |  | 95% | 98% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.10% |  | 0.10% |  | 0.13% |  | 96% | 99% |  |
| Class D | -0.08% |  | -0.06% |  | 0.07% |  | 96% | 98% |  |
| Class F | -0.04% |  | 0.04% |  | 0.06% |  | 95% | 98% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.17% |  | -0.09% |  | -0.06% |  | 97% | 104% |  |
| Class C | -0.09% |  | 0.38% |  | 0.19% |  | 99% | 99% |  |
| Class E | -0.21% |  | -0.02% |  | -0.32% |  | 95% | 96% |  |
| Overall | -0.15% |  | 0.08% |  | -0.04% |  | 97% | 100% |  |
| Class D | -0.11% |  | -0.46% |  | 0.35% |  | 96% | 100% |  |
| Class F | -0.06% |  | -0.02% |  | 0.43% |  | 95% | 99% |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.26% |  | 0.33% |  | -0.08% |  | 98% | 98% |  |
| Class C | -0.06% |  | 0.49% |  | 0.23% |  | 99% | 100% |  |
| Class E | -0.38% |  | -0.61% |  | -0.07% |  | 96% | 97% |  |
| Overall | -0.22% |  | 0.15% |  | 0.02% |  | 98% | 98% |  |
| Class D | -0.13% |  | -0.42% |  | 0.05% |  | 97% | 99% |  |
| Class F | 0.01% |  | 0.07% |  | -0.26% |  | 98% | 102% |  |

# Conclusions

This contribution proposes to simplify and improve coding performance of the inter-intra mode. The proposed technique leads to an average luma BD-rate gain of 0.10%, 0.15%, and 0.22% for the RA, LDB, and LDP configuration, respectively. The average encoding time is reportedly reduced without an average increase of decoding time. (EncT, DecT) is (96%, 99%), (97%, 100%), and (98%, 98%) under the RA, LDB, and LDP respectively.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).