| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 January 2019 | Document: JVET-M0100 |
| --- | --- |

| Title: | CE3-related: DM-dependent chroma intra prediction modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gagan Rath Fabrice Urban Fabien Racapé | Email: | gagan.rath@technicolor.com fabrice.urban@technicolor.com fabien.racape@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution presents new default prediction modes in chroma intra prediction. The default modes are adaptive to the value of the direct mode (DM). If the DM is directional, two adjacent modes of the DM at offset ± 2 are chosen, instead of the vertical and horizontal modes. Otherwise, if the dual-tree mode is supported, it is proposed to check the intra prediction mode of the luma block at the top left for a directional mode. If the mode is directional, then it is included in the set of four prediction modes along with its two adjacent modes. Otherwise the original default modes are chosen. Simulation results reportedly show an average chroma BD-rate variation of -0.74%, resp. -0.49%, in All Intra (AI), resp. Random Access (RA) configuration, compared with VTM-3.0 in same configurations.

# Introduction

VTM-3.0 specifies 4 default intra prediction modes for the chroma components besides the DM, LM, and MDLM modes. The four modes are fixed as PLANAR (index = 0), DC (index = 1), purely vertical (index 50) and purely horizontal modes (index 18).  If the direct mode (DM) is equal to any of these four modes, then that mode is replaced by the vertical diagonal mode (index = 66).  Each chroma block undergoing intra prediction is tested for rate-distortion performance with these four prediction modes along with the DM, LM, and MDLM modes, and the best prediction mode is signalled to the decoder. The DM mode is signalled using only one bit since it has the highest probability of occurrence.

In VTM-3.0, the DM is derived from the prediction mode of the luma block corresponding to the center position of the chroma block, as shown in Fig.1. In an earlier version of VVC software (in VTM 2.0.1) the DM was derived from the prediction mode of the luma block corresponding to the top-left corner of the chroma block. The proposals JVET-L0053 and JVET-L0272 presented at the Macao meeting demonstrated that deriving the DM as in Fig 1 leads to higher compression efficiency.

Figure 1: DM derivation from the luma block corresponding to the center of the chroma block (from JVET-L0272)

A chroma target block has eight prediction modes in VTM-3.0. The modes include the four default modes PLANAR, DC, purely vertical and purely horizontal modes besides the DM, the LM mode and two MDLM modes denoted as MDLM_L and MDLM_T. If the DM equals any of the four default modes, then that mode is replaced by the vertical diagonal mode. In the LM mode and the two MDLM modes, the chroma block is directly constructed from the collocated reconstructed luma block instead of constructing it based on the reference arrays on the top and the left of the target block. Table 1 lists the values of the candidate modes for different values of the DM.

Table 1: Chroma prediction modes in VTM 3.0 of VVC.

| Chroma prediction modes | DM = PLANAR_IDX | DM=VER_IDX | DM=HOR_IDX | DM=DC_IDX | DM = Angular (except HOR_IDX and VER_IDX) |
| --- | --- | --- | --- | --- | --- |
| chroma_mode[0] | VDIA_IDX | PLANAR_IDX | PLANAR_IDX | PLANAR_IDX | PLANAR_IDX |
| chroma_mode[1] | VER_IDX | VDIA_IDX | VER_IDX | VER_IDX | VER_IDX |
| chroma_mode[2] | HOR_IDX | HOR_IDX | VDIA_IDX | HOR_IDX | HOR_IDX |
| chroma_mode[3] | DC_IDX | DC_IDX | DC_IDX | VDIA_IDX | DC_IDX |
| chroma_mode[4] | LM | LM | LM | LM | LM |
| Chroma_mode[5] | MDLM_L | MDLM_L | MDLM_L | MDLM_L | MDLM_L |
| chroma_mode[6] | MDLM_T | MDLM_T | MDLM_T | MDLM_T | MDLM_T |
| chroma_mode[7] | DM | DM | DM | DM | DM |

All chroma target blocks are tested with the eight prediction modes for best R-D performance and the best mode is signaled with the following mode coding scheme:

Table 2: chroma mode coding in VTM 3.0 of VVC

| Chroma intra prediction mode | Binarization |
| --- | --- |
| DM (chroma_mode[7]) | 0 |
| LM (chroma_mode[4]) | 10 |
| MDLM_L(chroma_mode[5]) | 1110 |
| MDLM_T (chroma_mode[6]) | 1111 |
| chroma_mode[0] | 11000 |
| chroma_mode[1] | 11001 |
| chroma_mode[2] | 11010 |
| chroma_mode[3] | 11011 |

In this proposal, while deriving the chroma prediction modes, it is proposed to exploit the fact that the DM has the highest probability of occurrence. Instead of the vertical and horizontal modes, the adjacent modes of the DM at ±2 (or ± 1) offset are chosen, if the DM is directional. In case the DM is not directional, but is either PLANAR or DC, the prediction mode of the luma block associated with the top left corner of the chroma block is checked. If the mode is directional, it is included along with its adjacent modes at ± 2 (or ± 1) in the set of four prediction modes. Else, the default prediction modes are selected.

# Description and encoder options

Let lumaMode denote the value of the DM in the range [0-66]. The DM is the prediction mode of the luma block located at the center position of the chroma block as in VTM 3.0. Let lumaMode2 denote the prediction mode of the luma block located at the top left corner position of the chroma block, as in VTM 2.0.1. lumaMode2 is considered only if lumaMode is non-directional.  The four default modes are derived as follows.

offset = 61;

mod = 64;

Initialize

Chroma_mode[0] = (lumaMode == PLANAR_IDX ? DC : PLANAR) ;

Chroma_mode[1] = VER_IDX

Chroma_mode[2] = HOR_IDX

Chroma_mode[3] = VDIA_IDX;

If lumaMode > DC_IDX

chroma_mode[0] = PLANAR_IDX

chroma_mode[1] = DC_IDX

chroma_mode[2] = ((lumaMode + offset - 1) % mod) + 2;

chroma_mode[3] = (lumaMode % mod) + 2;

Else if lumaMode2 > DC_IDX

chroma_mode[0] = lumaMode2

chroma_mode[1] = !lumaMode

chroma_mode[2] = ((lumaMode2 + offset - 1) % mod) + 2;

chroma_mode[3] = (lumaMode2 % mod) + 2;

Above, !lumaMode equals DC_IDX if lumaMode is PLANAR_IDX or vice versa. Since the chroma components have half the resolution of the luma component, the neighbor modes of the DM at offset ± 2 are used.

The remaining four candidate modes chroma_mode[4-7] remain unchanged as LM, MDLM-L, MDLM-T, and DM,  respectively.

For information, the neighbor modes of the DM at offset ± 1 have also been tested, as follows:

If lumaMode > DC_IDX

chroma_mode[0] = PLANAR_IDX

chroma_mode[1] = DC_IDX

chroma_mode[2] = ((lumaMode + offset) % mod) + 2;

chroma_mode[3] = ((lumaMode - 1) % mod) + 2;

Else if lumaMode2 > DC_IDX

chroma_mode[0] = lumaMode2

chroma_mode[1] = !lumaMode

chroma_mode[2] = ((lumaMode2 + offset) % mod) + 2;

chroma_mode[3] = ((lumaMode2 - 1) % mod) + 2;

# Results

## The chroma intra prediction modes as presented above are implemented within VTM 3.0 software. This section reports the BD-rate performance compared to VTM 3.0 in AI and RA configurations, respectively, as defined in the Common Test Conditions. First, the results for the case when the adjacent modes are selected at offset ± 2 are presented. Then the results for the case when the adjacent modes are selected at offset ± 1 are presented.

Table 3: BD-rate performance of new chroma prediction modes in All Intra (AI) configuration with the adjacent modes of DM selected at ± 2 offset.

Table 4: BD-rate performance of new chroma prediction modes in random access (RA) configuration with the adjacent modes of DM selected at ± 2 offset.

Table 5: BD-rate performance of new chroma prediction modes in All Intra (AI) configuration with the adjacent modes of DM selected at ± 1 offset.

Table 6: BD-rate performance of new chroma prediction modes in random access (RA) configuration with the adjacent modes of DM selected at ± 1 offset.

# Conclusion

In this contribution, it is proposed to use two adjacent modes of the DM at offset ± 2 for chroma intra prediction, instead of the vertical and horizontal modes. Simulation results reportedly show an average chroma BD-rate variation of -0.74%, resp. -0.49%, in All Intra (AI), resp. Random Access (RA) configuration, compared with VTM-3.0 in same configurations. It is suggested that the chroma modes derivation using DM-dependent modes should be adopted in the reference test model.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).