| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0106 |
| --- | --- |

| Title: | CE4: STMVP without scaling (tests 4.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Name(s) F. Le Léannec, T. Poirier, F. Galpin | Tel: Email: | +33(0)299273198 fabrice.leleannec@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of the non-scaling STMVP motion vector prediction method, initially proposed in JVET-L207.

Two tests are reported, CE4.2.2.a and CE4.2.2.b. CE4.2.2.a adds one single STMVP candidate to the non-sub-block merge list. It reportedly brings 0.11% luma gain over VTM-3.0 in random access.

Test CE4.2.2.b adds to STMVP candidates to the non-sub-block merge candidate list. In addition, in the case of test 4.2.2.b, pairwise averaged motion vector candidates are deactivated. This test reportedly brings 0.15% luma gain over VTM-3.0.

No encoding nor decoding time change are brought in these two tests.

# Introduction

A simplified non-sub-block STMVP motion vector prediction merge candidate was proposed in JVET-L207.

This contribution reports the results of this STMVP motion vector prediction on top of VTM-3.0.

# Proposed STMVP technique

## Test 4.2.2.a

The proposed STMVP technique is based on JVT-L0207. It consists in constructing the STMVP merge candidate as follows.

- One top-neighboring motion vector (MV) of current CU is selected among spatial position (w/2,-1), (2*w,-1) and (w-1,-1), as shown on Figure 1. The first found position where all available MVs use the reference picture index 0 is selected.

- One left-neighboring motion vector (MV) of current CU is selected among spatial position (-1,h/2), (-1,2*h) and (-1,h-1), as shown on Figure 1. The first found position where all available MVs use the reference picture index 0 is selected.

- The same temporal MV predictor as in JVET-L0207 is used. The temporal candidate is set as always available, given that the Right-Bottom position of the current block is inside the picture. The y-position of the temporal candidate is clipped to ensure it lies inside the current CTU row (see Figure 2).

- The STMVP candidate is considered as available if at least 2 spatial-temporal MV predictors are found. If only one neighboring MV is retrieved, no STVMP candidate is included into the merge list.

- No MV scaling is applied.

- The STMVP candidate is computed as the average between the 2 or 3 retrieved spatial and temporal MV predictors.

Figure 1: spatial positions checked to construct the STMVP merge candidate

Figure 2: temporal predictor used to construct the STMVP candidate. If the bottom-right neighboring 4x4 block of current CU is below the current CTU row, then the 4x4 sub-block on the right of the bottom-right corner of current CU is used.

## Test 4.2.2.b

Test 4.2.2.b involves 2 additional aspects over test 4.2.2.a.

- A second STMVP candidate is added to the merge candidate list, if 3 motion vectors (thus including the temporal predictor) had been used to generate the first STMVP candidate. The additional STMVP candidate consists in the average of two spatial MV predictors used to construct the first STMVP candidate.

- Pairwise average candidates are disallowed.

Figure 3: construction of the second STMVP candidate as proposed in test 4.2.2.b

# Experimental results

## Test 4.2.2.a experimental results

This section reports the experimental results of test 4.2.2.a. As can be seen, the test 4.2.2.a brings 0.11% BD-rate gain in luma in random access. In low-delay, 0.05% loss is observed. The loss in low-delay B can is avoided by deactivating STMVP in the default configuration file (see Figure 5). Note encoding and decoding times are not reliable.

Figure 4: performance of test 4.2.2.a over VTM-3.0.

Figure 5: test 4.2.2.a performance in low-delay B, with STMVP deactivated by default

Figure 6 provides the performances of test 4.2.2.a with a merge list size equal to 8. A BD-rate gain of 0.17% is observed in RA. LDB runtimes are not reliable.

Figure 6: performance of test 4.2.2.a over VTM-3.0 with merge list size 8.

## Test 4.2.2.b experimental results

Figure 7 provides the performances of test 4.2.2.b over VTM-3.0. Test b brings 0.15% gain over VTM-3.0 in RA. In LDB configuration, Figure 8 shows no loss is obtained by deactivating STMVP by default. Note that in LDB, the results are not identical to those of VTM-3.0, due to the modified TMVP predictor in merge mode, and the deactivation of pairwise.

Figure 7 performance of test 4.2.2.b over VTM-3.0.

Figure 8: test 4.2.2.b performance in low-delay B, with STMVP deactivated by default

Finally, the performances of test 4.2.2.b over VTM-3.0 with a merge list size equal to 8 are given by Figure 9. An average bitrate saving of 0.23% is obtained in RA, and 0.06% in LDB. Note both pairwise and STMVP are off in these LDB simulations.

Figure 9: performance of test 4.2.2.b over VTM-3.0 with merge list size 8.

# Conclusion

The STMVP configuration which is recommended for adoption to the VTM-4.0 is the test 4.2.2.b. It is to be noted that the pairwise average candidate tool, which has been adopted in Macao, is giving 0.13% gain in tool-off configuration in the VTM-3.0, as reported by the Ahg 13.

The tables below report the gain of the proposed STMVP tool from test 4.2.2.b, over the VTM-3.0 without pairwise. -0.28% and -0.37% bitrate saving is reported in luma in random access, respectively with a merge list size equal to 6 and 8.

It is proposed to adopt test 4.2.2.b in VTM-4.0, with STMVP off in low-delay B configuration file.

Figure 10: performances of test 4.2.2.b over VTM-3.0 w/o pairwise average candidates (merge list size 6)

Figure 11: performances of test 4.2.2.b over VTM-3.0 w/o pairwise average candidates (merge list size 8)

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).