#### Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag[ xCb ][ yPb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ],

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ],

- the bi-prediction weight index gbiIdx.

The bi-prediction weight index gbiIdx is set equal to 0.

The motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are derived by the following ordered steps:

- The derivation process for merging candidates from neighbouring coding units as specified in clause 8.3.2.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, and the luma coding block height cbHeight as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2, with X being 0 or 1, and the bi-prediction weight indices gbiIdxA0, gbiIdxA1, gbiIdxB0, gbiIdxB1, gbiIdxB2.

- The reference indices, refIdxLXCol, with X being 0 or 1, and the bi-prediction weight index gbiIdxCol for the temporal merging candidate Col are set equal to 0.

- The derivation process for temporal luma motion vector prediction as specified in in clause 8.3.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL0Col as inputs, and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col. The variables availableFlagCol, predFlagL0Col and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col		(8-196)

predFlagL0Col = availableFlagL0Col		(8-197)

predFlagL1Col = 0		(8-198)

- When slice_type is equal to B, the derivation process for temporal luma motion vector prediction as specified in clause 8.3.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(8-199)

predFlagL1Col = availableFlagL1Col		(8-200)

- The derivation process for spatial temporal luma motion vector prediction as specified in in clause 8.3.2.4 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variables availableFlagCol, predFlagLXCol, mvLXCol as inputs (X being 0 and 1 when slice_type is equal to B), and the output being the availability flag availableFlagS, the reference indices refIdxLXS of the spatial-temporal merging candidate, the prediction list utilization flags predFlagLXS of the spatial temporal merging candidate, the motion vectors in 1/16 fractional-sample accuracy mvLXS of the spatial temporal merging candidate, the bi-prediction weight indices gbiIdxS of the spatial temporal merging candidate.

- The merging candidate list, mergeCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1
if( availableFlagB0 )
	mergeCandList[ i++ ] = B0		(8-201)
if( availableFlagA0 )
	mergeCandList[ i++ ] = A0
if( availableFlagB2 )
	mergeCandList[ i++ ] = B2

if( availableFlagS )
	mergeCandList[ i++ ] = S

if( availableFlagCol )
	mergeCandList[ i++ ] = Col

- The variable numCurrMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

- When numCurrMergeCand is less than (MaxNumMergeCand − 1) and NumHmvpCand is greater than 0, the following applies:

- The derivation process of history-based merging candidates as specified in 8.3.2.6 is invoked with mergeCandList, and numCurrMergeCand as inputs, and modified mergeCandList and numCurrMergeCand as outputs.

- numOrigMergeCand is set equal to numCurrMergeCand.

- The derivation process for pairwise average merging candidates specified in clause 8.3.2.4 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, numCurrMergeCand and numOrigMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0avgCandk and refIdxL1avgCandk, the prediction list utilization flags predFlagL0avgCandk and predFlagL1avgCandk and the motion vectors mvL0avgCandk and mvL1avgCandk of every new candidate avgCandk being added into mergeCandList. The bi-prediction weight index gbiIdx of every new candidate avgCandk being added into mergeCandList is set equal to 0. The number of candidates being added, numAvgMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand ). When numAvgMergeCand is greater than 0, k ranges from 0 to numAvgMergeCand − 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in clause 8.3.2.5 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm and the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added into mergeCandList. The bi-prediction weight index gbiIdx of every new candidate zeroCandm being added into mergeCandList is set equal to 0. The number of candidates being added, numZeroMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand − numAvgMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xCb ][ yCb ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xCb ][ yCb ] ] ) and X being replaced by 0 or 1:

refIdxLX = refIdxLXN		(8-202)

predFlagLX[ 0 ][ 0 ] = predFlagLXN		(8-203)

mvLX[ 0 ][ 0 ][ 0 ] = mvLXN[ 0 ]		(8-204)

mvLX[ 0 ][ 0 ][ 1 ] = mvLXN[ 1 ]		(8-205)

gbiIdx = gbiIdxN		(8-206)

- When mmvd_flag[ xCb ][ yCb ] is equal to 1, the following applies:

- The derivation process for merge motion vector difference as specified in 8.3.2.7 is invoked with the luma location ( xCb, yCb ), the luma motion vectors mvL0[ 0 ][ 0 ], mvL1[ 0 ][ 0 ], the reference indices refIdxL0, refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] as inputs, and the motion vector differences mMvdL0 and mMvdL1 as outputs.

- The motion vector difference mMvdLX is added to the merge motion vectors mvLX for X being 0 and 1 as follows:

mvLX[ 0 ][ 0 ][ 0 ] += mMvdLX[ 0 ]		(8-207)

mvLX[ 0 ][ 0 ][ 1 ] += mMvdLX[ 1 ]		(8-208)

#### Derivation process for spatial merging candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are as follows, with X being 0 or 1:

- the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2 of the neighbouring coding units,

- the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2 of the neighbouring coding units,

- the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2 of the neighbouring coding units,

- the motion vectors in 1/16 fractional-sample accuracy mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2 of the neighbouring coding units,

- the bi-prediction weight indices gbiIdxA0, gbiIdxA1, gbiIdxB0, gbiIdxB1, and gbiIdxB2.

For the derivation of availableFlagA1, refIdxLXA1, predFlagLXA1 and mvLXA1 the following applies:

- The luma location ( xNbA1, yNbA1 ) inside the neighbouring luma coding block is set equal to ( xCb − 1,  yCb + cbHeight − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA1, yNbA1 ) as inputs, and the output is assigned to the block availability flag availableA1.

- The variables availableFlagA1, refIdxLXA1, predFlagLXA1 and mvLXA1 are derived as follows:

- If availableA1 is equal to FALSE, availableFlagA1 is set equal to 0, both components of mvLXA1 are set equal to 0, refIdxLXA1 is set equal to −1 and predFlagLXA1 is set equal to 0, with X being 0 or 1, and gbiIdxA1 is set equal to 0.

- Otherwise, availableFlagA1 is set equal to 1 and the following assignments are made:

mvLXA1 = MvLX[ xNbA1 ][ yNbA1 ]		(8-209)

refIdxLXA1 = RefIdxLX[ xNbA1 ][ yNbA1 ]		(8-210)

predFlagLXA1 = PredFlagLX[ xNbA1 ][ yNbA1 ]	(8-211)

gbiIdxA1 = GbiIdx[ xNbA1 ][ yNbA1 ]			(8-212)

For the derivation of availableFlagB1, refIdxLXB1, predFlagLXB1 and mvLXB1 the following applies:

- The luma location ( xNbB1, yNbB1 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth − 1, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB1, yNbB1 ) as inputs, and the output is assigned to the block availability flag availableB1.

- The variables availableFlagB1, refIdxLXB1, predFlagLXB1 and mvLXB1 are derived as follows:

- If one or more of the following conditions are true, availableFlagB1 is set equal to 0, both components of mvLXB1 are set equal to 0, refIdxLXB1 is set equal to −1 and predFlagLXB1 is set equal to 0, with X being 0 or 1, and gbiIdxB1 is set equal to 0:

  - availableB1 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB1, yNbB1 ) have the same motion vectors and the same reference indices.

- Otherwise, availableFlagB1 is set equal to 1 and the following assignments are made:

mvLXB1 = MvLX[ xNbB1 ][ yNbB1 ]		(8-213)

refIdxLXB1 = RefIdxLX[ xNbB1 ][ yNbB1 ]		(8-214)

predFlagLXB1 = PredFlagLX[ xNbB1 ][ yNbB1 ]		(8-215)

gbiIdxB1 = GbiIdx[ xNbB1 ][ yNbB1 ]			(8-216)

For the derivation of availableFlagB0, refIdxLXB0, predFlagLXB0 and mvLXB0 the following applies:

- The luma location ( xNbB0, yNbB0 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB0, yNbB0 ) as inputs, and the output is assigned to the block availability flag availableB0.

- The variables availableFlagB0, refIdxLXB0, predFlagLXB0 and mvLXB0 are derived as follows:

- If one or more of the following conditions are true, availableFlagB0 is set equal to 0, both components of mvLXB0 are set equal to 0, refIdxLXB0 is set equal to −1 and predFlagLXB0 is set equal to 0, with X being 0 or 1, and gbiIdxB0 is set equal to 0:

  - availableB0 is equal to FALSE.

  - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB0, yNbB0 ) have the same motion vectors and the same reference indices.

- Otherwise, availableFlagB0 is set equal to 1 and the following assignments are made:

mvLXB0 = MvLX[ xNbB0 ][ yNbB0 ]		(8-217)

refIdxLXB0 = RefIdxLX[ xNbB0 ][ yNbB0 ]		(8-218)

predFlagLXB0 = PredFlagLX[ xNbB0 ][ yNbB0 ]		(8-219)

gbiIdxB0 = GbiIdx[ xNbB0 ][ yNbB0 ]		(8-220)

For the derivation of availableFlagA0, refIdxLXA0, predFlagLXA0 and mvLXA0 the following applies:

- The luma location ( xNbA0, yNbA0 ) inside the neighbouring luma coding block is set equal to ( xCb − 1,  yCb + cbWidth ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA0, yNbA0 ) as inputs, and the output is assigned to the block availability flag availableA0.

- The variables availableFlagA0, refIdxLXA0, predFlagLXA0 and mvLXA0 are derived as follows:

- If one or more of the following conditions are true, availableFlagA0 is set equal to 0, both components of mvLXA0 are set equal to 0, refIdxLXA0 is set equal to −1 and predFlagLXA0 is set equal to 0, with X being 0 or 1, and gbiIdxA0 is set equal to 0:

  - availableA0 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbA0, yNbA0 ) have the same motion vectors and the same reference indices.

- Otherwise, availableFlagA0 is set equal to 1 and the following assignments are made:

mvLXA0 = MvLX[ xNbA0 ][ yNbA0 ]		(8-221)

refIdxLXA0 = RefIdxLX[ xNbA0 ][ yNbA0 ]		(8-222)

predFlagLXA0 = PredFlagLX[ xNbA0 ][ yNbA0 ]	(8-223)

gbiIdxA0 = GbiIdx[ xNbA0 ][ yNbA0 ]		(8-224)

For the derivation of availableFlagB2, refIdxLXB2, predFlagLXB2 and mvLXB2 the following applies:

- The luma location ( xNbB2, yNbB2 ) inside the neighbouring luma coding block is set equal to ( xCb − 1, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB2, yNbB2 ) as inputs, and the output is assigned to the block availability flag availableB2.

- The variables availableFlagB2, refIdxLXB2, predFlagLXB2 and mvLXB2 are derived as follows:

- If one or more of the following conditions are true, availableFlagB2 is set equal to 0, both components of mvLXB2 are set equal to 0, refIdxLXB2 is set equal to −1 and predFlagLXB2 is set equal to 0, with X being 0 or 1, and gbiIdxB2 is set equal to 0:

  - availableB2 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB2, yNbB2 ) have the same motion vectors and the same reference indices.

  - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) have the same motion vectors and the same reference indices.

  - availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4 and merge_triangle_flag[ xCb ][ yCb ] is equal to 0.

- Otherwise, availableFlagB2 is set equal to 1 and the following assignments are made:

mvLXB2 = MvLX[ xNbB2 ][ yNbB2 ]

(8-225)

refIdxLXB2 = RefIdxLX[ xNbB2 ][ yNbB2 ]		(8-226)

predFlagLXB2 = PredFlagLX[ xNbB2 ][ yNbB2 ]		(8-227)

gbiIdxB2 = GbiIdx[ xNbB2 ][ yNbB2 ]		(8-228)

#### Derivation process for spatial temporal merging candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

- the predFlagL0Col  and predFlagL1Col variable the availability

Outputs of this process are as follows, with X being 0 or 1:

- the availability flags availableFlagS of the spatial-temporal merging candidate,

- the reference indices refIdxLXS of the spatial-temporal merging candidate,

- the prediction list utilization flags predFlagLXS of the spatial temporal merging candidate,

- the motion vectors in 1/16 fractional-sample accuracy mvLXS of the spatial temporal merging candidate,

- the bi-prediction weight indices gbiIdxS.

For each luma location ( xNbS0, yNbS0 ) sucessively equal to (xCb - 1, yCb -1) , (xCb + 2*cbWith, yCb - 1) and (xCb + cbWidth-1, yCb − 1 ), the following applies:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbS0, yNbS0 ) as inputs, and the output is assigned to the block availability flag availableS0.

- If one or more of the following conditions are true, availableFlagS0 is set equal to 0, both components of mvLXS0 are set equal to 0, refIdxLXS0 is set equal to −1 and predFlagLXS0 is set equal to 0, with X being 0 or 1, and gbiIdxS0 is set equal to 0:

  - availableS0 is equal to FALSE

  - PredFlagLX[ xNbS0 ][ yNbS0 ] is equal to true and RefIdxLX[ xNbA0 ][ yNbA0 ] != 0

- Otherwise, availableFlagS0 is set equal to 1, the current loop is stopped and the following assignments are made:

mvLXS0 = MvLX[ xNbS0 ][ yNbS0 ]

refIdxLXS0 = RefIdxLX[ xNbS0 ][ yNbS0 ]

predFlagLXS0 = PredFlagLX[ xNbS0 ][ yNbS0 ]

For each luma location ( xNbS1, yNbS1 ) successively equal to (xCb + cbWith/2, yCb -1) , (xCb + 2*cbWith, yCb - 1) and (xCb + cbWidth-1, yCb − 1 ), the following applies:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbS1, yNbS1 ) as inputs, and the output is assigned to the block availability flag availableS1.

- If one or more of the following conditions are true, availableFlagS1 is set equal to 0, both components of mvLXS1 are set equal to 0, refIdxLXS1 is set equal to −1 and predFlagLXS1 is set equal to 0, with X being 0 or 1, and gbiIdxS1 is set equal to 0:

  - availableS1 is equal to FALSE

  - PredFlagLX[ xNbS1 ][ yNbS1 ] is equal to true and RefIdxLX[ xNbA1 ][ yNbA1 ] != 0

- Otherwise, availableFlagS1 is set equal to 1, the current loop is stopped and the following assignments are made:

mvLXS1 = MvLX[ xNbS1 ][ yNbS1 ]

refIdxLXS1 = RefIdxLX[ xNbS1 ][ yNbS1 ]

predFlagLXS1 = PredFlagLX[ xNbS1 ][ yNbS1 ]

If availableS0 is equal to FALSE and availableS1 is equal to FALSE then the following applies:

- availableFlagS is set equal to FALSE

Otherwise if availableS0 or availebleS1 is equal to FALSE and availableFlagCol is equal to FALSE then the following applies:

- availableFlagS is set equal to FALSE

Otherwisde availableFlagS is set equal to TRUE and the following applies, with X being 0 or 1:

- mvTempX=0

- mvNumX=0

- predFlagLXS is set to FALSE

- if predFlagLXS0 is equal to TRUE then the following applies

- mvTempX = mvTempX + mvLXS0

- mvNumX = mvNumX + 1

- if predFlagLXS1 is equal to TRUE then the following applies

- mvTempX = mvTempX + mvLXS1

- mvNumX = mvNumX + 1

- if predFlagLXCol  is equal to TRUE then the following applies

- mvTempX = mvTempX + mvLXCol

- mvNumX = mvNumX + 1

- if mvNumX is equal to 3

- mvLXS = mvTempX * 43 / 128

- predFlagLXS is set equal to TRUE

- refIdxLXS = 0

- gbiIdxS = 0

- else if mvNumX is equal to 2

- mvLXS = mvTempX / 2

- predFlagLXS is set equal to TRUE

- refIdxLXS = 0

- gbiIdxS = 0

- else if mvNumX is equal to 1

- mvLXS = mvTempX

- predFlagLXS is set equal to TRUE

- refIdxLXS = 0

- gbiIdxS = 0