| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 09–18 January 2019 | Document: JVET-M0111-v2 |
| --- | --- |

| Title: | AHG13: On bi-prediction with weighted averaging and weighted prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Ye, Jie Chen, Mingyuan Yang 525 Almanor Avenue Sunnyvale, CA 94085 USA Philippe Bordes, Edouard François Takeshi Chujoh, Tomohiro Ikai | Tel: Email: | yan.ye@alibaba-inc.com philippe.bordes@technicolor.com chujoh.takeshi@sharp.co.jp |
| Source: | Alibaba Group U.S., Technicolor, Sharp Corporation |  |  |

_____________________________

# Abstract

At the 12th JVET meeting, bi-prediction with weighted averaging (BWA) (previously referred to as generalized bi-prediction or GBi in JVET-L0646) was adopted into the VVC draft 3. It was further agreed that “weighted prediction should also be put in the draft,” and that BWA “and weighted prediction would be mutually exclusive at the picture level.” As of VVC draft 3 version 7, weighted prediction was not yet incorporated into the draft text. Therefore, this contribution first proposes to add the HEVC-style weighted prediction into the VVC. Further, this contribution proposes three alternatives to ensure that BWA and weighted prediction are invoked in a “mutually exclusive” manner in the sense that the decoding process of both tools will not be invoked at the same time.

In v2, proponents from Sharp Corporation were added, and a presentation PPT file was added to the zip archive.

# Adding HEVC-style weighted prediction

In order to implement the HEVC-style weighted prediction, the following syntax changes should be made to PPS and slice header:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| init_qp_minus26 | se(v) |
| transform_skip_enabled_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| diff_cu_qp_delta_depth | ue(v) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_luma | ue(v) |
| slice_max_mtt_hierarchy_depth_luma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_luma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_luma | ue(v) |
| slice_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_chroma | ue(v) |
| slice_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_chroma | ue(v) |
| slice_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if ( slice_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enable_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( sps_sao_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| luma_weight_l0_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| chroma_weight_l0_flag[ i ] | u(1) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| if( luma_weight_l0_flag[ i ] ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_weight_l0_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| if( slice_type = = B ) { |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| if( luma_weight_l1_flag[ i ] ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_weight_l1_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

The accompanying WORD document implements the suggested changes on top of the version 6 of VVC draft 3.

# Invoking BWA and WP in a mutually exclusive manner

Though BWA and WP both apply weights to the motion compensated prediction signals, the specific operations are different. In the bi-prediction case, WP applies a linear weight and a constant offset to each of the prediction signals depending on the (weight, offset) parameters associated with the ref_idx that is used to generate the prediction signal. Whereas there are some range constraints on these (weight, offset) values, the allowed ranges are relatively large and there is no normalization constraint on the parameter values. WP parameters are signaled at the picture level (in slice headers). WP has been shown to provide very large coding gain for fading sequences, and is generally expected to be enabled for such content.

BWA uses a CU level index to indicate how to combine the two prediction signals in the case of bi-prediction. The BWA applies weighted averaging, i.e., the two weights add up to 1. BWA provides about 0.66% coding gain for the CTC RA configuration, but is much less effective than WP for the fading content (JVET-L0201).

According to the meeting notes of the 12th meeting, these two coding tools are to be used mutually exclusively in VVC. In this contribution, the phrase “mutually exclusive” is interpreted to mean that the decoding processes of the two tools will not be invoked at the same time, and three alternatives are proposed to ensure this.

In the first alternative, the PPS syntax element weight_bipred_flag is checked to determine if the gbi_idx is signaled at the current CU. The syntax signaling is modified as follows:

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_gbi_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && !weighted_bipred_flag && cbWidth * cbHeight >= 256 ) |  |
| gbi_idx[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |

The first alternative will disable BWA for all the reference pictures of the current slice, if the current slice refers to a PPS for which weighted_bipred_flag is set to 1.

In the second alternative, BWA is disabled only if both of the reference pictures used in the bi-prediction have turned on weighted prediction, i.e., the (weight, offset) parameters of these reference pictures have non-default values. This allows the bi-predicted CUs that use reference pictures with default WP parameters (i.e. WP is not invoked for these CUs) to still be able to use BWA.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_gbi_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && luma_weight_l0_flag [ ref_idx_l0[ x0 ][ y0 ] ] == 0 && chroma_weight_l0_flag [ ref_idx_l0[ x0 ][ y0 ] ] == 0 && luma_weight_l1_flag [ ref_idx_l1[ x0 ][ y0 ] ] == 0 && chroma_weight_l1_flag [ ref_idx_l1[ x0 ][ y0 ] ] == 0 && cbWidth * cbHeight >= 256 ) |  |
| gbi_idx[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |

In the third alternative, BWA is signaled similarly as for WP in the PPS with an additional flag (generalized_bi_prediction_flag), as follows:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(1) |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| if ( !weighted_bipred_flag ) |  |
| generalized_bi_prediction_flag | u(1) |
| Else |  |
| generalized_bi_prediction_flag = 0 |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( generalized_bi_prediction_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && cbWidth * cbHeight >= 256 ) |  |
| gbi_idx[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |

In this third alternative, the flag sps_gbi_enabled_flag (in SPS) is no longer required since BWA is enabled/disabled with generalized_bi_prediction_flag in PPS.

In P-slices, BWA is not active and WP is enabled/disabled with weighted_pred_flag (same semantic as HEVC).

In B-slices, if weighted_bipred_flag=1, then WP is enabled both for uni-directional and bi-prediction (same semantic as HEVC). The mutual exclusion of BWA and WP concerns the B-slice case only, that’s why generalized_bi_prediction_flag is inferred to 0 if weighted_bipred_flag=1. If weighted_bipred_flag=0, generalized_bi_prediction_flag can be 1 or 0 and it replaces “sps_gbi_enabled_flag” in the coding_unit() syntax.

generalized_bi_prediction_flag specifies whether bi-prediction with CU weights can be used for inter prediction. If generalized_bi_prediction_flag is equal to 0, the syntax shall be constrained such that no bi-prediction with CU weights is used in the CVS, and gbi_idx is not present in coding unit syntax of the CVS. Otherwise (generalized_bi_prediction_flag is equal to 1), bi-prediction with CU weights can be used in the CVS.

# Patent rights declaration(s)

Alibaba Group U.S. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).