| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0115-v2 |
| --- | --- |

| Title: | CE10-related: pipeline reduction for LIC and GBI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Franck Galpin, Tangi Poirier | Tel: Email: | +33.2.99.27.32.42 philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes to reduce pipeline for LIC by disabling GBI when LIC is enabled at CU level.

The proposed process has been implemented both in the JVET VTM3.0 on top of regular LIC (CE10.5.2) and combined with JVET-M0224 (LIC simplifications). The reported simulation results show an average BD-rate gain of -0.54%,-0.26%,-0.33% in Y,Cb,Cr in RA configuration and of -0.48%,-0.54%,-0.46% in LDB configuration implemented on top of (CE10.5.2), and an average BD-rate gain of -0.59%,-0.32%,-0.42% in Y,Cb,Cr in RA configuration and of -0.53%,-0.59%,-0.69% in LDB configuration when combined with JVET-M0224. The encoding and decoding times are slightly reduced compared to the regular LIC.

# Introduction / Problem Statement / …

The Local Illumination Compensation (LIC) is an inter tool which allows compensating temporal illumination variation locally, between samples from reference frame (inter-prediction) and current frame. In the worst case, in inter, the two processes LIC and GBI with non-default weights can be applied successively (Figure 1).

Figure 1: overview of the VTM-3.0 (CE10.5.2) pipeline in inter (top) and intra (bottom).

# Proposed pipeline reduction for LIC

When LIC and GBI with non-default weights are enabled together, two successive weighting and clipping stages are cascaded, what may be sub-optimal. It may be also counter-productive to apply additional GBI non-default weights on top of LIC weights (CE10.5.2 case, and conversely in case of JVET-M0224) that were specially computed to compensate illumination variation.

To overcome this inconsistency and to alleviate the inter pipeline when LIC is enabled, it is proposed:

- Mutual exclusion of LIC and GBI.

- LIC-flag is decoded first. If LIC-flag is true, then GBi-index is not decoded and inferred to defaults.

Figure 2: proposed reduced LIC pipeline in inter (top).

The Table 1 summarizes the results of the proposed reduced pipeline for LIC (left) implemented on VTM-3.0 (CE10.5.2) with CTC, on random access (RA) and low delay B (LDB), and BD-rate loss versus (CE10.5.2). The BD-rate gains of the proposed LIC pipeline are very close to regular LIC, while the encoder/decoder runtimes are reduced.

Besides alleviating the decoder pipeline worst case, the proposed modification allows reducing the encoder modes combinatory.

Table 1: BD-rate gains of the proposed reduced pipeline for LIC (left) on top of VTM-3.0 (CE10.5.2) and BD-rate loss compared to (CE10.5.2) (right).

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Proposed LIC excludes GBI |  |  |  |  | Proposed LIC excludes GBI difference with CE10.5.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.93% | -0.47% | -0.83% | 144% | 98% | 0.03% | 0.13% | 0.08% | 97% | 98% |
| Class A2 | -0.36% | -0.17% | -0.23% | 122% | 100% | 0.02% | -0.03% | -0.01% | 98% | 100% |
| Class B | -0.57% | -0.15% | -0.20% | 129% | 99% | 0.03% | 0.07% | 0.05% | 98% | 98% |
| Class C | -0.33% | -0.32% | -0.22% | 136% | 101% | 0.03% | 0.02% | 0.01% | 99% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.54% | -0.26% | -0.33% | 132% | 99% | 0.03% | 0.05% | 0.03% | 98% | 98% |
| Class D | -0.02% | -0.01% | -0.10% | 133% | 101% | 0.02% | 0.04% | 0.05% | 99% | 98% |
| Class F | -0.85% | -0.61% | -0.55% | 129% | 101% | 0.03% | -0.03% | -0.03% | 98% | 99% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Proposed LIC excludes GBI |  |  |  |  | Proposed LIC excludes GBI difference with CE10.5.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.75% | -0.87% | -0.59% | 144% | 99% | 0.03% | -0.02% | 0.21% | 98% | 98% |
| Class C | -0.47% | -0.39% | -0.55% | 157% | 101% | 0.02% | -0.17% | -0.02% | 99% | 100% |
| Class E | -0.03% | -0.21% | -0.11% | 104% | 98% | 0.05% | -0.16% | 0.16% | 99% | 96% |
| Overall | -0.48% | -0.54% | -0.46% | 137% | 99% | 0.03% | -0.11% | 0.12% | 99% | 98% |
| Class D | -0.11% | -0.14% | -0.26% | 147% | 99% | 0.00% | 0.30% | 0.14% | 99% | 97% |
| Class F | -2.65% | -1.84% | -1.82% | 147% | 99% | -0.01% | 0.27% | 0.10% | 99% | 99% |

The Table 2 summarizes the results of the proposed reduced pipeline for LIC combined with JVET-M0224 (left) and BD-rate loss versus JVET-M0224 (right), on random access (RA) and low delay B (LDB). The BD-rate gains of the proposed LIC pipeline remain very close to JVET-M0224, while GBI weighting and LIC weighting are not cascaded.

Table 2: BD-rate gains of the proposed reduced pipeline for LIC combined JVET-M0224 (left) and BD-rate loss compared to JVET-M0224 (right).

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Proposed LIC excludes GBI Combined with JVET-M0224 |  |  |  |  | Proposed LIC excludes GBI difference with JVET-M0224 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.06% | -0.59% | -0.88% | 146% | 99% | 0.06% | 0.08% | 0.15% | 98% | 100% |
| Class A2 | -0.37% | -0.10% | -0.30% | 124% | 99% | 0.04% | 0.08% | -0.01% | 98% | 99% |
| Class B | -0.62% | -0.23% | -0.35% | 129% | 99% | 0.03% | 0.00% | -0.03% | 99% | 101% |
| Class C | -0.36% | -0.40% | -0.27% | 136% | 99% | 0.03% | 0.00% | 0.04% | 99% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.59% | -0.32% | -0.42% | 133% | 99% | 0.04% | 0.03% | 0.03% | 98% | 100% |
| Class D | -0.08% | -0.05% | 0.00% | 133% | 103% | 0.00% | 0.09% | 0.04% | 99% | 102% |
| Class F | -1.00% | -0.69% | -0.60% | 129% | 100% | 0.08% | 0.07% | 0.12% | 98% | 101% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Proposed LIC excludes GBI Combined with JVET-M0224 |  |  |  |  | Proposed LIC excludes GBI difference with JVET-M0224 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.81% | -0.99% | -0.72% | 157% | 106% | 0.02% | -0.16% | 0.16% | 98% | 99% |
| Class C | -0.53% | -0.48% | -0.77% | 170% | 116% | 0.03% | 0.09% | -0.03% | 98% | 99% |
| Class E | -0.06% | -0.08% | -0.53% | 112% | 117% | 0.02% | 0.13% | -0.51% | 100% | 100% |
| Overall | -0.53% | -0.59% | -0.69% | 148% | 112% | 0.02% | -0.01% | -0.07% | 99% | 100% |
| Class D | -0.14% | -0.32% | -0.42% | 155% | 108% | 0.05% | 0.06% | 0.07% | 99% | 101% |
| Class F | -2.76% | -2.15% | -2.18% | 157% | 111% | 0.17% | -0.45% | -0.01% | 99% | 101% |

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).