| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0121r1 |
| --- | --- |

| Title: | AHG12: On Rectangular Tile Group |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmad Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a design of rectangular tile group with associated syntax update. The design can be summarised as follows:

- Define tile group as a group of integer number of complete tiles covering a rectangular region of a picture

- Signal tile group layout including number of tile groups, number of tiles per tile group, address of top-left tile and the bottom-right tile of each tile group in PPS;

- Provide CTB raster and tile scan conversion process, tile raster and tile group scan conversion process

- Propose tile group header and tile group data syntax update

# Introduction

In JVET-L0686, picture is divided into tile groups and tiles. A tile group is a sequence of tiles in tile raster scan of a picture. A tile is a sequence of CTUs that cover a rectangular region of a picture. For example, a picture may be divided into 9 tiles and 3 tile groups as shown in Figure 1.

Figure 1 A picture with 12 by 6 luma CTUs that is partitioned into 9 tiles and 3 tile groups

The tile group address is specified by the first tile address. Tile related syntax elements is included in tile group header as shown in Table 1.

Table 1 Tile Group Header Syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| tile_group_type | ue(v) |
| … |  |
| } |  |

The tile and slice design in HEVC is to address parallelism and error concealment needs, raster scan CTU slices was replaced by tile group in 12th JVET meeting.

Tiles or tile group can be used for various purposes, such as parallelization, region-of-interest driven content consumption, sub-picture based random access, and MCTS based sub-bitstream merging or stitching for viewport dependent streaming. Figure 2 to Figure 5 are some use cases from JVET-L0182, JVET-L0202 and OMAF specification. Since the majority of video processing is carried out on rectangular region of the pictures, the current tile group layout may not suit these applications.

Figure 2 Panorama with a particular Region of Interest

Figure 3 Cube-map projection and face packing

Figure 4 Example of merging of HEVC MCTS-based sub-picture tracks of the same resolution

Figure 5 Example of extractor track combining MCTSs from bitstreams of different resolution

# Rectangular tile group

JVET-K0260 and JVET-L0394 proposed new tile design to enable flexible tile layout to address specific needs. Instead of specifying flexible tile grid with additional overhead, one alternative is to group rectangular region of tiles into a tile group. This contribution proposes to define a tile group as an integer number of complete tiles covering a rectangular region of a picture. In the PPS, the tile raster scan index of the top-left and the bottom-right tile of a tile group are signalled. There should be no overlap or gap between tile groups, and tile groups are ordered by the tile_group_start_address in tile raster scan of a picture. Table 2 is the proposed tile group syntax elements highlighted in turquoise in PPS.

Table 2 Proposed picture parameter set syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1 [ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1 [ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| num_tile_groups_minus1 | ue(v) |
| for (i = 0; i <= num_tile_groups_minus1; i ++) { |  |
| num_tiles_in_tile_group_minus1 [ i ] | ue(v) |
| tile_group_start_address [ i ] | u(v) |
| if (num_tiles_in_tile_group_minus1 [ i ] != 0) |  |
| tile_group_end_address [ i ] | u(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

num_tile_groups_minus1 plus 1 specifies the number of tile groups in a picture. When not present, the value of num_tile_groups_minus1 is inferred equal to 0.

tile_group_start_address[ i ] and tile_group_end_address[ i ] specify the address of the first tile, or tile located at the top-left corner in the tile raster scan of a picture, and the address of the last tile, or the tile located at the bottom-right corner in the tile raster scan of a picture in i-th tile group. The length of tile_group_start_address[ i ] and tile_group_end_address[ i ] are Ceil( Log2 ( NumTilesInPic ) ) bits. The value of tile_group_start_address[ i ] and tile_group_end_address[ i ] shall be in the range of 0 to NumTilesInPic − 1, inclusive, and the value of tile_group_start_address [ i ] and tile_group_end_address[ i ] shall not be equal to the value of tile_group_start_address[ j ] and tile_group_end_address[ j ] of any other coded tile group NAL unit of the same coded picture. When not present, the value of tile_group_start_address[ i ] is inferred equal to 0, and the value of tile_group_end_address[ i ] is inferred to be equal to the value of tile_group_start_address[ i ].

Figure 6 is an example of a picture partitioned into 8x4 CTUs and 3 tile groups, each tile group covers a rectangular area of a picture.

Figure 6 Rectangular tile group grid example

Table 3 is the proposed tile group header syntax.

Table 3 Proposed Tile Group Header Syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_index | u(v) |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| tile_group_type | ue(v) |
| … |  |
| } |  |

tile_group_index specifies the index of the tile group. The length of tile_group_index is Ceil( Log2 ( NumTileGroupsInPic ) ) bits. The value of tile_group_index shall be in the range of 0 to NumTileGroupsInPic − 1, inclusive, and the value of tile_group_index shall not be equal to the value of tile_group_index of any other coded tile group NAL unit of the same coded picture. When tile_group_index is not present it is inferred to be equal to 0.

# CTB raster and tile scanning conversion process

Current tile group is defined as a sequence of of tiles in tile raster scan of a picture, the CTB raster and tile scanning conversion process specified in HEVC can be applied. In case tiles are not sequentially grouped into a tile group, the CTB raster and tile scanning conversion needs to be updated to support rectangular shaped tile group. Figure 7 illustrates CTB raster scan, tile scan for current sequential tile group and tile scan for proposed rectangular shaped tile group examples.

Figure 7 CTB raster and tile scanning examples

The following is specification text for the proposal relative to JVET-L0686 with changes highlighted in turquoise.

The list ColWidth[ i ] for i ranging from 0 to num_tile_columns_minus1, inclusive, specifying the width of the i-th tile column in units of CTBs, is derived as follows:

if( uniform_tile_spacing_flag )
	for( i = 0; i  <=  num_tile_columns_minus1; i++ )
		ColWidth[ i ] = ( ( i + 1 ) * PicWidthInCtbsY ) / ( num_tile_columns_minus1 + 1 ) − 
						( i * PicWidthInCtbsY ) / ( num_tile_columns_minus1 + 1 )
else {
	ColWidth[ num_tile_columns_minus1 ] = PicWidthInCtbsY	(6-1)
	for( i = 0; i < num_tile_columns_minus1; i++ ) {
		ColWidth[ i ] = tile_column_width_minus1[ i ] + 1
		ColWidth[ num_tile_columns_minus1 ]  −=  ColWidth[ i ]
	}
}

The list RowHeight[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of CTBs, is derived as follows:

if( uniform_tile_spacing_flag )
	for( j = 0; j  <=  num_tile_rows_minus1; j++ )
		RowHeight[ j ] = ( ( j + 1 ) * PicHeightInCtbsY ) / ( num_tile_rows_minus1 + 1 ) − 
						( j * PicHeightInCtbsY ) / ( num_tile_rows_minus1 + 1 )
else {
	RowHeight[ num_tile_rows_minus1 ] = PicHeightInCtbsY	(6-2)
	for( j = 0; j < num_tile_rows_minus1; j++ ) {
		RowHeight[ j ] = tile_row_height_minus1[ j ] + 1
		RowHeight[ num_tile_rows_minus1 ]  −=  RowHeight[ j ]
	}
}

The list ColBd[ i ] for i ranging from 0 to num_tile_columns_minus1 + 1, inclusive, specifying the location of the i-th tile column boundary in units of CTBs, is derived as follows:

for( ColBd[ 0 ] = 0, i = 0; i  <=  num_tile_columns_minus1; i++ )
		ColBd[ i + 1 ] = ColBd[ i ] + ColWidth[ i ]		(6-3)

The list RowBd[ j ] for j ranging from 0 to num_tile_rows_minus1 + 1, inclusive, specifying the location of the j-th tile row boundary in units of CTBs, is derived as follows:

for( RowBd[ 0 ] = 0, j = 0; j  <=  num_tile_rows_minus1; j++ )
	RowBd[ j + 1 ] = RowBd[ j ] + RowHeight[ j ]		(6-4)

The list TileGrpWidthInCtbs[ i ] and TileGrpHeightInCtbs[ i ] for i ranging from 0 to num_tile_groups_minus1, inclusive, specifying the width and height of the i-th tile group in units of CTBs, is derived as follows:

for ( i = 0; i <= num_tile_groups_minus1; i++ ) {
				FirstTileX[ i ] = tile_group_start_address[ i ] % ( num_tile_columns_minus1 + 1 ) 
				FirstTileY[ i ] = tile_group_start_address[ i ] / ( num_tile_columns_minus1 + 1 ) 
				LastTileX[ i ] = tile_group_end_address[ i ] % ( num_tile_columns_minus1 + 1 ) 
				LastTileY[ i ] = tile_group_end_address[ i ] / ( num_tile_columns_minus1 + 1 ) 
				for ( TileGrpWidthInCtbs[ i ] = 0, j = FirstTileX[ i ]; j <= LastTileX[ i ]; j++ )
					TileGrpWidthInCtbs[ i ] += ColWidth[ j ] 
				for ( TileGrpHeightInCtbs[ i ] = 0, j = FirstTileY[ i ] ; j <= LastTileY[ i ]; j++ )
					TileGrpHeightInCtbs[ i ] += RowHeight[ j ] 
			}

The list TileGrpWidthInTiles[ i ] and TileGrpHeightInTiles[ i ] for i ranging from 0 to num_tile_groups_minus1, inclusive, specifying the width and height of the i-th tile group in units of tiles, is derived as follows:

for ( i = 0; i <= num_tile_groups_minus1; i++ ) {

TileGrpWidthInTiles[ i ]  = LastTileX[ i ] – FirstTileX[ i ] + 1
				TileGrpHeightInTiles[ i ] = LastTileY[ i ] – FirstTileY[ i ]  + 1
			}

The list NumTilesInTileGroup[ i ] for i ranging from 0 to num_tile_groups_minus1, inclusive, specifying the number of tiles of the i-th tile group, is derived as follows:

for ( i = 0; i <= num_tile_groups_minus1; i++ )

NumTilesInTileGroup [ i ]  = TileGrpWidthInTiles[ i ] * TileGrpHeightInTiles[ i ]

The list CtbAddrRsToTs[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in CTB raster scan of a picture to a CTB address in tile scan, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ ) { tbX = ctbAddrRs % PicWidthInCtbsY tbY = ctbAddrRs / PicWidthInCtbsY for( i = 0; i <= num_tile_groups_minus1; i++ ) if ( tbX >= ColBd[ firstTileX [ i ] ] && tbY >= RowBd[ firstTileY[ i ] ] ) tileGrpIdx = i for (CtbAddrRsToTs[ ctbAddrRs ] = 0, i = 0; i < tileGrpIdx; i++ ) CtbAddrRsToTs[ ctbAddrRs ] += TileGrpWidthInCtbs[ i ] * TileGrpHeightInCtbs[ i ] for( j = FirstTileX[tileGrpIdx]; j <= LastTileX[tileGrpIdx]; j ++ ) if ( tbX > = ColBd [ j ] ) tileX = j for( i = FirstTileY[tileGrpIdx]; i <= LastTileY[tileGrpIdx]; i ++ ) if ( tbY >= RowBd[i] ) tileY = i for ( j = FirstTileX[ tileGrpIdx ]; j < tileX; j++ ) CtbAddrRsToTs[ ctbAddrRs ] += RowHeight[ tileY ] * ColWidth [ j ] for ( i = FirstTileY[tileGrpIdx]; i < tileY; i++ ) CtbAddrRsToTs[ ctbAddrRs ] += TileGrpWidthInCtbs[ tileGrpIdx ] * RowHeight[ i ] CtbAddrRsToTs[ ctbAddrRs ] += (tbY – RowBd[tileY]) * ColWidth[ tileX ] + tbX – ColBd[ tileX ] }

The list CtbAddrTsToRs[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a CTB address in CTB raster scan of a picture, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ ) CtbAddrTsToRs[ CtbAddrRsToTs[ ctbAddrRs ] ] = ctbAddrRs

The list TileId[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile ID in tile group scan, is derived as follows:

for( k = 0, tileIdx = 0; k <= num_tile_group_minus1; k++ ) for( i = 0; i < TileGroupHeightInTiles[ k ]; i++ ) for( j = 0; j < TileGroupWidthInTiles[ k ]; j++, tileIdx++ ) for( y = RowBd[ firstTileY[ k ] + i ]; y < RowBd[ firstTileY [ k ] + i + 1 ]; y++ ) for( x = ColBd[ firstTileX [ k ] + j ]; x < ColBd[ firstTileX [ k ] + j + 1 ]; x++ ) TileId[ CtbAddrRsToTs[ y * PicWidthInCtbsY+ x ] ] = tileIdx

The list NumCtusInTile[ tileIdx ] for tileIdx ranging from 0 to NumTilesInPic − 1, inclusive, specifying the conversion from a tile index to the number of CTUs in the tile, is derived as follows:

for( k = 0, tileIdx = 0; k  <= num_tile_group_minus1; k++ )
			for( i = 0; i < TileGrpHeightInTiles[ k ]; i++ ) 
				for( j = 0; j < TileGrpWidthInTiles[ k ]; j++, tileIdx++ ) 
					NumCtusInTile[tileIdx] =ColWidth[FirstTileX[ k ] +j ] * RowHeight[FirstTileY[ k ] +i ]

The list FirstCtbAddrTs[ tileIdx ] for tileIdx ranging from 0 to NumTilesInPic − 1, inclusive, specifying the conversion from a tile ID to the CTB address in tile scan of the first CTB in the tile are derived as follows:

for( k = 0, tileIdx = 0, FirstCtbAddrTs[0] = 0; k < NumTileGroupsInPic; k++ )

for( i  = 0, i < TileGroupHeightInTiles[ k ]; i++)
				for(j = 0, j < TileGroupWidthInTiles[ k ]; j++, tileIdx++ ) 
					FirstCtbAddrTs[ tileIdx + 1 ] = FirstCtbAddrTs[ tileIdx ] + NumCtusInTile[tileIdx]

# Tile raster and tile group scanning conversion process

The tile group scan is specific sequential ordering of tiles partitioning a picture in which the tiles are order consecutively in tile raster scan in a tile group. The tile raster of a picture and tile scanning of a tile group conversion needs to be updated as well to support rectangular tile group. Figure 8 illustrates tile raster scan, tile group scan for current tile group and tile group scan for proposed rectangular tile group examples.

Figure 8 Tile raster scan and tile group scan example

The list TileAddrRsToTs[ tileAddrRs ] for tileAddrRs ranging from 0 to NumTilesInPic – 1, inclusive, specifying the conversion from a tile address in tile raster scan of a picture to a tile address in tile group scan, is derived as follows:

for( tileAddrRs = 0; tileAddrRs < NumTilesInPic; tileAddrRs++ ) {
		tileX = tileAddrRs % (num_tile_columns_minus1 + 1)
		tileY = tileAddrRs / (num_tile_columns_minus1 + 1)
		for( i = 0, numTiles = 0; i <= num_tile_groups_minus1; i++ )
			if ( tileX >= FirstTileX[ i ] && tileY >= FirstTileY[ i ] ) 
				tileGrpIdx = i
		for( j = 0, TileAddrRsToTs[ tileAddrRs ] = 0; j <= tileGrpIdx; j++ )
			TileAddrRsToTs[ tileAddrRs ] += num_tiles_in_tile_group_minus1[ i ] + 1
		TileAddrRsToTs[ tileAddrRs ] += (tileY – FirstTileY[ tileGrpIdx ]) * tileGrpWidthInTiles[ tileGrpIdx ] + tileX – FirstTileX[ tileGrpIdx]
	}

The list TileAddrTsToRs[ tileAddrTs ] for tileAddrTs ranging from 0 to NumTilesInPic − 1, inclusive, specifying the conversion from a tile address in tile group scan to a tile address in raster scan of a picture, is derived as follows:

| for( tileAddrRs = 0; tileAddrRs < NumTilesInPic; tileAddrRs++ ) TileAddrTsToRs[ TileAddrRsToTs[ tileAddrRs ] ] = tileAddrRs |  |
| --- | --- |

# Tile group data syntax

Table 4 shows the proposed tile group data syntax structure with changes highlighted in turquoise.

Table 4 Proposed tile group data structure

| tile_group_data( ) { | Descriptor |
| --- | --- |
| tileIdx = TileAddrRsToTs[firstTileAddr[tile_group_index]] |  |
| for( i = 0; i < NumTilesInTileGroup[tile_group_index] ; i++, tileIdx++ ) { |  |
| ctbAddrInTs = FirstCtbAddrTs[ tileIdx ] |  |
| for( j = 0; j < NumCtusInTile[ tileIdx ]; j++, ctbAddrInTs++ ) { |  |
| CtbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| } |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < num_tiles_in_tile_group_minus1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).