| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0125 |
| --- | --- |

| Title: | CE2: History Based Affine Motion Candidate (Test 2.2.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, U.S.A Guichun Li, Xiaozhong Xu, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: Tel: Email: | 1-201-816-2000 jie.zhao@lge.com +1-650-798-3300 guichunli@tencent.com |
| Source: | LG Electronics. & Tencent |  |  |

_____________________________

# Abstract

This document reports results of CE2.2.3 – “History Based Affine Merge Candidate (Affine HMVP)”. In affine HMVP, a table of affine HMVP candidates is maintained and updated on-the-fly. After decoding an affine coded CU, the table is updated by adding the affine motion information as a new affine HMVP candidate to the last entry of the table. The affine HMVP candidates are applied in affine merge list generation and affine AMVP list generation. There are four subtests as follows:

- Applying affine HMVP candidates directly to affine merge list.

- Applying affine HMVP candidates directly to affine AMVP list.

- Combined test of 2.2.3a and 2.2.3b

- Replace existing inherited affine candidates with candidates inherited from affine HMVP candidates. In this way, it saves the storage of the affine CPMV (Control Point Motion Vector), which requires more than 6.9 KB for a CTU. Note that an affine HMVP requires 170 bytes or less.

Result summaries are as below.

- CE2.2.3.a: -0.06% on RA, -0.06% on LDB, -0.13% on LDP;

- CE2.2.3.b: -0.01% on RA, -0.00% on LDB, -0.02% on LDP;

- CE2.2.3.c: -0.07% on RA, -0.03% on LDB, -0.17% on LDP;

- CE2.2.3.d: 0.07% on RA, 0.09% on LDB, 0.00% on LDP.

# Introduction

In VTM3, a combined sub-block based merge list which contains both SbTVMP candidate and affine merge candidates is used for the signalling of sub-block based merge mode. The maximum allowed size of the sub-block based merge list is 5. For affine merge prediction, there can be up to five CPMV candidates and an index is signalled to indicate the one to be used for the current CU. The following three types of CPMV candidate are used to form the affine merge candidates in sub-block merge list:

1)	Inherited affine merge candidates that extrapolated from the CPMVs of the neighbour CUs

2)	Constructed affine merge candidates CPMVs that are derived using the translational MVs of the neighbour CUs

3)	Zero MVs

In VTM3, the affine AMVP candidate list size is 2 and it is generated by using the following four types of CPMV candidate in order:/

1)	Inherited affine AMVP candidates that extrapolated from the CPMVs of the neighbour CUs

2)	Constructed affine AMVP candidates CPMVPs that are derived using the translational MVs of the neighbour CUs

3)	Translational MVs from neighboring CUs

4)	Zero MVs.

Affine HMVP was previously proposed in JVET-L0305 [2]. A table to store motion information of coded affine CUs is updated after coding an affine coded CU, and being used in affine merge list and AMVP list generation. There are 4 subtests: for test a, b, c, affine HMVP candidates are added as additional candidates into the merge list or affine AMVP list. In test d, affine HMVP candidates are used to derive inherited affine candidates and replace existing inherited affine candidates.

# Proposed

## Affine HMVP Table

After coding an affine coded CU, motion information of the current CU is used to update the affine HMVP table. Similar to the regular HMVP table updating process, when adding a new motion candidate to the table, a constrained FIFO rule is utilized wherein redundancy check is firstly applied to find whether there is an identical affine HMVP in the table. If found, the identical affine HMVP is removed from the table and all the affine HMVP candidates afterwards are moved forward, i.e., with indices reduced by 1. In our test, the affine HMVP table size is set to 5, same as the sub-block merge list size, and the table is reset at the beginning of a CTU row.

For each entry of Affine history table, the following motion information is stored. Their memory requirements are also listed.

- Affine CPMVs mv0, mv1, mv2 for 2 reference lists			(36 bits * 2 * 3)

- Reference index of List 0 and/or List1					(4 bits * 2)

- Inter prediction direction (L0, L1 or Bi-pred)				(2 bits)

- Affine type, whether 4-parameter or 6-parameteer affine			(1 bit)

- Position (x, y)								(16 bits * 2)

- CU Width and height							(5 bits * 2)

- GBI index (optional)							(3 bits)

Total: 36*2*3 + 4*2 + 2 + 1 + 16*2 + 5*2 + 3 = 272 bits

So the memory requirements for affine HMVP table of difference sizes are:

- Table size 1: 	272 bits 	~ 34 bytes

- Table size 4: 	1088 bits 	~ 136 bytes

- Table size 5: 	1360 bits 	~ 170 bytes

## Subtests

### CE 2.2.3.a Apply Affine HMVP to merge list

In this subtest, the Affine HMVP candidates are directly added to the merge list as additional candidates. They are added after the constructed affine candidates and before the zero MV candidates. There is no pruning check when adding Affine HMVP candidates to the merge list. Adding directly to the merge list means that CPMVs of Affine HMVP candidates are to be applied to the current CU directly regardless of the history block’s shape and size.

Table 2.1 - Complexity Analysis for CE2.2.3.a

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of pruning stages | Max number of MV scaling | Others |
| --- | --- | --- | --- | --- | --- |
| No change | 5 extra candidates | No change | No change | No change |  |

### CE 2.2.3.b Apply Affine HMVP to affine AMVP list

In this subtest, the Affine HMVP candidate are added directly to the affine AMVP list as additional candidates. They are added before the TMVP candidate. Only HMVP candidates with the same reference picture as the AMVP target reference picture are used to construct the AMVP candidate list. There is no pruning check when adding the Affine HMVP candidates e to the list.

Table 2.2 - Complexity Analysis for CE2.2.3.b

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of pruning stages | Max number of MV scaling | Others |
| --- | --- | --- | --- | --- | --- |
| No change | 2 extra candidate | No change | No change | No change |  |

### CE 2.2.3c Combination of CE 2.2.3.a and CE 2.2.3.b

### This test is a combination of CE 2.2.3.a and CE 2.2.3.b, and affine HMVP is added into both sub-block merge list and affine AMVP list as additional candidates.

### CE 2.2.3.d Replace existing inherited affine candidates with inherited affine HMVP candidates

In VTM3, the inherited affine merge candidates and inherited affine AMVP candidates are derived from the left and above neighboring blocks coded in affine mode. In this test, those existing inherited affine candidates are replaced with inherited candidates derived from the affine HMVP. The affine merge and AMVP candidates inherited from affine HMVP take the same positions as the current inherited affine candidates. Maximum two inherited candidates from affine HMVP are allowed in both affine merge and affine AMVP.

Entries in Affine HMVP tables are checked starting from the latest entry, and only if an Affine HMVP candidate is neighboring to the current CU, it is used to derive the inherited candidate. Whether the HMVP is neighbor of the current CU can be determined because position and size information are stored in the affine HMVP table. For the HMVP that are identified as neighbors of the current CU, its width, height and CPMVs are then used to derive the CPMVs of the current CU in the same way as current affine inheritance method in VTM3.

The benefit of this approach is memory reduction since after a CU is coded/parsed, there is no need to store its CPMVs. In VTM3, affine CPMVs are stored in a separate buffer (adopted from JVET-L0047 [3]). As analyzed in JVET-L0047:

“For 128x128 CTU size, the buffers for storing all CPMVs inside the CTU are about 6,144 bytes (just counting motion vectors, not other parameters such as PU sizes and reference indices for the sake of explanation).  Because the CPMVs are only used for the affine motion data in heritance and the non-adjacent CPMVs can be disposed, in actual implementations the CPMVs do not need to be stored for the entire CTU. In order to minimize the storage requirements, a decoder can store only a top-row and left-column of vector context, rather than a full-CTU.  This concept is shown in Figure 1.  The figure illustrates the vector context state after decoding CU #10.  The CPMV context is shown in green (8 pixel granularity). As each CU is decoded, the corresponding left and top context are updated, shifting the blue and green lines further down and to the right, working the way across the CTU and eventually across the picture. In this type of memory-optimized implementation, the CPMVs would be stored for each 8-pixel segment.  There would be 16 such segments for the top-row, 16 for the left-column, and 16 segments for storing top-left context. This CPMV storage would require roughly 24 bytes per segment (3 CPMVs – 8 bytes each), so 1,152 bytes (24 bytes * 48 segments) for the top/left CPMV context within the CTU. ”

Note that the above analysis was assuming 16 bits for each MV. Due to the storage of high precision MV in current VVC, the memory requirement for each MV is 18 bits instead of 16 bits, which makes each CPMV require 9 bytes. So that full buffering of CPMV for the whole CTU requires 6912 Bytes, and the memory optimized buffering requires 1296 Bytes.

For the memory-optimized implementation, the encoder/decoder would need to figure out which CPMVs can be discarded and update the vector context after coding a CU. This is not so straightforward. It would need a complicated logic to update the vector context, and retrieve the correct the MV from the vector. In contrary, if using affine HMVP, it would be very easy to update the affine HMVP tables to be used for affine motion inheritance later. For affine HMVP table of size 5, including MVs and all the other necessary information, the storage is only about 170 bytes, which is a significant reduction comparing to the 6.9K+ bytes for storing CPMVs for the whole CTU or nearly 1.3K bytes for the complicated affine vector context.

Table 2.3-Memory requirement comparison

|  | Number of Segments | Bytes | Counted Information |
| --- | --- | --- | --- |
| Full Buffer | 256 | 6912 | For CPMV only |
| Optimized Buffer | 48 | 1296 | For CPMV only |
| Affine HMVP buffer | 5 | 170 | CPMV + related info. |

Fig. 1 Top-row / left-column motion vector context storage

Inheritances from Affine HMVP candidates replace the existing inherited candidates, so there is no change to the number of potential number of candidates.

Table 2.4 - Complexity Analysis for CE2.2.3.d

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of pruning stages | Max number of MV scaling | Others |
| --- | --- | --- | --- | --- | --- |
| No change | No change | No change | No change | No change | Reduce CPMV buffer to ~170bytes |

# Experimental Results

Proposed methods in this CE are implemented into VTM3, and tested following CE2 description [4] and JVET common test condition [5]. In the code, affine HMVP table size is defined as 5.

Table 3.1 Result of CE2.2.3.a – Apply affine HMVP to merge

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.21% | -0.10% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.08% | 0.01% | 0.04% | 100% | 99% |  |  |  |  |  |
| Class B | -0.07% | 0.02% | -0.02% | 101% | 100% | -0.04% | -0.17% | 0.09% | 102% | 101% |
| Class C | -0.03% | -0.02% | 0.00% | 100% | 99% | -0.06% | 0.33% | 0.08% | 101% | 101% |
| Class E |  |  |  |  |  | -0.10% | 0.00% | 0.09% | 101% | 99% |
| Overall | -0.06% | -0.04% | -0.02% | 100% | 100% | -0.06% | 0.04% | 0.09% | 101% | 100% |
| Class D | -0.05% | -0.12% | -0.01% | 101% | 101% | -0.03% | 0.12% | 0.33% | 101% | 99% |
| Class F | -0.06% | -0.05% | -0.12% | 101% | 100% | -0.03% | -0.22% | 1.13% | 103% | 101% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |  |  |  |  |  |
| Class B | -0.13% | 0.14% | -0.12% | 101% | 101% |  |  |  |  |  |
| Class C | -0.08% | -0.06% | -0.01% | 101% | 102% |  |  |  |  |  |
| Class E | -0.17% | -0.29% | -0.84% | 101% | 100% |  |  |  |  |  |
| Overall | -0.13% | -0.03% | -0.26% | 101% | 101% |  |  |  |  |  |
| Class D | -0.09% | 0.06% | -0.10% | 100% | 101% |  |  |  |  |  |
| Class F | 0.00% | -0.36% | -0.17% | 100% | 98% |  |  |  |  |  |

Table 3.2 Result of CE2.2.3.b – Apply affine HMVP to affine AMVP

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.10% | -0.04% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.03% | 0.08% | 0.12% | 100% | 100% |  |  |  |  |  |
| Class B | -0.02% | 0.03% | -0.03% | 100% | 100% | 0.00% | -0.20% | 0.20% | 102% | 100% |
| Class C | 0.00% | 0.00% | 0.04% | 99% | 100% | -0.02% | 0.18% | 0.04% | 102% | 102% |
| Class E |  |  |  |  |  | 0.05% | 0.07% | -0.03% | 101% | 100% |
| Overall | -0.01% | 0.01% | 0.02% | 100% | 100% | 0.00% | 0.00% | 0.09% | 102% | 101% |
| Class D | -0.02% | -0.05% | -0.04% | 100% | 100% | -0.02% | -0.08% | 0.19% | 100% | 101% |
| Class F | 0.00% | -0.02% | 0.01% | 100% | 100% | 0.04% | -0.09% | -0.10% | 101% | 101% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |  |  |  |  |  |
| Class B | -0.02% | 0.22% | -0.30% | 102% | 103% |  |  |  |  |  |
| Class C | -0.01% | 0.10% | 0.00% | 100% | 102% |  |  |  |  |  |
| Class E | -0.02% | -0.35% | -0.82% | 101% | 101% |  |  |  |  |  |
| Overall | -0.02% | 0.04% | -0.33% | 101% | 102% |  |  |  |  |  |
| Class D | -0.03% | -0.33% | -0.08% | 99% | 100% |  |  |  |  |  |
| Class F | 0.02% | -0.08% | 0.00% | 99% | 98% |  |  |  |  |  |

Table 3.3 - Result of CE2.2.3.c – Apply affine HMVP to both affine merge and AMVP

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.09% | -0.05% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.06% | -0.05% | -0.02% | 101% | 101% |  |  |  |  |  |
| Class B | -0.08% | -0.04% | -0.12% | 100% | 101% | -0.03% | -0.30% | -0.18% | 101% | 101% |
| Class C | -0.06% | 0.00% | 0.04% | 100% | 99% | -0.04% | 0.17% | 0.00% | 101% | 104% |
| Class E |  |  |  |  |  | -0.02% | 0.07% | -0.40% | 100% | 101% |
| Overall | -0.07% | -0.04% | -0.04% | 100% | 100% | -0.03% | -0.05% | -0.18% | 101% | 102% |
| Class D | -0.02% | -0.05% | 0.00% | 100% | 100% | -0.05% | -0.09% | -0.22% | 100% | 101% |
| Class F | -0.06% | -0.07% | -0.10% | 100% | 100% | -0.11% | 0.23% | 0.24% | 101% | 101% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |  |  |  |  |  |
| Class B | -0.16% | 0.23% | 0.13% | 101% | 103% |  |  |  |  |  |
| Class C | -0.14% | 0.16% | -0.14% | 100% | 101% |  |  |  |  |  |
| Class E | -0.23% | 0.03% | -0.64% | 100% | 101% |  |  |  |  |  |
| Overall | -0.17% | 0.16% | -0.15% | 101% | 102% |  |  |  |  |  |
| Class D | -0.09% | -0.47% | 0.26% | 101% | 102% |  |  |  |  |  |
| Class F | -0.03% | -0.35% | -0.03% | 99% | 99% |  |  |  |  |  |

Table 3.4 - Result of CE2.2.3.d – Replacing existing inherited affine with candidates inherited from affine HMVP

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.08% | 0.03% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.12% | 0.10% | 0.17% | 101% | 101% |  |  |  |  |  |
| Class B | 0.07% | 0.09% | 0.02% | 101% | 101% | 0.15% | -0.06% | -0.09% | 101% | 100% |
| Class C | 0.05% | 0.11% | 0.04% | 99% | 99% | 0.12% | 0.22% | 0.17% | 101% | 103% |
| Class E |  |  |  |  |  | -0.05% | 0.17% | 0.00% | 100% | 101% |
| Overall | 0.07% | 0.10% | 0.06% | 100% | 100% | 0.09% | 0.09% | 0.02% | 101% | 101% |
| Class D | 0.11% | 0.06% | 0.15% | 99% | 98% | 0.28% | -0.02% | 0.41% | 100% | 100% |
| Class F | 0.02% | 0.05% | -0.01% | 100% | 99% | 0.11% | 0.11% | 0.01% | 100% | 101% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |  |  |  |  |  |
| Class B | 0.03% | 0.20% | 0.41% | 102% | 102% |  |  |  |  |  |
| Class C | 0.06% | 0.25% | 0.05% | 101% | 102% |  |  |  |  |  |
| Class E | -0.11% | 0.12% | -0.52% | 100% | 100% |  |  |  |  |  |
| Overall | 0.00% | 0.20% | 0.06% | 102% | 102% |  |  |  |  |  |
| Class D | 0.07% | -0.11% | 0.38% | 100% | 100% |  |  |  |  |  |
| Class F | 0.14% | -0.61% | -0.02% | 99% | 98% |  |  |  |  |  |

.

# Conclusion

In this CE, several methods of applying affine HMVP candidates to sub-block merge and affine AMVP list are proposed. For subtest a) to c), adding affine HMVP directly into merge list and AMVP list as additional candidate, there is 0.07% small gain combined. It is not significant.

For subtest d), replacing existing inherited affine candidates with candidates inherited from the affine HMVP, it reduces the storage of CPMVs significantly to about only 170 bytes and makes it very straightforward to store CPMVs for future affine inheritance. This method has a small loss of 0.07% for RA, and 0.09% for LD. We propose to adopt the CE2.2.3.d into VVC draft.

# Reference

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3),” JVT-L1002, Macao, CN, Oct 2018.

- J. Zhao, S. Paluri, S. Kim, G. Li, X. Xu, X. Li, S. Liu, “CE4-related: History Based Affine Merge Candidate,” JVET-L0305, Macao, CN, Oct 2018.

- M. Zhou, B. Heng, “CE4-related: A clean up for affine mode,” JVET-L0047, Macao, CN, Oct 2018.

- Y. He, C.-Y. Chen, C.-C Chen, “Description of Core Experiment 2 (CE2): Sub-block based motion prediction,” JVT-L1024, Macao, CN, Oct 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-L1010 Macao, CN, Oct 2018.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).