| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0128-v1 |
| --- | --- |

| Title: | On reference picture management for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang, Hendry (Huawei) Sachin Deshpande (Sharp) Miska. M. Hannuksela (Nokia) Gahyun Ryu, Woongil Choi (Samsung) Xianglin Wang, Yi-Wen Chen (Kwai) Li Zhang (Bytedance) Ping Wu, Ming Li (ZTE) Seung-Hwan Kim (LG) Jill Boyce (Intel) Alexis M. Tourapis, David Singer (Apple) Edouard François, Pierre Andrivon (Technicolor) Yu-Wen Huang, Chih-Wei Hsu, Ching-Yeh Chen, Tzu-Der Chuang Lulin Chen (MediaTek) Kei Kawamura (KDDI) Yu-Chen Sun, Jian Lou (Alibaba) | Tel: Email: | +1-908-803-3888 yekui.wang@huawei.com |
| Source: | Huawei Technologies Co., Ltd., Sharp Labs of America, Inc., Nokia, Samsung Electronics Co., Ltd., Kwai Inc., Bytedance Inc., ZTE Corporation, LG Electronics Inc., Intel, Apple Inc., Technicolor, MediaTek Inc., KDDI Corporation, and Alibaba Group |  |  |

_____________________________

# Abstract

This contribution proposes a reference picture management approach for VVC based on direct signalling and derivation of reference picture lists 0 and 1, without use of reference picture set (RPS) as in HEVC or the sliding window plus memory management control operation (MMCO) process as in AVC.

It is asserted that the proposed approach is significantly simpler compared to the approaches in HEVC and AVC. The SPS, PSS, and slice header (SH) syntaxes and the text for semantics and derivation processes of the proposed approach are 45 lines (SPS: 7, PPS: 4, SH: 21, ref_pic_list_struct: 13) and about 4 pages, respectively, while the numbers for the RPS-based approach in HEVC are 82 lines (SPS: 11, PPS: 3, SH: 30, ref_pic_lists_modification: 12, st_ref_pic_set: 26) and about 9.5 pages, respectively. From reference software point of view, the codes are 1374 vs 2251 lines in the proponents' implementation of the proposed approach and the implementation of the RPS-based approach, respectively, both in the VTM-2.0.1 reference software.

The specification text and the software package are both enclosed in the submission. The specification text is based on JVET-K1001-v6, with change marks.

On signalling overhead, when only one SPS and only one PPS are used, the following results were reported for the different CTC configurations:

- For the RA configuration, the proposed approach needs 732 bits in the SPS, 8 bits in the PPS, and on average 6.9 bits in each slice header, while the numbers for RPS based approach are 258 bits (a difference of 474 bits, or 59.3 bytes) in the SPS, 7 bits in the PPS (a difference of 1 bit) and on average 7.8 bits (a difference of 0.9 bits) in each slice header, respectively.

- For the LD-B configuration, the proposed approach needs 284 bits in the SPS, 12 bits in the PPS, and on average 6.4 bits in each slice header, while the numbers for RPS based approach are 134 bits in the SPS (a difference of 150 bits, or 18.8 bytes), 11 bits in the PPS (a difference of 1 bit) and on average 6.4 bits (same) in each slice header, respectively.

- For the LD-P configuration, the proposed approach needs 284 bits in the SPS, 12 bits in the PPS, and on average 6.0 bits in each slice header, while the numbers for RPS based approach are 134 bits in the SPS (a difference of 150 bits, or 18.8 bytes), 11 bits in the PPS (a difference of 1 bit) and on average 6.0 bits (same) in each slice header, respectively.

# Introduction

Currently, the reference picture management mechanism, including reference picture lists construction and reference picture marking, which is needed, has not yet been decided for the draft VVC specification. The mechanism in the VTM reference software is the same as the RPS based approach in HEVC.

In the following subsections, we first discuss picture identification, the basic concept of reference picture management, and the approaches in AVC and HEVC, then "brainstorm" potential directions for VVC and layout the reasons why we chose the approach that is being proposed.

## Picture identification and reference picture management

In a video codec specification, pictures need to be identified for multiple purposes, including for use as a reference picture in inter prediction, for output of pictures from the decoded picture buffer (DPB), for scaling of motion vectors (MVs), for scaling of sample values in weighted prediction, etc.

In AVC and HEVC, pictures can be identified by picture order count (POC).

For use of reference pictures in inter prediction, scaling of MVs, and scaling of sample values in weighted prediction, the reference indices (each to a reference picture list) need to be signalled in a block level. To enable that, the reference picture lists, two in AVC and HEVC, reference picture list 0 and 1, need to be derived. To maintain the right reference pictures in the DPB without requiring unnecessarily large amount of DPB memory, reference picture marking is needed.

In AVC and HEVC, pictures in the DPB can be marked as "used for short-term reference", "used for long-term reference", or "unused for reference". Once a picture has been marked "unused for reference", it can no longer be used for prediction, and when it is also no longer needed for output, it can be removed from the DPB.

## Reference picture management in AVC

In AVC, there are two types of reference pictures, short-term reference pictures (STRPs) and long-term reference pictures (LTRPs). A reference picture may be marked as "unused for reference" when it becomes no longer needed for prediction reference. The conversion among these three statuses (short-term, long-term, and unused for reference) is controlled by the decoded reference picture marking process. There are two alternative decoded reference picture marking mechanisms, the implicit sliding window process and the explicit memory management control operation (MMCO) process. The sliding window process marks a short-term reference picture as "unused for reference" when the number of reference frames is equal to a given maximum number (max_num_ref_frames in the SPS). The short-term reference pictures are stored in a first-in, first-out manner so that the most recently decoded short-term pictures are kept in the DPB.

The explicit MMCO process may include multiple MMCO commands. An MMCO command may mark one or more short-term or long-term reference picture as "unused for reference", mark all the pictures as "unused for reference", or mark the current reference picture or an existing short-term reference picture as long-term, and assign a long-term picture index to that long-term reference picture.

In AVC the reference picture marking operations as well as the processes for output and removal of pictures from the DPB are performed after a picture has been decoded.

The entire process of AVC reference picture management based on the sliding window plus the MMCO process can be summarized by the following ordered steps:

- The POC value of the current picture is derived.

- The gaps in frame_num are derived when needed.

- FrameNum, FrameNumWrap, PicNum, LongTermFrameIdx, and LongTermPicNum are derived.

- Reference picture lists are initialized (for slices in frames, excluding the process for slices in fields).

  - RefPicList0 for P and SP slices

    - STRPs go before LTRPs

    - STRPs go in descending order of PicNum values

    - LTRPs go in ascending order of LongTermPicNum values

  - RefPicList0 for B slices

    - STRPs

      - Pictures with POC values less than the current picture go before pictures with POC values greater than the current picture

      - Pictures with POC values less than the current picture go in descending order of POC values

      - Pictures with POC values greater than the current picture go in ascending order of POC values

    - LTRPs in ascending order of LongTermPicNum values

  - RefPicList1 for B slices

    - SRTPs

      - Pictures with POC values less than the current picture go after pictures with POC values greater than the current picture

      - Pictures with POC values less than the current picture go in descending order of POC values

      - Pictures with POC values greater than the current picture go in ascending order of POC values

    - LTRPs in ascending order of LongTermPicNum values

    - When RefPicList1 has more than one entry and RefPicList1 is identical to RefPicList0, the first two entries RefPicList1[ 0 ] and RefPicList1[ 1 ] are switched.

- The order of some of the reference pictures in each reference picture list is modified when indicated, and each list is truncated to contain the indicated number of active entries.

- Reference pictures are marked after all slices of the current picture that is a reference picture are decoded.

  - If the current picture is an IDR picture, all reference pictures are marked as "unused for reference", and the current picture is marked as "used for short-term reference" or "used for long-term reference" depending on the value of long_term_reference_flag.

  - Otherwise, either the sliding window or the MMCO operation is applied, and the current picture is marked as "used for short-term reference" if not marked as "used for long-term reference" by MMCO command 6.

- All pictures in the DPB that are marked as "unused for reference" and no long needed for output are removed from the DPB.

## Reference picture management in HEVC

A different approach for reference picture management than the AVC approach was introduced to HEVC. The approach is based on a mechanism named reference picture set (RPS). The most fundamental difference with the RPS concept compared to the sliding window plus MMCO process of AVC is that, for each particular slice a complete set of the reference pictures that are used by the current picture or any subsequent picture is provided. Thus, a complete set of all pictures that must be kept in the DPB for use by the current or future picture is signalled. This is different from the AVC scheme where only relative changes to the DPB are signalled. With the RPS concept, basically no information from earlier pictures in decoding order is needed to maintain the correct status of reference pictures in the DPB.

The entire process of HEVC RPS-based reference picture management can be summarized as follows (excluding the part for current picture referencing, aka intra block copy):

- The POC value of the current picture is derived.

- The RPS for the current picture is derived. This contains five subsets: PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll, and during the process reference pictures may be marked as "unused for reference" or "used for long-term reference".

  - PocLtCurr and PocLtFoll are derived.

  - All reference pictures that are included in RefPicSetLtCurr or RefPicSetLtFoll are marked as "used for long-term reference".

  - PocStCurrBefore, PocStCurrAfter, and PocStFoll derived.

  - All reference pictures in the DPB that are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetStFoll are marked as "unused for reference".

- All pictures in the DPB that are marked as "unused for reference" and no long needed for output are removed from the DPB.

- When the current picture is a BLA or CRA picture, unavailable reference pictures are generated when needed.

- Reference picture lists are initialized.

  - RefPicListTemp0

    - In the order of RefPicSetStCurrBefore, RefPicSetStCurrAfter, RefPicSetLtCurr

    - Repeatedly add in the above order if NumPicTotalCurr is less than the indicated number of active entries

  - RefPicListTemp1

    - In the order of RefPicSetStCurrAfter, RefPicSetStCurrBefore, RefPicSetLtCurr

    - Repeatedly add in the above order if NumPicTotalCurr is less than the indicated number of active entries

- Final reference picture lists are constructed, either by truncating the initial lists to contain the respective numbers of active entries, or based on direct signalling of the active entries through indices to the initial lists.

- After all slices of the current picture have been decoded, the current decoded picture is marked as "used for short-term reference".

- For output order conformance, all pictures in the DPB that are marked as "unused for reference" and no long needed for output are removed from the DPB, by the additional bumping process.

## What reference picture management approach should be used for VVC?

Many options are possible. For example, VVC could use the AVC reference picture management approach based on sliding window plus the MMCO operation. VVC could also use the HEVC reference picture management approach based on RPS. Another option is to define a new approach that is different from both the AVC and the HEVC approaches.

Firstly we thought about using the HEVC approach. After some thinking, we gave up mainly because of the following factors:

- We had heard complaints about the number of text pages, including the large number of lines of pseudo code, for the semantics and decoding process for RPS signalling and derivation.

- We had also heard complaints saying that the inter-RPS prediction feature was quite difficult to understand and yet it was mainly just for saving some amount of bits in the SPS .

Then we thought about using the AVC approach with potential simplifications. After some thinking, we gave up mainly because of the following factors:

- Although nowadays people no longer display error-concealed pictures to avoid uncontrollable error concealment equality, loss of the decoded picture buffer status seems nasty, which, e.g., affects picture loss detection that remains useful. As long as some inter-coding of the buffer status is used, this issue remains there.

- The frame_num semantics is complicated and handling of gaps in frame_num is needed. It does not seem easy to get rid of these issues.

Just for information, the frame_num semantics in the AVC specification is copied and pasted below:

frame_num is used as an identifier for pictures and shall be represented by log2_max_frame_num_minus4 + 4 bits in the bitstream. frame_num is constrained as follows:

The variable PrevRefFrameNum is derived as follows:

–	If the current picture is an IDR picture, PrevRefFrameNum is set equal to 0.

–	Otherwise (the current picture is not an IDR picture), PrevRefFrameNum is set as follows:

–	If the decoding process for gaps in frame_num specified in clause ‎8.2.5.2 was invoked by the decoding process for an access unit that contained a non-reference picture that followed the previous access unit in decoding order that contained a reference picture, PrevRefFrameNum is set equal to the value of frame_num for the last of the "non-existing" reference frames inferred by the decoding process for gaps in frame_num specified in clause ‎8.2.5.2.

–	Otherwise, PrevRefFrameNum is set equal to the value of frame_num for the previous access unit in decoding order that contained a reference picture.

The value of frame_num is constrained as follows:

–	If the current picture is an IDR picture, frame_num shall be equal to 0.

–	Otherwise (the current picture is not an IDR picture), referring to the primary coded picture in the previous access unit in decoding order that contains a reference picture as the preceding reference picture, the value of frame_num for the current picture shall not be equal to PrevRefFrameNum unless all of the following three conditions are true:

- The current picture and the preceding reference picture belong to consecutive access units in decoding order.

- The current picture and the preceding reference picture are reference fields having opposite parity.

- One or more of the following conditions is true:

–	The preceding reference picture is an IDR picture,

–	The preceding reference picture includes a memory_management_control_operation syntax element equal to 5,

NOTE 3 – When the preceding reference picture includes a memory_management_control_operation syntax element equal to 5, PrevRefFrameNum is equal to 0.

–	There is a primary coded picture that precedes the preceding reference picture and the primary coded picture that precedes the preceding reference picture does not have frame_num equal to PrevRefFrameNum,

–	There is a primary coded picture that precedes the preceding reference picture and the primary coded picture that precedes the preceding reference picture is not a reference picture.

When the value of frame_num is not equal to PrevRefFrameNum, it is a requirement of bitstream conformance that the following constraints shall be obeyed:

- There shall not be any previous field or frame in decoding order that is currently marked as "used for short-term reference" that has a value of frame_num equal to any value taken on by the variable UnusedShortTermFrameNum in the following:

UnusedShortTermFrameNum = ( PrevRefFrameNum + 1 ) % MaxFrameNum
while( UnusedShortTermFrameNum  !=  frame_num ) 	(‎7-24)
	UnusedShortTermFrameNum = ( UnusedShortTermFrameNum + 1 ) % MaxFrameNum

- The value of frame_num is constrained as follows:

–	If gaps_in_frame_num_value_allowed_flag is equal to 0, the value of frame_num for the current picture shall be equal to ( PrevRefFrameNum + 1 ) % MaxFrameNum.

–	Otherwise (gaps_in_frame_num_value_allowed_flag is equal to 1), the following applies:

–	If frame_num is greater than PrevRefFrameNum, there shall not be any non-reference pictures in the bitstream that follow the previous reference picture and precede the current picture in decoding order in which either of the following conditions is true:

–	The value of frame_num for the non-reference picture is less than PrevRefFrameNum,

–	The value of frame_num for the non-reference picture is greater than the value of frame_num for the current picture.

–	Otherwise (frame_num is less than PrevRefFrameNum), there shall not be any non-reference pictures in the bitstream that follow the previous reference picture and precede the current picture in decoding order in which both of the following conditions are true:

–	The value of frame_num for the non-reference picture is less than PrevRefFrameNum,

–	The value of frame_num for the non-reference picture is greater than the value of frame_num for the current picture.

A picture including a memory_management_control_operation equal to 5 shall have frame_num constraints as described above and, after the decoding of the current picture and the processing of the memory management control operations, the picture shall be inferred to have had frame_num equal to 0 for all subsequent use in the decoding process, except as specified in clause ‎7.4.1.2.4.

NOTE 4 – When the primary coded picture is not an IDR picture and does not contain memory_management_control_operation syntax element equal to 5, the value of frame_num of a corresponding redundant coded picture is the same as the value of frame_num in the primary coded picture. Alternatively, the redundant coded picture includes a memory_management_control_operation syntax element equal to 5 and the corresponding primary coded picture is an IDR picture.

- We had heard complaints about the AVC reference picture list reordering/modification process being not easy to understand, and it is our understanding that that's why the straightforward reference picture list modification process in HEVC, by directly signalling of the active entries through indices to the initial lists, was agreed during the standardization of HEVC. The same list modification mechanism as in HEVC could also be applied to the AVC approach, though. The determining factors for use to give up thinking more about starting from the AVC approach are the first two items above.

When continuing thinking and studying on the topic, we realized that Nokia had a proposal in JCTVC-G643, proposing a reference picture management approach that directly signals, derives, and uses three reference picture lists, namely reference picture list 0, reference picture list 1, and an idle reference picture list, for management of reference pictures. It seemed to us that an approach along that direction could be significantly simpler than both the AVC and HEVC approaches. Particularly, we observed that a third reference picture list is actually not needed, as inactive reference pictures that would only be used for inter prediction reference by future pictures in decoding order can be included entries in either list 0 or 1 with indices greater than the greatest index value of the active entries.

# Proposal

The proposed direct-RPL-based reference picture management approach is summarized as follows:

- Two reference picture lists, list 0 and list 1, are directly signalled and derived. They are not based on RPS as in HEVC or the sliding window plus MMCO process as in AVC.

- Reference picture marking is directly based on reference picture lists 0 and 1, utilizing both active and inactive entries in the reference picture lists, while only active entries may be used as reference indices in inter prediction of CTUs.

- Information for derivation of the two reference picture lists is signalled by syntax elements and syntax structures in the SPS, the PPS, and the slice header. Predefined RPL structures are signalled in the SPS, for use by referencing in the slice header.

- The two reference picture lists are generated for all types of slices, i.e., B, P, and I slices.

- The two reference picture lists are constructed without using a reference picture list initialization process or a reference picture list modification process.

- Long-term reference pictures (LTRPs) are identified by POC LSBs. When needed, additional POC LSBs are signalled for LTRPs, determined at a picture by picture basis.

Detailed specification text for the proposed direct-RPL-based reference picture management approach, with change marks relative to JVET-K1001-v6, is provided as an attachment of this contribution.

# Implementation and test results

## RPL-based reference picture management implementation

The proposed direct-RPL-based reference picture management approach has been implemented on top of VTM-2.0.1. The implementation replaces the RPS based signalling, reference picture marking and reference picture list construction. All changed codes are guarded by macro EXPLICIT_RPL_ENABLE. The software implementation uses 1374 lines of code for the direct-RPL-based approach compared to the RPS-based implementation that uses 2251 lines of code in the VTM-2.0.1.

The implementation produces identical decoded YUV pictures as the ones produced by the RPS-based implementation.

## Simulation results for bit counts of the signalling

The bit counts comparisons of the RPL-based and RPS-based reference picture management approaches are shown in Table 1, Table 2, and Table 3 for the random access, low delay B, and low delay P configurations, respectively. To capture the regularity of bit counts in slice headers, for the random access configuration, we encoded 65 pictures, which covers 4 group of pictures (GOPs) plus 1 IDR picture; likewise, for the two low delay configurations, we encoded 21 pictures which covers 5 GOPs plus 1 IDR picture.

The reported bit counts for the RPS-based approach include bit numbers for the following syntax elements:

- For SPS: num_short_term_ref_pic_sets, all syntax elements in st_ref_pic_set( i ), long_term_ref_pics_present_flag, num_long_term_ref_pics_sps, lt_ref_pic_poc_lsb_sps[ i ], and used_by_curr_pic_lt_sps_flag[ i ]

- For PPS: num_ref_idx_l0_default_active_minus1, num_ref_idx_l1_default_active_minus1, and lists_modification_present_flag

- For slice header: short_term_ref_pic_set_sps_flag, all syntax elemens in st_ref_pic_set( num_short_term_ref_pic_sets ), short_term_ref_pic_set_idx, num_long_term_sps, num_long_term_pics, lt_idx_sps[ i ], poc_lsb_lt[ i ], used_by_curr_pic_lt_flag[ i ], delta_poc_msb_present_flag[ i ], delta_poc_msb_cycle_lt[ i ], all syntax elements in ref_pic_lists_modification( ), num_ref_idx_active_override_flag, num_ref_idx_l0_active_minus1, and num_ref_idx_l1_active_minus1

The reported bit counts for the RPL-based approach include bit numbers for the following syntax elements:

- For SPS: long_term_ref_pics_flag, rpl1_same_as_rpl0_flag, num_ref_pic_lists_in_sps[ i ], and all syntax elements in ref_pic_list_struct( i, j )

- For PPS: num_ref_idx_default_active_minus1[ i ], additional_lt_poc_lsb_len and rpl1_idx_present_flag

- For slice header: ref_pic_list_sps_flag[ i ], ref_pic_list_idx[ i ], all syntax elemens in ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ), additional_poc_lsb_present_flag[ i ][ j ], additional_poc_lsb_val[ i ][ j ], num_ref_idx_active_override_flag, and num_ref_idx_active_minus1[ i ]

Table 1 Bit counts for the RA configuration

| Random Access Configuration (current CTC) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Description | RPS-based approach |  | RPL-based approach |  |
| Bits in SPS | 258 |  | 732 |  |
| Bits in PPS | 7 |  | 8 |  |
| Bits in slice headers | 506 |  | 449 |  |
| Picture 0 | 0 | (Avg bits / slice) 7.8 | 0 | (Avg bits / slice) 6.9 |
| GOP #1 | 114 |  | 114 |  |
| GOP #2 | 139 |  | 112 |  |
| GOP #3 | 114 |  | 112 |  |
| GOP #4 | 139 |  | 111 |  |

Table 2 Bit counts for the low-delay B configuration

| Low-Delay B Configuration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Description | RPS-based approach |  | RPL-based approach |  |
| Bits in SPS | 134 |  | 284 |  |
| Bits in PPS | 11 |  | 12 |  |
| Bits in slice headers | 134 |  | 134 |  |
| Picture 0 | 0 | (Avg bits / slice) 6.4 | 0 | (Avg bits / slice) 6.4 |
| GOP #1 | 38 |  | 38 |  |
| GOP #2 | 24 |  | 24 |  |
| GOP #3 | 24 |  | 24 |  |
| GOP #4 | 24 |  | 24 |  |
| GOP #5 | 24 |  | 24 |  |

Table 3 Bit counts for the low-delay P configuration

| Low-Delay P Configuration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Description | RPS-based approach |  | RPL-based approach |  |
| Bits in SPS | 134 |  | 252 |  |
| Bits in PPS | 11 |  | 12 |  |
| Bits in slice headers | 127 |  | 127 |  |
| Picture 0 | 0 | (Avg bits / slice) 6.0 | 0 | (Avg bits / slice) 6.0 |
| GOP #1 | 31 |  | 31 |  |
| GOP #2 | 24 |  | 24 |  |
| GOP #3 | 24 |  | 24 |  |
| GOP #4 | 24 |  | 24 |  |
| GOP #5 | 24 |  | 24 |  |

The bit counts for the RPL-based approach in SPS are higher than the RPS-based approach partly because there are two sets of RPL structures signalled in the SPS and the RPL structures are coded without using an inter-RPL structure prediction.

# Discussion on getting rid of the long-term reference picture concept

It is suggested to discuss whether to get rid of the long-term reference picture concept to further simplify the specification text.

Another text version that has this idea implemented is enclosed, summarized as follows:

- There would be no distinction between LTRPs and STRPs. All reference pictures are just called reference pictures.

- A flag is signalled for each entry of a reference picture list to specify whether that reference index is used for POC based scaling of motion vectors and sample values.

- The LTRP functionality is still kept, while a new functionality is introduced, i.e., any reference picture can be determined to be used or not used for POC based scaling of motion vectors and sample values.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd., Sharp Labs of America, Inc., Nokia, Samsung Electronics Co., Ltd., Kwai Inc., Bytedance Inc., ZTE Corporation, LG Electronics Inc., Intel, Apple Inc., Technicolor, MediaTek Inc., KDDI Corporation, and Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).