| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0134-v2 |
| --- | --- |

| Title: | AHG12: On explicit signalling of tile IDs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Ye-Kui Wang 10180 Telesis Court San Diego, CA, 92121, USA Jianle Chen 2330 Central Expy Santa Clara, CA 95050, USA Maxim Sychev Building 1/7, Altufievskoe Shosse, Moscow 127106, Russia | Tel: Email: | +1-858-375-8065 fnu.hendry@huawei.com +1-908-803-3888 yekui.wang@huawei.com +1-858-999-4243 jianle.chen@huawei.com +7-925-008-1124 sychev.maxim@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes an approach to allow signalling tile IDs explicitly such that tile IDs can be different from tile indices. It is asserted that the proposed approach enables extraction of motion-constrained tile sets without having to change the VCL NAL units.

# Problem statement

In the agreed tile group design from the 12th JVET meeting in Macau [1], when there is more than one tile in a picture, the address of the tile group is signalled in the tile group header by using the syntax element tile_group_address. For convenience, the semantics of tile_group_address is copied and pasted below:

tile_group_address specifies the tile address of the first tile in the tile group. The length of tile_group_address is Ceil( Log2 ( NumTilesInPic ) ) bits. The value of tile_group_address shall be in the range of 0 to NumTilesInPic − 1, inclusive, and the value of tile_group_address shall not be equal to the value of tile_group_address of any other coded tile group NAL unit of the same coded picture. When tile_group_address is not present it is inferred to be equal to 0.

The "tile address" in the above semantics is tile index. Using tile index as the address for each tile group would mean repeating the issue related to signalling of slice address in HEVC. MPEG contribution M43397 [2] described the problem with HEVC tiles (i.e., MCTS) during the MCTS extraction to form a new bitstream. The following is copied from the [2]:

"Recently, a few new use cases have been presented that require modification of an AVC or HEVC slice segment header between encoding and decoding, either on the client-side directly before passing the bitstream to the decoder, or in some network-based media processing entity.

One example of such a use case is tiled streaming, where a (panoramic) video is encoded using HEVC tiles, but the decoder only decodes a portion of these tiles. By rewriting the HEVC slice segment header (SSH) as well as the SPS/PPS, the bitstream can be manipulated in order to change the subset of tiles being decoded as well as their spatial arrangement in the decoded video frame.

The primary reason for this CPU processing cost is the fact that AVC and HEVC slice segment headers use variable length fields and have a byte alignment field at the end. This means that whenever a certain field in the SSH is changed, this may have an impact on the byte alignment field at the end of the SSH, which will then have to be rewritten as well. And because all fields are variable length encoded, the only way to know the location of the byte alignment field is to parse all preceding fields. This results in significant processing overhead, especially when tiles are used and a second worth of video might contain hundreds of NALs."

It is the authors' belief that such problem should be avoided for the tile group design in VVC. We therefore propose an approach to enable extraction of MCTSs without having to change the tile group headers, and consequently VCL NAL units do not need to be changed at all.

# Proposal: Signalling tile IDs explicitly

## Overview

The proposed solution is summarized as follows:

- Differentiate the concept of tile index and tile ID. The tile ID of a tile may or may not be equal to the tile index of the tile.

- When tile IDs are different from tile indices, a mapping between tile indices and tile IDs is signalled in the PPS.

- For signalling of tile group address in tile group headers, instead of using tile index, tile ID is used. In this way, when the tile group is extracted from its original bitstream, although the values of tile indices may change, the values of tile IDs can be kept the same by updating the mapping between tile indices and tile IDs in the PPS referred to by the tile group. Note that in MCTS sub-bitstream extraction, parameter sets anyway need to be updated (by changing or replacing).

Figure 1 illustrates how signalling tile IDs explicitly would solve the problem of having to rewrite tile group headers when a tile group belonging to an MCTS is extracted. When the second tile group of the shown picture is extracted from its original bitstream, if tile_group_address is tile index, then because in the extracted bitstream tile index values are different than in the original bitstream, the tile_group_address has to be changed. However, if tile_group_address is tile ID instead of tile index, then nothing needs to be changed in the tile group header.

Figure 1 – Example of tile groups without explicitly signalled tile IDs in the original bitstream and with explicitly signalled tile IDs in the extracted bitstream.

## Proposed text changes

To accommodate the proposed tile ID signalling, the necessary changes to the VVC draft specification is as follows (added text are highlighted in yellow whereas removed text are strikethrough and highlighed in red:

Clause 6.5.1	CTB raster and tile scanning process

...

The list TileId[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile ID, and the list NumCtusInTile[ tileIdx ] for tileIdx ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a tile index to the number of CTUs in the tile, are derived as follows:

for( j = 0, tileIdx = 0; j  <=  num_tile_rows_minus1; j++ )
	for( i = 0; i  <=  num_tile_columns_minus1; i++, tileIdx++ ) {
		for( y = RowBd[ j ]; y < RowBd[ j + 1 ]; y++ )	(6-7)
			for( x = ColBd[ i ]; x < ColBd[ i + 1 ]; x++ )
				TileId[ CtbAddrRsToTs[ y * PicWidthInCtbsY+ x ] ] =
					explicit_tile_id_flag ? tile_id_val[ tileIdx ] : tileIdx
		FirstCtbAddrTs[ tileIdx ] = CtbAddrRsToTs[ RowBd[ j ] * PicWidthInCtbsY + ColBd[ i ] ]
	}

The list NumCtusInTile[ tileIdx ] for tileIdx ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a tile index to the number of CTUs in the tile, is derived as follows:

for( j = 0, tileIdx = 0; j  <=  num_tile_rows_minus1; j++ )
	for( i = 0; i  <=  num_tile_columns_minus1; i++, tileIdx++ ) 	(6-8)
		NumCtusInTile[ tileIdx ] = ColWidth[ i ] * RowHeight[ j ]

The set TileIdToIdx[ tileId ] for a set of NumTilesInPic tileId values specifying the conversion from a tile ID to a tile index is derived as follows:

for( ctbAddrTs = 0, tileIdx = 0, tileStartFlag = 1; ctbAddrTs < PicSizeInCtbsY; ctbAddrTs++ ) {
	if( tileStartFlag ) {
		TileIdToIdx[ TileId[ ctbAddrTs ] ] = tileIdx	(6-9)
		tileStartFlag = 0
	}
	tileEndFlag = ctbAddrTs  = =  PicSizeInCtbsY − 1  | |  TileId[ ctbAddrTs + 1 ]  !=  TileId[ ctbAddrTs ]
	if( tileEndFlag ) {
		tileIdx++
		tileStartFlag = 1
	}
}

...

Clause 7.3.2.2	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| explicit_tile_id_flag | u(1) |
| if( explicit_tile_id_flag ) { |  |
| tile_id_len_minus1 | ue(v) |
| for( i = 0; i < NumTilesInPic; i++ ) |  |
| tile_id_val[ i ] | u(v) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Clause 7.3.3	Tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| tile_group_address | u(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| ... |  |

Clause 7.3.3	Tile group data syntax

| tile_group_data( ) { | Descriptor |
| --- | --- |
| tileIdx = TileIdToIdx[ tile_group_address ] |  |
| for( i = 0; i <= num_tiles_in_tile_group_minus1; i++, tileIdx++ ) { |  |
| ctbAddrInTs = FirstCtbAddrTs[ tileIdx ] |  |
| for( j = 0; j < NumCtusInTile[ tileIdx ]; j++, ctbAddrInTs++ ) { |  |
| CtbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| } |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < num_tiles_in_tile_group_minus1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |

Clause 7.4.3.2	Picture parameter set RBSP semantics

...

explicit_tile_id_flag equal to 1 specifies that tile ID for each tile is explicitliy signalled. explicit_tile_id_flag equal to 0 specifies that tile IDs are not explicitliy signalled.

NOTE – For a bitstream that is a result of an MCTS sub-bitstream extraction, the value of explicit_tile_id_flag has to be set equal to 1 for active PPSs unless the bitstream contains the upper-left corner tile in the original bitstream.

tile_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element tile_id_val[ i ] and the syntax element tile_group_address in tile group headers referring to the PPS. The value of tile_id_len_minus1 shall be in the range of Ceil( Log2( NumTilesInPic ) to 15, inclusive. When not present, the value of tile_id_len_minus1 is inferred to be equal to Ceil( Log2( NumTilesInPic ).

[Ed.: Note that the value of tile_id_len_minus1 may be greater than Ceil( Log2( NumTilesInPic ), because the current bitstream may be the result of an MCTS sub-bitstream extraction, and in that case, the tile IDs, which can be the tile indices in the original bitstream, would need to be represented by Ceil( Log2( OrgNumTilesInPic ) bits, where OrgNumTilesInPic is the NumTilesInPic of the original bitstream, which is greater than NumTilesInPic of the current bitstream.]

tile_id_val[ i ] specifies the tile ID of the i-th tile of pictures referring to the PPS. The length of tile_id_val[ i ] is tile_id_len_minus1 + 1 bits.

For any integers m and n in the range of 0 to NumTilesInPic − 1, inclusive, tile_id_val[ m ] shall not be equal to tile_id_val[ n ] when m is not equal to n, and tile_id_val[ m ] shall be less than tile_id_val[ n ] when m is less than n.

The following variables are derived by invoking the CTB raster and tile scanning conversion process as specified in clause 6.5.1:

–	The list ColWidth[ i ] for i ranging from 0 to num_tile_columns_minus1, inclusive, specifying the width of the i-th tile column in units of CTBs,

–	the list RowHeight[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of CTBs,

–	the list ColBd[ i ] for i ranging from 0 to num_tile_columns_minus1 + 1, inclusive, specifying the location of the i-th tile column boundary in units of CTBs,

–	the list RowBd[ j ] for j ranging from 0 to num_tile_rows_minus1 + 1, inclusive, specifying the location of the j-th tile row boundary in units of CTBs,

–	the list CtbAddrRsToTs[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in the CTB raster scan of a picture to a CTB address in the tile scan,

–	the list CtbAddrTsToRs[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in the tile scan to a CTB address in the CTB raster scan of a picture,

–	the list TileId[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile ID,

–	the list NumCtusInTile[ tileIdx ] for tileIdx ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a tile index to the number of CTUs in the tile,

–	the list FirstCtbAddrTs[ tileIdx ] for tileIdx ranging from 0 to NumTilesInPic − 1, inclusive, specifying the conversion from a tile ID to the CTB address in tile scan of the first CTB in the tile,

–	the set TileIdToIdx[ tileId ] for a set of NumTilesInPic tileId values specifying the conversion from a tile ID to a tile index and the list FirstCtbAddrTs[ tileIdx ] for tileIdx ranging from 0 to NumTilesInPic − 1, inclusive, specifying the conversion from a tile ID to the CTB address in tile scan of the first CTB in the tile,

–	the lists ColumnWidthInLumaSamples[ i ] for i ranging from 0 to num_tile_columns_minus1, inclusive, specifying the width of the i-th tile column in units of luma samples,

–	the list RowHeightInLumaSamples[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of luma samples.

Clause 7.4.4	Tile group header semantics

...

tile_group_address specifies the tile ID address of the first tile in the tile group. The length of tile_group_address is Ceil( Log2 ( NumTilesInPic ) ) tile_id_len_minus1 + 1 bits. The value of tile_group_address shall be in the range of 0 to NumTilesInPic – 12tile_id_len_minus1 + 1 – 1 inclusive, and the value of tile_group_address shall not be equal to the value of tile_group_address of any other coded tile group NAL unit of the same coded picture. When tile_group_address is not present it is inferred to be equal to 0.

...

# Reference

- JVET-L0686, "Spec text for the agreed starting point on slicing and tiling", the 12th JVET meeting, October 2018, Macau.

- M43397, "Adding slice segment header length information to isobmff", the 123rd MPEG meeting, July 2018, Ljubljana.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).