| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0142-v1 |
| --- | --- |

| Title: | CE3: Modified CCLM downsampling filter for “type-2” content (Test 2.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE3 test 2.4. Different downsampling filters in CCLM are investigated for the linear model determination and prediction to align the location of the downsampled reconstructed luma samples with that of the chroma samples. For HDR sequences represented using chroma sample location type 2, results based on the wPSNR metric show the following {Y, U, V} BD-rate savings over the default CCLM downsampling filter:

- Determination: 3-tap; prediction: 3-tap; AI: {-0.2%, -4.2%, -8.6%}; RA: {-0.1%, -4.2%, -5.8%}

- Determination: 3-tap; prediction: 5-tap; AI: {-0.2%, -4.7%, -8.6%}; RA: {-0.1%, -4.7%, -6.0%}

- Determination: 5-tap; prediction: 5-tap; AI: {-0.2%, -5.5%, -9.6%}; RA: {-0.2%, -5.1%, -6.6%}

# Introduction

To reduce the cross-component redundancy, the cross-component linear model (CCLM) prediction mode has been integrated in the VTM for intra prediction. In the CCLM mode, the chroma samples are predicted based on the reconstructed luma samples of the same CU. For 4:2:0 chroma sampling, the reconstructed luma samples need to be downsampled to match the sampling of the chroma signal. In VTM, the CCLM prediction assumes chroma sample location type 0 (aka “type-0”), since it is the most commonly used in legacy content in ITU-R BT.601 or ITU-R BT.709 container. However, if a different chroma sample location type is used, this could result in a misalignment between the chroma samples and the downsampled luma samples, which may reduce the coding performance. This contribution reports the coding performance of different downsampling filters in CCLM for the linear model determination and prediction to align the location of the downsampled reconstructed luma samples with that of the chroma samples for “type-2” content, as studied in CE3 test 2.4 [1].

# Technical description

To avoid misalignment between the chroma samples and the downsampled luma samples in CCLM for “type-2” content, the following downsampling filters are investigated for the linear model determination and the prediction:

| 3-tap: | recL'i,j= recL2i-1,2j+2∙recL2i,2j+recL2i+1,2j+2≫2 |
| --- | --- |
| 5-tap: | recL'i,j= recL2i,2j-1+recL2i-1,2j+4∙recL2i,2j+recL2i+1,2j+recL2i,2j+1+4≫3 |
| 9-tap: | recL'i,j= recL2i-1,2j-1+2∙recL2i,2j-1+recL2i+1,2j-1+2∙recL2i-1,2j+4∙recL2i,2j+2∙recL2i+1,2j+recL2i-1,2j+1+2∙recL2i,2j+1+recL2i+1,2j+1+8≫4 |

To avoid increasing the number of line buffer, these modifications are not applied at the top CTU boundary.

# Syntax

An exemplary syntax to enable support for different chroma sample location types is shown in Table 1.

Table 1. Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| chroma_format_idc | ue(v) |
| … |  |
| lm_chroma_enabled_flag | u(1) |
| if( lm_chroma_enabled_flag && chroma_format_idc = = 1 ) { |  |
| lm_chroma_horizontal_offset_flag | u(1) |
| lm_chroma_vertical_offset_flag | u(1) |
| } |  |
| … |  |
| } |  |

lm_chroma_horizontal_offset_flag equal to 1 specifies that the top-left downsampled luma sample in CCLM is horizontally shifted by 0.5 units of luma samples relatively to the top-left luma sample. lm_chroma_horizontal_offset_flag equal to 0 specifies that the top-left downsampled luma sample in CCLM is horizontally co-sited with the top-left luma sample.

lm_chroma_vertical_offset_flag equal to 1 specifies that the top-left downsampled luma sample in CCLM is vertically shifted by 0.5 units of luma samples relatively to the top-left luma sample. lm_chroma_horizontal_offset_flag equal to 0 specifies that the top-left downsampled luma sample in CCLM is vertically co-sited with the top-left luma sample.

# Simulation results

The software is implemented based on VTM-3.0 and JEM-comparable HDR implementation (WCG_EXT=1). The encoding was done for the HDR-B sequences under HDR/WCG video CTC [2].

The anchor uses the default chroma sample location type 0 CCLM downsampling filter. The test uses the chroma sample location type 2 CCLM downsampling filter since the HDR/WCG video sequences are in ITU-R BT.2020 container. Results for the all intra and random access configurations are reported in the following sub-sections for tests 2.4.a (determination: 3-tap; prediction: 3-tap), 2.4.b (determination: 3-tap; prediction: 5-tap), and 2.4.c (determination: 5-tap; prediction: 5-tap). Supplemental results are provided in the accompanying Excel sheets for other combinations of filters for determination and prediction, including for 9-tap filter, but show less interesting performance.

## Test 2.4.a (determination: 3-tap; prediction: 3-tap)

Table 2. BD-rate for HDR-B all intra.

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Market | -0.4% | 0.0% | 0.0% | -3.0% | -1.2% | 0.0% | -2.6% | -0.8% |
| SunRise | -0.6% | 0.0% | 0.0% | -3.7% | -4.8% | 0.0% | -1.8% | -2.4% |
| ShowGirl2 | -1.0% | 0.0% | 0.0% | -6.6% | -1.1% | 0.0% | -4.7% | -0.9% |
| BalloonFestival | 0.3% | -0.1% | 0.0% | 0.7% | -1.9% | 0.0% | 1.3% | -1.5% |
| Hurdles | -11.5% | -1.2% | -1.0% | -8.6% | -28.0% | -1.0% | -7.2% | -23.3% |
| Starting | -3.8% | -0.4% | -0.3% | -1.6% | -14.7% | -0.3% | -1.5% | -8.9% |
| Cosmos1 | -0.9% | 0.0% | 0.0% | -6.8% | -8.8% | 0.0% | -7.3% | -8.9% |
| Average HDR-B | -2.5% | -0.2% | -0.2% | -4.2% | -8.6% | -0.2% | -3.4% | -6.7% |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |  |  |

Table 3. BD-rate for HDR-B random access.

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Market | -0.1% | 0.0% | 0.0% | -2.3% | 0.1% | -0.1% | -2.0% | 0.3% |
| SunRise | -1.7% | 0.2% | 0.0% | -5.4% | -6.3% | 0.0% | -3.1% | -2.7% |
| ShowGirl2 | -0.9% | 0.0% | 0.0% | -8.3% | -1.8% | 0.0% | -6.0% | -1.9% |
| BalloonFestival | 0.1% | 0.0% | 0.0% | 0.7% | -1.6% | 0.0% | 1.1% | -1.4% |
| Hurdles | -7.8% | -0.6% | -0.4% | -7.5% | -17.3% | -0.4% | -6.2% | -14.2% |
| Starting | -2.5% | -0.6% | -0.4% | -1.2% | -7.3% | -0.4% | -1.3% | -5.0% |
| Cosmos1 | -0.8% | -0.1% | -0.1% | -5.2% | -6.7% | 0.0% | -5.3% | -6.6% |
| Average HDR-B | -1.9% | -0.2% | -0.1% | -4.2% | -5.8% | -0.1% | -3.3% | -4.5% |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |  |  |

## Test 2.4.b (determination: 3-tap; prediction: 5-tap)

Table 4. BD-rate for HDR-B all intra.

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Market | -0.5% | 0.0% | 0.0% | -2.7% | -1.7% | 0.0% | -2.3% | -1.1% |
| SunRise | -0.6% | 0.1% | 0.0% | -4.5% | -5.9% | 0.0% | -2.6% | -2.7% |
| ShowGirl2 | -0.7% | 0.1% | 0.0% | -5.9% | -1.3% | 0.0% | -4.1% | -0.9% |
| BalloonFestival | 0.2% | -0.1% | 0.0% | 0.5% | -2.0% | 0.0% | 1.2% | -1.7% |
| Hurdles | -9.9% | -1.1% | -0.9% | -8.3% | -26.5% | -0.9% | -7.2% | -22.5% |
| Starting | -2.4% | -0.3% | -0.2% | -2.3% | -10.7% | -0.2% | -2.0% | -6.8% |
| Cosmos1 | -1.2% | 0.1% | 0.1% | -10.0% | -11.9% | 0.1% | -10.8% | -12.2% |
| Average HDR-B | -2.2% | -0.2% | -0.2% | -4.7% | -8.6% | -0.2% | -4.0% | -6.8% |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |  |  |  |

Table 5. BD-rate for HDR-B random access.

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Market | 0.1% | 0.0% | 0.0% | -2.5% | 0.2% | 0.0% | -2.1% | 0.1% |
| SunRise | -1.0% | 0.3% | 0.1% | -6.4% | -6.8% | 0.1% | -4.2% | -3.4% |
| ShowGirl2 | -1.0% | 0.0% | 0.0% | -8.1% | -1.5% | 0.0% | -5.8% | -1.0% |
| BalloonFestival | 0.1% | 0.1% | 0.0% | 0.4% | -1.3% | 0.0% | 1.0% | -0.7% |
| Hurdles | -7.4% | -0.5% | -0.4% | -7.6% | -18.5% | -0.4% | -6.2% | -15.4% |
| Starting | -1.8% | -0.5% | -0.4% | -1.8% | -6.4% | -0.4% | -1.7% | -4.5% |
| Cosmos1 | -0.8% | 0.1% | 0.0% | -6.8% | -7.6% | 0.0% | -7.0% | -7.5% |
| Average HDR-B | -1.7% | -0.1% | -0.1% | -4.7% | -6.0% | -0.1% | -3.7% | -4.6% |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |

## Test 2.4.c (determination: 5-tap; prediction: 5-tap)

Table 6. BD-rate for HDR-B all intra.

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Market | -0.5% | 0.0% | 0.0% | -3.2% | -2.2% | 0.0% | -2.6% | -1.7% |
| SunRise | -0.7% | -0.1% | 0.0% | -5.1% | -5.9% | -0.1% | -3.2% | -2.8% |
| ShowGirl2 | -0.8% | -0.1% | 0.0% | -7.1% | -1.5% | -0.1% | -5.7% | -1.4% |
| BalloonFestival | 0.2% | -0.1% | 0.0% | 0.4% | -1.7% | 0.0% | 1.0% | -1.3% |
| Hurdles | -11.8% | -1.3% | -1.1% | -10.5% | -29.1% | -1.1% | -8.9% | -24.6% |
| Starting | -3.7% | -0.5% | -0.4% | -3.4% | -15.0% | -0.4% | -2.8% | -9.1% |
| Cosmos1 | -1.1% | 0.0% | 0.0% | -9.5% | -11.8% | 0.0% | -10.3% | -12.0% |
| Average HDR-B | -2.6% | -0.3% | -0.2% | -5.5% | -9.6% | -0.2% | -4.6% | -7.6% |
| Enc Time[%] | 99% |  |  |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |  |  |

Table 7. BD-rate for HDR-B random access.

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Market | -0.3% | 0.0% | 0.0% | -3.0% | -0.2% | 0.0% | -2.6% | -0.2% |
| SunRise | -1.6% | 0.0% | -0.2% | -6.9% | -7.2% | -0.2% | -4.3% | -3.6% |
| ShowGirl2 | -0.8% | -0.1% | 0.0% | -8.2% | -1.8% | -0.1% | -5.8% | -1.3% |
| BalloonFestival | -0.1% | -0.1% | 0.0% | 0.2% | -1.7% | 0.0% | 0.7% | -1.3% |
| Hurdles | -8.5% | -0.7% | -0.5% | -9.1% | -19.8% | -0.5% | -7.5% | -16.4% |
| Starting | -2.8% | -0.7% | -0.6% | -3.0% | -8.6% | -0.5% | -2.6% | -5.8% |
| Cosmos1 | -0.7% | 0.1% | 0.0% | -6.1% | -6.8% | 0.0% | -6.3% | -7.0% |
| Average HDR-B | -2.1% | -0.2% | -0.2% | -5.1% | -6.6% | -0.2% | -4.1% | -5.1% |
| Enc Time[%] | 102% |  |  |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |  |  |

# Conclusion

This contribution reports simulation result of CE13 test 2.4, which evaluates modified CCLM downsampling filter for “type-2” content. The results based on the wPSNR metric show the following {Y, U, V} BD-rate savings over the default CCLM downsampling filter:

- Determination: 3-tap; prediction: 3-tap; AI: {-0.2%, -4.2%, -8.6%}; RA: {-0.1%, -4.2%, -5.8%}

- Determination: 3-tap; prediction: 5-tap; AI: {-0.2%, -4.7%, -8.6%}; RA: {-0.1%, -4.7%, -6.0%}

- Determination: 5-tap; prediction: 5-tap; AI: {-0.2%, -5.5%, -9.6%}; RA: {-0.2%, -5.1%, -6.6%}

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding”, JVET-L1023, Oct. 2018, Macau, CN.

- A. Segall, E. François, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-L1011, Oct. 2018, Macau, CN.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).