| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2018 | Document: JVET-M0151-v1 |
| --- | --- |

| Title: | CE8-related: Virtual search area for current picture referencing (CPR) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van Ted Hsieh Wei-Jung Chien Vadim Seregin Hongtao Wang Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Tel: Email: | +1-858-658-5664 lphamvan@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a method to extend the CPR search area by padding the area using the line buffer and the column to the left of the current CTU. Without any additional memory cost, the luma BD-rate changes for [CTC, Class F, class SCC 1080p] with the use of the proposed technique are reported as:

CPR on and PLT off:

AI: (-0.26%, -12.36%, -37.66%) and (0.02%, -0.39%, -1.23%) over VTM3 and VTM3+CPR on, resp.

RA: (0.11%, -10.11%, -22.48%) and (0.02%, -0.29%, -0.60%) over VTM3 and VTM3+CPR on, resp.

LB: (0.11%, -5.30%, -13.39%) and (-0.03%, -0.20%, -0.35%) over VTM3 and VTM3+CPR on, resp.

CPR on and PLT on:

AI:(-0.16%, -15.56%, -44.93%) and (0.03%, -0.09%, -0.31%) over VTM3 and VTM3+CPR+PLT on, resp.

RA:(0.23%, -11.99%, -25.91%) and (0.03%, -0.12%, -0.35%) over VTM3 and VTM3+CPR+PLT on, resp.

LB:(0.30%, -6.39%, -14.92%) and (0.01%, -0.27%, -0.17%) over VTM3 and VTM3+CPR+PLT on, resp.

In addition, this document also reports the results of the proposed padding scheme when 4 lines above and 4 columns to the left of the current CTU are included into the CPR search area. the luma BD-rate changes for [CTC, Class F, class SCC 1080p] are reported as:

CPR on and PLT off:

AI: (-0.31%, -12.75%, -38.73%) and (-0.04%, -0.88%, -3.05%) over VTM3 and VTM3+CPR on, resp.

RA: (0.07%, -10.38%, -23.23%) and (-0.01%, -0.65%, -1.64%) over VTM3 and VTM3+CPR on, resp.

LB: (0.10%, -5.42%, -13.96%) and (-0.04%, -0.34%, -1.04%) over VTM3 and VTM3+CPR on, resp.

CPR on and PLT on:

AI:(-0.23%, -15.84%, -45.56%) and (-0.04%, -0.49%, -1.51%) over VTM3 and VTM3+CPR+PLT on, resp.

RA:(0.20%, -12.24%, -26.40%) and (0.00%, -0.45%, -1.07%) over VTM3 and VTM3+CPR+PLT on, resp.

LB:(0.25%, -6.55%, -15.30%) and (0.01%, -0.50%, -0.63%) over VTM3 and VTM3+CPR+PLT on, resp.

# Introduction

The current picture reference (CPR) mode was adopted into BMS software in the last Ljubljana meeting [1]. In the last meeting in Macao, the CPR reference is restricted in the reconstructed area of the current CTU. This limitation significantly reduces the performance of CPR coding compared to full-frame search.

# Proposed Method

2.1 Padding the CPR search area using one buffer line

In order to improve the performance of CPR in restricted CPR reference mode without any additional cost, this contribution utilizes the line buffer to extend the CPR reference. In particularly, one top line above and one column to the left of the current CTU are included into the CPR reference as depicted in Figure 1. In addition, the search area is extended to the top by copying the line buffer above the current CTU. On the other hand, the search area is extended to the left by copying the left column of the current CTU. The extended areas have the  sizes of (128, PUHeight) and (PUWidth, 128) to the top and left, respectively.

Figure 1 Extended CPR reference of CE8.1.3

2.1 Padding the CPR search area using  4 buffer lines

In real implementation, 4 lines above the current CTU may be needed for deblocking filter. Utilizing these lines will increase the CPR coding performance without any additional cost. This test includes these 4 lines above and 4 columns to the left of the current CTU. The CPR reference is then extended further using the proposed padding scheme in Section 2.1.

# Experimental Results

The proposed technique has been implemented on top of VTM3.0. These tests are evaluated with PLT off and PLT off. The gain of the proposed padding scheme with the use of 1 and 4 buffer lines are summarized in Table 1 and Table 2, respectively.

Without any additional memory cost, the luma BD-rate changes for [CTC, Class F, class SCC 1080p] with the use of the proposed padding technique with 1 buffer line are reported as:

CPR on and PLT off:

AI: (-0.26%, -12.36%, -37.66%) and (0.02%, -0.39%, -1.23%) over VTM3 and VTM3+CPR on, resp.

RA: (0.11%, -10.11%, -22.48%) and (0.02%, -0.29%, -0.60%) over VTM3 and VTM3+CPR on, resp.

LB: (0.11%, -5.30%, -13.39%) and (-0.03%, -0.20%, -0.35%) over VTM3 and VTM3+CPR on, resp.

CPR on and PLT on:

AI:(-0.16%, -15.56%, -44.93%) and (0.03%, -0.09%, -0.31%) over VTM3 and VTM3+CPR+PLT on, resp.

RA:(0.23%, -11.99%, -25.91%) and (0.03%, -0.12%, -0.35%) over VTM3 and VTM3+CPR+PLT on, resp.

LB:(0.30%, -6.39%, -14.92%) and (0.01%, -0.27%, -0.17%) over VTM3 and VTM3+CPR+PLT on, resp.

The luma BD-rate changes for [CTC, Class F, class SCC 1080p] with the use of the proposed padding technique with 1 buffer line are reported as:

CPR on and PLT off:

AI: (-0.31%, -12.75%, -38.73%) and (-0.04%, -0.88%, -3.05%) over VTM3 and VTM3+CPR on, resp.

RA: (0.07%, -10.38%, -23.23%) and (-0.01%, -0.65%, -1.64%) over VTM3 and VTM3+CPR on, resp.

LB: (0.10%, -5.42%, -13.96%) and (-0.04%, -0.34%, -1.04%) over VTM3 and VTM3+CPR on, resp.

CPR on and PLT on:

AI:(-0.23%, -15.84%, -45.56%) and (-0.04%, -0.49%, -1.51%) over VTM3 and VTM3+CPR+PLT on, resp.

RA:(0.20%, -12.24%, -26.40%) and (0.00%, -0.45%, -1.07%) over VTM3 and VTM3+CPR+PLT on, resp.

LB:(0.25%, -6.55%, -15.30%) and (0.01%, -0.50%, -0.63%) over VTM3 and VTM3+CPR+PLT on, resp.

Table 1(a) Virtual CPR search area with 1 buffer line – CPR on and PLT off

|  | All Intra Main10 - VTM configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR on + PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.18% | 0.28% | 132% | 103% | 0.03% | 0.28% | 0.22% | 105% | 103% |
| Class A2 | -0.47% | -0.42% | -0.49% | 136% | 104% | 0.01% | 0.08% | 0.05% | 104% | 103% |
| Class B | -0.27% | -0.32% | -0.27% | 141% | 106% | 0.02% | 0.07% | 0.08% | 104% | 105% |
| Class C | -0.04% | -0.17% | -0.22% | 145% | 103% | 0.05% | 0.08% | 0.02% | 104% | 99% |
| Class E | -0.68% | -0.66% | -0.62% | 149% | 107% | -0.04% | 0.15% | -0.03% | 104% | 104% |
| Overall | -0.26% | -0.28% | -0.26% | 141% | 105% | 0.02% | 0.12% | 0.07% | 104% | 103% |
| Class D | -0.11% | -0.13% | 0.18% | 140% | 101% | -0.01% | 0.10% | -0.03% | 103% | 103% |
| Class F | -12.36% | -12.23% | -12.21% | 164% | 107% | -0.39% | -0.28% | -0.19% | 106% | 107% |
| Class SCC | -37.66% | -36.43% | -36.72% | 145% | 97% | -1.23% | -0.55% | -0.66% | 102% | 107% |
|  | Random Access Main 10 - VTM configuration |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR on + PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | 0.12% | 0.29% | 98% | 105% | 0.00% | 0.17% | 0.06% | 101% | 105% |
| Class A2 | 0.01% | 0.11% | 0.15% | 95% | 103% | 0.01% | 0.04% | -0.09% | 97% | 103% |
| Class B | 0.05% | 0.02% | -0.12% | 100% | 104% | 0.03% | 0.03% | 0.09% | 101% | 105% |
| Class C | 0.23% | -0.03% | 0.15% | 101% | 101% | 0.04% | 0.03% | 0.07% | 99% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.11% | 0.04% | 0.09% | 99% | 103% | 0.02% | 0.06% | 0.04% | 100% | 103% |
| Class D | 0.16% | -0.29% | 0.12% | 102% | 101% | 0.01% | -0.01% | -0.04% | 101% | 103% |
| Class F | -10.11% | -9.99% | -10.12% | 105% | 103% | -0.29% | -0.08% | -0.18% | 100% | 107% |
| Class SCC | -22.48% | -21.82% | -22.38% | 103% | 105% | -0.60% | -0.33% | -0.38% | 100% | 108% |
|  | Low delay B Main10 - VTM configuration |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR on + PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.07% | -0.04% | 0.03% | 109% | 107% | 0.00% | 0.28% | 0.16% | 102% | 104% |
| Class C | 0.07% | 0.34% | 0.59% | 112% | 109% | -0.06% | -0.05% | 0.25% | 102% | 108% |
| Class E | 0.24% | 0.97% | 0.41% | 104% | 102% | -0.04% | -0.08% | 0.61% | 99% | 103% |
| Overall | 0.11% | 0.34% | 0.31% | 108% | 107% | -0.03% | 0.08% | 0.30% | 101% | 105% |
| Class D | 0.13% | -0.52% | 0.37% | 111% | 97% | 0.00% | -0.38% | -0.32% | 99% | 97% |
| Class F | -5.30% | -5.56% | -5.44% | 107% | 105% | -0.20% | -0.22% | 0.01% | 99% | 104% |
| Class SCC | -13.39% | -13.57% | -13.88% | 108% | 104% | -0.35% | -0.26% | -0.32% | 100% | 109% |

Table 1(b) Virtual CPR search area with 1 buffer line – CPR on and PLT on

|  | All Intra Main10 - VTM configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR on + PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | 0.26% | 0.33% | 138% | 103% | 0.05% | 0.19% | 0.19% | 101% | 98% |
| Class A2 | -0.37% | -0.33% | -0.39% | 144% | 107% | 0.02% | 0.11% | 0.05% | 102% | 108% |
| Class B | -0.18% | -0.25% | -0.13% | 148% | 106% | 0.03% | 0.06% | 0.17% | 102% | 101% |
| Class C | 0.07% | -0.10% | -0.12% | 151% | 104% | 0.07% | 0.03% | 0.05% | 102% | 99% |
| Class E | -0.55% | -0.62% | -0.45% | 156% | 107% | -0.05% | -0.10% | 0.12% | 103% | 102% |
| Overall | -0.16% | -0.21% | -0.15% | 148% | 106% | 0.03% | 0.05% | 0.12% | 102% | 101% |
| Class D | -0.03% | -0.10% | 0.22% | 146% | 102% | -0.02% | -0.06% | 0.04% | 101% | 95% |
| Class F | -15.56% | -14.03% | -14.13% | 176% | 97% | -0.09% | -0.02% | -0.05% | 103% | 100% |
| Class SCC | -44.93% | -41.11% | -41.34% | 160% | 78% | -0.31% | -0.14% | -0.19% | 102% | 100% |
|  | Random Access Main 10 - VTM configuration |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR on + PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.26% | 0.21% | 0.43% | 101% | 104% | 0.05% | 0.01% | 0.06% | 99% | 102% |
| Class A2 | 0.15% | 0.26% | 0.31% | 99% | 104% | 0.04% | 0.11% | 0.03% | 97% | 107% |
| Class B | 0.14% | 0.10% | -0.12% | 105% | 105% | 0.03% | 0.10% | -0.04% | 102% | 106% |
| Class C | 0.36% | 0.02% | 0.37% | 108% | 104% | 0.00% | 0.03% | -0.01% | 100% | 102% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.23% | 0.13% | 0.20% | 104% | 105% | 0.03% | 0.06% | 0.00% | 100% | 104% |
| Class D | 0.41% | 0.10% | 0.37% | 108% | 100% | -0.01% | 0.14% | 0.41% | 99% | 99% |
| Class F | -11.99% | -11.69% | -11.97% | 111% | 102% | -0.12% | 0.10% | 0.03% | 99% | 103% |
| Class SCC | -25.91% | -24.62% | -25.03% | 106% | 99% | -0.35% | -0.21% | -0.32% | 99% | 104% |
|  | Low delay B Main10 - VTM configuration |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR on + PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.18% | -0.08% | 0.15% | 115% | 110% | 0.00% | 0.16% | 0.04% | 101% | 107% |
| Class C | 0.27% | 0.66% | 0.51% | 119% | 108% | -0.02% | 0.12% | 0.13% | 100% | 105% |
| Class E | 0.52% | 1.66% | 0.16% | 111% | 100% | 0.05% | 0.60% | 0.49% | 100% | 106% |
| Overall | 0.30% | 0.60% | 0.27% | 115% | 107% | 0.01% | 0.26% | 0.19% | 101% | 106% |
| Class D | 0.36% | 1.28% | 0.86% | 118% | 99% | -0.03% | 0.62% | 0.22% | 99% | 102% |
| Class F | -6.39% | -6.84% | -7.53% | 113% | 103% | -0.27% | -0.04% | -0.40% | 99% | 103% |
| Class SCC | -14.92% | -14.78% | -14.82% | 110% | 100% | -0.17% | -0.12% | -0.09% | 99% | 108% |

Table 2(a) Virtual CPR search area with 4 buffer lines – CPR on and PLT off

|  | All Intra Main10 - VTM configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR + PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.08% | 0.06% | 131% | 100% | -0.01% | 0.02% | 0.00% | 104% | 100% |
| Class A2 | -0.51% | -0.53% | -0.58% | 136% | 103% | -0.03% | -0.02% | -0.04% | 104% | 101% |
| Class B | -0.34% | -0.42% | -0.38% | 141% | 103% | -0.04% | -0.03% | -0.03% | 104% | 102% |
| Class C | -0.12% | -0.22% | -0.31% | 144% | 103% | -0.03% | 0.02% | -0.06% | 103% | 99% |
| Class E | -0.74% | -0.84% | -0.69% | 149% | 107% | -0.10% | -0.03% | -0.09% | 105% | 104% |
| Overall | -0.31% | -0.41% | -0.38% | 140% | 103% | -0.04% | -0.01% | -0.04% | 104% | 101% |
| Class D | -0.15% | -0.19% | 0.15% | 141% | 101% | -0.04% | 0.04% | -0.06% | 103% | 102% |
| Class F | -12.75% | -12.69% | -12.63% | 163% | 106% | -0.88% | -0.87% | -0.74% | 105% | 106% |
| Class SCC | -38.73% | -37.67% | -37.94% | 143% | 96% | -3.05% | -2.59% | -2.65% | 101% | 106% |
|  | Random Access Main 10 - VTM configuration |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR + PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | 0.04% | 0.29% | 98% | 104% | 0.00% | 0.08% | 0.06% | 100% | 105% |
| Class A2 | 0.00% | 0.04% | 0.16% | 95% | 103% | -0.01% | -0.03% | -0.08% | 97% | 102% |
| Class B | -0.01% | -0.13% | -0.24% | 100% | 105% | -0.03% | -0.13% | -0.03% | 102% | 106% |
| Class C | 0.19% | -0.07% | 0.01% | 102% | 104% | 0.00% | 0.00% | -0.07% | 100% | 103% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.07% | -0.04% | 0.01% | 99% | 104% | -0.01% | -0.03% | -0.03% | 100% | 104% |
| Class D | 0.14% | -0.14% | 0.19% | 102% | 100% | -0.01% | 0.14% | 0.04% | 100% | 102% |
| Class F | -10.38% | -10.46% | -10.56% | 105% | 103% | -0.65% | -0.66% | -0.74% | 100% | 107% |
| Class SCC | -23.23% | -22.68% | -23.25% | 104% | 105% | -1.64% | -1.48% | -1.56% | 100% | 108% |
|  | Low delay B Main10 - VTM configuration |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR + PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.08% | -0.02% | 0.08% | 109% | 110% | 0.01% | 0.30% | 0.22% | 103% | 107% |
| Class C | 0.10% | 0.47% | 0.71% | 111% | 109% | -0.02% | 0.08% | 0.38% | 102% | 108% |
| Class E | 0.15% | 0.83% | -0.07% | 104% | 101% | -0.13% | -0.22% | 0.12% | 99% | 102% |
| Overall | 0.10% | 0.36% | 0.26% | 108% | 107% | -0.04% | 0.10% | 0.25% | 101% | 106% |
| Class D | 0.18% | 0.07% | 0.12% | 111% | 99% | 0.04% | 0.22% | -0.56% | 99% | 98% |
| Class F | -5.42% | -5.56% | -5.81% | 107% | 104% | -0.34% | -0.24% | -0.38% | 99% | 103% |
| Class SCC | -13.96% | -14.22% | -14.45% | 108% | 102% | -1.04% | -1.04% | -1.01% | 99% | 107% |

Table 2(b) Virtual CPR search area with 4 buffer lines – CPR on and PLT on

|  | All Intra Main10 - VTM configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR on + PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.02% | 0.09% | 138% | 102% | -0.01% | -0.06% | -0.04% | 101% | 97% |
| Class A2 | -0.42% | -0.48% | -0.50% | 144% | 104% | -0.03% | -0.04% | -0.07% | 102% | 105% |
| Class B | -0.25% | -0.32% | -0.29% | 147% | 106% | -0.04% | -0.01% | 0.00% | 102% | 100% |
| Class C | -0.02% | -0.21% | -0.19% | 150% | 104% | -0.02% | -0.09% | -0.02% | 101% | 99% |
| Class E | -0.61% | -0.74% | -0.57% | 155% | 106% | -0.11% | -0.21% | 0.00% | 102% | 101% |
| Overall | -0.23% | -0.33% | -0.29% | 147% | 104% | -0.04% | -0.07% | -0.02% | 102% | 100% |
| Class D | -0.03% | -0.06% | 0.13% | 145% | 101% | -0.02% | -0.02% | -0.04% | 100% | 95% |
| Class F | -15.84% | -14.37% | -14.51% | 176% | 99% | -0.49% | -0.46% | -0.53% | 103% | 102% |
| Class SCC | -45.56% | -41.94% | -42.16% | 157% | 79% | -1.51% | -1.55% | -1.57% | 100% | 103% |
|  | Random Access Main 10 - VTM configuration |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR on + PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.21% | 0.17% | 0.35% | 101% | 104% | 0.00% | -0.03% | -0.01% | 99% | 102% |
| Class A2 | 0.12% | 0.04% | 0.28% | 99% | 104% | 0.01% | -0.11% | 0.00% | 97% | 107% |
| Class B | 0.12% | 0.04% | -0.31% | 105% | 105% | 0.01% | 0.04% | -0.22% | 102% | 106% |
| Class C | 0.35% | 0.15% | 0.28% | 108% | 104% | -0.01% | 0.16% | -0.10% | 99% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.20% | 0.10% | 0.10% | 104% | 104% | 0.00% | 0.03% | -0.10% | 100% | 104% |
| Class D | 0.37% | 0.10% | 0.41% | 108% | 100% | -0.05% | 0.14% | 0.46% | 98% | 99% |
| Class F | -12.24% | -11.99% | -12.25% | 111% | 102% | -0.45% | -0.30% | -0.35% | 99% | 103% |
| Class SCC | -26.40% | -25.23% | -25.65% | 105% | 99% | -1.07% | -1.04% | -1.16% | 99% | 104% |
|  | Low delay B Main10 - VTM configuration |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR on + PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.17% | 0.00% | 0.12% | 115% | 108% | 0.00% | 0.24% | 0.01% | 101% | 105% |
| Class C | 0.25% | 0.64% | 0.58% | 119% | 111% | -0.05% | 0.10% | 0.20% | 100% | 108% |
| Class E | 0.40% | 0.65% | 0.17% | 111% | 99% | -0.07% | -0.39% | 0.52% | 100% | 105% |
| Overall | 0.25% | 0.38% | 0.29% | 115% | 107% | -0.03% | 0.04% | 0.20% | 101% | 106% |
| Class D | 0.40% | 0.16% | 0.23% | 117% | 98% | 0.01% | -0.48% | -0.41% | 99% | 102% |
| Class F | -6.55% | -7.04% | -7.35% | 113% | 104% | -0.50% | -0.28% | -0.24% | 99% | 104% |
| Class SCC | -15.30% | -15.22% | -15.24% | 110% | 100% | -0.63% | -0.67% | -0.60% | 99% | 107% |

# Conclusions

This contribution proposes a method to extend the CPR search area by padding the area using the line buffer and the column to the left of the current CTU. The experimental results show that the proposed padding technique provides coding performance gain without any additional memory cost.

References

[1] Xiaozhong Xu, Xiang Li and Shan Liu, “CE8-2.2: Current picture referencing using reference index signaling”, JVET-K0076, Ljubljana, SL, July 2018

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).