| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0156 |
| --- | --- |

| Title: | AHG17: On component type indication for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Sehoon Yea Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The contribution proposes a syntax element (nuh_component_type) in the NAL unit header, which indicates the component type of the picture in the NAL unit, such as YUV, RGB, depth map, alpha map, 3D geometry map, occupancy map, etc.; in other words, any visual data beyond the traditional YUV color video. In point cloud coding (PCC), for example, YUV, XYZ geometry map (projected), and occupancy map may simultaneously be present in the coded bitstream. Depending on the purpose of the visual data consumption, only specific component data needs to be extracted from the entire bitstream. It is asserted that by exposing the component type information in NAL unit header level according to this proposal, any intelligent visual system can easily access a specific component data, by parsing and interpreting NAL unit headers only. Also, a file writer can easily write a sub-bitstream in a separate track corresponding to the component type.

# Proposal

The contribution proposes a syntax element (nuh_component_type) in NAL unit header, which indicate a component type of the corresponding NAL unit, such as YUV, RGB, depth map, alpha map, 3D geometry information, occupancy map and so on. Any VCL or non_VCL NAL unit corresponding to the same component shall have the same value of nuh_component_type.

Depending on the purpose of the visual data consumption, only specific component data needs to be extracted from the entire bitstream. It is asserted that by exposing the component type information in NAL unit header level according to this proposal, any intelligent visual system can easily access a specific component data, by parsing and interpreting NAL unit headers only. For example, any computer vision based application may access 3D geometry map only, in order to interpret the 3D geometry information of object and use the information for an AR service.

#### NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(32) |
| nuh_reserved_zero_7bits | u(7) |
| nuh_component_type | u(4) |
| nuh_reserved_zero_4bits | u(4) |
| } |  |

Or, in accordance with the NAL unit header design in JVET-M0520:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nuh_unit_class | u(4) |
| nuh_temporal_id_plus1 | u(3) |
| . . . |  |
| if( nuh_unit_class == 9 ) { /* Complex */ | u(1) |
| nuh_component_type_present_flag | u(1) |
| if( nuh_component_type_present_flag ) { |  |
| nuh_component_type | u(4) |
| } |  |
| } |  |
| byte_align() |  |
| } |  |

nuh_component_type specifies the component type of the NAL unit, as specified in Table 1

Table 1 – component type

| nuh_component_type | Component type |
| --- | --- |
| 0 | YUV color |
| 1 | Depth map |
| 2 | XYZ geometry map |
| 3 | Occupancy map |
| 4 | Transparency map |
| 5-7 | Reserved |

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).